3GPP TSG-CT Meeting #101	CP-232152

# Bangalore, India, 11th – 12th September 2023					Revision of C4-233434, C4-233555

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0680 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS rediscovery indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As defined in clause 5.2.8.2.3 of 3GPP TS 23.502, EAS rediscovery indication is defined as optional IE in Nsmf_PDUSession_Update service operation. It is also defined in clause 6.7.2.6 of 3GPP TS 23.548: if this is an inter-PLMN change, and the UE indicated support of the feature, the V-SMF may provide an EAS rediscovery indication, to the H-SMF. … The H-SMF furthermore provides V-SMF with EAS rediscovery indication to be sent to the UE if received at step 7. … The target V-SMF sends the DNS information for the UE (received at step 9) and EAS rediscovery indication via PCO in a PDU Session Modification Command sent to the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include EAS rediscovery indication in HsmfUpdateData from V-SMF to H-SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.72, 6.1.6.2.73, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution introduces backward compatible new features to the OpenAPI file of Nsmf_PDUSession API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2: to avoid overlapping with other CR, revert the change on Table NOTE of Table 6.1.6.2.72-1 and define a New NOTE on Table 6.1.6.2.73-1. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.1.6.2.72	Type: HrsboReqInfo

Table 6.1.6.2.72-1: Definition of type HrsboReqInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hrsboAuthReqInd | boolean | C | 0..1 | This IE shall be present, for a HR PDU session, if the V-SMF requests HR SBO authorization: - in a Create Request; and - in an Update Request sent during a V-SMF insertion (i.e. H-PLMN to V-PLMN mobility) and during an Inter-PLMN V-SMF change (i.e. mobility between different V-PLMNs). When present, this IE shall be set to true to indicate that authorization of Session Breakout for HR Session in VPLMN is requested. |
| vEasdfAddr | IpAddress | C | 0..1 | This IE shall be present during a request for HR-SBO authorization, if available, or if the V-EASDF IP address previously signaled to the H-SMF needs to be changed e.g. as a result of a V-SMF change. When present, this IE shall contain the IP Address of the (new) V-EASDF. (NOTE) |
| easRediscoveryInd | boolean | C | 0..1 | This IE shall be present, for a HR PDU session, if the UE indicates the support of EAS rediscovery and the V-SMF requests EAS rediscovery in an Update Request sent during a V-SMF insertion (i.e. H-PLMN to V-PLMN mobility) and during an Inter-PLMN V-SMF change (i.e. mobility between different V-PLMNs). When present, this IE shall be set to true to indicate that EAS rediscovery is requested. (NOTE) |
| NOTE: The V-EASDF address is sent from H-SMF to UE in n1SmInfoToUe attribute. |  |  |  |  |

* * * Next Change * * * *

6.1.6.2.73	Type: HrsboRspInfo

Table 6.1.6.2.73-1: Definition of type HrsboRspInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hrsboAuthResult | boolean | C | 0..1 | This IE shall be present, for a HR PDU session, if the H-SMF received a request for HR SBO authorization: - in a Create Response; and - in an Update Response sent during a V-SMF insertion (i.e. H-PLMN to V-PLMN mobility) and during an Inter-PLMN V-SMF change (i.e. mobility between different V-PLMNs). When present, this IE shall indicate whether HR-SBO request is authorized as follows: - true: authorized. - false: Not authorized. |
| hDnsAddr | IpAddress | C | 0..1 | This IE shall be present if available. When present, this IE shall contain the DNS server address of HPLMN. |
| hPlmnAddr | IpAddress | O | 0..1 | When present, this IE shall contain the HPLMN address information (e.g. H-UPF IP address on N6). The V-SMF may configure the V-EASDF to build EDNS Client Subnet option based on this HPLMN address information for target FQDN of DNS queries which are not authorized for HR-SBO. |
| vplmnOffloadingInfo | VplmnOffloadingInfo | O | 0..1 | When present, this IE shall include VPLMN specific Offloading Information. |
| NOTE X: The H-SMF sends the EAS rediscovery indication received from the V-SMF to the UE in n1SmInfoToUe attribute. |  |  |  |  |

* * * Next Change * * * *

A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.3.0-alpha.4'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

HrsboReqInfo:

description: HR-SBO Information in request message

type: object

properties:

hrsboAuthReqInd:

type: boolean

enum:

- true

vEasdfAddr:

$ref: '#/components/schemas/IpAddress'

easRediscoveryInd:

type: boolean

enum:

- true

[…]

* * * End of Changes * * * *