3GPP TSG-CT Meeting #101	CP-232161

# Bangalore, India, 11th Sep 2023 - 12th Sep 2023                                     was C3-233478

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1001 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Slice Usage Control parameter provisioning OpenAPI update. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per TS 23.502, cl 4.15.6.3g An AF having a S-NSSAI dedicated to it may be authorized, to control the parameters to remove the S-NSSAI from the Allowed NSSAI or release a PDU Session associated with the S-NSSAI. In this case, these parameters may be provided by an authorized AF via the NEF and be stored as part of the subscriber data. The provision procedure of the Network Slice Usage Control Parameters is realized by external parameter provision procedure as described in clause 4.15.6.2. The AMF and SMF use the Network Slice Usage Control parameter as described in clause 5.15.15 of TS 23.501 [2]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New API is defined to support the new slice based parameters provisioning for on Network slice usage control feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non compliant with stage-2 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.32 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 29.571 CR 0458 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces new backwards compatible feature with a new OpenAPI SliceParamProvision API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#02 – Correction to the OpenAPI was done to update sign “#” which was missing in the reference to the SliceUsageControlInfo data type. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# A.32	SliceParamProvision API

openapi: 3.0.0

info:

title: Slice Parameters Provisionings

version: 1.0.0-alpha.1

description: |

API for Slice Parameters Provisionings.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V18.2.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-slice-pp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

paths:

/pp:

get:

summary: Request to retrieve all the active Slice Parameters Provisionings

resources at the NEF.

operationId: GetSliceParamProvisionings

tags:

- Slice Parameters Provisionings (Collection)

responses:

'200':

description: >

OK. All the active Slice Parameters Provisioning resources managed by the

NEF are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SlicePpData'

minItems: 1

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a new Slice Parameters Provisioning.

tags:

- Slice Parameters Provisionings (Collection)

operationId: CreateSliceParamProvisioning

requestBody:

description: >

Representation of the new Slice Parameters Provisioning to be created at

the NEF.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SlicePpData'

responses:

'201':

description: >

Created. Successful creation of a new Individual Slice Parameters

Provisioning resource.

content:

application/json:

schema:

$ref: '#/components/schemas/SlicePpData'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/3gpp-slice-pp/v1/pp/{ppId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/pp/{ppId}:

parameters:

- name: ppId

in: path

description: >

Represents the identifier of the Individual Slice Parameters

Provisioning resource.

required: true

schema:

type: string

get:

summary: Request to retrieve an existing Individual Slice Parameters

Provisioning resource.

operationId: GetIndSliceParamProvisioning

tags:

- Individual Slice Usage Control Parameters Provisioning (Document)

responses:

'200':

description: >

OK. Successful retrieval of the requested Individual Slice

Parameters Provisioning resource.

content:

application/json:

schema:

$ref: '#/components/schemas/SlicePpData'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request to update an existing Individual Slice Parameters

Provisioning resource.

tags:

- Individual Slice Parameters Provisioning (Document)

operationId: UpdateIndSliceParamProvisioning

requestBody:

description: >

Represents the updated Individual Slice Parameters Provisioning

resource representation.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SlicePpData'

responses:

'200':

description: >

OK. The Individual Slice Parameters Provisioning resource is successfully

updated and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SlicePpData'

'204':

description: >

No Content. The Individual Slice Parameters Provisioning resource

is successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Slice Parameters

Provisioning resource.

tags:

- Individual Slice Parameters Provisioning (Document)

operationId: ModifyIndSliceParamProvisioning

requestBody:

description: >

Contains the requested modifications to the Individual Slice Parameters

Provisioning resource.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SlicePpDataPatch'

responses:

'200':

description: >

OK. The Individual Slice Parameters Provisioning resource is successfully

modified and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SlicePpData'

'204':

description: >

No Content. The Individual Slice Parameters Provisioning resource

is successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Slice Parameters

Provisioning resource.

tags:

- Individual Slice Parameters Provisioning (Document)

operationId: DeleteIndSliceParamProvisioning

responses:

'204':

description: >

No Content. The Individual Slice Parameters Provisioning resource

is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

SlicePpData:

description: Represents the Slice Parameters Provisioning data.

type: object

properties:

afId:

type: string

sliceUsgCtrlData:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SliceUsageControlInfo'

minProperties: 1

description: >

Contains the Network Slice Usage Control information to be provisioned.

The key of the map shall be the AF dedicated S-NSSAI to which the Network Slice Usage

Control information is related and that is provided within the snssai attribute of the

corresponding map value encoded via the SliceUsageControlInfo data structure.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

SlicePpDataPatch:

description: >

Represents the requested modifications to an existing Slice Parameters

Provisioning data.

type: object

properties:

sliceUsgCtrlData:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SliceUsageControlInfo'

minProperties: 1

description: >

Contains the updated Network Slice Usage Control information to be provisioned.

The key of the map shall be the AF dedicated S-NSSAI to which the Network Slice Usage

Control information are related and that is provided within the snssai attribute of the

corresponding map value encoded via the SliceUsageControlInfo data structure.

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# * * * * End of changes * * * *