3GPP TSG-CT Meeting #101	CP-232254

# Bangalore, India, 11th – 12th September 2023							(Revision of C3-233745)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.558 | CR | 0109 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selected EES declaration service OpenAPI file definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 23.558 clause 8.8.3.10, 8.8.4.26 and 8.8.4.27 introduced Selected EES declaration service procedures and information flows to declare the selected EES to the CAS during the ACR from EAS to CAS and SA6 TS 23.558 CR 0419 completing corresponding Ecas_SelectedEES API in clause 8.8.5 of TS 23.558. Hence related implementation could be carried out in this specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the OpenAPI description for the Ecas_SelectedEES service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not supporting API definitions for stage 2 required selected EES declaration service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.288 CR 0419 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible feature in the OpenAPI file of Ecas_SelectedEES API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision to C3-233745: Corrected Open API errors: a. '$ref: '#/components/schemas/SelEESDecl' corrected as '$ref: '#/components/schemas/SelEESDecInfo' b. adding the missing "schemas" in '$ref: 'TS29571_CommonData.yaml#/components/Gpsi' corrected as '$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** First Change ***

# A.14	Ecas_SelectedEES API

openapi: 3.0.0

info:

title: Selected EES Service

version: 1.0.0-alpha.1

description: |

Selected EES Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V18.3.0; Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ecas-selected-ees/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

paths:

/declare:

post:

summary: Declare the selected EES to the CAS.

operationId: DeclareSelectedEES

tags:

- Declare Selected EES

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SelEESDecInfo'

responses:

'204':

description: >

No Content. The Seleted EES Declaration request is received

successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SelEESDecInfo:

description: >

Represents the information elements for the selected EES declaration.

type: object

properties:

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

seleEesId:

type: string

seleEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

easId:

type: string

acId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ueId

- seleEesId

- seleEndpoint

- easId

*** End of Changes ***