3GPP TSG-CT WG4 Meeting #121	C4-240808

Athens, Greece, 26th Feb – 01st Mar 2024	was C4-240127, C4-240613

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.532 | CR | 0086 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New feature for Nmbsmf_MBSSession to support MOCN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MB-SMF needs to be upgraded to support the MOCN procedure as specified in clause 6.18 of 3GPP TS 23.247 [14], the MB-SMF needs: 1. to support "Associated Session ID" attribute in the MbsSession data type, to populate this information element to the NG-RAN 2. to able to handle NGAP IE "Broadcast Transport Request Transfer" and include the corresponding response in the Namf_MBSBroadcast ContextStatusNotificationResponse. See below requirements from TS29.518. "If the ContextStatusNotification contains one or more N2 MBS Session Management containers encapsulating a Broadcast Session Transport Request Transfer IE, the MB-SMF shall return a "200 OK" with the ContextStatusNotificationResponse containing one or more N2 MBS Session Management containers encapsulating a Broadcast Session Transport Response Transfer IE or a Broadcast Session Transport Failure Transfer IE. For each N2 MBS Session Management container sent towards the AMF, the MB-SMF shall insert the identifier of the NG-RAN node to which the information in the container relates in the corresponding entry of the n2MbsSmInfoList attribute." Only a subset of the MB-SMFs is deployed in a PLMN to support MOCN feature, therefore it is is important for the NF service consumer of Nmbsmf_MBSSession service to find a "proper" MB-SMF to satisfy the required feature for a given MBS Session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifies new features for Nmbsmf_MBSSession services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MB-SMF with the proper support of the desired feature may not be selected, this leads the failure of delivery of MBS session service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution has no impact to the Nmbsmf_MBSSession API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: rewording the reason for change and the description of the feature. |  |  |  |  |  |  |  |  |

# *** First Change ***

### 6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the Nmbsmf_MBSSession API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| x | MOCNS | Multiple Operators Core Network Support This feature indicates that the MB-SMF supports the resource sharing across broadcast MBS Sessions during network sharing as specified in clauses 6.18 and 7.3.7 of 3GPP TS 23.247 [14]. The NF service consumer may use the feature for selecting an MB-SMF supporting MOCN function, e.g. support of receiving the associatedSessionId attribute in the MbsSession data type (as defined in 3GPP TS 29.571 [18]) and communicating the same to the NG-RAN. |

# *** End of Changes ***