3GPP TSG-CT Meeting #103	CP-240140

Maastricht, Netherlands; 18th – 19th March 2024	was C4-240280

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0217 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add correlation ID to location UP service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, CATT, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell, CATT, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-03-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Nlmf_Location_UPNotify service operation defined by SA2 requires Notification Correlation Information as a mandatory input parameter. However, this parameter is not included as an input in the Nlmf_Location_UPConfig and Nlmf_Location_UPSubscribe operations. CT4 requested SA2 to explain how LMF obtains Notification Correlation Information and what its value is. In S2-2313489, SA2 clarified that the Notification correlation information is an ID and will be sent from the NF service consumer in Nlmf_Location_UPConfig and Nlmf_Location_UPSubscribe service operaions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the Notification Correlation Information to Notification Correlation ID. Add the Notification Correlation ID input parameter to Nlmf_Location_UPConfig, Nlmf_Location_UPSubscribe and Nlmf_Location_UPNotify. Modify the data type of the notification correlation ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LMF does not know what the Notification correlation information in the UPNotify service operation is and how to set it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.8.2, 5.2.2.9.2, 6.1.5.1.2, 6.1.5.2.2, 6.1.5.2.3.1, 6.1.6.2.49, 6.1.6.2.50, 6.1.6.2.52, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the OpenAPI definition of the Nlmf_Location API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: revert the change in 5.2.2.7.2, the changes will be covered in TS 29.572 CR 0225 Merge the changes to 6.1.5.1.2 and 6.1.5.2.2 clauses from TS 29.572 CR 0219. Rev 2: the CR has been agreed in CT4#121 ,this revision corrects the callback URI attribute name in the OpenApi as following: callbacks: UPNotify: '{$request.body#/upNotifyCallBackUri}': |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.2.8.2	Notification of LCS-UP connection

This procedure notifies the NF Service Consumer (i.e. AMF) about status or modification of a secure LCS-UP connection for a target UE.

Figure 5.2.2.8.2-1: UPNotify

1.	The LMF shall send a POST request to the upNotifyCallBackUri. The request body shall include a notification correlation ID, LCS-UP connection status and may include a target LMF identifier.

2a.	On success, "204 No content" shall be returned by the NF Service Consumer.

2b.	On failure or redirection, one of the appropriate HTTP status code listed in Table 6.1.5.1.3.1-2 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure indicating appropriate additional error information.

* * * Next Change * * * *

##### 5.2.2.9.2	Set up LCS-UP connection

This procedure allows a consumer NF to set up, modify or terminate a secure LCS-UP connection for a target UE.

Figure 5.2.2.9.2-1: LCS-UP connection

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI associated with the "up-configure" custom operation to set up, modify or terminate LCS-UP connection. The request content shall include a UP notify callback URI, a notification correlation ID, and the UE identity.

2a.	On success, "204 No Content" shall be returned if no additional information needs to be returned in the response.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.4.7.2-2 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.4.7.2-2.

* * * Next Change * * * *

##### 6.1.5.1.2	Notification Definition

Callback URI: {hgmlcCallBackURI}

See clause 5.2.2.2.2 for the description of how the LMF obtains the Callback URI of the NF Service Consumer (e.g. GMLC).

* * * Next Change * * * *

##### 6.1.5.2.2	Notification Definition

Callback URI: {upNotifyCallBackUri}

See clause 5.2.2.7.2 for the description of how the LMF obtains the Callback URI of the NF Service Consumer (e.g. AMF).

* * * Next Change * * * *

###### 6.1.5.2.3.1	POST

This method sends a notification of LCS-UP connection status to the NF service consumer.

***.Text is skipped for clarity ***

* * * Next Change * * * *

##### 6.1.6.2.49	Type: UpNotifyData

Table 6.1.6.2.49-1: Definition of type UpNotifyData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notifCorrelationId | CorrelationID | M | 1 | Notification correlation ID. |
| upConnectionStatus | UpConnectionStatus | M | 1 | UP Connection Status |
| targetLMFId | LMFIdentification | C | 0..1 | This IE shall be present if the UpConnectionStatus is set to “MOVE”. When present, this IE shall indicate the Target LMF identifier. |

* * * Next Change * * * *

##### 6.1.6.2.50	Type: UpSubscription

Table 6.1.6.2.50-1: Definition of type UpSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upNotifyCallBackUri | Uri | M | 1 | Callback URI of the NF Service Consumer |
| notifCorrelationId | CorrelationID | M | 1 | Notification correlation ID. |
| supi | Supi | M | 1 | SUPI |
| gpsi | Gpsi | C | 0..1 | GPSI shall be included if available. |

* * * Next Change * * * *

##### 6.1.6.2.52	Type: UpConfig

Table 6.1.6.2.52-1: Definition of type UpConfig

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upNotifyCallBackUri | Uri | M | 1 | Callback URI of the NF Service Consumer. |
| notifCorrelationId | CorrelationID | M | 1 | Notification correlation ID. |
| supi | Supi | C | 0..1 | SUPI (NOTE) |
| gpsi | Gpsi | C | 0..1 | GPSI (NOTE) |
| amfReallocationInd | boolean | O | 0..1 | Indicates AMF reallocation indication. When present, it shall be set as follows: - true: AMF reallocated. - false (default): AMF not reallocated. |
| lcsUpConnectionInd | LcsUpConnectionInd | O | 0..1 | LCS-UP connection indication |
| targetLMFId | LMFIdentification | O | 0..1 | Target LMF identifier. |
| NOTE: At least one of the supi or gpsi shall be present in the UpConfig structure. |  |  |  |  |

* * * Next Change * * * *

# A.2	Nlmf_Location API

openapi: 3.0.0

info:

version: '1.3.0-alpha.5'

title: 'LMF Location'

description: |

LMF Location Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.572 V18.4.0; 5G System; Location Management Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.572/'

servers:

- url: '{apiRoot}/nlmf-loc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nlmf-loc

[…]

/up-subscription:

post:

summary: subscribe about status of a secure LCS-UP connection for a target UE

operationId: UpSubscription

tags:

- UP Subscribe

security:

- {}

- oAuth2ClientCredentials:

- nlmf-loc

- oAuth2ClientCredentials:

- nlmf-loc

- nlmf-loc:up-subscription:invoke

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/UpSubscription'

required: true

responses:

'204':

description: Expected response to successful UP Subscription

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

UPNotify:

'{$request.body#/upNotifyCallBackUri}':

post:

requestBody:

description: UP Connection Status Notification

content:

application/json:

schema:

$ref: '#/components/schemas/UpNotifyData'

[…]

LocMeasurements:

description: PRU Location Measurements.

type: object

properties:

rstd:

type: integer

HighAccuracyGnssMetrics:

description: High Accuracy GNSS Positioning Metrics.

type: object

properties:

nrOfUsedSatellites:

type: integer

minimum: 0

maximum: 64

hdopi:

type: integer

minimum: 1

maximum: 256

pdopi:

type: integer

minimum: 1

maximum: 256

age:

type: integer

minimum: 0

maximum: 99

fixType:

$ref: '#/components/schemas/FixType'

UpNotifyData:

description: UP Subscription

type: object

required:

- notifCorrelationId

- upConnectionStatus

properties:

notifCorrelationId:

$ref: '#/components/schemas/CorrelationID'

upConnectionStatus:

$ref: '#/components/schemas/UpConnectionStatus'

targetLMFId:

$ref: '#/components/schemas/LMFIdentification'

UpSubscription:

description: UP Subscription

type: object

required:

- upNotifyCallBackUri

- notifCorrelationId

- supi

properties:

upNotifyCallBackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrelationId:

$ref: '#/components/schemas/CorrelationID'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

RelatedUE:

description: Related UE Information

type: object

required:

- applicationlayerId

- relatedUEType

properties:

applicationlayerId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationlayerId'

relatedUEType:

$ref: '#/components/schemas/RelatedUEType'

UpConfig:

description: UP Config

type: object

required:

- upNotifyCallBackUri

- notifCorrelationId

properties:

upNotifyCallBackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrelationId:

$ref: '#/components/schemas/CorrelationID'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

amfReallocationInd:

type: boolean

default: false

lcsUpConnectionInd:

$ref: '#/components/schemas/LcsUpConnectionInd'

targetLMFId:

$ref: '#/components/schemas/LMFIdentification'

RangeDirection:

description: Represents a range and direction from a point A to a point B.

type: object

properties:

range:

type: number

azimuthDirection:

$ref: '#/components/schemas/Angle'

elevationDirection:

$ref: '#/components/schemas/Angle'

2DRelativeLocation:

description: Represents a relative 2D location with uncertainty ellipse.

type: object

properties:

semiMinor:

$ref: '#/components/schemas/Uncertainty'

semiMajor:

$ref: '#/components/schemas/Uncertainty'

orientationAngle:

$ref: '#/components/schemas/Angle'

[…]

* * * End of Changes * * * *