3GPP TSG-CT Meeting #103	CP-240237

# Maastricht, NL, March 18 – 19, 2024												(revision of C3-241625)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.574 | CR | 0087 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Ndccf_DataManagement_Transfer service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As indicated in S2-2313417 and in clause 8.2.6 of TS 23.288, the new service operation Ndccf_DataManagement_Transfer was added for DCCF to support source DCCF transfering of UE data subscription context to the target DCCF. Therefore, this new service operation shall be defined in CT3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define a new service operation Ndccf_DataManagement_Transfer. - Update the NdccfDataSubscriptionNotification data type to support the data subscription transfer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirement not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2.1.1, 4.2.1.2, 4.2.2.1, 4.2.2.6(new), 5.1.3.1, 5.1.3.6(new), 5.1.3.7(new), 5.1.6.1, 5.1.6.2.5, 5.1.6.2.18(new), 5.1.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the OpenAPI file for Ndccf_DataManagement API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 2: Update the "CreateDCCFDataManagementTransfer" in the PUT method to "UpdateDCCFDataManagementTransfer". |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

4.1	Introduction

The Ndccf services are used for the DCCF to provide collected data.

The following services are specified for the DCCF:

Table 4.1-1: Services provided by DCCF

| Service Name | Description | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- | --- |
| Ndccf_DataManagement | This service enables the NF service consumers to subscribe to/unsubscribe from notifications for different collected data from the DCCF. | Subscribe | Subscribe / Notify | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, LMF, ADRF |
|  |  | Unsubscribe |  |  |
|  |  | Notify |  |  |
|  |  | Fetch | Request / Response | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, LMF, ADRF |
|  |  | Transfer | Request / Response | DCCF |
| Ndccf_ContextManagement | This service enables the NF service consumers to register/deregister collected data in the DCCF. | Register | Request / Response | NWDAF, ADRF |
|  |  | Update |  |  |
|  |  | Deregister |  |  |

# *** Next Changes ***

4.2.1.1	Overview

The Ndccf_DataManagement service, as defined in 3GPP TS 23.288 [14], is provided by the Data Collection Coordination Function (DCCF).

This service:

-	allows NF service consumers to subscribe, modify and unsubscribe for analytics or data releated events;

-	notifies NF service consumers with a corresponding subscription about observed events; and

-	allows NF service consumers to retrieve analytics or data according to the Fetch Instructions provided by DCCF.

-	allows NF service consumers to transfer UE data subscription context to the target DCCF

# *** Next Changes ***

4.2.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture, including the DCCF architecture, is defined in 3GPP TS 23.288 [14].

Known consumers of the Ndccf_DataManagement service are:

-	Policy Control Function (PCF)

-	Network Slice Selection Function (NSSF)

-	Access and Mobility Management Function (AMF)

-	Session Management Function (SMF)

-	Network Exposure Function (NEF)

-	Application Function (AF)

-	Location Management Function (LMF)

-	Network Data Analytics Function (NWDAF)

-	Analytics Data Repository Function (ADRF)

-	Data Collection Coordination Function (DCCF)

The Ndccf_DataManagement service is provided by the DCCF and consumed by the NF service consumers as shown in figure 4.2.1.2-1 for the SBI representation model and in figure 4.2.1.2-2 for the reference point representation model.

Figure 4.2.1.2-1: Ndccf_DataManagement service architecture, SBI representation

Figure 4.2.1.2-2: Ndccf_DataManagement service architecture, reference point representation

# *** Next Changes ***

4.2.2.1	Introduction

Service operations defined for the Ndccf_DataManagement Service are shown in table 4.2.2.1-1.

Table 4.2.2.1-1: Ndccf_DataManagement Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Ndccf_DataManagement_Subscribe | This service operation is used by an NF service consumer to subscribe to, or modify a subscription in the DCCF for event notifications. | NF service consumer (NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, LMF, ADRF) |
| Ndccf_DataManagement_Unsubscribe | This service operation is used by an NF service consumer to unsubscribe from event notifications. | NF service consumer (NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, LMF, ADRF) |
| Ndccf_DataManagement_Notify | This service operation is used by the DCCF to report the detected event(s) to the NF service consumer if subscribed before. | DCCF |
| Ndccf_DataManagement_Fetch | This service operation is used by an NF service consumer to retrieve collected data from the DCCF. | NF service consumer (NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, LMF, ADRF) |
| Ndccf_DataManagement_Transfer | This service operation is used by the source DCCF to transfer UE data subscription context to the target DCCF | NF service consumer (DCCF) |

# *** Next Changes ***

4.2.2.6	Ndccf_DataManagement_Transfer service operation

4.2.2.6.1	General

The Ndccf_DataManagement_Transfer service operation is used by NF service consumer to request the transfer of UE data subscription context to the target DCCF.

4.2.2.6.2	Creation a request for UE data subscription context transfer

Figure 4.2.2.6.2-1 shows a scenario where the NF Service Consumer (i.e. DCCF) sends a request to the DCCF to request the transfer of UE data subscription context from the NF Service Consumer to the NF Service Producer.

Figure 4.2.2.6.2-1: NF service consumer requests a subscription data transfer

The NF service consumer shall invoke the Ndccf_DataManagement_Transfer service operation to request the transfer of UE data subscription context. The NF service consumer shall send an HTTP POST request with "{apiRoot}/ndccf-datamanagement/<apiVersion>/transfers" as Resource URI representing the "DCCF Data Management Transfers", as shown in figure 4.2.2.6.2-1, step 1, to create a request for an "Individual DCCF Data Management Transfer" according to the information in the message body. The NdccfDataManagementTransfer data structure provided in the request body shall include:

-	the identifier of a subscription that needs to be transferred within the "subscriptionId" attribute:

-	the subscription information that needs to be transferred within the "subInfo" attribute.

Upon the reception of an HTTP POST request with: "{apiRoot}/ndccf-datamanagement/<apiVersion>/transfers" as Resource URI and DataManagementTransfer data structure as a request body, in the successful case the DCCF shall create a new Individual DCCF Data Management Transfer resource and send an HTTP "201 Created" response with the URI for the created resource in the "Location" header field, as shown in figure 4.2.2.6.2-1, step 2;

If errors occur when processing the HTTP POST request, the DCCF shall send an HTTP error response as specified in clause 5.1.7

4.2.2.6.3	Update a request for subscription data transfer

Figure 4.2.2.6.3-1 shows a scenario where the NF Service Consumer (i.e. DCCF) sends a request to the DCCF to update a request for the transfer of UE data subscription from the NF Service Consumer to the NF Service Producer.

Figure 4.2.2.6.3-1: NF service consumer updates a request for a subscription data transfer

The NF service consumer shall invoke the Ndccf_DataManagement_Transfer service operation to update a request for the transfer of subscription. The NF service consumer shall send an HTTP PUT request with "{apiRoot}/ndccf-datamanagement/<apiVersion>/transfers/{transferId}" as Resource URI representing the "Individual DCCF Data Management Transfer", as shown in figure 4.2.2.6.3-1, step 1, to update the "Individual DCCF Data Management Transfer" resouorce identified by the {transferId}. The NdccfDataManagementTransfer data structure provided in the request body shall include the same contents as described in c lause 4.2.2.6.2.

Upon the reception of an HTTP PUT request with: "ndccf-datamanagement/<apiVersion>/transfers/{transferId}" as Resource URI and NdccfDataManagementTransfer data structure as request body, the DCCF shall update the Individual DCCF Data Management Transfer resource identified by "transferId", and shall respond with:

- HTTP "204 No Content" response); or

- HTTP "200 OK" response with a response body containing a representation of the updated subscription in the NdccfDataManagementTransfer data type.

If errors occur when processing the HTTP PUT request, the DCCF shall send an HTTP error response as specified in clause 5.1.7

If the DCCF determines the received HTTP PUT request needs to be redirected, the DCCF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [6].

4.2.2.6.4	Cancel a request for subscription data transfer

Figure 4.2.2.6.4-1 shows a scenario where the NF Service Consumer (i.e. DCCF) sends a request to the DCCF to cancel request the transfer of UE data subscription from the NF Service Consumer to the NF Service Producer.

Figure 4.2.2.6.4-1: NF service consumer cancels a request for a subscription data transfer

The NF service consumer shall invoke the Ndccf_DataManagement_Transfer service operation to cancel a request for the transfer of subscription data. The NF service consumer shall send an HTTP DELETE request with "{apiRoot}/ndccf-datamanagement/<apiVersion>/transfers/{transferId}" as Resource URI representing the "Individual DCCF Data Management Transfer", as shown in figure 4.2.2.6.4-1, step 1, to cancel the "Individual DCCF Data Management Transfer" resouorce identified by the {transferId}.

Upon the reception of an HTTP DELETE request with: "{apiRoot}/ndccf-datamanagement/<apiVersion>/transfers/{transferId}" as Resource URI, if the DCCF successfully processed and accepted the received HTTP DELETE request, the DCCF shall:

-	remove the corresponding Individual DCCF Data Management Transfer resource; and

-	respond with HTTP "204 No Content" status code, as shown in figure 4.2.2.6.4-1, step 2.

If errors occur when processing the HTTP DELETE request, the DCCF shall send an HTTP error response as specified in clause 5.1.7

If the DCCF determines the received HTTP DELETE request needs to be redirected, the DCCF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [6].

# *** Next Changes ***

5.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 5.1.3.1-1 depicts the resource URIs structure for the Ndccf_DataManagement API.

Figure 5.1.3.1-1: Resource URI structure of the Ndccf_DataManagement API

Table 5.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| DCCF Analytics Subscriptions | /analytics-subscriptions | POST | Creates a new Individual DCCF Analytics Subscription resource. |
| Individual DCCF Analytics Subscription | /analytics-subscriptions/{subscriptionId} | PUT | Modifies an existing Individual DCCF Analytics Subscription resource. |
|  |  | DELETE | Deletes an Individual DCCF Analytics Subscription identified by {subscriptionId}. |
| DCCF Data Subscriptions | /data-subscriptions | POST | Creates a new Individual DCCF Data Subscription resource. |
| Individual DCCF Data Subscription | /data-subscriptions/{subscriptionId} | PUT | Modifies an existing DCCF Data Subscription resource. |
|  |  | DELETE | Deletes an Individual DCCF Data Subscription identified by {subscriptionId}. |
| DCCF Data Management Transfers | /transfers | POST | Provides information about the UE data subscription context transfer(s), potentially creating a new Individual DCCF Data Management Transfer resource. |
| Individual DCCF Data Management Transfer | /transfers/{transferId} | PUT | Modifies an existing DCCF Data Management Transfer resource. |
|  |  | DELETE | Deletes an Individual DCCF Data Management Transfer resource identified by {transferId}. |

# *** Next Changes ***

5.1.3.6	Resource: DCCF Data Management Transfers

5.1.3.6.1	Description

The DCCF Data Management Transfers resource represents all the requests to transfer the UE data subscription context at a given DCCF.

5.1.3.6.2	Resource definition

Resource URI: {apiRoot}/ndccf-datamanagement/<apiVersion>/transfers

The <apiVersion> shall be set as described in clause 5.1.1.

This resource shall support the resource URI variables defined in table 5.1.3.6.2-1.

Table 5.1.3.6.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1.1 |

5.1.3.6.3	Resource Standard Methods

5.1.3.6.3.1	POST

This method shall support the URI query parameters specified in table 5.1.3.6.3.1-1.

Table 5.1.3.6.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.6.3.1-2 and the response data structures and response codes specified in table 5.1.3.6.3.1-3.

Table 5.1.3.6.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NdccfDataManagementTransfer | M | 1 | Information about UE data subscription context that are requested to be transferred. |

Table 5.1.3.6.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NdccfDataManagementTransfer | M | 1 | 201 Created | The creation of an Individual DCCF Data Management Transfer resource is confirmed and a representation of that resource is returned. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] also apply. |  |  |  |  |

Table 5.1.3.6.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnwdaf-eventssubscription/<apiVersion>/transfers/{transferId}. |

5.1.3.6.4	Resource Custom Operations

None in this release of the specification.

5.1.3.7	Resource: Individual DCCF Data Managment Transfer

5.1.3.7.1	Description

The Individual DCCF Data Managment Transfer resource represents a single request to transfer the UE data subscription context at a given DCCF.

5.1.3.7.2	Resource definition

Resource URI: {apiRoot}/nnwdaf-eventssubscription/<apiVersion>/transfers/{transferId}

The <apiVersion> shall be set as described in clause 5.1.1.

This resource shall support the resource URI variables defined in table 5.1.3.7.2-1.

Table 5.1.3.7.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1.1. |
| transferId | string | Identifies a request to transfer the UE data subscription context. |

5.1.3.7.3	Resource Standard Methods

5.1.3.7.3.1	PUT

This method shall support the URI query parameters specified in table 5.1.3.5.3.1-1.

Table 5.1.3.7.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.7.3.1-2 and the response data structures and response codes specified in table 5.1.3.7.3.1-3.

Table 5.1.3.7.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NdccfDataManagementTransfer | M | 1 | Parameters to replace in an Individual DCCF Data Management Transfer resource. |

Table 5.1.3.7.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NdccfDataManagementTransfer | M | 1 | 200 OK | The Individual DCCF Data Management Transfer resource was modified successfully and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | The Individual DCCF Data Management Transfer resource was modified successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during Individual DCCF Data Management Transfer modification. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during Individual DCCF Data Management Transfer modification. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [6]). |  |  |  |  |

Table 5.1.3.7.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative DCCF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [6]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target DCCF (service) instance towards which the request is redirected. |

Table 5.1.3.7.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative DCCF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [6]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target DCCF (service) instance towards which the request is redirected. |

5.1.3.7.3.2	DELETE

This method shall support the URI query parameters specified in table 5.1.3.7.3.2-1.

Table 5.1.3.7.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.7.3.2-2 and the response data structures and response codes specified in table 5.1.3.7.3.2-3.

Table 5.1.3.7.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.1.3.7.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The Individual DCCF Data Management Transfer resource matching the transferId was deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during Individual DCCF Data Management Transfer deletion. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during Individual DCCF Data Management Transfer deletion. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [6]). |  |  |  |  |

Table 5.1.3.7.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative DCCF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [6]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target DCCF (service) instance towards which the request is redirected. |

Table 5.1.3.7.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative DCCF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [6]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target DCCF (service) instance towards which the request is redirected. |

# *** Next Changes ***

5.1.6.1	General

This clause specifies the application data model supported by the Ndccf_DataManagement API.

Table 5.1.6.1-1 specifies the data types defined for the Ndccf_DataManagement service based interface protocol.

Table 5.1.6.1-1: Ndccf_DataManagement specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AggregationLevel | 5.1.6.3.4 | Contains an aggregation level for processing instructions (e.g. per UE, per Area of Interest). |  |
| DataCollectionPurpose | 5.1.6.3.5 | Represents the purpose for data collection. |  |
| DccfEvent | 5.1.6.2.13 | Represents the event type exposed by DCCF |  |
| DeletionAlert | 5.1.6.2.16 | Contains information about data or analytics that are about to be deleted. | EnhDataMgmt |
| EventParamReport | 5.1.6.2.10 | Represents a summarized report for one event parameter. |  |
| FormattingInstruction | 5.1.6.2.6 | Contains data or analytics formatting Instructions. |  |
| NdccfAnalyticsSubscription | 5.1.6.2.2 | Represents an Individual DCCF Analytics Subscription resource. |  |
| NdccfAnalyticsSubscriptionNotification | 5.1.6.2.4 | Represents a notification that corresponds with an Individual DCCF Analytics Subscription resource. |  |
| NdccfDataManagementTransfer | 5.1.6.2.18 | Represents an individual DCCF Data Management Transfer resource. |  |
| NdccfDataSubscription | 5.1.6.2.3 | Represents an Individual DCCF Data Subscription resource. |  |
| NdccfDataSubscriptionNotification | 5.1.6.2.5 | Represents a notification that corresponds with an Individual DCCF Data Subscription resource. |  |
| NotifResponse | 5.1.6.2.17 | Contains information about the planned action upon receiving a notification. | EnhDataMgmt |
| NotifSummaryReport | 5.1.6.2.9 | Represents summarized notifications based on processing instructions. |  |
| ParameterProcessingInstruction | 5.1.6.2.8 | Contains an event parameter name and the respective event parameter values and sets of attributes to be used in summarized reports. |  |
| ProcessingInstruction | 5.1.6.2.7 | Contains instructions related to the processing (e.g. clubbing) of notifications. |  |
| ReportingOptions | 5.1.6.2.11 | Represents reporting options for notifications that are processed. |  |
| StorageHandlingInformation | 5.1.6.2.15 | Contains storage handling information for data or analytics. | EnhDataMgmt |
| SummarizationAttribute | 5.1.6.3.3 | Represents attribute in the summarized report. |  |
| TermCause | 5.1.6.3.6 | A cause for which the DCCF will collect no further data for this subscription. | TerminationCause |

Table 5.1.6.1-2 specifies data types re-used by the Ndccf_DataManagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Ndccf_DataManagement service based interface.

Table 5.1.6.1-2: Ndccf_DataManagement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AfEvent | 3GPP TS 29.517 [21] | Represents an AF Event id |  |
| AmfEventType | 3GPP TS 29.518 [19] | Represents an AMF Event id |  |
| DataSubscription | 3GPP TS 29.575 [25] | Represents a data subscription to one of various possible data sources. |  |
| DataNotification | 3GPP TS 29.575 [25] | Represents a data subscription notification of one of various possible data sources. |  |
| DateTime | 3GPP TS 29.571 [17] | Identifies the time. |  |
| DurationSec | 3GPP TS 29.571 [17] | Time in seconds. |  |
| EventNotifyDataType | 3GPP TS 29.515 [29] | Represents a GMLC Event id | LocEvents |
| EventType | 3GPP TS 29.503 [20] | Represents an UDM Event id |  |
| EventType | 3GPP TS 29.564 [30] | Represents a UPF Event id | UpEvents |
| FetchInstruction | 3GPP TS 29.576 [26] | The fetch instruction indicates whether the data or analytics can be fetched by the consumer. |  |
| NefEvent | 3GPP TS 29.591 [22] | Represents a NEF Event id |  |
| NetworkAreaInfo | 3GPP TS 29.554 [28] | Identifies a network area. |  |
| NfInstanceId | 3GPP TS 29.571 [17] | NF instance identifier. |  |
| NfSetId | 3GPP TS 29.571 [17] | NF set identifier. |  |
| NnwdafEventsSubscription | 3GPP TS 29.520 [15] | Represents a NWDAF analytics subscription. |  |
| NnwdafEventsSubscriptionNotification | 3GPP TS 29.520 [15] | Represents a NWDAF analytics subscription notification. |  |
| NumberAverage | 3GPP TS 29.520 [15] | Represents average and variance of a parameter value. |  |
| PendingNotificationCause | 3GPP TS 29.520 [15] | Represents the Pending Notification Cause for the stored unsent events. | EnhDataMgmt |
| RedirectResponse | 3GPP TS 29.571 [17] | Contains redirection related information. |  |
| SACEvent | 3GPP TS 29.536 [27] | Represents a NSAC Event id |  |
| SmfEvent | 3GPP TS 29.508 [18] | Represents a SMF Event id |  |
| Supi | 3GPP TS 29.571 [17] | Contains a SUPI. |  |
| SupportedFeatures | 3GPP TS 29.571 [17] | Used to negotiate the applicability of the optional features defined in table 5.1.8-1. |  |
| TimeWindow | 3GPP TS 29.122 [23] | Represents a time window. |  |
| Uinteger | 3GPP TS 29.571 [17] | Unsigned Integer. |  |
| Uri | 3GPP TS 29.571 [17] | URI. |  |

# *** Next Changes ***

5.1.6.2.5	Type NdccfDataSubscriptionNotification

Table 5.1.6.2.5-1: Definition of type NdccfDataSubscriptionNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataNotif | DataNotification | C | 0..1 | Data subscription notification. (NOTE 1, NOTE 3, NOTE 4) |  |
| dataReports | array(NotifSummaryReport) | C | 1..N | List of reports with summarized data from multiple notifications received from data producer. (NOTE 1) |  |
| fetchInstruct | FetchInstruction | C | 0..1 | Instructions for the NF service consumer to fetch the notifications itself. (NOTE 1, NOTE 2) |  |
| delAlert | DeletionAlert | C | 0..1 | Information about data that is about to be deleted. (NOTE 1, NOTE 2) | EnhDataMgmt |
| newSubscriptionUri | Uri | C | 0..1 | The URI of the subscription resource located in the target DCCF. This parameter shall be present if the notification is for informing the successful transfer of a subscription from the source DCCF to the target DCCF. (NOTE 1) | SubscriptionTransfer |
| dataNotifCorrId | string | M | 1 | Notification correlation identifier. |  |
| pendDataNotifCause | PendingNotificationCause | O | 0..1 | Represents the Pending Data Notification Cause for the stored unsent data. | EnhDataMgmt |
| reUserConsentPurs | array(DataCollectionPurpose) | O | 1..N | The purposes of data collection for which the user consent is revoked. This attribute may only be provided if subscription is requested to be terminated due to user consent revocation, i.e. the value of "termCause" is "USER_CONSENT_REVOKED". | TerminationCause |
| terminationReq | boolean | O | 0..1 | If provided and set to "true", it indicates that the subscription is requested to be terminated, i.e. no further notifications related to this subscription will be provided. The default value is "false". |  |
| termCause | TermCause | O | 0..1 | A cause for which the DCCF will collect no further data for this subscription. Its presence indicates that the DCCF requests the termination of the subscription. This attribute may only be provided if the "terminationReq" attribute is set to "true". | TerminationCause |
| timeStamp | DateTime | M | 1 | It represents the time when DCCF completed preparation of the requested analytics. (NOTE 3) |  |
| NOTE 1: Exactly one of these attributes shall be provided. NOTE 2: This "fetchInstruct" attribute shall not be included in the response body of a Fetch operation. NOTE 3: If the DCCF has received the notifications from another source without a timestamp, then the DCCF adds itself a timestamp based on the time it received the notification in the "timeStamp" attribute within the DataNotification data type. NOTE 4: The "upfEventNotifs" attribute within the "dataNotification" attribute is applicable only if the "UpEvents" feature is supported. The "gmlcEventNotifs" attribute within the "dataNotification" attribute is applicable only if the "LocEvents" feature is supported. |  |  |  |  |  |

# *** Next Changes ***

5.1.6.2.18	Type: NdccfDataManagementTransfer

Table 5.1.6.2.18-1: Definition of type NdccfDataManagementTransfer

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | The identifier of a subscription that needs to be transferred. |  |
| subInfo | NdccfDataSubscription | M | 1 | The subscription information that needs to be transferred. |  |

Editor’s note: Whether all the attributes contained in the NdccfDataSubscription data type are applicable is FFS.

# *** Next Changes ***

5.1.8	Feature negotiation

The optional features in table 5.1.8-1 are defined for the Ndccf_DataManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 5.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | UserConsent | Indicates the support of detailed handling of user consent, e.g. indications that user consent has been checked and error responses related to the user consent is not granted. |
| 2 | DataAnaCollect | This feature indicates support for the enhancement of data and analytics collection. |
| 3 | EnhDataMgmt | Indicates the support of enhanced data management mechanisms, including support of pending notification, muting and storage handling. |
| 4 | TerminationCause | This feature indicates support for data collection subscription termination requests with termination cause and the purposes of data collection if the user consent is revoked sent by the DCCF to the NF service consumer. |
| 5 | LocEvents | This feature indicates the support of location events. |
| 6 | UpEvents | Indicates the support of UPF events. |
| 7 | SubscriptionTransfer | This feature indicates support for the data subscription transfer. |

# *** Next Changes ***

A.2	Ndccf_DataManagement API

openapi: 3.0.0

info:

version: 1.1.0-alpha.5

title: Ndccf_DataManagement

description: |

DCCF Data Management Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.574 V18.4.0; 5G System; Data Collection Coordination Services; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.574/'

#

servers:

- url: '{apiRoot}/ndccf-datamanagement/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

#

security:

- oAuth2ClientCredentials:

- ndccf-datamanagement

- {}

#

paths:

/analytics-subscriptions:

post:

summary: Creates a new Individual DCCF Analytics Subscription resource.

operationId: CreateDCCFAnalyticsSubscription

tags:

- DCCF Analytics Subscriptions (Collection)

requestBody:

description: Contains the information for the creation the resource.

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfAnalyticsSubscription'

required: true

responses:

'201':

description: Create a new Individual DCCF Analytics Subscription resource.

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/ndccf-datamanagement/<apiVersion>/analytics-subscriptions/{subscriptionId}

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfAnalyticsSubscription'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

dccfAnalyticsNotification:

'{$request.body#/anaNotifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfAnalyticsSubscriptionNotification'

responses:

'200':

description: The notification is acknowledged and a planned action is provided.

content:

application/json:

schema:

$ref: '#/components/schemas/NotifResponse'

'204':

description: The receipt of the notification is acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

Fetch:

'{request.body#/fetchInstruct/fetchUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

type: string

minItems: 1

description: Fetch correlation identifiers.

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfAnalyticsSubscriptionNotification'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/analytics-subscriptions/{subscriptionId}:

delete:

summary: Deletes an existing Individual DCCF Data Subscription.

operationId: DeleteDCCFAnalyticsSubscription

tags:

- Individual DCCF Analytics Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

String identifying an analytics subscription to the Ndccf_DataManagement Service.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual DCCF Analytics Subscription resource matching the

subscriptionId was deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Updates an existing Individual DCCF Analytics Subscription resource.

operationId: UpdateDCCFAnalyticsSubscription

tags:

- Individual DCCF Analytics Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfAnalyticsSubscription'

parameters:

- name: subscriptionId

in: path

description: >

String identifying an analytics subscription to the Ndccf_DataManagement Service.

required: true

schema:

type: string

responses:

'200':

description: >

The Individual DCCF Analytics Subscription resource was modified successfully and a

representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfAnalyticsSubscription'

'204':

description: >

The Individual DCCF Analytics Subscription resource was modified successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/data-subscriptions:

post:

summary: Creates a new Individual DCCF Data Subscription resource.

operationId: CreateDCCFDataSubscription

tags:

- DCCF Data Subscriptions (Collection)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfDataSubscription'

required: true

responses:

'201':

description: Creates a new Individual DCCF Data Subscription resource.

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/ndccf-datamanagement/<apiVersion>/data-subscriptions/{subscriptionId}

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfDataSubscription'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

dccfDataNotification:

'{$request.body#/dataNotifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfDataSubscriptionNotification'

responses:

'200':

description: The notification is acknowledged and a planned action is provided.

content:

application/json:

schema:

$ref: '#/components/schemas/NotifResponse'

'204':

description: The receipt of the notification is acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

Fetch:

'{request.body#/fetchInstruct/fetchUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

type: string

minItems: 1

description: Fetch correlation identifiers.

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfAnalyticsSubscriptionNotification'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/data-subscriptions/{subscriptionId}:

delete:

summary: Deletes an existing Individual DCCF Data Subscription resource.

operationId: DeleteDCCFDataSubscription

tags:

- Individual DCCF Data Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: String identifying a data subscription to the Ndccf_DataManagement Service.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual DCCF Data Subscription resource matching the subscriptionId

was deleted.

'200':

description: >

The Individual DCCF Data Subscription resource matching the subscriptionId was deleted

and including the stored unsent data events in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfDataSubscriptionNotification'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Updates an existing Individual DCCF Data Subscription resource.

operationId: UpdateDCCFDataSubscription

tags:

- Individual DCCF Data Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfDataSubscription'

parameters:

- name: subscriptionId

in: path

description: >

String identifying a data subscription to the Ndccf_DataManagement Service.

required: true

schema:

type: string

responses:

'200':

description: >

The Individual DCCF Data Subscription resource was modified successfully and a

representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfDataSubscription'

'204':

description: >

The Individual DCCF Data Subscription resource was modified successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/transfers:

post:

summary: Creates a new Individual DCCF Data Management Transfer resource.

operationId: CreateDCCFDataManagementTransfer

tags:

- DCCF Data Management Transfers (Collection)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfDataManagementTransfer'

required: true

responses:

'201':

description: Creates a new Individual DCCF Data Management Transfer resource.

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/ndccf-datamanagement/<apiVersion>/tranfers/{tranferId}

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfDataManagementTransfer'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/transfers/{transferId}:

put:

summary: Updates an existing Individual DCCF Data Management Transfer resource.

operationId: UpdateDCCFDataManagementTransfer

tags:

- Individual DCCF Data Management Transfer (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfDataManagementTransfer'

parameters:

- name: transferId

in: path

description: >

String identifying a data management transfer to the Ndccf_DataManagement Service.

required: true

schema:

type: string

responses:

'200':

description: >

The Individual DCCF Data Management Transfer resource was modified successfully and a

representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/NdccfDataManagementTransfer'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an existing Individual DCCF Data Management Transfer resource.

operationId: DeleteDCCFDataManagementTransfer

tags:

- Individual DCCF Data DataManagementTransfer (Document)

parameters:

- name: transferId

in: path

description: >

String identifying a data management transfer to the Ndccf_DataManagement Service.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual DCCF Data Subscription resource matching the transferId was

deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

#

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

ndccf-datamanagement: Access to the ndccf-datamanagement API

#

schemas:

#

NdccfAnalyticsSubscription:

description: Represents an Individual DCCF Analytics Subscription.

type: object

required:

- anaSub

- anaNotifUri

- anaNotifCorrId

properties:

anaSub:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NnwdafEventsSubscription'

anaNotifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

anaNotifCorrId:

type: string

description: Notification correlation identifier.

notifEndpoints:

type: array

items:

$ref: '#/components/schemas/NotifyEndpoint'

minItems: 1

description: The information of notification endpoints.

formatInstruct:

$ref: '#/components/schemas/FormattingInstruction'

procInstructs:

type: array

items:

$ref: '#/components/schemas/ProcessingInstruction'

minItems: 1

description: Processing instructions to be used for sending event notifications.

targetNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

targetNfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

adrfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

ardfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

storeInd:

type: boolean

description: >

The indication for analytics storage. This attribute shall be provided and set to "true"

if the consumer requests to store the analytics in an ADRF but both the "adrfId" and

"adrfSetId" attributes are not provided. The default value is "false".

storeHandl:

$ref: '#/components/schemas/StorageHandlingInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

timePeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

dataCollectPurposes:

type: array

items:

$ref: '#/components/schemas/DataCollectionPurpose'

minItems: 1

description: >

The purposes of data collection. This attribute may only be provided if user consent is

required depending on local policy and regulations and the consumer has not

checked user consent.

checkedConsentInd:

type: boolean

description: Indication that the NF service consumer has already checked the user consent.

immReport:

$ref: '#/components/schemas/NdccfAnalyticsSubscriptionNotification'

#

NdccfDataSubscription:

description: Represents an Individual DCCF Data Subscription.

type: object

required:

- dataNotifUri

- dataNotifCorrId

- dataSub

properties:

dataSub:

$ref: 'TS29575_Nadrf_DataManagement.yaml#/components/schemas/DataSubscription'

dataNotifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

dataNotifCorrId:

type: string

description: Notification correlation identifier.

notifEndpoints:

type: array

items:

$ref: '#/components/schemas/NotifyEndpoint'

minItems: 1

description: The information of notification endpoints.

formatInstruct:

$ref: '#/components/schemas/FormattingInstruction'

procInstructs:

type: array

items:

$ref: '#/components/schemas/ProcessingInstruction'

minItems: 1

description: Processing instructions to be used for sending event notifications.

targetNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

targetNfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

adrfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

ardfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

storeInd:

type: boolean

description: >

The indication for analytics storage. This attribute shall be provided and set to "true"

if the consumer requests to store the analytics in an ADRF but both the "adrfId" and

"adrfSetId" attributes are not provided. The default value is "false".

storeHandl:

$ref: '#/components/schemas/StorageHandlingInformation'

timePeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

dataCollectPurposes:

type: array

items:

$ref: '#/components/schemas/DataCollectionPurpose'

minItems: 1

description: >

The purposes of data collection. This attribute may only be provided if user consent

is required depending on local policy and regulations and the consumer has not

checked user consent.

checkedConsentInd:

type: boolean

description: Indication that the NF service consumer has already checked the user consent.

immReport:

$ref: '#/components/schemas/NdccfDataSubscriptionNotification'

#

NdccfAnalyticsSubscriptionNotification:

description: Represents a notification for a DCCF analytics subscription.

type: object

required:

- anaNotifCorrId

- timeStamp

oneOf:

- required: [anaNotifications]

- required: [anaReports]

- required: [fetchInstruct]

properties:

anaNotifCorrId:

type: string

description: Notification correlation identifier.

anaNotifications:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NnwdafEventsSubscriptionNotification'

minItems: 1

description: List of analytics subscription notifications.

anaReports:

type: array

items:

$ref: '#/components/schemas/NotifSummaryReport'

minItems: 1

description: >

List of reports with summarized data from multiple analytics notifications that the DCCF

has received from NWDAF.

fetchInstruct:

$ref: 'TS29576_Nmfaf_3caDataManagement.yaml#/components/schemas/FetchInstruction'

terminationReq:

type: boolean

description: >

If provided and set to true, it indicates the termination of the data management

subscription that requested by the DCCF, i.e. no further notifications related to this

subscription will be provided, apart from sending final report (if available).

The default value is false.

delAlert:

$ref: '#/components/schemas/DeletionAlert'

termCause:

$ref: '#/components/schemas/TermCause'

pendDataNotifCause:

$ref: 'TS29520_Nnwdaf_DataManagement.yaml#/components/schemas/PendingNotificationCause'

reUserConsentPurs:

type: array

items:

$ref: '#/components/schemas/DataCollectionPurpose'

minItems: 1

description: The purposes of data collection for which the user consent is revoked.

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

#

NdccfDataSubscriptionNotification:

description: Represents a notification for a DCCF data subscription.

type: object

required:

- dataNotifCorrId

- timeStamp

oneOf:

- required: [dataNotif]

- required: [dataReports]

- required: [fetchInstruct]

properties:

dataNotifCorrId:

type: string

description: Notification correlation identifier.

newSubscriptionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

dataNotif:

$ref: 'TS29575_Nadrf_DataManagement.yaml#/components/schemas/DataNotification'

dataReports:

type: array

items:

$ref: '#/components/schemas/NotifSummaryReport'

minItems: 1

description: >

List of reports with summarized data from multiple notifications received from data

producer.

delAlert:

$ref: '#/components/schemas/DeletionAlert'

fetchInstruct:

$ref: 'TS29576_Nmfaf_3caDataManagement.yaml#/components/schemas/FetchInstruction'

terminationReq:

type: boolean

description: >

If provided and set to true, it indicates the termination of the data management

subscription that requested by the DCCF, i.e. no further notifications related to this

subscription will be provided, apart from sending final report (if available).

The default value is false.

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

#

FormattingInstruction:

description: Contains data or analytics formatting instructions.

type: object

properties:

consTrigNotif:

type: boolean

description: >

Indicates that notifications shall be buffered until the NF service consumer requests

their delivery.

reportingOptions:

$ref: '#/components/schemas/ReportingOptions'

#

ReportingOptions:

description: Represents reporting options for processed notifications.

type: object

oneOf:

- required: [notifyWindow]

- required: [notifyPeriod]

- required: [notifyPeriodInc]

- required: [depEventSubId]

properties:

notifyWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

notifyPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

notifyPeriodInc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

depEventSubId:

type: string

description: >

Notifications for the present subscription are sent only upon occurrence of events of the

subscription with identifier that matches this attribute.

minClubbedNotif:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxClubbedNotif:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

#

ProcessingInstruction:

description: Contains instructions related to the processing of notifications.

type: object

required:

- eventId

- procInterval

properties:

eventId:

$ref: '#/components/schemas/DccfEvent'

procInterval:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

paramProcInstructs:

type: array

items:

$ref: '#/components/schemas/ParameterProcessingInstruction'

minItems: 1

description: >

List of event parameter names, and for each event parameter name, respective event

parameter values and sets of the attributes to be used in the summarized reports.

NotifyEndpoint:

description: The information of notification endpoint.

type: object

required:

- notifUri

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrId:

type: string

description: Notification correlation identifier.

#

DccfEvent:

description: >

Identifies the (event exposure or analytics) event that the processing instructions

shall apply to. Contains all event IDs related to DCCF.

type: object

oneOf:

- required: [nwdafEvent]

- required: [smfEvent]

- required: [amfEvent]

- required: [nefEvent]

- required: [afEvent]

- required: [sacEvent]

- required: [nrfEvent]

- required: [udmEvent]

- required: [gmlcEvent]

- required: [upfEvent]

properties:

nwdafEvent:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

smfEvent:

$ref: 'TS29508_Nsmf_EventExposure.yaml#/components/schemas/SmfEvent'

amfEvent:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/AmfEventType'

nefEvent:

$ref: 'TS29591_Nnef_EventExposure.yaml#/components/schemas/NefEvent'

udmEvent:

$ref: 'TS29503_Nudm_EE.yaml#/components/schemas/EventType'

afEvent:

$ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/AfEvent'

sacEvent:

$ref: 'TS29536_Nnsacf_SliceEventExposure.yaml#/components/schemas/SACEvent'

nrfEvent:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NotificationEventType'

gmlcEvent:

$ref: 'TS29515_Ngmlc_Location.yaml#/components/schemas/EventNotifyDataType'

upfEvent:

$ref: 'TS29564_Nupf_EventExposure.yaml#/components/schemas/EventType'

#

ParameterProcessingInstruction:

description: >

Contains an event parameter name and the respective event parameter values and sets of

attributes to be used in summarized reports.

type: object

required:

- name

- values

- sumAttrs

properties:

name:

type: string

description: >

A JSON pointer value that references an attribute within the notification object to which

the processing instruction is applied.

values:

type: array

items: {}

minItems: 1

description: A list of values for the attribute identified by the name attribute.

sumAttrs:

type: array

items:

$ref: '#/components/schemas/SummarizationAttribute'

minItems: 1

description: Attributes requested to be used in the summarized reports.

aggrLevel:

$ref: '#/components/schemas/AggregationLevel'

supis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

description: Indicates the UEs for which processed reports are requested.

temporalAggrLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

areas:

type: array

items:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

minItems: 1

description: Indicates the Areas of Interest for which processed reports are requested.

#

NotifSummaryReport:

description: Represents summarized notifications based on processing instructions.

type: object

required:

- eventId

- procInterval

- eventReports

properties:

eventId:

$ref: '#/components/schemas/DccfEvent'

procInterval:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

eventReports:

type: array

items:

$ref: '#/components/schemas/EventParamReport'

minItems: 1

description: List of event parameter reports.

#

EventParamReport:

description: Represents a summarized report for one event parameter.

type: object

required:

- name

- values

properties:

name:

type: string

description: The name of the reported parameter.

values:

type: array

items: {}

minItems: 1

description: The list of values of the reported parameter.

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

area:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

spacing:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NumberAverage'

duration:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NumberAverage'

avgAndVar:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NumberAverage'

mostFreqVal: {}

leastFreqVal: {}

count:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minValue:

type: string

description: The minimum value of the parameter.

maxValue:

type: string

description: The maximum value of the parameter.

#

StorageHandlingInformation:

description: Contains storage handling information about data or analytics.

type: object

properties:

lifetime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

delNotifInd:

type: boolean

description: Indicates if deletion alerts are requested.

#

DeletionAlert:

description: Contains information about data or analytics that are about to be deleted.

type: object

properties:

alertStorTransId:

type: string

description: >

Storage transaction identifier that can be used to retrieve data or analytics.

required:

- alertStorTransId

#

NotifResponse:

description: >

Contains information about planned actions related to data or analytics

that are about to be deleted.

type: object

properties:

retrievalInd:

type: boolean

description: >

Indicates if the NF service consumer has determined to retrieve data

or analytics that are about to be deleted.

required:

- retrievalInd

#

NdccfDataManagementTransfer:

description: Represents an Individual DCCF Data Management Transfer.

type: object

properties:

subscriptionId:

type: string

description: The identifier of a subscription that needs to be transferred.

subInfo:

$ref: '#/components/schemas/NdccfDataSubscription'

required:

- subscriptionId

- subInfo

#

SummarizationAttribute:

anyOf:

- type: string

enum:

- SPACING

- DURATION

- OCCURRENCES

- AVG_VAR

- FREQ_VAL

- MIN_MAX

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the attribute in the summarized report.

Possible values are:

- SPACING: Average and variance of the time interval separating two consecutive occurrences

of the same event and parameter value, or periodicity for periodic reporting.

- DURATION: Average and variance of the time for which the parameter value applies.

- OCCURRENCES: Number of countable occurrences for the parameter.

- AVG_VAR: Average and variance of the parameter.

- FREQ_VAL: Most and least frequent values.

- MIN_MAX: Maximum and minimum parameter values.

#

AggregationLevel:

anyOf:

- type: string

enum:

- UE

- AOI

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the aggregation level for processing instructions.

Possible values are:

- UE: Indicates that the summarized reports shall be provided per UE.

- AOI: Indicates that the summarized reports shall be provided per Area of Interest.

#

DataCollectionPurpose:

anyOf:

- type: string

enum:

- ANALYTICS_GENERATION

- MODEL_TRAINING

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the purpose for data collection.

Possible values are:

- ANALYTICS_GENERATION: The data is collected for generating the analytics.

- MODEL_TRAINING: The data is collected for ML model training.

#

TermCause:

anyOf:

- type: string

enum:

- USER_CONSENT_REVOKED

- DCCF_OVERLOAD

- OTHER

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the cause for the subscription termination request by DCCF for data collection.

Possible values are:

- USER_CONSENT_REVOKED: The user consent has been revoked.

- DCCF_OVERLOAD: The DCCF is overloaded.

- OTHER: Indicates that the termination is due to other reason.

# *** End of Changes ***