3GPP TSG-CT Meeting #103	CP-240253

Maastricht, NL, 18th – 19th Mar 2024

# Source:	Samsung

# Title:	Revised WID on CT Aspects of Edge Computing Phase 2

# Document for:	Approval

# Agenda Item:	18.22

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Revised WID on CT Aspects of Edge Computing Phase 2

Acronym:	EDGE_Ph2

Unique identifier:	980005

Potential target Release:	Rel-18

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| EDGE_Ph2 | S2 | 970026 | Edge Computing Phase 2 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940059 | Study on Edge Computing Phase 2 | Corresponding study for architecture enhancements and procedures (SA2) |
| 950023 | Study on Security Enhancement of support for Edge Computing — phase 2 | Corresponding study for security aspects of Edge Computing phase 2 (SA3) |
| 990039 | Security Aspects of Support for Edge Computing in 5GC phase 2 | Security aspects of Edge Computing phase 2 (SA3) |

# 3	Justification

The stage 2 study on Edge Computing Phase 2 is captured in 3GPP TR 23.700-48 and the related normative work is specified normative aspects of the following in 3GPP TS 23.548, TS 23.501, TS 23.502 and TS 23.503:

-	The support of accessing an EHE in a VPLMN when roaming, including the scenario using a PDU Session with a PSA in the HPLMN and the scenario using a LBO PDU Session

-	The support of Fast and efficient network exposure improvements

-	The support of policies for finer granular sets of UEs

-	The support of influencing UPF and EAS (re)location for collections of UEs

-	The support of providing improvements related to GSMA OPG for EHE operated by a separate party

-	The support of allowing an AF to obtain and maintain a mapping table between IP address/IP range and DNAI

The 3GPP CT WGs need to do the normative work to complete the stage 3 work in Rel-18.

# 4	Objective

The objective of the work item is to develop the stage 3 specifications for the stage 2 requirements agreed under the stage 2 work item EDGE_Ph2. The following areas of work are expected to be covered:

CT4:

-	SMF Selection Subscription data updates to include the indication for HR-local traffic routing allowed and Session Management Subscription data updates to include the indication for HR-local traffic routing authorized

-	Impacts to SMF to define a new feature for the discovery and selection of a V-SMF and an H-SMF supporting local traffic routing for HR PDU sessions

-	Impacts to H-SMF to provide HR-local traffic offloading authorization information (e.g. Home Routed Session Breakout (HR-SBO) authorized indication, offload identifier, Vplmn Offloading Information including allowed FQDN ranges), DNS server address of HPLMN and HPLMN address information to V-SMF

-	Impacts to V-SMF to send the HR-SBO authorization request indication, the stored list of offload identifiers, V-EASDF address or local DNS server address to the H-SMF

-	Impacts to V-SMF/V-EASDF to support the HR-local traffic routing on creation of DNS context with HPLMN ID, unspecified address or mapped UE IP address and N6 traffic routing information

-	Impacts to source AMF to provide HR-SBO allowed indication to the target AMF on HO procedure.

-	Impacts to V-SMF/H-SMF to support UL CL/BP and local UPF insertion in HR roaming

-	Impacts to AMF/V-SMF/H-SMF to support sending of target DNAI information from H-SMF to AMF

-	Impacts to V-SMF to send EAS rediscovery Indication and the EAS information to be refreshed for EAS re-discovery to H-SMF to indicate that the EAS rediscovery is requested

-	Impacts to UDM/UDR to support the storage of subscription data of ECS Address Configuration Information (EACI) per PLMN ID and providing the EACI per PLMN ID by using the parameter provisioning procedure

-	Impacts to V-SMF to send or update the VPLMN EACI obtained from V-AF to the H-SMF

-	Impacts to UDR for granular UE set, common EAS/DNAI and DNAI-EAS mapping

-	Impacts to SMF/EASDF to support 5GC selection/AF selection of the common DNAI for UE collection or 5GC selection of a common EAS for a collection of UEs for both multiple SMFs and single SMF cases

-	Impacts to H-SMF to send Internal Group Identifier(s) for group of UE(s) to the V-SMF.

-	Extending ECS configuration information with security parameters

CT3:

-	Impacts to H-PCF to send to H-SMF roaming offload policy to indicate traffic which is authorized to perform local traffic routing in VPLMN

-	Impacts to AF/H-NEF/H-SMF on providing the target DNAI to support AF triggered EAS Re-discovery

-	Impacts to AF deployed in the VPLMN on providing the EACI in the VPLMN to the V-SMF via the V-NEF for HR case

-	Impacts to AF/NEF/UDR/PCF to support combination of Group Ids or any UE, and (service information and/or "Subscriber categories")

-	Impacts to PCF/SMF/NEF/AF to identify the collections of UEs using common EAS/DNAI

-	Impacts to SMF/NEF/PCF/AF to provide common EAS/DNAI to support using common EAS/DNAI determined by AF or 5GC for traffic routing or EAS discovery

-	Impacts to SMF/NEF/AF to support Edge Relocation within the same hosting PLMN's EHEs

-	Impacts to NEF/UDR/AF on mapping information between EAS IP/IP range and DNAI

-	Impacts on Nnef_TrafficInfluence service to support AF may provide common DNAI/EAS for the UE collection

-	Extending ECS configuration information with security parameters

CT1:

-	Extending ECS configuration information with security parameters

The potential impacts to CT4 and CT3 will be updated during the normative work in SA2. Stage 3 work will start only when the normative stage 2 requirements are available.

The following two aspects need further SA2 work under FS_eUEPO and FS_XRM SIDs and will be included in the related WIDs:

- URSP enhancement to support the scenarios where the EHE is accessed via an LBO PDU Session depends on further SA2 work on FS_eUEPO.

- Fast and efficient network exposure improvements via the Local UPF/NEF depends on further SA2 work on FS_XRM.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A |  |  |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 29.502 | 1. Impacts on V-SMF and H-SMF to support HR-SBO; 2. Support UL CL/BP and local UPF insertion in HR roaming 3. Support of target DNAI information from H-SMF to AMF via V-SMF 4. Impacts on V-SMF and H-SMF to support EAS rediscovery 5. Impacts on V-SMF and H-SMF on VPLMN EACI | TSG#103 (March 2024) | CT4 responsibility |
| 29.503 | 1. User subscription information updates for indicating HR-local traffic routing allowed or HR-local traffic routing authorized; 2. Support parameter provisioning and storage of subscription data of ECS Address Configuration Information (EACI) per PLMN ID. 3. Extending ECS configuration information with security parameters | TSG#104 (June 2024) | CT4 responsibility |
| 29.504 | Updates to the UDR data model. | TSG#103 (March 2024) | CT4 responsibility |
| 29.518 | Provides HR-SBO allowed indication from source AMF to the target AMF | TSG#103 (March 2024) | CT4 responsibility |
| 29.571 | Specification work on common data type definition for the new introduced parameters. | TSG#103 (March 2024) | CT4 responsibility |
| 29.556 | 1. Impacts to support the HR-local traffic routing; 2. Support 5GC selection of a common EAS for a collection of UEs | TSG#103 (March 2024) | CT4 responsibility |
| 29.508 | Impacts on Nsmf_eventExposure services to support notification of AF/EAS change and supporting common EAS/DNAI. | TSG#103 (March 2024) | CT3 responsibility |
| 29.512 | 1. Provides roaming offload policy to perform local traffic routing in VPLMN to SMF; 2. Potential impacts to support combination of Group Ids or any UE, and (service information and/or "Subscriber categories"); 3. Potential impacts to support the collections of UEs. | TSG#103 (March 2024) | CT3 responsibility |
| 29.514 | Specification work on Policy Authorization updates | TSG#103 (March 2024) | CT3 responsibility |
| 29.513 | Impacts on policy and Charging Control signalling flows. | TSG#103 (March 2024) | CT3 responsibility |
| 29.522 | 1. Impacts on providing the target DNAI to H-SMF to support AF triggered EAS Re-discovery; 2. Impacts on providing the EACI in the VPLMN to the V-SMF for HR case; 3. Supports combination of Group Ids or any UE, and (service information and/or "Subscriber categories"); 4. Impacts on Nnef_TrafficInfluence service to support common EAS/DNAI; 5. Impacts on mapping information between EAS IP/IP range and DNAI. 6. Extending ECS configuration information with security parameters | TSG#104 (June 2024) | CT3 responsibility |
| 29.519 | Data storage update in UDR | TSG#103 (March 2024) | CT3 responsibility |
| 24.501 | Extending ECS configuration information with security parameters | TSG#104 (June 2024) | CT1 responsibility |
| 24.008 | Extending ECS configuration information with security parameters | TSG#104 (June 2024) | CT1 responsibility |

# 6	Work item Rapporteur(s)

Qi Caixia, caixia.qi@huawei.com

# 7	Work item leadership

CT4

# 8	Aspects that involve other WGs

SA3 for the security aspects.

SA5 for the charging aspects.

# 9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

China Unicom

China Telecom

ZTE

NTT DOCOMO

CATT

OPPO

Apple

Lenovo

Motorola Mobility

Nokia

Nokia Shanghai Bell

Ericsson

China Mobile

KDDI

NEC

Charter Communications

Samsung