3GPP TSG-SA/CT/RAN Meeting #103	CP-240285

# Maastricht, NL, March 18 – 19, 2024

Title:	Summary for Rel-18 WID ShDatID_H

Type:	Work Item Summary

Agenda Item:	21

Source:	Huawei

Contact:	hao.jing@huawei.com

Summary based on the input provided by Huawei in CP-240285.

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 990004 | Enhancement of Shared Data ID and Handling | ShDatID_H | CT4 | CP-232030 | JING, Hao, Huawei |

Introduction

This work item introduce the enhancements of shared data, it is concluded to optimize the shared data retrieval procedure on Nudm_SDM Service operation for shared data identified by shared data IDs conveyed within AccessAndMobilitySubscriptionData

Description

CT4 has introduced shared data concept since Release 15, by including Shared Data IDs as reference of individual subscription data type, for example, sharedAmData and sharedSmsMngSubsData. When the consumer of the Nudm_SDM service (e.g. AMF) retrieves AccessAndMobilitySubscriptionData from the UDM, it may detect that the retrieved data contains a shared data ID pointing to shared AccessAndMobilitySubscriptionData. If these shared AccessAndMobilitySubscriptionData are not yet available (i.e. cached) at the NF consumer, a second GET message to retrieve the shared AccessAndMobilitySubscriptionData from the UDM is required.

The second GET message can be avoided by retrieving both the AccessAndMobilitySubscriptionData and the shared AccessAndMobilitySubscriptionData with a single GET request by adding shared data IDs as a query parameter to the GET request.

In summary, it is concluded that the extra signalling to retrieve shared AccessAndMobilitySubscriptionData is optimized as defined in 3GPP TS 29.503 [1].

References

[1]	3GPP TS 29.503: "Unified Data Management Services".