3GPP TSG-CT Meeting #103	CP-240292

# Maastricht, Netherlands; 18th – 19th March 2024								 (was CP-240257, C1-241361)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0852 | rev | 3 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to +CUNPER AT Command |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Apple |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2, SUECR |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1] As per TS 24.501, there is a possibility that the unavailability period duration may not be provided by the UE and that the unavailability period duration is determined by the network and sent to UE. “The AMF may determine the periodic registration update timer value based on the stored value of the received unavailability period duration, or based on a network determined unavailability period duration when the unavailability period durationis not provided by the UE”. Changes in Rev-2 to CT Plenary #103: Add the work item code 5GSAT_Ph2 as first work item code. No change in CR contents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the +CUNPER AT Command to align with stage-2 requirements. a] Network can send unavailability period duration b] Add network sent unavailability period duration to unsolicited return code |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | User will be unaware of the correct unavailability period duration if it has been calculated and sent by network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.86 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Please note: CR #0850 (Correction to +CUNPER command and +CDISCO command) should be applied to TS 27.007 after CR #0852 (Updates to +CUNPER AT Command) has been implemented. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes in Rev-2 to CT Plenary #103: Add the work item code 5GSAT_Ph2 as first work item code. No change in CR contents. Changes in Rev-3 to CT Plenary #103: Correct the CR header to represent the correct meeting and location |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.86	Unavailability Period +CUNPER

Table 8.86-1: +CUNPER parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CUNPER=<n>,<unavailability_type>, <requested_unper_duration>,<unper_start> | +CME ERROR: <err> |
| +CUNPER? | +CUNPER: <n>,<unper_support_nw>,<unavailability_type>,<requested_unper_duration>,<unper_start>,<result>[,<nw_sent_unper_duration>] +CME ERROR: <err> |
| +CUNPER=? | +CUNPER: (list of supported <requested_unper_duration>s) |

Description

The set command is used to provide the the unavailability type, unavailability period duration, and the start of the unavailability period that are sent to the network in the next registration or UE initiated de-registration procedure, see 3GPP TS 24.501 [161] clause 5.3.26 and 3GPP TS 24.301 [83]. The set command is also used to enable or disable the unsolicited response +CUNPER: <unper_support_nw>[,<nw_sent_unper_duration>]. The unavailability period specifies the period during which the UE may become unavailable when an event is triggered.

Refer clause 9.2 for possible <err> values.

Read command returns the <requested_unper_duration>, <unavailability_type> and <unper_start> that were provided to the network in the last registration or UE initiated de-registration procedure and <unper_suppport_nw> specifies whether the network indicated support for the unavailability period in the last registration procedure.  It also returns the value of unavailability period duration sent by the network in <nw_sent_unper_duration>.

Test command returns supported values as a compound value.

Defined values

<n>: integer type, to enable or disable the unsolicited response:

+CUNPER: <unper_support_nw>[,<nw_sent_unper_duration>].

0	disable the unsolicited response.

1	enable the unsolicited response.

<requested_unper_duration>: string type; indicates the unavailability period duration provided by the UE. The <requested_unper_duration> value is coded as three bytes (octets 3 to 5) of the Time duration information element in 3GPP TS 24.301 [83].  The default value is "000000000000000000000000", which indicates that the unavailability period duration is not known.

<unavailability_type>: integer type. Indicates the reason for unavailability.

0	UE reasons.

1	discontinuous coverage.

The <unavailability_type> is applicable only in NG-RAN. It shall be considered as set to "discontinuous coverage" by the MT in satellite E-UTRAN.

<unper_start>: string type; indicates the time until the start of the next unavailability period. The <unper_start> value is coded as three bytes (octets 3 to 5) of the Time duration information element in 3GPP TS 24.301 [83].  The default value is "000000000000000000000000".

<unper_support_nw>: integer type. Indicates the unavailability period feature support by the network see 3GPP TS 24.501 [161] clause 5.3.26 and 3GPP TS 24.301 [83].

0	indicates that the unavailability period feature is not supported by the network.

1	indicates that the unavailability period feature is supported by the network.

<result>: integer type; indicates the final result of successful indication of unavailability period to network.

0	de-registration procedure successful (see 3GPP TS 24.501 [161] clause 5.3.26)

1	registration procedure successful (see 3GPP TS 24.501 [161] clause 5.5.1.3.4)

2	registration procedure not successful (see 3GPP TS 24.501 [161] 5.5.1.3.5, clause 5.5.1.3.7).

<nw_sent_unper_duration>: string type; indicates the network sent unavailability period duration. This is coded as three bytes (octets 3 to 5) of the Time duration information element in 3GPP TS 24.301 [83]. The default value is "000000000000000000000000", which indicates that the unavailability period duration is not known or not received from the network. This is just an indication of the value sent by the network but as per 3GPP TS 24.501 [161], UE may choose to use or discard this as per implementation.

Implementation

Optional.

This command is only applicable to UEs in NG-RAN and in satellite E-UTRAN.

* * * End of Changes * * * *