3GPP TSG-CT WG4 Meeting #122	C4-241513

# Changsha, P.R.China; 15th – 19th April 2024                                           was C4-241510

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.571 | CR | 0553 | rev | 4 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | dlAmbr for HR-SBO PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The dlAmbr is representing an Authorized DL Session AMBR for Offloading, i.e. DL Aggregate Maximum Bit Rate for the Non-GBR QoS Flows of the PDU Session authorized for offloading to the local part of DN in VPLMN, this is valid for the whole HR-SBO PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to use a separate vplmnDlAmbr. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous requirements may leads to interoperability problem. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.85, 5.4.4.x(new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The contribution introduces a backwards compatible correction to the following APIs: TS29502_Nsmf_PDUSession.yaml TS29512_Npcf_SMPolicyControl.yaml TS29571_CommonData.yaml. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 5.4.4.85	Type: VplmnOffloadingInfo

Table 5.4.4.85-1: Definition of type VplmnOffloadingInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| offloadIdentifier | OffloadIdentifier | O | 0..1 | Offload Identifier uniquely identifying the VPLMN Offloading information provided by the HPLMN. |
| vplmnId | PlmnId | O | 0..1 | V-PLMN ID. When absent, the PLMN ID of the PLMN serving the UE shall be assumed. |
| allowedTraffic | boolean | O | 0..1 | When present, this IE shall be set as follows: - true (default): the VplmnOffloadingInfo describes the traffic allowed to be offloaded - false: the VplmnOffloadingInfo describes the traffic disallowed to be offloaded |
| ipv4AddressRanges | array(Ipv4AddressRange) | O | 1..N | List of ranges of IPv4 addresses allowed (or disallowed) to be routed to the local part of DN in the VPLMN |
| ipv4AddrMasks | array(Ipv4AddrMask) | O | 1..N | List of ranges of IPv4 addresses allowed (or disallowed) to be routed to the local part of DN in the VPLMN, whereby each range of IPv4 addresses corresponds to the IPv4 addresses of an IPv4 subnet defined by an IPv4 address and subnet mask. |
| ipv6AddressRanges | array(Ipv6AddressRange) | O | 1..N | List of ranges of IPv6 addresses allowed (or disallowed) to be routed to the local part of DN in the VPLMN |
| ipv6PrefixRanges | array(Ipv6PrefixRange) | O | 1..N | List of ranges of IPv6 prefixes allowed (or disallowed) to be routed to the local part of DN in the VPLMN |
| fqdnList | array(Fqdn) | O | 1..N | List of FQDNs allowed (or disallowed) to be routed to the local part of DN in the VPLMN |
| fqdnPatterns | array(FqdnPatternMatchingRule) | O | 1..N | List of FQDN patterns of FQDNs allowed (or disallowed) to be routed to the local part of DN in the VPLMN |
| NOTE: If none of the ipv4AddressRanges, ipv4AddrMasks, ipv6AddressRanges, ipv6PrefixRanges, fqdnList and fqdnPatterns IEs is present, all the traffic of the PDU session is allowed to be routed to the local part of DN in the VPLMN. At least one of these IEs shall be present when the allowedTraffic IE is set to false. |  |  |  |  |

# *** Next Change ***

#### 5.4.4.x	Type: VplmnDlAmbr

Table 5.4.4.x-1: VplmnDlAmbr

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| vplmnId | PlmnId | M | 1 | V-PLMN ID. |
| sessionDlAmbr | BitRate | M | 1 | Authorized DL Session AMBR for Offloading, i.e. DL Aggregate Maximum Bit Rate for the Non-GBR QoS Flows of the PDU Session authorized for offloading to the local part of DN in VPLMN. |

# *** Next Change ***

# A.2	Data related to Common Data Types

openapi: 3.0.0

info:

version: '1.5.0-alpha.6'

title: 'Common Data Types'

description: |

Common Data Types for Service Based Interfaces.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.571 Common Data Types for Service Based Interfaces, version 18.5.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.571/'

******Skipped for Clarity*******

VplmnOffloadingInfo:

description: VPLMN Specific Offloading Information

type: object

nullable: true

properties:

offloadIdentifier:

$ref: '#/components/schemas/OffloadIdentifier'

vplmnId:

$ref: '#/components/schemas/PlmnId'

allowedTraffic:

type: boolean

default: true

ipv4AddressRanges:

type: array

items:

$ref: '#/components/schemas/Ipv4AddressRange'

minItems: 1

ipv4AddrMasks:

type: array

items:

$ref: '#/components/schemas/Ipv4AddrMask'

minItems: 1

ipv6AddressRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6AddressRange'

minItems: 1

ipv6PrefixRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6PrefixRange'

minItems: 1

fqdnList:

type: array

items:

$ref: '#/components/schemas/Fqdn'

minItems: 1

fqdnPatterns:

type: array

items:

$ref: '#/components/schemas/FqdnPatternMatchingRule'

minItems: 1

VplmnDlAmbr:

description: an Authorized DL Session AMBR for Offloading for the VPLMN

type: object

nullable: true

properties:

vplmnId:

$ref: '#/components/schemas/PlmnId'

sessionDlAmbr:

$ref: '#/components/schemas/BitRate'

required:

- vplmnId

- sessionDlAmbr

******Skipped for Clarity*******

# *** End of Change ***