3GPP TSG-CT WG4 Meeting #122	C4-241353

# Changsha, P.R.China; 15th – 19th April 2024											   was C4-241040

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0758 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDU session release condition due to network slice replacement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The text in clause 5.2.2.3.12 is a bit mis-leading: Such handling of PDU session release may also happen in network slice replacement procedure (see clause 4.3.3.3 of 3GPP TS 23.502 [3]) when the AMF determines the existing PDU session cannot serve the alternative S-NSSAI. From the text, it is not clear if the PDU session is currently associated with the alternative S-NSSAI while the alternative S-NSSAI becomes unavailable, or the PDU session is currently associated with original S-NSSAI and is to be changed to the alternative S-NSSAI while for certain reason the alternative cannot be used by the PDU session. According to TS 23.502 clause 4.3.3.3, the PDU session release happens when the AMF wants to assoicate the PDU session with an alternative S-NSSAI while due to certain reason (e.g. V-SMF/I-SMF cannot support the alternative S-NSSAI) the PDU session cannot be retained. In this case, the AMF finally decides to trigger PDU session release procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the mis-leading text in clause 5.2.2.3.12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-leading text exists in clause 5.2.2.3.12. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution does not introduce any change to the OpenAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1: - Revert the original changes, and further clarify the exact situation happens when the PDU session cannot be retained with the alternative S-NSSAI due to certain reason e.g. V-SMF cannot support the alternative S-NSSAI. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.2.3.12	AMF requested PDU Session Release due to slice not available

If a network slice is detected as not available (e.g. due to slice congestion or operational disable the slice), the NF Service Consumer (i.e. AMF) shall use this procedure to release the PDU session associated to the unavailable network slice. Such handling of PDU session release may also happen in network slice replacement procedure if the AMF determines the PDU session cannot be retained with the alternative S-NSSAI due to certain reason (e.g. V-SMF/I-SMF cannot support the alternative S-NSSAI) (see clause 4.3.3.3 of 3GPP TS 23.502 [3]).

The requirements specified in clause 5.2.2.3.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.3.1-1, with the following modifications.

The POST request shall contain:

-	the release IE set to true;

-	the cause IE set to S_NSSAI_CONGESTION or REL_DUE_TO_SLICE_NOT_AVAILABLE;

-	optionally the skipN2PduSessionResRelInd IE with the value "true" to skip RAN resources release for the PDU session, e.g. for a PDU session with active UP associated with a slice that is no longer available after a handover.

* * * End of Changes * * * *