3GPP TSG-CT WG4 Meeting #122	C4-241291

# Changsha, P.R.China; 15th – 19th April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.573 | CR | 0189 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on modificationsBlock |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, Roaming5G |  |  |  |  |  | Date: |  |  | 2024-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | modificationsBlock attribute defined in clause 6.2.5.2.2 is reused by roaming hub to support the Error message formatting by the Roaming Intermediary. There are several places define the name of attribute as modificationBlock, which shall be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the name of attribute modificationBlock to modificationsBlock |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent within the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.3.2, 5.5.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution does not change the OpenAPI. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.5.3.3.2	Error message formatting by the Roaming Intermediary

If a Roaming Intermediary needs to generate an N32-f related error message upon receiving an N32-f response, the Roaming Intermediary shall construct a new N32-f request as defined in clause 5.3.2.3 for a SEPP with the following modifications:

-	the DataToIntegrityProtectBlock (see Table 6.2.5.2.2-1) shall only contain the MetaData with the N32fContextId and messageId of the N32-f response message for which an error was detected.

-	the patch instructions in the modificationsBlock (see Table 6.2.5.2.2-1) shall be based on an  DataToIntegrityProtectBlock only containing the MetaData with the n32fContextId and messageId.

-	the modifications in the "modificationsBlock" shall result in encoding a N32-c request for N32-f Error Reporting, i.e. it shall contain patch instructions:

-	adding the requestLine to form an HTTP POST request "{n32c-apiRoot}/n32c-handshake/v1/n32f-error"; the {n32c-apiRoot} shall be set to a dummy N32-c apiRoot defined as the N32-f apiRoot with the authority part prepended with the label "n32c".

EXAMPLE:	If the n32-f apiRoot is https://sepp-n32f.5gc.mnc203.mcc422.3gppnetwork.org:443, the dummy N32-c apiRoot is:

https://n32c.sepp-n32f.5gc.mnc203.mcc422.3gppnetwork.org:443

NOTE:	The dummy N32-c apiRoot in the N32-c message encapsulated in the N32-f request needs not contain addressing information of any actual N32-c service instance of the p-SEPP. Instead, this indicates to the receiving N32-f service instance of the p-SEPP that it needs to look up for a N32-c service instance and route the request towards that N32-c service instance. See step 9 of clause 5.5.3.3.1.

-	adding headers, if applicable; and

-	adding the payload that shall be the content of the N32-f Error Reporting Request, i.e N32fErrorInfo.

-	the modificationsBlock shall contain the JWS signature of the Roaming Intermediary.

The Roaming Intermediary shall then send its N32-f request towards the p-SEPP, (using the same N32-f apiRoot as the one that was used in the original N32-f request sent to the p-SEPP) possibly via another intermediate Roaming Intermediary.

* * * Next Change * * * *

5.5.3.4.2	Error message formatting by the Roaming Intermediary

If a Roaming Intermediary needs to generate a service error message upon receiving an N32-f request, the Roaming Intermediary shall construct a service error response (to be sent within a successful N32-f response) as defined in clause 5.3.2.3 for a SEPP with the following modifications:

-	the DataToIntegrityProtectBlock (see Table 6.2.5.2.3-1) shall only contain metadata with N32-f message ID and N32-f context ID of the N32-f request message;

-	the patch instructions in the modificationsBlock (see Table 6.2.5.2.3-1) shall be based on the intermediary originated DataToIntegrityProtectBlock only containing the MetaData with the N32-f message ID and N32-f context ID.

-	the modifications in the "modificationsBlock" shall result in encoding the service error response, i.e. it shall contain patch instructions;

-	adding the statusLine to form the desired service error response (e.g. 403 Forbidden response);

-	adding SBI headers, if applicable; and

-	adding the payload that shall be the content of the service error response (e.g. ProblemDetails with the reason why the registration request is rejected);

-	the modificationsBlock shall contain the JWS signature of the Roaming Intermediary.

The Roaming Intermediary shall then send its N32-f response towards the c-SEPP, possibly via another intermediate Roaming Intermediary, encapsulating the service error response.

Editor's note: Roaming Intermediary originated error message e.g. to terminate a session is FFS.

* * * End of Changes * * * *