3GPP TSG-CT WG4 Meeting #123	C4-242456

# Hyderabad, India; 27th – 31st May 2024	(was C4-242130)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 1069 | rev | 3 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Content of JSON Patch requests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2024-03-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The PATCH operation on the the "Individual subscription" resource (see clause 6.2.3.3) is defined as a JSON Patch operation. According to the JSON Patch RFC 6902, the different operations to be performed in the PATCH request MUST be expressed as a <op, path, value> tuple. This is essential, in order to claim that this PATCH operation conforms to the "application/json-patch+json" media type. Also, adhering to this sytax/format, allows to use off-the-shelf software that implements the JSON Patch functionality, without modification. However, the current definition of the PATCH operation in this TS (in Namf_EventExposure) completely violates this approach, since it defines additional attributes, to be used instead of the standard "value" attribute. To claim compliance with the JSON Patch standard format, one and only one attribute, called "value", MUST be used, no matter which element of a JSON document needs to be modified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the additional attributes, and keep only the "value" attribute, defined as <any type> in its schema defintion (i.e.: { }), similarly to how it is defined in the re-usable data type "PatchItem" in TS 29.571. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PATCH operation is incorrectly defined, making it impossible to claim compliance with the JSON Patch format; this leads to interoperability problems (e.g. if a receiver of the PATCH requests detects that the payload does not conform to the syntax advertised by the media type). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.2.14, 6.2.6.2.18, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections with impacts on the following APIs: - TS29518_Namf_EventExposure.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.6.2.14	Type: AmfUpdateEventSubscriptionItem

Table 6.2.6.2.14-1: Definition of type AmfUpdateEventSubscriptionItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| op | string | M | 1 | This IE indicates the patch operation as defined in IETF RFC 6902 [14] to be performed on attributes of the "Individual subscription" resource (see clause 6.2.3.3). This IE shall support the following values: Enum: "add" Enum: "replace" Enum: "remove" |  |
| path | string | M | 1 | This IE contains a JSON pointer value (as defined in IETF RFC 6901 [40]) that references a location of an attribute of the "Individual subscription" resource (see clause 6.2.3.3) on which the patch operation shall be performed. This IE shall contain the JSON pointer to a valid index of the "/eventList" array in the AMF Event Subscription, formatted with following pattern: '\/eventList\/[0-]$¦\/eventList\/[1-9][0-9]*$' Example: "/eventList/0" stands for the first member of the array; "/eventList/10" stands for the 11th member of the array; "/eventList/-" stands for a new (non-existent) member after the last existing array element. Only allowed with "add" operation. To update the PRA Information, this IE shall contain the JSON pointer to a valid key of the "/presenceInfoList" object in the AMF Event Subscription, the key shall be formatted as the "praId" attribute within the PresenceInfo data type. These attributes shall only be updated when both the NF Service Consumer and the AMF support the MPRA feature. Pattern: '^(\/eventList\/0¦\/eventList\/[1-9][0-9]*){1}(\/presenceInfoList\/0¦\/presenceInfoList\/[1-9][0-9]*)?$' Example: "/eventList/10/presenceInfoList/123" stands for the PresenceInfo with PRA ID 123 for the 11th member of the array. (NOTE 1) To remove list of group member UE(s) from a group subscription, this IE shall contain the JSON pointer to the "/excludeSupiList" object or "/excludeGpsiList" object in the AMF Event Subscription. These attributes shall only be updated when both the NF Service Consumer and the AMF support the DGEM feature. Pattern: '^\/excludeSupiList¦\/excludeGpsiList$' (NOTE 2) To add list of group member UE(s) into a group subscription, this IE shall contain the JSON pointer to the "/includeSupiList" object or "/includeGpsiList" object in the AMF Event Subscription. These attributes shall only be updated when both the NF Service Consumer and the AMF support the DGEM feature. Pattern: '^\/includeSupiList¦\/includeGpsiList$' (NOTE 3) To modify or remove the notifyForSupiList IE, the notifyForGroupList IE or the notifyForSnssaiDnnList IE, this IE shall contain the JSON pointer to the "/notifyForSupiList", "/notifyForGroupList" or "/notifyForSnssaiDnnList" attribute in the AMF Event Subscription. When modifying these IEs, the new list of SUPIs, Internal Groups or DNN/S-NSSAIs shall replace any earlier received list of SUPIs, Internal Groups or DNNs/S-NSSAIs respectively. These attributes shall only be updated when both the NF Service Consumer and the AMF support the AOIEF feature. Pattern: '^(\/eventList\/0¦\/eventList\/[1-9][0-9]*){1}(\/notifyForSupiList)$' Pattern: '^(\/eventList\/0¦\/eventList\/[1-9][0-9]*){1}(\/notifyForGroupList)$' Pattern: '^(\/eventList\/0¦\/eventList\/[1-9][0-9]*){1}(\/notifyForSnssaiDnnList)$' (NOTE 4) |  |
| value | Any Type | C | 0..1 | This IE contains the new value of the attribute (as indicated in "path") to be added or updated of an existing AMF event to be modified. It shall be present if the patch operation is "add" or "replace" |  |
| NOTE 1: Update of PRA information by extending the schema of the path IE with JSON pointer to a valid key of the "/presenceInfoList" object shall only be used if the AMF supports the MPRA feature. NOTE 2: Remove group member UE(s) by extending the schema of the path IE with JSON pointer to the "/excludeSupiList" object or "/excludeGpsiList" object shall only be used if the AMF supports the DGEM feature. NOTE 3: Add group member UE(s) by extending the schema of the path IE with JSON pointer to the "/includeSupiList" object or "/includeGpsiList" object shall only be used if the AMF supports the DGEM feature. NOTE 4: Modifying the list of SUPIs or DNNs/S-NSSAIs shall only be used if the AMF supports the AOIEF feature. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.2.6.2.18	Type: AmfUpdateEventOptionItem

Table 6.2.6.2.18-1: Definition of type AmfUpdateEventOptionItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| op | string | M | 1 | This IE indicates the patch operation as defined in IETF RFC 6902 [14] to be performed on attributes of the "Individual subscription" resource (see clause 6.2.3.3). This IE shall support the following values: Enum: "replace" |  |
| path | string | M | 1 | This IE contains a JSON pointer value (as defined in IETF RFC 6901 [40]) that references a location of attributes of the "Individual subscription" resource (see clause 6.2.3.3) on which the patch operation shall be performed. This IE shall contain the JSON pointer to "/options/expiry" attribute of the event subscription resource. Pattern: "^\/options\/expiry$" To update the notifFlag attribute, this IE shall contain the JSON pointer to "/options/notifFlag" attribute of the event subscription resource. This attribute shall only be updated when both the NF Service Consumer and the AMF support the EneNA feature. Pattern: "^\/options\/notifFlag$" To update the mutingExcInstructions attribute, this IE shall contain the JSON pointer to "/options/mutingExcInstructions" attribute of the event subscription resource. This attribute shall only be updated when both the NF Service Consumer and the AMF support the ENAPH3 feature. Pattern: "^\/options\/mutingExcInstructions$" |  |
| value | Any Type | M | 1 | This IE contains the new value of the attribute (as indicated in "path") to be updated. |  |

* * * Next Change * * * *

# A.3	Namf_EventExposure API

(... text not shown for clarity ...)

AmfUpdateEventSubscriptionItem:

description: Document describing the modification(s) to an AMF Event Subscription

type: object

properties:

op:

type: string

enum:

- add

- remove

- replace

path:

type: string

pattern: '^\/eventList\/-|(\/eventList\/0|\/eventList\/[1-9][0-9]*){1}(\/presenceInfoList\/0|\/presenceInfoList\/[1-9][0-9]* |\/notifyForSupiList|\/notifyForGroupList|\/notifyForSnssaiDnnList)?|\/excludeSupiList|\/excludeGpsiList|\/includeSupiList|\/includeGpsiList$'

value: {}

required:

- op

- path

AmfUpdateEventOptionItem:

description: Document describing the modifications to AMF event subscription options

type: object

properties:

op:

type: string

enum:

- replace

path:

type: string

pattern: '^(\/options\/expiry|\/options\/notifFlag|\/options\/mutingExcInstructions)$'

value: {}

required:

- op

- path

- value

(... text not shown for clarity ...)

* * * End of Changes * * * *