3GPP TSG-CT Meeting #104	CP-241223

# Shanghai, P.R. China; 17th – 18th June 2024							(revision of C4-241409)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.175 | CR | 0002 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the Nimsas_MediaControl service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In clause 5.3.2.1, it states that the media flow maybe “originatied by the MF or MRF”, while the MRF supporting data channel has been removed in the WI scope, so the MRF shall be removed. - The description of the resource Individual call session is not clear, which should be clarified. - The description of mdc2Protocol is not correct |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove MRF in 5.3.2.1 - Clarify the description of the resource Indivial call session. - Correct the description of mdc2Protocol |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1, 6.2.3.1, 6.2.6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2: remove the change in 6.2.6.2.4 which is overlapped with C4-242416. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.3.2.1	Introduction

The Service operations defined for the Nimsas_MediaControl service are as follows:

-	MediaInstruction: It provides instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event, and that may require media resource reservation on MF.

NOTE:	For the case when a specific media flow needs to be terminated in MF media resource (i.e. termination of a Data Channel media descriptor offered by one of the IMS subscribers) or originated by the MF (i.e. origination of a data channel media descriptor to be offered towards one of the IMS subscribers), the consumer must provide a complete MF media specification including information required by MF to know how to terminate or originate the media flow.

Table 5.3.2.1-1: Service operations supported by the Nimsas_MediaControl service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| MediaInstruction | Instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event | Request/Response | DCSF |

* * * Next Change * * * *

6.2.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.3.1-1 depicts the resource URIs structure for the Nimsas_MediaControl service API.

Figure 6.2.3.1-1: Resource URI structure of the Nimsas_MediaControl API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Individual call session (Document) | /call-sessions/{sessionId}/media-instruction | media-instruction (POST) | Instructs the IMS AS on how to control the media resources. |

* * * End of Changes * * * *