3GPP TSG-CT Meeting #104	CP-241232

# Shanghai, P.R. China; 17th – 18th June 2024										was C1-242814

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.549 | CR | 0021 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network slice capability enablement services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Lenovo, China Mobile |  |  |  |  |  |  |  |  |  |
| Work item code: | NSCALE |  |  |  |  |  | Date: |  |  | 2024-06-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 24.549 is not following principles and guidelines for services definition according to TS 29.501. As the result some important feature are missing. In addition the attribute for "configuration cause" does not exist in stage 2. Please see clause 16 in 3GPP TS 23.343, entitled "Network slice capability enablement". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Restruring API definitions according to TS 29.501by removing clause 6 and creating a new clause 7. Removal of Online and Offline behavior. Removal of CoAP details. Removal of "configuration cause". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent stage 3 remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6, 7.2 (new), 7.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was agreed as document C1-242814 in CT1#148. However, clause 7.1 and clause 7.4 are overlapping with later agreed CR#0029 for 3GPP TS 24.549 with Tdoc number C1-243763. Therefore clause 7.1 and clause 7.4 should be removed from this CR. In addition, in clause 3.2 abbreviation ETN is placed after DNN to be in the right alphabetical order. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GCN	5G Core Network

AF	Application Function

DNN	Data Network Name

ETN	Event Triggered Network

HTTP	Hypertext Transfer Protocol

KQI	Key Quality Indicator

NSCE	Network Slice Capability Enablement

PCF	Policy Control Function

QoE	Quality of Experience

SEAL	Service Enabler Architecture Layer

SNSCE-C	SEAL Network Slice Capability Enablement Client

SNSCE-S	SEAL Network Slice Capability Enablement Server

S-NSSAI	Single Network Slice Selection Assistance Information

UE	User Equipment

URSP	UE Route Selection Policy

VAL	Vertical Application Layer

XCAP	XML Configuration Access Protocol

XDMC	XML Document Management Client

XDMS	XML Document Management Server

XML	Extensible Markup Language

* * * Next Change * * * *

# 6	Void

* * * Next Change * * * *

# 7	Network slice capability enablement services

* * * Next Change * * * *

## 7.2	Network slice adaptation service

### 7.2.1	Service description

#### 7.2.1.1	Overview

The network slice adaptation procedure is a SEAL service of providing network slice capability enablement capabilities for network slice re-mapping from one VAL service to one or more other VAL services, according to 3GPP TS 23.434 [2] and 3GPP TS 23.435 [13]. The network server entity, providing the functionality for the network slice re-mapping, acts as an AF communicating with 5GCN to provide guidance to update and modify the S-NSSAIs and the DNNs of the route selection descriptors of the URSP rules, 3GPP TS 24.526 [3], for one or more application traffics per UE.

NOTE:	In this release, S-NSSAI and DNN are only used as the route selection descriptor.

### 7.2.2	Service operations

#### 7.2.2.1	Introduction

The service operation, defined for ETN_Configuration API for network capability configuration, is shown in table 7.2.2.1-1.

Table 7.2.2.1-1: Operations for network capability configuration

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Event_Triggered_Network_Slice_Adaptation | This service operation is used by SNSCE-C to trigger the event of the network slice configuration. | SNSCE-C |

#### 7.2.2.2	Event_Triggered_Network_Slice_Adaptation

##### 7.2.2.2.1	General

These clauses describe the procedures on the SNSCE-C and SNSCE-S side when an event triggered request for network slice configuration is sent by the SNSCE-C to the SNSCE-S. The event triggered network slice configuration request causes a network slice adaptation and sent by the SNSCE-C acting as application client requesting a new or a change in network slice configuration.

##### 7.2.2.2.2	Network slice adaptation using Event_Triggered_Network_Slice_Adaptation service operation

To request for the network slice adaptation, the SNSCE-C shall send an HTTP PUT request message according to procedures specified in IETF RFC 9110 [8]. In the HTTP PUT request message, the SNSCE-C:

NOTE 1:	How the requested network slice is known by the SNSCE-C is out of scope of this release.

a)	shall set the Request-URI to the URI identifying the SNSCE-S according to the pattern"{apiRoot}/su_nsc/val-services/{valServiceId}/configurations/{configurationId}", where:

1)	{valServiceId} set to the identity of the VAL application; and

2)	{configurationId}set to the identity of slice adaptation configuration,

b)	shall set the "Host" header field to the URI identifying of SNSCE-S and the port information;

c)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [7]; and

d)	shall include a body containing the data type NwSliceAdptEvent as defined in clause 8.1.1.6.2.2,

Upon receipt an HTTP PUT request:

a)	with a Request-URI according to "{apiRoot}/su_nsc/val-services/{valServiceId}/configurations/{configurationId} identifying:

1)	"valServiceId" identifying the VAL application; and

2)	"configurationId" identifying the slice adaptation configuration; and

b)	with a body containing the data type NwSliceAdptEvent as defined in clause 8.1.1.6.2.2, the SNSCE-S shall determine the sender identity of the sender is authorized or not as specified in 3GPP TS 24.547 [4].

If:

a)	the sender is not an authorized user, the SNSCE-S shall respond with an HTTP 403 (Forbidden) response message and avoid the rest of steps; or

b)	the sender is an authorized user, the SNSCE-S:

1)	shall attempt to update the network S-NSSAI for one or more VAL UEs with the identities listed in the VAL UE list for the VAL service, identified by VAL service ID by using the parameters for requested S-NSSAI, requested DNN, and requested application requirements from the HTTP PUT request message;

NOTE 2:	To update the application traffic, the SNSCE-S can act as an AF and use the reference point N33 as shown in 3GPP TS 23.434 [2] to influence a VAL UE's URSP rules for the application traffic by providing a guidance on the route selection parameters S-NSSAI and DNN as described in clause 4.15.6.10 of 3GPP TS 23.502 [2A].

NOTE 3:	Whether and how the SNSCE-S can update the network S-NSSAI for all VAL UEs for the VAL service, is out of the scope of this release.

2)	shall send the updated network S-NSSAI and any DNN to the PCF, if the update is successful, 3GPP TS 23.434 [2]; and

3)	shall send:

i)	if the request is successfully processed, an HTTP 204 No Content response message indicating the successful status; or

ii)	if errors occur when processing the request, request, an appropriate error response as specified in clause 8.1.1.7.

* * * Next Change * * * *

## 7.3	Retrieval of data and information

### 7.3.1	Service description

#### 7.3.1.1	Overview

The network slice capability enablement procedures is a SEAL service of providing slice capabilities based on 5GS management system services and 5GS network services, according to 3GPP TS 23.435 [13] e.g., retrieving the KQI data of services, the QoE data, the end user information and fault reports from NSCE client, notifying the slice modification and delivering slice information to NSCE client.

The procedures on how the NSCE server retrieves network and service related KQI or performance data, QoE data, and fault information from the NSCE client apply for the following NSCE procedures:

a)	network slice related performance and analytics monitoring job creation request procedure specified in 3GPP TS 23.435 [13] clause 9.7.2.1;

b)	information collection from NSCE server(s) subscribe request and response procedure specified in 3GPP TS 23.435 [13] clause 9.8.2.1;

c)	network slice fault management capability exposure procedure specified in 3GPP TS 23.435 [13] clause 9.15.2.1; and

d)	slice requirements verification and alignment capability exposure procedure specified in 3GPP TS 23.435 [13] clause 9.16.2.1.

The procedures at the SNSCE-Cand SNSCE-S side follow the mechanism specified in clause 5.5 of 3GPP TS 26.531 [15] and HTTP procedures specified in clause 4.3 and clause 7 of 3GPP TS 26.532 [16]. In the procedures, the SNSCE-C acts as the data collection client, and the SNSCE-S acts as data collection AF.

### 7.3.2	Service operations

#### 7.3.2.1	Introduction

The service operations, defined for the APIs of data collection and reporting service specified in 3GPP TS 26.532 [16], for retrieval of data and information, is shown in table 7.3.2.1-1.

Table 7.3.2.1-1: Operations for retrieval of data and information

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Ndcaf_DataReporting_CreateSession | This service operation is used by SNSCE-C to obtain the configuration the requested data and information for retrieval. | SNSCE-C |
| Ndcaf_DataReporting_RetrieveSession | This service operation is used by SNSCE-C to update the configuration of the requested data and information for retrieval. | SNSCE-C |
| Ndcaf_DataReporting_Report | This service operation is used by SNSCE-C to report the requested data and information for retrieval. | SNSCE-C |

#### 7.3.2.2	Ndcaf_DataReporting_CreateSession

##### 7.3.2.2.1	General

These clauses describe the procedures on the SNSCE-C and SNSCE-S side when a request for obtaining the configuration of the requested data and information for retrieval, is sent by the SNSCE-C to the SNSCE-S.

##### 7.3.2.2.2	Configuration of the requested data and information retrieval using Ndcaf_DataReporting_CreateSession service operation

In order to obtain the configuration of requested data and information for retrieval, the SNSCE-C shall send an HTTP POST request message to invoke Ndcaf_DataReporting_CreateSession service operation as described in clause 3.3.2.2 and clause 7.2.2.3.1 of 3GPP TS 26.532 [16].

Upon receipt an HTTP POST request message on Ndcaf_DataReporting_CreateSession service operation, the SNSCE-S shall send HTTP "201 Created" status code and provide the configuration of requested data and information for retrieval as described in clause 4.3.2.2 and clause 7.2.2.3.1 of 3GPP TS 26.532 [16].

#### 7.3.2.3	Ndcaf_DataReporting_RetrieveSession

##### 7.3.2.3.1	General

These clauses describe the procedures on the SNSCE-C and SNSCE-S side when a request for updating the configuration of the requested data and information for retrieval, is sent by the SNSCE-C to the SNSCE-S.

##### 7.3.2.3.2	Updated configuration of the requested data and information retrieval using Ndcaf_DataReporting_RetrieveSession service operation

In order to update the configuration of requested data and information for retrieval, the SNSCE-C may send an HTTP GET request message to invoke Ndcaf_DataReporting_RetrieveSession service operation as described in clause 4.3.2.3 and clause 7.2.3.3.1 of 3GPP TS 26.532 [16].

Upon receipt an HTTP GET request message on Ndcaf_DataReporting_RetrieveSession service operation, the SNSCE-S shall send HTTP "201 Created" status code and provide the updated configuration, if available, as described in clause 4.3.2.3 and clause 7.2.3.3.1 of 3GPP TS 26.532 [16].

#### 7.3.2.4	Ndcaf_DataReporting_Report

##### 7.3.2.4.1	General

These clauses describe the procedures on the SNSCE-C and SNSCE-S side when a request for reporting the configuration of the requested data and information for retrieval, is sent by the SNSCE-C to the SNSCE-S.

##### 7.3.2.4.2	Reporting the requested data and information retrieval using Ndcaf_DataReporting_Report service operation

After the configuration, the SNSCE-C shall send an HTTP POST request message in accordance with this configuration to invoke Ndcaf_DataReporting_Report service operation as described in clause 4.3.3 and clause 7.2.3.4.1 of 3GPP TS 26.532 [16].

Upon receipt an HTTP POST request message on Ndcaf_DataReporting_Report service operation, the SNSCE-S shall send HTTP "204 No Content" status code and may provide the updated configuration as described in clause 4.3.3 and clause 7.2.3.4.1 of 3GPP TS 26.532 [16].

* * * End of Changes * * * *