3GPP TSG-CT Meeting #104	CP-241237

# Shanghai, P.R. China; 17th – 18th June 2024										was C1-243039

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.549 | CR | 0022 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HTTP resource representation and encoding for network slice configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Work item code: | NSCALE |  |  |  |  |  | Date: |  |  | 2024-06-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 24.549 is not following principles and guidelines for services definition according to TS 29.501. As the result some important feature are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Restruring API definitions according to TS 29.501 by: - Removing contet of annex A; and - Adding new clause 8 and 9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent stage 3 remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8(new), 9 (new), A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was agreed as document C1-242793 in CT1#148, however it had to be adjusted by removing the added reference to 3GPP TS 29.571 since that reference already exists in the spec. This CR was agreed document C1-243039 in CT1#149, however this revision introduces name change of reference 22 from “openAPI” to “openAPI Specification” to be consistent with the text in C.1 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[1A]	3GPP TR 21.900: "Technical Specification Group working methods".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[2A]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[3]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[3A]	3GPP TS 24.546: "Configuration management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[4]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[5]	Void.

[6]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[7]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[8]	IETF RFC 9110:"HTTP Semantics".

[8A]	IETF RFC 9111: "HTTP Caching".

[8B]	IETF RFC 9112: "HTTP/1.1".

[8C]	IETF RFC 9113: "HTTP/2".

[9]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[10]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[11]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[12]	OMA OMA-TS-XDM_Core-V2_1-20120403-A: "XML Document Management (XDM) Specification".

[13]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[14]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[15]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[16]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[X]	3GPP TS 29.122: " T8 reference point for Northbound Application Programming Interfaces (APIs)".

[XX]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[Y]	3GPP TS 29.549:" Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification".

[W]	3GPP TS 33.434: "Service Enabler Architecture Layer for Verticals (SEAL); Security Aspects".

[V]	Open API Specification: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

* * * Next Change * * * *

# 8	API Definitions

## 8.1	Event triggered network slice configuration APIs

## 8.1.1	ETN_Configuration API

### 8.1.1.1	Introduction

The information in this clause provides a description for the HTTP parameters transmitted by the SNSCE-C to the SNSCE-S to trigger a network slice configuration such as the network slice adaptation for one or more VAL UEs within a VAL service.

The HTTP URIs used in HTTP protocol for the event triggered network (ETN) slice configuration service shall have the resource URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [X]:

- {apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

where:

a)	{apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [X];

b)	<apiName> shall be "su_nsc";

c)	<apiVersion> shall be "v1"; and

d)	<apiSpecificSuffixes> shall be set as described in clause 8.1.1.3.

#### 8.1.1.2	Usage of HTTP

##### 8.1.1.2.1	General

For SNSCE service configuration API, support of HTTP/1.1 (IETF RFC 9110 [8], IETF RFC 9111 [8A] and IETF RFC 9112 [8B]) over TLS is mandatory and support of HTTP/2 (IETF RFC 9113 [8C]) over TLS is recommended.

A functional entity desiring to use HTTP/2 shall use the HTTP upgrade mechanism to negotiate applicable HTTP version as described in IETF RFC 9113 [8C].

##### 8.1.1.2.2	Content type

The bodies of HTTP request and successful HTTP responses shall be encoded in JSON format (see IETF RFC 8259 [10]).

The MIME media type that shall be used within the related Content-Type header field is "application/json", as defined in IETF RFC 8259 [10].

#### 8.1.1.3	Resources

##### 8.1.1.3.1	Overview

The Resource URI structure of the ETN_Configuration API is as shown in Figure 8.1.1.3.1-1:

Figure 8.1.1.3.1-1: Resource URI structure of the ETN_Configuration API

Table 8.1.1.3.1-1 provides an overview of the resources and applicable HTTP method.

Table 8.1.1.3.1-1: Resources and method overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Configuration | /val-services/{valServiceId}/configurations/{configurationId} | PUT (NOTE) | Performs configuration. |
| NOTE: In this release, the only configuration is the slice adaptation as described in 3GPP TS 23.434 [2]. |  |  |  |

##### 8.1.1.3.2	Resource: Configuration

###### 8.1.1.3.2.1	Description

The Configuration resource allows an SNSCE-C a specific configuration identified by the identity "configurationId".

###### 8.1.1.3.2.2	Resource definition

Resource URI: {apiRoot}/su_nsc/<apiVersion>/val-services/{valServiceId}/configurations/{configurationId}

This resource shall support the resource URI variables defined in the table 8.1.1.3.2.2-1.

Table 8.1.1.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 of 3GPP TS 29.122 [X]. |
| apiVersion | string | See clause 5.2.4 of 3GPP TS 29.122 [X]. |
| valServiceId | string | Identifier of a VAL service. |
| configurationId | string | Identifier of a configuration |

###### 8.1.1.3.2.3	Resource standard methods

8.1.1.3.2.3.1	PUT

This operation is for triggering network slice adaptation event and shall support the URI query parameters specified in table 8.1.1.3.2.3.1-1.

Table 8.1.1.3.2.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 8.1.1.3.2.3.1-2and the response data structures and response codes specified in table 8.1.1.3.2.3.1-3.

Table 8.1.1.3.2.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NwSliceAdptEvent | M | 1 | Triggering the network slice adaptation event. |

Table 8.1.1.3.2.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a | M | 1 | 204 No Content | The configuration of the VAL UEs with VAL UE List within the VAL service identified by the value "valServiceId" and for the network slice configuration identified by the value "configurationId", was successful. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing an alternative SNSCE-S to which the request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [X]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing an alternative SNSCE-S to which the request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [X]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [X] shall also apply. |  |  |  |  |

Table 8.1.1.3.2.3.1-3: Headers supported by 307 Response Code for this operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing an alternative NSCE server to which the request should be redirected. |

Table 8.1.1.3.2.3.1-4: Headers supported by 308 Response Code for this operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing an alternative NSCE server to which the request should be redirected. |

###### 8.1.1.3.2.4	Resource Custom Operations

None.

### 8.1.1.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

#### 8.1.1.5	Notifications

None.

#### 8.1.1.6	Data model

##### 8.1.1.6.1	General

This clause specifies the application data model supported by the API.

Table 8.1.1.6.1-1 specifies the data types defined for the ETN_Configuration API.

Table 8.1.1.6.1-1: ETN_Configuration API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| NwSliceAdptEvent | 8.1.1.6.2.2 | Event triggered network slice adaptation |  |

Table 8.1.1.6.1-2 specifies data types re-used by the ETN_Configuration API service.

Table 8.1.1.6.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Dnn | 3GPP TS 29.571 [14] | Used to Identify a DNN. |  |
| DurationSec | 3GPP TS 29.122 [X] | Represents a period of time in units of seconds. |  |
| LocationArea | 3GPP TS 29.122 [X] | Represents location information. |  |
| RatType | 3GPP TS 29.571 [14] | Identifies the RAT Type. |  |
| Snssai | 3GPP TS 29.571 [14] | Used to Identify the S-NSSAI. |  |
| ValTargetUe | 3GPP TS 29.549 [Y] | Used to indicate either VAL User ID or VAL UE ID. |  |

##### 8.1.1.6.2	Structured data types

###### 8.1.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 8.1.1.6.2.2	Type: NwSliceAdptEvent

Table 8.1.1.6.2.2-1: Definition of type NwSliceAdptEvent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUeIds | array(ValTargetUe) | M | 1..N | One or more VAL UEs, for which a given event triggered network slice configuration applies. The VAL service is identified by the value "valServiceId" and the network slice configuration is identified by the value "configurationId". |  |
| sliceId | Snssai | M | 1 | The identifier of the slice or slice instance, to which the event triggered network slice adaptation is applied. |  |
| dnn | Dnn | O | 0..1 | Requested DNN |  |
| appReqs | AppReqs | O | 0..1 | Requirement related to the requested application |  |

###### 8.1.1.6.2.3	Type: AppReqs

Table 8.1.1.6.2.3-1: Definition of type AppReqs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timeIntervals | array(DurationSec) | O | 1..N | The requested time intervals as the start time and end time. |  |
| area | LocationArea | O | 0..1 | The requested geographical or service area. |  |
| ratType | RatType | O | 0..1 | The requested access type. |  |
| preservIpAdd | boolean | O | 0..1 | Indication to preserve the UE IP address. If it is not requested, it is included as set to "false". Otherwise, the UE IP address is preserved. |  |

Editor’s note [CR#0017, WID: NSCALE]:	Whether to contain the UE IP address preservation indicator depends on the clarification from SA6.

##### 8.1.1.6.3	Simple data types and enumerations

None.

#### 8.1.1.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 8.1.1.6.5	Binary data

##### 8.1.1.6.5.1	Binary Data Types

Table 8.1.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 8.1.1.7	Error Handling

##### 8.1.1.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [X].

In addition, the requirements in the following clauses shall apply.

##### 8.1.1.7.2	Protocol Errors

In this release of the specification, there are no additional protocol errors applicable for the ETN_Configuration API.

##### 8.1.1.7.3	Application Errors

The application errors defined for ETN_Configuration API are listed in table 8.1.1.7.3-1.

Table 8.1.1.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 8.1.1.8	Feature Negotiation

General feature negotiation procedures are defined in clause 5.2.7 of 3GPP TS 29.122 [X]. Table 8.1.1.8-1 lists the supported features for ETN_ServiceConfiguration API.

Table 8.1.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 8.1.1.9	Security

#### 8.1.1.9.1	General

Usage of HTTP over TLS and the TLS profiles shall be as specified in clause 5.1.1.4 of 3GPP TS 33.434 [W].

## 8.2	Data collection and reporting APIs

## 8.2.1	Ndcaf_DataReporting API

3GPP TS 26.532 [16] specifies Ndcaf_DataReporting API for Ndcaf_DataReporting service which includes:

-	Ndcaf_DataReporting_CreateSession service operation;

-	Ndcaf_DataReporting_RetrieveSession service operation; and

-	Ndcaf_DataReporting_Report service operation.

* * * Next Change * * * *

### 9	Usage of common API framework

#### 9.1	General

Usage of common API framework shall be supported by the event triggered network slice configuration service API as described in clause 8 in 3GPP TS 29.549 [Y].

* * * Next Change * * * *

Annex A (normative):
 Void

* * * End of Changes * * * *