3GPP TSG-CT Meeting #105	CP-242217

# Melbourne, Australia; 9th – 10th September 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6669 | rev | 6 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of test eCall with CEN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Qualcomm Incorporated, Vodafone, Deutsche Telekom, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Qualcomm Incorporated, Vodafone, Deutsche Telekom, OPPO |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN, IMSProtoc19 |  |  |  |  |  | Date: |  |  | 2024-08-29 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CEN/TC 278/WG 15 sent an LS to SA#104 (in SP-240522) and CT#104 (in CP-241245) on Next Generation eCall. The LS asks 3GPP to remove the differences for cases 1 to 5 by aligning with CEN requirements in CEN/TS 17184. This CR addresses alignment for case 1 for test eCall. While a test eCall is not an emergency call (i.e. not an eCall over IMS), most signalling requirements for an eCall over IMS remain applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for test eCall over IMS using existing regular IMS call establishment and transfer of updated MSD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCall over IMS will be less aligned with CEN requirements which may result in poorer eCall service and/or extra problems in compliance with EC directives for the EU which reference CEN and not ETSI specifications in the case of next generation eCall. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision X (CT#105): Resubmission to TSG-CT Meeting #105 of the CR endorsed in TSG-CT WG1 Meeting #150 without any technical changes |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

### 5.1.6A	Test eCall

This clause specifies a feature which is subject to national regulations in CEN EN 17184:2024 [xx]. If the upper layers request establishment of an IMS call of the test eCall type while in limited service state, the request shall be rejected. Otherwise, a CS and IM CN subsystem capable UE shall follow the conventions and rules specified in 3GPP TS 22.101 [1A] and TS 23.167 [4B] to select the same domain as for an emergency eCall attempt, with the exception of not selecting PS domain when VoIMS=N. If the CS domain is selected, the UE shall attempt a test eCall setup as a non-emergency services call using appropriate access technology specific procedures.

If the IM CN subsystem is selected and the UE is not currently registered, the UE shall perform an initial registration, as described in subclause 5.1.1.2. The UE shall then apply the procedures as specified in subclause 5.1.2A and 5.1.3 to send an INVITE with the following additions:

1)	the UE shall set the Request-URI to non-emergency service URN (e.g. SIP, Service, Tel URIs) that is defined by a national regulator, PSAP or eCall test provider for testing purposes;

NOTE 1:	The Request-URI can be set to "urn:service:test.sos.ecall" as specified in RFC 8147 [244] or the URI can be configured in USIM files; see 3GPP TS 31.102 [15C].

2)	the UE shall insert in the INVITE request, a To header field with the same service URN as in the Request-URI;

3)	the UE shall insert in the INVITE request, a From header field that includes the public user identity or the tel URI associated with the public user identity, as described in subclause 4.2; and

4)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244]; and

NOTE 2:	Further content for the INVITE such as set the test bit in the MSD is defined in CEN EN 15722:2015 [245] and others is defined in RFC 8147 [244].

Then the UE shall proceed as follows:

1)	if the UE receives a 200 (OK) response to the INVITE request not containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to either "true" or "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall send the MSD using audio media stream encoded as described in 3GPP TS 26.267 [9C];

2)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful;

3)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as unsuccessful;

4)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall report the response and successful MSD transfer to upper layers; and

5)	in all other cases, the UE shall perform domain selection as specified in 3GPP TS 23.167 [4B], with the exception of not selecting PS domain when VoIMS=N and shall re-attempt the test eCall if a new domain or new RAT is selected.

While a test eCall remains established and if the IP-CAN indicates the eCall support indication, a UE shall support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

NOTE 3:	For the updated MSD the test bit is set in the MSD according to CEN EN 15722:2015 [245].

* * * End of Change(s) * * *