3GPP TSG-CT WG1 Meeting #91 	C1-151656

# Bratislava (Slovakia), 13-17 April 2015                                         Revision of C1-151643

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.302 | CR | 0420 | rev | 3 | Current version: | 12.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Priority defined in preferredSSIDlist |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | WLAN_NS-CT |  |  |  |  |  |  | Date: |  | 2015-03-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | During the last SA2#107 meeting, SA2 agreed to add priority in the PreferredSSIDlist, quote from TS 23.402 sub-clause 4.8.2.1.6: b) Additional attributes: - PreferredSSIDList: A prioritized list of SSIDs preferred for selection. Therefore, the related stage 3 description needs to be changed to be alignment with stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the priority to the preferredSSIDlist. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with the stage 2 definition. This results in problems as the use of PreferredSSIDList becomes inefficient and the WLANSP rules unnecessarily increase in size. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

# 5.1.3.2.3.3	Automatic mode WLAN selection

If the ANDSF rules control the WLAN access selection and traffic routing as described in subclause 6.10.2, then the selected WLAN(s) are WLAN(s) that fulfil the selection criteria with the highest priority configured in the active ANDSF WLANSP rule.

If the RAN rules control the WLAN access selection and traffic routing as described in subclause 6.10.2, then the selected WLAN(s) are WLAN(s) matching WLAN identifiers in an entry of the list of the WLAN identifiers received along with the move-traffic-to-WLAN indication as described in subclause 6.10.4.

The UE determines the selected WLAN(s) according to the following steps:

1)	the UE shall construct prioritized list of available WLANs as follows:

a)	if the ANDSF rules control the WLAN access selection and traffic routing as described in subclause 6.10.2, the UE shall use the procedures specified in IEEE 802.11-2012 [57] to discover the available WLANs. The UE may perform ANQP procedures as specified in IEEE 802.11-2012 [57] to discover the attributes and capabilities of available WLANs. The UE shall compare the attributes and capabilities of the available WLANs with the highest priority selection criterion that has not been used yet in the active WLANSP rule, and construct a prioritized list of available WLANs that fulfil the selection criteria. If there are multiple highest priority selection criteria, it is up to the UE implementation which one to use. In particular, if:

-	the group of selection criteria include the HomeNetworkIndication and it is set to "1" (see 3GPP TS 24.312 [13]); and

-	the HomeNetworkPreference:

i)	does not include 3GPP_RPLMN_Preferred; or

ii)	includes 3GPP_RPLMN_Preferred and it is set to "0" (see 3GPP TS 24.312 [13]);

then a WLAN is included, if:

-	the other selection criteria in the active WLANSP rule are met; and

-	the domain name list (see IEEE 802.11-2012 [57]) includes:

i)	the home domain name derived from its IMSI; or

ii)	any realm in the EquivalentHomeSPs as specified in 3GPP TS 24.312 [13].

The priority of a WLAN in the list is set to the WLAN priority defined in the preferredSSIDlist of the matching selection criteria.. There may be one or more selected WLANs in the list; and

b)	if the RAN rules control the WLAN access selection and traffic routing as described in subclause 6.10.2, the UE shall use the procedures specified in IEEE 802.11-2012 [57] to discover available WLANs. The UE shall construct a prioritized list of available WLANs and populate it with each discovered WLAN which matches all WLAN identifiers included in an entry of the list of the WLAN identifiers received along with the move-traffic-to-WLAN indication as described in subclause 6.10.4. The priority of a discovered WLAN in the prioritized list of available WLANs is decided by the UE in an implementation specific way;

2)	if the ANDSF rules control the WLAN access selection and traffic routing as described in subclause 6.10.2, and if the following conditions are fulfilled:

-	the UE supports the PDN connection establishment over WLAN using the applicable S2a procedures specified in 3GPP TS 23.402 [6];

-	the "S2a connection preference" indicator exists and indicates that PDN connection establishment over WLAN using the applicable S2a procedures specified in 3GPP TS 23.402 [6] is preferred; and

-	one or more WLANs in the list constructed in step 1) support S2a connectivity;

then the UE considers the WLANs that have the highest priority and indicate the HPLMN or RPLMN in the PLMN list with S2a connectivity IE (see annex H) as the selected WLAN(s).

Otherwise, the UE considers the WLAN(s) that has or have the highest priority as the selected WLAN(s). And

NOTE 1:	WLAN advertises PLMN(s) towards which the S2a connectivity is supported using ANQP-element "3GPP Cellular Network" with the PLMN List with S2a Connectivity IE in the payload, according to annex H.

NOTE 2:	Advertising S2a connectivity over a WLAN using EAP signalling is not supported in this version of the specification.

3)	if the ANDSF rules control the WLAN access selection and traffic routing as described in subclause 6.10.2, if there are no WLAN(s selected in step 2), the UE may repeat the procedure from step 1) taking into consideration selection criteria with lower priority from the active WLANSP rule.

NOTE 3:	UE implementation can optimize the steps described above, e.g. by combining the ANQP procedures.

***** Next change *****