3GPP TSG-CT WG1 Meeting #92 	C1-152471

# Sanya, P.R of China, 25-29 May 2015                                                      (was C1-152082)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 2153 | rev | 2 | Current version: | 12.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE back-off timer for EPS session management rejections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Samsung Telecommunications, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SINE |  |  |  |  |  |  | Date: |  | 2015-05-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In CT1#90 meeting, UE data retry handling for ESM procedures have been agreed. It is specified that the network may include a back-off timer value in a session management reject message to regulate the time interval at which the UE may retry the same procedure. It is also specifed that this back-off timer is used to describe a logical model of the required UE behaviour and this model does not imply any specific implementation, e.g. as a timer or timestamp. Then a subsequent note is used in Section 6.5.1.4.3, 6.5.3.4.3, 6.5.4.4.3 and 10.3 to provide hint on possible implementation of this back-off timer that it can either refer to use of timer T3396 or a different packet system specific timer within the UE: NOTE 4: Reference to back-off timer in this section can either refer to use of timer T3396 or to use of a different packet system specific timer within the UE. So, the UE can re-use T3396 as a back-off timer or it can use different packet system specific timers as back-off timers. This is left up to UE implementation. This description is somehow not very accurate and could lead to misinterpretation by the implementors. In current specification, T3396 is specified and used as packet system independent timer and have congestion control related timer stopping conditions. So when the text simply says T3396 can be reused, it could be interpreted by the implementors as no design changes are needed for T3396 to be used as UE retry back-off timer, which could lead to mis-implementation and potentially different UE behavior. It needs to be clarified that if T3396 is to be used as retry back-off timer, then existing T3396 cannot be reused, i.e. extension to T3396 would be required in order to satisfy back-off timer requirements of managing packet system specific instances and different timer stopping conditions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the note in Section 6.1.3.1.3.3, 6.1.3.2.2.3, 6.1.3.3.3.3 and 11.2.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description causing mis-interpretation and inconsistent implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1.4.3, 6.5.3.4.3, 6.5.4.4.3, 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

##### 6.5.1.4.3	Handling of network rejection due to ESM cause other than cause #26

If the ESM cause value is not #26 "insufficient resources", the UE shall behave as follows:

1)	if the PDN CONNECTIVITY REQUEST message was sent standalone, the UE shall take different actions depending on the timer value received in Back-off timer value IE:

i)	if the timer value indicates neither zero nor deactivated, the UE shall start back-off timer with the value provided in the Back-off timer value IE for PDN connectivity request for APN and PLMN combination and not send another PDN CONNECTIVITY REQUEST message for the same APN until back-off timer expires, the UE is switched off or the USIM is removed;

ii)	if the timer value indicates that this timer is deactivated, the UE shall not send another PDN CONNECTIVITY REQUEST message for the same APN until the UE is switched off or the USIM is removed; and

iii)	if the timer value indicates zero, the UE may send another PDN CONNECTIVITY REQUEST message for the same APN; and

2)	if the PDN CONNECTIVITY REQUEST message was sent together with an ATTACH REQUEST, the UE shall take different actions depending on the timer value received for back-off timer and integrity protection of the ATTACH REJECT message:

i)	if the ATTACH REJECT message is not integrity protected, the UE shall start back-off timer with a random value from a default range specified in table 11.2.3(see 3GPP TS 24.008 [13]), and:

a)	shall not initiate a new attach procedure with the same APN that was sent by the UE, until back-off timer expires, the UE is switched off or the USIM is removed; and

b)	shall not initiate a new attach procedure without an APN if the UE did not provide any APN during the attach procedure, until back-off timer expires, the UE is switched off or the USIM is removed; and

ii)	if the ATTACH REJECT message is integrity protected, the UE shall proceed as follows:

a)	if the timer value indicates neither zero nor deactivated, the UE shall start back-off timer with the value provided in the Back-off timer value IE for PDN connectivity request for APN and PLMN combination and:

-	shall not initiate a new attach procedure with the same APN that was sent by the UE, until back-off timer expires, the UE is switched off or the USIM is removed; and

-	shall not initiate a new attach procedure without an APN if the UE did not provide any APN during the attach procedure, until back-off timer expires, the UE is switched off or the USIM is removed;

b)	if the timer value indicates that this timer is deactivated, the UE:

-	shall not initiate a new attach procedure with the same APN that was sent by the UE, until the UE is switched off or the USIM is removed; and

-	shall not initiate a new attach procedure without an APN if the UE did not provide any APN during the attach procedure, until the UE is switched off or the USIM is removed; and

c)	if the timer value indicates that this timer is zero, the UE shall proceed as specified in subclause 5.5.1.2.6 item d.

If the Back-off timer value IE is not included and PDN CONNECTIVITY REQUEST was sent standalone, then the MS shall ignore the Re-attempt indicator IE provided by the network, if any and proceed as follows depending on the received ESM cause value:

-	if the UE is registered in the HPLMN or in a PLMN that is within the EHPLMN list, the ESM cause value is #8 "operator determined barring", #27 "missing or unknown APN", #32 "service option not supported" or #33 "requested service option not subscribed", the UE shall behave as described in the conditions listed above using the configured SM Retry Timer value for back-off timer as specified in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [112] if it is available. Otherwise, if the UE is not registered in its HPLMN or a PLMN that is within the EHPLMN list or the SM Retry Timer value is not configured, for ESM cause values #8 "operator determined barring", #27 "missing or unknown APN", #32 "service option not supported" and #33 "requested service option not subscribed", it shall behave as described in the conditions listed above using the default value of 12 minutes for the back-off timer.

# The UE shall not stop any back-off timer upon a PLMN change or inter-system change.

If the Back-off timer value IE is not included and PDN CONNECTIVITY REQUEST was sent together with an ATTACH REQUEST, the UE shall proceed as specified in subclause 5.5.1.2.6, item d.

If the back-off timer is started upon receipt of PDN CONNECTIVITY REJECT (i.e. timer value was provided by the network, configuration value is available or default value is used for back-off timer as explained above), and the ESM cause value is not #26 "insufficient resources", the UE behaves as follows:

-	the UE may re-attempt PDN CONNECTIVITY REQUEST message after selecting a new PLMN but the UE shall keep the back-off timer running for the previous PLMN(s).

-	the UE shall not send another PDN CONNECTIVITY REQUEST message for the same APN in the PLMN in which it was rejected until the timer expires, UE is switched off or the USIM is removed.

NOTE 1:	The above bullet is applicable to the case when the UE selects a new PLMN but the back-off timer is already running. This can happen if the UE selects a PLMN in which it was rejected previously for the same APN with a back-off timer.

-	if the network indicates that re-attempt after inter-system change is allowed, the UE is allowed to send a ACTIVATE PDP CONTEXT REQUEST message for the same APN after inter-system change to Iu or A/Gb mode but the UE shall keep the back-off timer running in the S1 mode;

-	the UE shall not send another PDN CONNECTIVITY REQUEST message for the same APN in the S1 mode until the timer expires, UE is switched off or the USIM is removed;

NOTE 2:	The above bullet applies for the case when the UE performs inter-system change again from Iu or A/Gb mode in GERAN/UTRAN back to S1 mode in E-UTRAN and the back-off timer for the same APN is already running.

-	if the network indicates that re-attempt at inter-system change is not allowed, the UE shall not send ACTIVATE PDP CONTEXT REQUEST message for the same APN in Iu or A/Gb mode and the UE shall not send PDN CONNECTIVITY REQUEST message for the same APN in S1 mode until the timer expires, UE is switched off or the USIM is removed; and

-	if the UE is registered in its HPLMN or in a PLMN that is within the EHPLMN list and the network does not include the Re-attempt indicator IE to indicate whether re-attempt at inter-system change is allowed, the UE shall apply the configured value, if it is available, for inter-system change as specified in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [112] to determine whether re-attempt at inter-system change is allowed. If the network does not include the Re-attempt indicator IE and the NAS configuration MO as specified in 3GPP TS 24.368 [15A] is not available and the value for inter-system change is not configured in the USIM file NASCONFIG, then the UE behaviour is unspecified; and

-	If the UE is not registered in its HPLMN or in a PLMN that is within the EHPLMN list and the network does not include the Re-attempt indicator IE then the UE behaviour is unspecified.

NOTE 3:	The back-off timer is used to describe a logical model of the required UE behaviour. This model does not imply any specific implementation, e.g. as a timer or timestamp.

NOTE 4:	Reference to back-off timer in this section can either refer to use of timer T3396 or to use of a different packet system specific timer within the UE. Whether the UE uses T3396 as a back-off timer or it uses different packet system specific timers as back-off timers is left up to UE implementation.

When the timer T3396 or back-off timer is running, the UE is allowed to initiate attach procedure or PDN connectivity procedure if:

-	the UE is a UE configured to use AC11 – 15 in selected PLMN; or

-	the procedure is for emergency bearer services.

If the ESM cause value is #50 "PDN type IPv4 only allowed" or #51 "PDN type IPv6 only allowed", the UE shall not automatically send another PDN CONNECTIVITY REQUEST message for the same APN that was sent by the UE using the same PDN type until:

-	the UE is registered to a new PLMN;

-	the PDN type which is used to access to the APN is changed;

-	the UE is switched off; or

-	the USIM is removed.

Furthermore as an implementation option, for the SM cause values #50 "PDP type IPv4 only allowed" and #51 "PDP type IPv6 only allowed", the UE may decide not to automatically send another PDN CONNECTIVITY REQUEST message for the same APN that was sent by the UE using the same PDN type, if the UE is registered to a new PLMN which is in the list of equivalent PLMNs.

NOTE 5:	Request to send another PDN CONNECTIVITY REQUEST message with a specific PDN type has to come from upper layers.

If the ESM cause value is #65 "maximum number of EPS bearers reached", the UE shall determine the PLMN's maximum number of EPS bearer contexts in S1 mode (see subclause 6.5.0) as the number of active EPS bearer contexts it has.

NOTE 6:	In some situations, when attempting to establish multiple EPS bearer contexts, the number of active EPS bearer contexts that the UE has when ESM cause #65 is received is not equal to the maximum number of EPS bearer contexts reached in the network.

NOTE 7:	When the network supports emergency bearer services, it is not expected that ESM cause #65 is returned by the network when the UE requests a PDN connection for emergency bearer services.

The PLMN's maximum number of EPS bearer contexts in S1 mode applies to the PLMN in which the ESM cause #65 "maximum number of EPS bearers reached" is received. When the UE is switched off or when the USIM is removed, the UE shall clear all previous determinations representing PLMNs maximum number of EPS bearer contexts in S1 mode. When the UE selects a new PLMN, the UE may clear previous determinations representing any PLMN's maximum number(s) of EPS bearer contexts in S1 mode.

If the ESM cause value is #66 "requested APN not supported in current RAT and PLMN combination" and the Back-off timer value IE is not included, the UE shall not send another PDN CONNECTIVITY REQUEST message for the same APN in the current PLMN in S1 mode until the UE is switched off or the USIM is removed. If the ESM cause value is #66 "requested APN not supported in current RAT and PLMN combination" and Back-off timer value IE is included, the UE shall not send another PDN CONNECTIVITY REQUEST for the same APN in the current PLMN or any PLMN in the list of equivalent PLMNs in S1 mode until back-off timer expires, the UE is switched off or the USIM is removed. Furthermore as an implementation option, the UE may decide not to automatically send another PDN CONNECTIVITY REQUEST message for the same APN in a PLMN which is in the list of equivalent PLMNs.

If the ESM cause value is #54 "PDN connection does not exist", the UE shall take different actions as follows:

-	if the PDN CONNECTIVITY REQUEST message was sent standalone, the UE shall set the request type to "initial request" in the subsequent PDN CONNECTIVITY REQUEST message to establish a PDN connectivity to the same APN;

-	if the PDN CONNECTIVITY REQUEST message was sent together with an ATTACH REQUEST message, the UE shall set the request type to "initial request" in the PDN CONNECTIVITY REQUEST message which is included in the subsequent ATTACH REQUEST message to establish a PDN connectivity to the same APN.

NOTE 8:	User interaction is necessary in some cases when the UE cannot re-activate the EPS bearer context(s) automatically.

*** Next Change ***

##### 6.5.3.4.3	Handling of network rejection due to cause other than ESM cause #26

# If the ESM cause value is not #26 "insufficient resources", the UE shall behave as follows.

-	if the back-off timer value indicates neither zero nor deactivated, the UE shall start back-off timer with the value provided in the back-off timer value IE for the BEARER RESOURCE ALLOCATION REQUEST messages for the same APN and PLMN combination and not try to send another BEARER RESOURCE ALLOCATION REQUEST messages in the PLMN for the same APN until back-off timer expires, the UE is switched off or the USIM is removed;

-	if the timer value indicates that this timer is deactivated, the UE shall not send another BEARER RESOURCE ALLOCATION REQUEST messages in the PLMN for the same APN until the UE is switched off or the USIM is removed; or

-	if the timer value indicates zero, the UE may send another BEARER RESOURCE ALLOCATION REQUEST messages in the PLMN for the same APN.

If the back-off timer value IE is not included, then the UE shall ignore the Re-attempt Indicator IE provided by the network, if any, and proceed as follows depending on the received ESM cause value:

-	if the UE is registered in the HPLMN or in a PLMN that is within the EHPLMN list, the ESM cause value is #8 "operator determined barring", #27 "missing or unknown APN", #32 "service option not supported" or #33 "requested service option not subscribed", the UE shall behave as described in the conditions listed above using the configured SM Retry Timer value for back-off timer as specified in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [112] if it is available. Otherwise, if the UE is not registered in its HPLMN or a PLMN that is within the EHPLMN list or the SM Retry Timer value is not configured, for ESM cause values #8 "operator determined barring", #27 "missing or unknown APN", #32 "service option not supported" and #33 "requested service option not subscribed", it shall behave as described in the conditions listed above using the default value of 12 minutes for the back-off timer.

# The MS shall not stop any back-off timer upon a PLMN change or inter-system change.

If the back-off timer is started upon receipt of BEARER RESOURCE ALLOCATION REJECT (e.g. timer value was provided by the network, configuration value is available or default value is used as explained above), and the ESM cause value is not #26 "insufficient resources", the UE behaves as follows:

-	the UE may send a BEARER RESOURCE ALLOCATION REQUEST message after selecting a new PLMN but the UE shall keep the back-off timer running for the previous PLMN(s).

-	the UE shall not send another BEARER RESOURCE ALLOCATION REQUEST message for the same APN in the PLMN in which it was rejected until the timer expires, UE is switched off or the USIM is removed.

NOTE 1:	The above bullet is applicable to the case when the UE selects a new PLMN but the back-off timer is already running. This can happen if the UE selects a PLMN in which it was rejected previously for the same APN with a back-off timer.

-	if the network indicates that re-attempt at inter-system change is allowed, the UE is allowed to send a ACTIVATE SECONDARY PDP CONTEXT REQUEST message for the same APN after inter-system change to Iu or A/Gb mode but the UE shall keep the back-off timer running in the S1 mode;

-	the UE shall not send another BEARER RESOURCE ALLOCATION REQUEST message for the same APN in the S1 mode until the timer expires, UE is switched off or the USIM is removed;

NOTE 2:	The above bullet applies for the case when the UE performs inter-system change again from Iu or A/Gb mode in GERAN/UTRAN back to S1 mode in E-UTRAN and the back-off timer for the same APN is already running.

-	if the network indicates that re-attempt at inter-system change is not allowed, the UE shall not send ACTIVATE SECONDARY PDP CONTEXT REQUEST message for the same APN in Iu or A/Gb mode and BEARER RESOURCE ALLOCATION REQUEST message for the same APN in S1 mode until the timer expires, UE is switched off or the USIM is removed;

-	if the UE is registered in its HPLMN or in a PLMN that is within the EHPLMN list and the network does not include the Re-attempt indicator IE to indicate whether re-attempt at inter-system change is allowed, the UE shall apply the configured value, if it is available, for inter-system change as specified in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [112] to determine whether re-attempt at inter-system change is allowed. If the network does not include the Re-attempt indicator IE and the NAS configuration MO as specified in 3GPP TS 24.368 [15A] is not available and the value for inter-system change is not configured in the USIM file NASCONFIG, then the UE behaviour is unspecified; and

-	If the UE is not registered in its HPLMN or in a PLMN that is within the EHPLMN list and the network does not include the Re-attempt indicator IE then the UE behaviour is unspecified.

NOTE 3:	The back-off timer is used to describe a logical model of the required UE behaviour. This model does not imply any specific implementation, e.g. as a timer or timestamp.

NOTE 4:	Reference to back-off timer in this section can either refer to use of timer T3396 or to use of a different packet system specific timer within the UE. Whether the UE uses T3396 as a back-off timer or it uses different packet system specific timers as back-off timers is left up to UE implementation.

If the ESM cause value is #65 "maximum number of EPS bearers reached", the UE shall determine the PLMN's maximum number of EPS bearer contexts in S1 mode (see subclause 6.5.0) as the number of active EPS bearer contexts it has.

NOTE 5:	In some situations, when attempting to establish multiple EPS bearer contexts, the number of active EPS bearer contexts in the UE when cause #65 is received is not equal to the maximum number of EPS bearer contexts reached in the network.

The PLMN's maximum number of EPS bearer contexts in S1 mode applies to the PLMN in which the ESM cause #65 "maximum number of EPS bearers reached" is received. When the UE is switched off or when the USIM is removed, the UE shall clear all previous determinations representing any PLMN's maximum number of EPS bearer contexts in S1 mode. When the UE selects a new PLMN, the UE may clear previous determinations representing any PLMN's maximum number of EPS bearer contexts in S1 mode.

# The further actions to be performed by the UE are implementation dependent as part of upper layers responsibility.

*** Next Change ***

##### 6.5.4.4.3	Handling of network rejection due to cause other than ESM cause #26

# If the ESM cause value is not #26 "insufficient resources", the UE shall behave as follows.

-	if the the timer value indicates neither zero nor deactivated, the UE shall start back-off timer with the value provided in the back-off timer value IE for the BEARER RESOURCE MODIFICATION REQUEST messages and APN and PLMN combination and not try to send another BEARER RESOURCE MODIFICATION REQUEST messages in the PLMN for the same APN until back-off timer expires, the UE is switched off or the USIM is removed;

-	if the timer value indicates that this timer is deactivated, the UE shall not send another BEARER RESOURCE MODIFICATION REQUEST messages in the PLMN for the same APN until the UE is switched off or the USIM is removed; or

-	if the timer value indicates zero, the UE may send another BEARER RESOURCE MODIFICATION REQUEST messages in the PLMN for the same APN.

If the Back-off timer value IE is not included, then the MS shall ignore the Re-attempt indicator IE provided by the network, if any, and proceed as follows depending on the received ESM cause value:

-	if the UE is registered in the HPLMN or in a PLMN that is within the EHPLMN list, the ESM cause value is #8 "operator determined barring", #27 "missing or unknown APN", #32 "service option not supported" or #33 "requested service option not subscribed", the UE shall behave as described in the conditions listed above using the configured SM Retry Timer value for back-off timer as specified in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [112] if it is available. Otherwise, if the UE is not registered in its HPLMN or a PLMN that is within the EHPLMN list or the SM Retry Timer value is not configured, for ESM cause values #8 "operator determined barring", #27 "missing or unknown APN", #32 "service option not supported" and #33 "requested service option not subscribed", it shall behave as described in the conditions listed above using the default value of 12 minutes for the back-off timer.

# The MS shall not stop any back-off timer upon a PLMN change or inter-system change.

If the back-off timer is started upon receipt of BEARER RESOURCE MODIFICATION REJECT (e.g. timer value was provided by the network, configuration value is available or default value is used as explained above), and the ESM cause value is not #26 "insufficient resources", the UE behaves as follows:

-	the UE may send BEARER RESOURCE MODIFICATION REQUEST message after selecting a new PLMN but the UE shall keep the back-off timer running for the previous PLMN(s).

-	The UE shall not send another BEARER RESOURCE MODIFICATION REQUEST message for the same APN in the PLMN in which it was rejected until the timer expires, UE is switched off or the USIM is removed

NOTE 1:	The above bullet is applicable for the case when the UE selects a new PLMN but the backoff timer is already running. This can happen if the UE selects a PLMN in which it was rejected previously for the same APN with a back-off timer.

-	if the network indicates that re-attempt at inter-system change is allowed, the UE is allowed to send a MODIFY PDP CONTEXT REQUEST message for the same APN after inter-system change to Iu or A/Gb mode but the UE shall keep the back-off timer running in the S1 mode;

-	the UE shall not send another BEARER RESOURCE MODIFICATION REQUEST message for the same APN in the S1 mode until the timer expires, UE is switched off or the USIM is removed;

NOTE 2:	The above bullet applies for the case when the UE performs inter-system change again from Iu or A/Gb mode back to S1 mode and the back-off timer for the same APN is already running.

-	if the network indicates that re-attempt at inter-system change is not allowed, the UE shall not send MODIFY PDP CONTEXT REQUEST message for the same APN in Iu or A/Gb mode and the UE shall not send BEARER RESOURCE MODIFICATION REQUEST message in S1 mode until the timer expires, UE is switched off or the USIM is removed;

-	if the UE is registered in its HPLMN or in a PLMN that is within the EHPLMN list and the network does not include the Re-attempt indicator IE to indicate whether re-attempt at inter-system change is allowed, the UE shall apply the configured value, if it is available, for inter-system change as specified in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [112] to determine whether re-attempt at inter-system change is allowed. If the network does not include the Re-attempt indicator IE and the NAS configuration MO as specified in 3GPP TS 24.368 [15A] is not available and the value for inter-system change is not configured in the USIM file NASCONFIG, then the UE behaviour is unspecified; and

-	If the UE is not registered in its HPLMN or in a PLMN that is within the EHPLMN list and the network does not include the Re-attempt indicator IE then the UE behaviour is unspecified.

NOTE 3:	The back-off timer is used to describe a logical model of the required UE behaviour. This model does not imply any specific implementation, e.g. as a timer or timestamp.

NOTE 4:	Reference to back-off timer in this section can either refer to use of timer T3396 or to use of a different packet system specific timer within the UE. Whether the UE uses T3396 as a back-off timer or it uses different packet system specific timers as back-off timers is left up to UE implementation.

The further actions to be performed by the UE are implementation dependent as part of upper layers responsibility.

*** Next Change ***

## 10.3	Timers of EPS session management

# Table 10.3.1: EPS session management timers – UE side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| T3480 | 8s | PROCEDURE TRANSACTION PENDING | BEARER RESOURCE ALLOCATION REQUEST sent | ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST received or MODIFY EPS BEARER CONTEXT REQUEST received or BEARER RESOURCE ALLOCATION REJECT received | Retransmission of BEARER RESOURCE ALLOCATION REQUEST |
| T3481 | 8s | PROCEDURE TRANSACTION PENDING | BEARER RESOURCE MODIFICATION REQUEST sent | ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST received or MODIFY EPS BEARER CONTEXT REQUEST received or DEACTIVATE EPS BEARER CONTEXT REQUEST received or BEARER RESOURCE MODIFICATION REJECT received | Retransmission of BEARER RESOURCE MODIFICATION REQUEST |
| T3482 | 8s | PROCEDURE TRANSACTION PENDING | An additional PDN connection is requested by the UE which is not combined in attach procedure | ACTIVE DEFAULT EPS BEARER CONTEXT REQUEST received or PDN CONNECTIVITY REJECT received | Retransmission of PDN CONNECTIVITY REQUEST |
| T3492 | 6s | PROCEDURE TRANSACTION PENDING | PDN DISCONNECT REQUEST sent | DEACTIVATE EPS BEARER CONTEXT REQUEST received or PDN DISCONNECT REJECT received | Retransmission of PDN DISCONNECT REQUEST |
| Back-off timer |  |  | defined in 3GPP TS 24.008 [13] |  |  |
| NOTE 1: Typically, the procedures are aborted on the fifth expiry of the relevant timer. Exceptions are described in the corresponding procedure description. |  |  |  |  |  |

NOTE 1:	The back-off timer is used to describe a logical model of the required UE behaviour. This model does not imply any specific implementation, e.g. as a timer or timestamp.

NOTE 2:	Reference to back-off timer in this section can either refer to use of timer T3396 or to use of a different packet system specific timer within the UE. Whether the UE uses T3396 as a back-off timer or it uses different packet system specific timers as back-off timers is left up to UE implementation.

# Table 10.3.2: EPS session management timers – network side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| T3485 | 8s | BEARER CONTEXT ACTIVE PENDING | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST sent ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST sent | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT received or ACTIVATE DEFAULT EPS BEARER CONTEXT REJECT received or ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT received or ACTIVATE DEDICATED EPS BEARER CONTEXT REJECT received | Retransmission of the same message |
| T3486 | 8s | BEARER CONTEXT MODIFY PENDING | MODIFY EPS BEARER CONTEXT REQUEST sent | MODIFY EPS BEARER CONTEXT ACCEPT received or MODIFY EPS BEARER CONTEXT REJECT received | Retransmission of MODIFY EPS BEARER CONTEXT REQUEST |
| T3489 | 4s | PROCEDURE TRANSACTION PENDING | ESM INFORMATION REQUEST sent | ESM INFORMATION RESPONSE received | Retransmission of ESM INFORMATION REQUEST on 1st and 2nd expiry only |
| T3495 | 8s | BEARER CONTEXT INACTIVE PENDING | DEACTIVATE EPS BEARER CONTEXT REQUEST sent | DEACTIVATE EPS BEARER CONTEXT ACCEPT received | Retransmission of DEACTIVATE EPS BEARER CONTEXT REQUEST |
| NOTE 1: Typically, the procedures are aborted on the fifth expiry of the relevant timer. Exceptions are described in the corresponding procedure description. |  |  |  |  |  |

*** End Change ***