3GPP TSG CT Meeting #74	CP-160810

# Vienna, AUSTRIA; 5th – 6th December 2016	(was C1-165413)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.008 | CR | 3026 | rev | 3 | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of value "0" of the TFT operation code in the traffic flow template IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NBIFOM-CT |  |  |  |  |  |  | Date: |  | 2016-12-02 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | For the NBIFOM procedures such as the IP flow mobility procedure and IP flow mapping, handling of change of access usability and handling of RAN rule indication procedure, the BEARER RESOURCE MODIFICATION REQUEST and BEARER RESOURCE ALLOCATION REQUEST messages defined by TS 24.301 are used to transport the requested routing rule, or to send the access usability change and the RAN rule indication, in which the traffic aggregate description IE may not be needed. Hence, it is unclear about how to set the traffic aggregate description in those procedures when this IE is not needed. It is proposed to clarify in TS 24.301 how the the traffic aggregate description IE is set. However, the changes in TS 24.301 impact TS 24.008 as TS 24.301 reuses the traffic flow template IE for the purpose of the traffic flow aggregate description, and the proposal is to set the TFT operation code to the value "0". At present the TS 24.008 defintion of "0" is spare. This needs to be updated to indicate that the use of the value "0" means "Ignore this IE". The use of value "0" for TFT operation code works for legacy, as the legacy network has to ignore the traffic flow aggregate description if the TFT operation code is set "0" as the value "0" was defined as "spare" in previous releases. For new networks, it should be indicated that the network has to ignore the traffic flow aggregate description when the TFT operation code is set to the value "0" (behaviour as "spare"). Changes to the agreed version 2 at CT1#101: - The reason for change is clarified by indicating that using the value "0" for TFT operation code works for legacy, as the legacy network has to ignore the traffic flow aggregate description if the TFT operation code is set to "0" as the value "0" was defined as "spare" in previous releases; - The CN box is marked as impacted; and - Text is added in the specification to explicitly indicate that the network has to ignore the traffic flow aggregate description when it is to set the TFT operation code to the value "0". |  |  |  |  |  |  |  |  |
| Summary of change: |  | The use of value "0" of the TFT operation code is used when the traffic flow aggregate IE has presence requirement "M" in a message, but this IE does not serve any useful purpose in the specific procedure for which the message is sent (as defined in TS 24.301 for NBIFOM procedures described in the subclauses 6.5.3.2 and 6.5.4.2). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use of "0" is spare for the TFT operation code so it has no the meaning of "Ignore this IE" when set. However, when the traffic flow aggregate description is not needed, the value "0" for the TFT operation code should be used in the messages defined by TS 24.301 for the case of NBIFOM procedures described in the subclauses 6.5.3.2 and 6.5.4.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.6.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

#### 10.5.6.12	Traffic Flow Template

The purpose of the traffic flow template information element is to specify the TFT parameters and operations for a PDP context. In addition, this information element may be used to transfer extra parameters to the network (e.g. the Authorization Token; see 3GPP TS 24.229 [95]). The TFT may contain packet filters for the downlink direction, the uplink direction or packet filters that are applicable to both directions. The packet filters determine the traffic mapping to PDP contexts. The downlink packet filters shall be used by the network and the uplink packet filters shall be used by the MS. A packet filter that is applicable to both directions shall be used by the network as a downlink packet filter and by the MS as an uplink packet filter.

The traffic flow template is a type 4 information element with a minimum length of 3 octets. The maximum length for the IE is 257 octets.

NOTE 1:	The IE length restriction is due to the maximum length that can be encoded in a single length octet.

NOTE 2:	A maximum size IPv4 packet filter can be 32 bytes. Therefore, 7 maximum size IPv4 type packet filters, plus the last packet filter which can contain max 30 octets can fit into one TFT IE, i.e. if needed not all packet filter components can be defined into one message. A maximum size IPv6 packet filter can be 60 bytes. Therefore, only 4 maximum size IPv6 packet filters can fit into one TFT IE. However, using "Add packet filters to existing TFT", it's possible to create a TFT data structure including 16 maximum size IPv4 or IPv6 filters.

The traffic flow template information element is coded as shown in figure 10.5.144/3GPP TS 24.008 and table 10.5.162/3GPP TS 24.008.

NOTE 3:	The 3GPP TS 24.301 [120] reuses the traffic flow template information element for the purpose of the traffic flow aggregate description, where the use of individual TFT parameters, e.g. the packet filter identifier in the parameter list, can differ from this specification.

|  |  | 8 | 7 | 6 | 5 |  | 4 |  | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Traffic flow template IEI |  |  |  |  |  |  |  |  |  |  | Octet 1 |  |
|  |  | Length of traffic flow template IE |  |  |  |  |  |  |  |  |  |  | Octet 2 |  |
|  |  | TFT operation code |  |  |  | E bit |  | Number of packet filters |  |  |  |  | Octet 3 |  |
|  |  | Packet filter list |  |  |  |  |  |  |  |  |  |  | Octet 4 Octet z |  |
|  |  | Parameters list |  |  |  |  |  |  |  |  |  |  | Octet z+1 Octet v |  |

Figure 10.5.144/3GPP TS 24.008: Traffic flow template information element

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | Packet filter identifier 1 |  |  |  | Octet 4 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | 0 | 0 | 0 | 0 | Packet filter identifier 2 |  |  |  | Octet 5 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | … |  |  |  |  |  |  |  |  |
|  | 0 | 0 | 0 | 0 | Packet filter identifier N |  |  |  | Octet N+3 |
|  | Spare |  |  |  |  |  |  |  |  |

Figure 10.5.144a/3GPP TS 24.008: Packet filter list when the TFT operation is "delete packet filters from existing TFT" (z=N+3)

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | Packet filter direction 1 |  | Packet filter identifier 1 |  |  |  | Octet 4 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Packet filter evaluation precedence 1 |  |  |  |  |  |  |  | Octet 5 |
|  | Length of Packet filter contents 1 |  |  |  |  |  |  |  | Octet 6 |
|  | Packet filter contents 1 |  |  |  |  |  |  |  | Octet 7 Octet m |
|  | 0 | 0 | Packet filter direction 2 |  | Packet filter identifier 2 |  |  |  | Octet m+1 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Packet filter evaluation precedence 2 |  |  |  |  |  |  |  | Octet m+2 |
|  | Length of Packet filter contents 2 |  |  |  |  |  |  |  | Octet m+3 |
|  | Packet filter contents 2 |  |  |  |  |  |  |  | Octet m+4 Octet n |
|  | … |  |  |  |  |  |  |  | Octet n+1 Octet y |
|  | 0 | 0 | Packet filter direction N |  | Packet filter identifier N |  |  |  | Octet y+1 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Packet filter evaluation precedence N |  |  |  |  |  |  |  | Octet y+2 |
|  | Length of Packet filter contents N |  |  |  |  |  |  |  | Octet y+3 |
|  | Packet filter contents N |  |  |  |  |  |  |  | Octet y+4 Octet z |

Figure 10.5.144b/3GPP TS 24.008: Packet filter list when the TFT operation is "create new TFT", or "add packet filters to existing TFT" or "replace packet filters in existing TFT"

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Parameter identifier 1 |  |  |  |  |  |  |  | Octet z+1 |
|  | Length of Parameter contents 1 |  |  |  |  |  |  |  | Octet z+2 |
|  | Parameter contents 1 |  |  |  |  |  |  |  | Octet z+3 Octet k |
|  | Parameter identifier 2 |  |  |  |  |  |  |  | Octet k+1 |
|  | Length of Parameter contents 2 |  |  |  |  |  |  |  | Octet k+2 |
|  | Parameter contents 2 |  |  |  |  |  |  |  | Octet k+3 Octet p |
|  | … |  |  |  |  |  |  |  | Octet p+1 Octet q |
|  | Parameter identifier N |  |  |  |  |  |  |  | Octet q+1 |
|  | Length of Parameter contents N |  |  |  |  |  |  |  | Octet q+2 |
|  | Parameter contents N |  |  |  |  |  |  |  | Octet q+3 Octet v |

Figure 10.5.144c/3GPP TS 24.008: Parameters list

Table 10.5.162/3GPP TS 24.008: Traffic flow template information element

TFT operation code (octet 3) Bits 8 7 6 0 0 0 Ignore this IE 0 0 1 Create new TFT 0 1 0 Delete existing TFT 0 1 1 Add packet filters to existing TFT 1 0 0 Replace packet filters in existing TFT 1 0 1 Delete packet filters from existing TFT 1 1 0 No TFT operation 1 1 1 Reserved The TFT operation code "No TFT operation" shall be used if a parameters list is included but no packet filter list is included in the traffic flow template information element. The TFT operation code "Ignore this IE" shall be used by the MS if the Traffic flow aggregate information element has presence requirement "M" in a message, but the information element does not serve any useful purpose in the specific procedure for which the message is sent (see 3GPP TS 24.301 [120], subclauses 6.5.3.2 and 6.5.4.2). If the TFT operation code indicates "Ignore this IE", the MS shall also set the E bit and the number of packet filters to zero. If the TFT operation code is set to "Ignore this IE" and the the E bit and the number of packet filters to zero, then the network shall ignore the contents of the traffic flow template information element. E bit (bit 5 of octet 3) The E bit indicates if a parameters list is included in the TFT IE and it is encoded as follows: 0 parameters list is not included 1 parameters list is included Number of packet filters (octet 3) The number of packet filters contains the binary coding for the number of packet filters in the packet filter list. The number of packet filters field is encoded in bits 4 through 1 of octet 3 where bit 4 is the most significant and bit 1 is the least significant bit. For the "delete existing TFT" operation and for the "no TFT operation", the number of packet filters shall be coded as 0. For all other operations, the number of packet filters shall be greater than 0 and less than or equal to 15. Packet filter list (octets 4 to z) The packet filter list contains a variable number of packet filters. For the "delete existing TFT" operation and the "no TFT operation", the packet filter list shall be empty. For the "delete packet filters from existing TFT" operation, the packet filter list shall contain a variable number of packet filter identifiers. This number shall be derived from the coding of the number of packet filters field in octet 3. For the "create new TFT", "add packet filters to existing TFT" and "replace packet filters in existing TFT" operations, the packet filter list shall contain a variable number of packet filters. This number shall be derived from the coding of the number of packet filters field in octet 3. Each packet filter is of variable length and consists of - a packet filter identifier and direction (1 octet); - a packet filter evaluation precedence (1 octet); - the length of the packet filter contents (1 octet); and - the packet filter contents itself (v octets). The packet filter identifier field is used to identify each packet filter in a TFT. The least significant 4 bits are used. The packet filter direction is used to indicate, in bits 5 and 6, for what traffic direction the filter applies: 00 - pre Rel-7 TFT filter 01 - downlink only 10 - uplink only 11 - bidirectional Bits 8 through 7 are spare bits. The packet filter evaluation precedence field is used to specify the precedence for the packet filter among all packet filters in all TFTs associated with this PDP address. Higher the value of the packet filter evaluation precedence field, lower the precedence of that packet filter is. The first bit in transmission order is the most significant bit. The length of the packet filter contents field contains the binary coded representation of the length of the packet filter contents field of a packet filter. The first bit in transmission order is the most significant bit. The packet filter contents field is of variable size and contains a variable number (at least one) of packet filter components. Each packet filter component shall be encoded as a sequence of a one octet packet filter component type identifier and a fixed length packet filter component value field. The packet filter component type identifier shall be transmitted first. In each packet filter, there shall not be more than one occurrence of each packet filter component type. Among the "IPv4 remote address type" and "IPv6 remote address type" packet filter components, only one shall be present in one packet filter. Among the "single local port type" and "local port range type" packet filter components, only one shall be present in one packet filter. Among the "single remote port type" and "remote port range type" packet filter components, only one shall be present in one packet filter. The term local refers to the MS and the term remote refers to an external network entity. Packet filter component type identifier Bits 8 7 6 5 4 3 2 1 0 0 0 1 0 0 0 0 IPv4 remote address type 0 0 0 1 0 0 0 1 IPv4 local address type 0 0 1 0 0 0 0 0 IPv6 remote address type 0 0 1 0 0 0 0 1 IPv6 remote address/prefix length type 0 0 1 0 0 0 1 1 IPv6 local address/prefix length type 0 0 1 1 0 0 0 0 Protocol identifier/Next header type 0 1 0 0 0 0 0 0 Single local port type 0 1 0 0 0 0 0 1 Local port range type 0 1 0 1 0 0 0 0 Single remote port type 0 1 0 1 0 0 0 1 Remote port range type 0 1 1 0 0 0 0 0 Security parameter index type 0 1 1 1 0 0 0 0 Type of service/Traffic class type 1 0 0 0 0 0 0 0 Flow label type All other values are reserved. The description and valid combinations of packet filter component type identifiers in a packet filter are defined in 3GPP TS 23.060 [74] subclause 15.3.2. For "IPv4 remote address type", the packet filter component value field shall be encoded as a sequence of a four octet IPv4 address field and a four octet IPv4 address mask field. The IPv4 address field shall be transmitted first. For "IPv4 local address type", the packet filter component value field shall be encoded as defined for "IPv4 remote address type". Both the MS and network indication for support of the Local address in TFTs are required to use this packet filter component. For "IPv6 remote address type", the packet filter component value field shall be encoded as a sequence of a sixteen octet IPv6 address field and a sixteen octet IPv6 address mask field. The IPv6 address field shall be transmitted first. For "IPv6 remote address/prefix length type", the packet filter component value field shall be encoded as a sequence of a sixteen octet IPv6 address field and one octet prefix length field. The IPv6 address field shall be transmitted first. This parameter shall be used, instead of IPv6 remote address type, when both the MS and network indication for support of the Local address in TFT are present. For "IPv6 local address/prefix length type", the packet filter component value field shall be encoded as defined for "IPv6 remote address /prefix length". Both the MS and network indication for support of the Local address in TFTs are required to use this packet filter component. NOTE: Local IP address and mask can be used when IPv6 prefix delegation is used (see 3GPP TS 23.060 [74] subclause 9.2.1.2). For "Protocol identifier/Next header type", the packet filter component value field shall be encoded as one octet which specifies the IPv4 protocol identifier or IPv6 next header. For "Single local port type" and "Single remote port type", the packet filter component value field shall be encoded as two octet which specifies a port number. For "Local port range type" and "Remote port range type", the packet filter component value field shall be encoded as a sequence of a two octet port range low limit field and a two octet port range high limit field. The port range low limit field shall be transmitted first. For "Security parameter index", the packet filter component value field shall be encoded as four octet which specifies the IPSec security parameter index. For "Type of service/Traffic class type", the packet filter component value field shall be encoded as a sequence of a one octet Type-of-Service/Traffic Class field and a one octet Type-of-Service/Traffic Class mask field. The Type-of-Service/Traffic Class field shall be transmitted first. For "Flow label type", the packet filter component value field shall be encoded as three octet which specifies the IPv6 flow label. The bits 8 through 5 of the first octet shall be spare whereas the remaining 20 bits shall contain the IPv6 flow label. Parameters list (octets z+1 to v) The parameters list contains a variable number of parameters that may be transferred. If the parameters list is included, the E bit is set to 1; otherwise, the E bit is set to 0. Each parameter included in the parameters list is of variable length and consists of: - a parameter identifier (1 octet); - the length of the parameter contents (1 octet); and - the parameter contents itself (v octets). The parameter identifier field is used to identify each parameter included in the parameters list and it contains the hexadecimal coding of the parameter identifier. Bit 8 of the parameter identifier field contains the most significant bit and bit 1 contains the least significant bit. In this version of the protocol, the following parameter identifiers are specified: - 01H (Authorization Token); - 02H (Flow Identifier); and - 03H (Packet Filter Identifier). If the parameters list contains a parameter identifier that is not supported by the receiving entity the corresponding parameter shall be discarded. The length of parameter contents field contains the binary coded representation of the length of the parameter contents field. The first bit in transmission order is the most significant bit. When the parameter identifier indicates Authorization Token, the parameter contents field contains an authorization token, as specified in 3GPP TS 29.207 [100]. The first octet is the most significant octet of the authorization token and the last octet is the least significant octet of the authorization token. The parameters list shall be coded in a way that an Authorization Token (i.e. a parameter with identifier 01H) is always followed by one or more Flow Identifiers (i.e. one or more parameters with identifier 02H). If the parameters list contains two or more consecutive Authorization Tokens without any Flow Identifiers in between, the receiver shall treat this as a semantical TFT error. When the parameter identifier indicates Flow Identifier, the parameter contents field contains the binary representation of a flow identifier. The Flow Identifier consists of four octets. Octets 1 and 2 contains the Media Component number as specified in 3GPP TS 29.207 [100]. Bit 1 of octet 2 is the least significant bit, and bit 8 of octet 1 is the most significant bit. Octets 3 and 4 contains the IP flow number as specified in 3GPP TS 29.207 [100]. Bit 1 of octet 4 is the least significant bit, and bit 8 of octet 3 is the most significant bit. When the parameter identifier indicates Packet Filter Identifier, the parameter contents field contains the binary representation of one or more packet filter identifiers. Each packet filter identifier is encoded in one octet, in the 4 least significant bits. This parameter is used by the MS and the network to identify one or more packet filters in a TFT when modifying the QoS of a PDP context without modifying the packet filter itself.