3GPP TSG CT Meeting #77	CP-172138

# Sapporo, JP; 11th – 12th September 2017								(rev of CP-172130) (rev of C1-173784)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 2891 | rev | 4 | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extending maximum bitrate in EPS QoS IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Vodafone, AT&T, MediaTek Inc., NTT DOCOMO, ZTE, Ericsson, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Work item code: | EDCE5-CT |  |  |  |  |  |  | Date: |  | 2017-09-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | [RfC for CP-172130 Rev4] Cover sheet update. [RfC for CP-172130 Rev3] SA2 requested CT1 to specify the encoding scheme in support of the maximum bit rate in a future proof manner. The following text added in C1-173784 is not future proof if RAN revisites the max bit rate that the NR can support. “In this version of the protocol, the UE and the network shall interpret - any maximum bit rate value and guaranteed bit rate value for uplink larger than 10 Gbps as 10 Gbps; and - any maximum bit rate value and guaranteed bit rate value for the downlink larger than 20 Gbps as 20 Gbps.” [RfC for C1-173784 Rev2] NR supports 20Gbps for downlink and 10Gbps for uplink per UE according to the requirements in TR38.913 in Rel-15. The maximum bit rate defined in the NAS protocol has to be updated in order to support 20 Gbps data rate. Furthermore, the incoming LS from SA2 in S2-173685 indicated that GTP-C can signal data rates up to 4 Tbps today and asks CT1 whetheran alignment is desirable. In June meeting, RAN3 have agreed to increase signalling support on S1 and X2 to 4Tbps. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [SoC for CP-172130 Rev3] The above mentioned text is replaced by a NOTE. This note points to the reference of the RAN specification in which the maximum bit rate a NR capable UE can support is given. [SoC for C1-173784 Rev2] An extended Quality of Service information element is defined to support bit rates of 20 Gbps. Additinally, an extended APN-AMBR is aldo defined. It is also proposed to entend the maximum bitrate encoding to support high bit rates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Supporting of 20 Gpbs with the NR cannot be met. The coding in support of the maximum bit rate is not future proof. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.3.3.1,8.3.3.11 (new), 8.3.6.1, 8.3.6.17 (new), 8.3.6.18 (new), 8.3.8.1, 8.3.8.6 (new), 8.3.10.1, 8.3.10.9 (new), 8.3.18.1, 8.3.18.14 (new), 8.3.18.15 (new) 9.9.4.2a (new), 9.9.4.3a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * 1s Change * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[1A]	3GPP TS 22.011: "Service accessibility".

[1B]	Void.

[1C]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[2]	3GPP TS 23.003: "Numbering, addressing and identification".

[3]	3GPP TS 23.038: "Alphabets and language-specific information".

[4]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service Description; Stage 2".

[5]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[6]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[7]	3GPP TS 23.203: "Policy and charging control architecture".

[8]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[8A]	3GPP TS 23.221: "Architectural requirements".

[8B]	3GPP TS 23.251: "Network Sharing; Architecture and Functional Description".

[9]	3GPP TS 23.272: "Circuit Switched Fallback in Evolved Packet System; Stage 2".

[10]	3GPP TS 23.401: "GPRS enhancements for E-UTRAN access".

[11]	3GPP TS 23.402: "GPRS architecture enhancements for non-3GPP accesses".

[11A]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[12]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[13]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[13A]	3GPP TS 24.011: "Point-to-Point Short Message Service (SMS) support on mobile radio interface".

[13B]	3GPP TS 24.167: "3GPP IMS Management Object (MO); Stage 3".

[13C]	3GPP TS 24.171: "NAS Signalling for Control Plane LCS in Evolved Packet System (EPS)".

[13D]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[13E]	3GPP TS 24.173: "IMS Multimedia telephony communication service and supplementary services; Stage 3".

[14]	3GPP TS 24.303: "Mobility Management based on DSMIPv6; User Equipment (UE) to network protocols; Stage 3".

[15]	3GPP TS 24.304: "Mobility management based on Mobile IPv4; User Equipment (UE) - foreign agent interface; Stage 3".

[15A]	3GPP TS 24.368: "Non-Access Stratum (NAS) configuration Management Object (MO)".

[15B]	3GPP TS 25.304: "User Equipment (UE) procedures in idle mode and procedures for cell reselection in connected mode".

[15C]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[15D]	3GPP TS 24.341: "Support of SMS over IP networks; Stage 3".

[16]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[16A]	3GPP TS 29.118: "Mobility Management Entity (MME) – Visitor Location Register (VLR) SGs interface specification".

[16B]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[16C]	3GPP TS 29.272: "Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[16D]	3GPP TS 29.274: "Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[17]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[18]	3GPP TS 33.102: "3G security; Security architecture".

[19]	3GPP TS 33.401: "3GPP System Architecture Evolution; Security architecture".

[20]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description".

[21]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[22]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[22A]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[23]	3GPP TS 36.413: "Evolved Universal Terrestrial Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[23A]	3GPP TS 45.008: "Radio Access Network; Radio subsystem link control".

[24]	Void.

[24A]	IETF RFC 3633: "IPv6 Prefix Options for Dynamic Host Configuration Protocol (DHCP) version 6".

[25]	Void.

[26]	Void.

[27]	Void.

[28]	Void.

[29]	ISO/IEC 10646: "Information technology – Universal Multiple-Octet Coded Character Set (UCS)".

[30]	ITU-T Recommendation E.212: "The international identification plan for mobile terminals and mobile users".

[31]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[32]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to Proximity-services (ProSe) Function Protocol aspects; Stage 3".

[33]	3GPP TS 23.380: "IMS restoration procedures".

[34]	3GPP TS 23.161: "Network-Based IP Flow Mobility (NBIFOM); Stage 2".

[35]	3GPP TS 24.105: "Application specific Congestion control for Data Communication (ACDC) Management Object (MO)".

[36]	3GPP TS 24.161: "Network-Based IP Flow Mobility (NBIFOM); Stage 3".

[37]	IETF RFC 4995: "The RObust Header Compression (ROHC) Framework".

[38]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[39]	IETF RFC 4996: "RObust Header Compression (ROHC): A Profile for TCP/IP (ROHC-TCP)".

[40]	IETF RFC 3095: "RObust Header Compression (RoHC): Framework and four profiles: RTP, UDP, ESP and uncompressed".

[41]	IETF RFC 3843: "RObust Header Compression (RoHC): A Compression Profile for IP".

[42]	IETF RFC 4815: "RObust Header Compression (ROHC): Corrections and Clarifications to RFC 3095".

[43]	IETF RFC 5225: "RObust Header Compression (ROHC) Version 2: Profiles for RTP, UDP, IP, ESP and UDP Lite".

[44]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[45]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[46]	3GPP TS 22.101: "Service aspects; Service principles".

[47]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[48]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3".

[49]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[50]	3GPP TS 24.623: "Extensive Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[51]	3GPP TS 24.250: "Protocol for Reliable Data Service between UE and SCEF; Stage 3".

[52]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies".

* * * Next Change * * *

#### 8.3.3.1	Message definition

This message is sent by the network to the UE to request activation of a dedicated EPS bearer context associated with the same PDN address(es) and APN as an already active default EPS bearer context. See table 8.3.3.1.

Message type:	ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST

Significance:		dual

Direction:			network to UE

# Table 8.3.3.1: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | EPS bearer identity | EPS bearer identity 9.3.2 | M | V | 1/2 |
|  | Procedure transaction identity | Procedure transaction identity 9.4 | M | V | 1 |
|  | Activate dedicated EPS bearer context request message identity | Message type 9.8 | M | V | 1 |
|  | Linked EPS bearer identity | Linked EPS bearer identity 9.9.4.6 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.9.2.9 | M | V | 1/2 |
|  | EPS QoS | EPS quality of service 9.9.4.3 | M | LV | 2-14 |
|  | TFT | Traffic flow template 9.9.4.16 | M | LV | 2-256 |
| 5D | Transaction identifier | Transaction identifier 9.9.4.17 | O | TLV | 3-4 |
| 30 | Negotiated QoS | Quality of service 9.9.4.12 | O | TLV | 14-22 |
| 32 | Negotiated LLC SAPI | LLC service access point identifier 9.9.4.7 | O | TV | 2 |
| 8- | Radio priority | Radio priority 9.9.4.13 | O | TV | 1 |
| 34 | Packet flow Identifier | Packet flow Identifier 9.9.4.8 | O | TLV | 3 |
| 27 | Protocol configuration options | Protocol configuration options 9.9.4.11 | O | TLV | 3-253 |
| C- | WLAN offload indication | WLAN offload acceptability 9.9.4.18 | O | TV | 1 |
| 33 | NBIFOM container | NBIFOM container 9.9.4.19 | O | TLV | 3-257 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.9.4.26 | O | TLV-E | 4-65538 |
| xx | Extended QoS | Extended quality of service 9.9.4.3a | O | TLV | 12 |

#### 8.3.3.2	Transaction identifier

If the UE supports A/Gb mode or Iu mode or both, a network supporting mobility from S1 mode to A/Gb mode or Iu mode or both shall include this IE

#### 8.3.3.3	Negotiated QoS

If the UE supports A/Gb mode or Iu mode or both, a network supporting mobility from S1 mode to A/Gb mode or Iu mode or both shall include the corresponding R99 QoS parameter values of a PDP context.

#### 8.3.3.4	Negotiated LLC SAPI

If the UE supports A/Gb mode, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.3.5	Radio priority

If the UE supports A/Gb mode, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.3.6	Packet flow identifier

If the UE supports A/Gb mode and BSS packet flow procedures, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.3.7	Protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.3.8	WLAN offload indication

This IE shall be included in the message when the network wishes to indicate if the UE is allowed to offload the traffic of the associated PDN connection to WLAN(s), as specified in subclause 9.9.4.18.

#### 8.3.3.9	NBIFOM container

This information element is used to transfer information associated with network-based IP flow mobility, see 3GPP TS 24.161 [36].

#### 8.3.3.10	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.3.11	Extended quality of service

This IE is included in the message when the network wishes to use the extended maximum and guaranteed bit rate.

* * * Next Change * * *

#### 8.3.6.1	Message definition

This message is sent by the network to the UE to request activation of a default EPS bearer context. See table 8.3.6.1.

Message type:	ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST

Significance:		dual

Direction:			network to UE

# Table 8.3.6.1: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message content

| IEI | Information Element |  | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator |  | Protocol discriminator 9.2 | M | V | 1/2 |
|  | EPS bearer identity |  | EPS bearer identity 9.3.2 | M | V | 1/2 |
|  | Procedure transaction identity |  | Procedure transaction identity 9.4 | M | V | 1 |
|  | Activate default EPS bearer context request message identity |  | Message type 9.8 | M | V | 1 |
|  | EPS QoS |  | EPS quality of service 9.9.4.3 | M | LV | 2-14 |
|  | Access point name | Access point name 9.9.4.1 |  | M | LV | 2-101 |
|  | PDN address | PDN address 9.9.4.9 |  | M | LV | 6-14 |
| 5D | Transaction identifier | Transaction identifier 9.9.4.17 |  | O | TLV | 3-4 |
| 30 | Negotiated QoS | Quality of service 9.9.4.12 |  | O | TLV | 14-22 |
| 32 | Negotiated LLC SAPI |  | LLC service access point identifier 9.9.4.7 | O | TV | 2 |
| 8- | Radio priority | Radio priority 9.9.4.13 |  | O | TV | 1 |
| 34 | Packet flow Identifier | Packet flow Identifier 9.9.4.8 |  | O | TLV | 3 |
| 5E | APN-AMBR | APN aggregate maximum bit rate 9.9.4.2 |  | O | TLV | 4-8 |
| 58 | ESM cause | ESM cause 9.9.4.4 |  | O | TV | 2 |
| 27 | Protocol configuration options | Protocol configuration options 9.9.4.11 |  | O | TLV | 3-253 |
| B- | Connectivity type |  | Connectivity type 9.9.4.2A | O | TV | 1 |
| C- | WLAN offload indication |  | WLAN offload acceptability 9.9.4.18 | O | TV | 1 |
| 33 | NBIFOM container |  | NBIFOM container 9.9.4.19 | O | TLV | 3-257 |
| 66 | Header compression configuration |  | Header compression configuration 9.9.4.22 | O | TLV | 5-257 |
| 9- | Control plane only indication |  | Control plane only indication 9.9.4.23 | O | TV | 1 |
| 7B | Extended protocol configuration options |  | Extended protocol configuration options 9.9.4.26 | O | TLV-E | 4-65538 |
| 6E | Serving PLMN rate control |  | Serving PLMN rate control 9.9.4.28 | O | TLV | 4 |
| xx | Extended APN-AMBR |  | Extended APN aggregate maximum bit rate 9.9.4.2a | O | TLV | 8 |
| yy | Extended QoS |  | Extended quality of service 9.9.4.3a | O | TLV | 12 |

#### 8.3.6.2	Transaction identifier

If the UE supports A/Gb mode or Iu mode or both, a network supporting mobility from S1 mode to A/Gb mode or Iu mode or both shall include this IE.

#### 8.3.6.3	Negotiated QoS

If the UE supports A/Gb mode or Iu mode or both, a network supporting mobility from S1 mode to A/Gb mode or Iu mode or both shall include the corresponding R99 QoS parameter values of a PDP context.

#### 8.3.6.4	Negotiated LLC SAPI

If the UE supports A/Gb mode, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.6.5	Radio priority

If the UE supports A/Gb mode, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.6.6	Packet flow identifier

If the UE supports A/Gb mode and BSS packet flow procedures, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.6.7	APN-AMBR

This IE is included in the message when the network wishes to transmit the APN-AMBR to the UE for possible uplink policy enforcement.

#### 8.3.6.8	ESM cause

The network shall include this IE, if the network allocated a PDN address of a PDN type which is different from the PDN type requested by the UE.

#### 8.3.6.9	Protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

This IE shall be included if the network supports Local IP address in the traffic flow aggregate description and TFT filter, the PDN Type is different from Non-IP, and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.6.10	Connectivity type

The network shall include the Connectivity type IE if:

-	the network is configured to indicate when a PDN connection is a LIPA PDN connection; and

-	the present PDN connection is a LIPA PDN connection.

#### 8.3.6.11	WLAN offload indication

This IE shall be included in the message when the network wishes to indicate if the UE is allowed to offload the traffic of the associated PDN connection to WLAN(s), as specified in subclause 9.9.4.18.

#### 8.3.6.12	NBIFOM container

This information element is used to transfer information associated with network-based IP flow mobility, see 3GPP TS 24.161 [36].

#### 8.3.6.13	Header compression configuration

The network may include the Header compression configuration IE if:

-	the network accepts an IP PDN type; and

-	control plane CIoT EPS optimization is selected.

#### 8.3.6.14	Control plane only indication

The network shall include the Control plane only indication IE if the associated PDN connection is only for control plane CIoT EPS optimization.

#### 8.3.6.15	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

This IE shall be included if the network supports Local IP address in the traffic flow aggregate description and TFT filter, the PDN Type is different from Non-IP, and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.6.16	Serving PLMN rate control

This IE shall be included when the network wishes to indicate the maximum number of uplink ESM DATA TRANSPORT messages including User data container IEs the UE is allowed to send per 6 minute interval.

#### 8.3.6.17	Extended APN aggregate maximum bit rate

This IE is included in the message when the network wishes to transmit the extended APN-AMBR to the UE for possible uplink policy enforcement.

#### 8.3.6.18	Extended quality of service

#### This IE is included in the message when the network wishes to use the extended maximum and guaranteed bit rate.

* * * Next Change * * *

#### 8.3.8.1	Message definition

This message is sent by the UE to the network to request the allocation of a dedicated bearer resource. See table 8.3.8.1.

Message type:	BEARER RESOURCE ALLOCATION REQUEST

Significance:		dual

Direction:			UE to network

# Table 8.3.8.1: BEARER RESOURCE ALLOCATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | EPS bearer identity | EPS bearer identity 9.3.2 | M | V | 1/2 |
|  | Procedure transaction identity | Procedure transaction identity 9.4 | M | V | 1 |
|  | Bearer resource allocation request message identity | Message type 9.8 | M | V | 1 |
|  | Linked EPS bearer identity | Linked EPS bearer identity 9.9.4.6 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.9.2.9 | M | V | 1/2 |
|  | Traffic flow aggregate | Traffic flow aggregate description 9.9.4.15 | M | LV | 2-256 |
|  | Required traffic flow QoS | EPS quality of service 9.9.4.3 | M | LV | 2-14 |
| 27 | Protocol configuration options | Protocol configuration options 9.9.4.11 | O | TLV | 3-253 |
| C- | Device properties | Device properties 9.9.2.0A | O | TV | 1 |
| 33 | NBIFOM container | NBIFOM container 9.9.4.19 | O | TLV | 3-257 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.9.4.26 | O | TLV-E | 4-65538 |
| xx | Extended QoS | Extended quality of service 9.9.4.3a | O | TLV | 12 |

#### 8.3.8.2	Protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.8.3	Device properties

The UE shall include this IE if the UE is configured for NAS signalling low priority.

#### 8.3.8.4	NBIFOM container

This information element is used to transfer information associated with network-based IP flow mobility, see 3GPP TS 24.161 [36].

#### 8.3.8.5	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.8.6	Extended quality of service

This IE is included in the message when the UE wishes to use the extended maximum and guaranteed bit rate.

* * * Next Change * * *

#### 8.3.10.1	Message definition

This message is sent by the UE to the network to request the modification of a dedicated bearer resource, or to request re-negotiation of header compression configuration associated to an EPS bearer context if the network has previously indicated support of Control plane CIoT EPS optimization and Header compression for control plane CIoT EPS optimization. See table 8.3.10.1.

Message type:	BEARER RESOURCE MODIFICATION REQUEST

Significance:		dual

Direction:			UE to network

# Table 8.3.10.1: BEARER RESOURCE MODIFICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | EPS bearer identity | EPS bearer identity 9.3.2 | M | V | 1/2 |
|  | Procedure transaction identity | Procedure transaction identity 9.4 | M | V | 1 |
|  | Bearer resource modification request message identity | Message type 9.8 | M | V | 1 |
|  | EPS bearer identity for packet filter | Linked EPS bearer identity 9.9.4.6 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.9.2.9 | M | V | 1/2 |
|  | Traffic flow aggregate | Traffic flow aggregate description 9.9.4.15 | M | LV | 2-256 |
| 5B | Required traffic flow QoS | EPS quality of service 9.9.4.3 | O | TLV | 3-15 |
| 58 | ESM cause | ESM cause 9.9.4.4 | O | TV | 2 |
| 27 | Protocol configuration options | Protocol configuration options 9.9.4.11 | O | TLV | 3-253 |
| C- | Device properties | Device properties 9.9.2.0A | O | TV | 1 |
| 33 | NBIFOM container | NBIFOM container 9.9.4.19 | O | TLV | 3-257 |
| 66 | Header compression configuration | Header compression configuration 9.9.4.22 | O | TLV | 5-257 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.9.4.26 | O | TLV-E | 4-65538 |
| xx | Extended QoS | Extended quality of service 9.9.4.3a | O | TLV | 12 |

#### 8.3.10.2	Required traffic flow QoS

This IE is included in the message when the UE requests a change of QoS for the indicated traffic flows and does not request to release all bearer resources for the EPS bearer context.

#### 8.3.10.3	ESM cause

This IE is included in the message when the UE requests the release of a dedicated bearer resource.

#### 8.3.10.4	Protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.10.5	Device properties

The UE shall include this IE if the UE is configured for NAS signalling low priority.

#### 8.3.10.6	NBIFOM container

This information element is used to transfer information associated with network-based IP flow mobility, see 3GPP TS 24.161 [36].

#### 8.3.10.7	Header compression configuration

This IE is included in the message if the UE wishes to re-negotiate header compression configuration associated to an EPS bearer context and both the UE and the network supports Control plane CIoT EPS optimization and header compression.

#### 8.3.10.8	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.10.9	Extended quality of service

This IE is included in the message when the UE wishes to use the extended maximum and guaranteed bit rate.

* * * Next Change * * *

#### 8.3.18.1	Message definition

This message is sent by the network to the UE to request modification of an active EPS bearer context, or to request re-negotiation of header compression configuration associated to an EPS bearer context if the UE has previously indicated support of Control plane CIoT EPS optimization and Header compression for control plane CIoT EPS optimization. See table 8.3.18.1.

Message type:	MODIFY EPS BEARER CONTEXT REQUEST

Significance:		dual

Direction:			network to UE

# Table 8.3.18.1: MODIFY EPS BEARER CONTEXT REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | EPS bearer identity | EPS bearer identity 9.3.2 | M | V | 1/2 |
|  | Procedure transaction identity | Procedure transaction identity 9.4 | M | V | 1 |
|  | Modify EPS bearer context request message identity | Message type 9.8 | M | V | 1 |
| 5B | New EPS QoS | EPS quality of service 9.9.4.3 | O | TLV | 3-15 |
| 36 | TFT | Traffic flow template 9.9.4.16 | O | TLV | 3-257 |
| 30 | New QoS | Quality of service 9.9.4.12 | O | TLV | 14-22 |
| 32 | Negotiated LLC SAPI | LLC service access point identifier 9.9.4.7 | O | TV | 2 |
| 8- | Radio priority | Radio priority 9.9.4.13 | O | TV | 1 |
| 34 | Packet flow Identifier | Packet flow Identifier 9.9.4.8 | O | TLV | 3 |
| 5E | APN-AMBR | APN aggregate maximum bit rate 9.9.4.2 | O | TLV | 4-8 |
| 27 | Protocol configuration options | Protocol configuration options 9.9.4.11 | O | TLV | 3-253 |
| C- | WLAN offload indication | WLAN offload acceptability 9.9.4.18 | O | TV | 1 |
| 33 | NBIFOM container | NBIFOM container 9.9.4.19 | O | TLV | 3-257 |
| 66 | Header compression configuration | Header compression configuration 9.9.4.22 | O | TLV | 5-257 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.9.4.26 | O | TLV-E | 4-65538 |
| xx | Extended APN-AMBR | Extended APN aggregate maximum bit rate 9.9.4.2a | O | TLV | 8 |
| yy | Extended QoS | Extended quality of service 9.9.4.3a | O | TLV | 12 |

#### 8.3.18.2	New EPS QoS

When the EPS QoS of the EPS bearer context is modified, the network shall include the modified EPS QoS assigned to the EPS bearer context.

#### 8.3.18.3	TFT

This IE provides the UE with packet filters.

#### 8.3.18.4	New QoS

If the UE supports A/Gb mode or Iu mode or both and when the corresponding R99 QoS of the EPS bearer context is modified, a network supporting mobility from S1 mode to A/Gb mode or Iu mode or both shall include the corresponding R99 QoS parameter values of a PDP context.

#### 8.3.18.5	Negotiated LLC SAPI

If the UE supports A/Gb mode and when the negotiated LLC SAPI is modified, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.18.6	Radio priority

If the UE supports A/Gb mode and when the radio priority is modified, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.18.7	Packet flow identifier

If the UE supports A/Gb mode and BSS packet flow procedures, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.18.8	APN-AMBR

This IE is included when the APN-AMBR has been changed by the network.

#### 8.3.18.9	Protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.18.10	WLAN offload indication

This IE shall be included in the message when the network wishes to indicate if the UE is allowed to offload the traffic of the associated PDN connection to WLAN(s), as specified in subclause 9.9.4.18.

#### 8.3.18.11	NBIFOM container

This information element is used to transfer information associated with network-based IP flow mobility, see 3GPP TS 24.161 [36].

#### 8.3.18.12	Header compression configuration

This IE is included in the message if the network wishes to re-negotiate header compression configuration associated to an EPS bearer context and both the UE and the network support Control plane CIoT EPS optimization and header compression.

#### 8.3.18.13	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.18.14	Extended APN aggregate maximum bit rate

This IE is included in the message when the extended APN-AMBR has been changed by the network.

#### 8.3.18.15	Extended quality of service

This IE is included in the message when the network wishes to use the extended maximum and guaranteed bit rate.

* * * Next Change * * *

#### 9.9.4.2a	Extended APN aggregate maximum bit rate

The purpose of the extended APN aggregate maximum bit rate information element is to indicate the initial subscribed APN-AMBR when the UE establishes a PDN connection or to indicate the new APN-AMBR if it is changed by the network.

The extended APN aggregate maximum bit rate information element is coded as shown in figure 9.9.4.2a.1 and table 9.9.4.2a.1.

The extended APN aggregate maximum bit rate is a type 4 information element with a length of 8 octets.

NOTE:	Refer to 3GPP TR 38.913 [52] for the maximum bit rate a NR capable UE can support.

Editor's Note [WI: EDCE5-CT, CR#2891]:	3GPP TR 38.913 [52] will be replaced by the normative RAN specifications when available.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended APN aggregate maximum bit rate IEI |  |  |  |  |  |  |  | octet 1 |
| Length of extended APN aggregate maximum bit rate contents |  |  |  |  |  |  |  | octet 2 |
| Unit for extended APN-AMBR for downlink |  |  |  |  |  |  |  | octet 3 |
| Extended APN-AMBR for downlink |  |  |  |  |  |  |  | octet 4 |
| Extended APN-AMBR for downlink (continued) |  |  |  |  |  |  |  | octet 5 |
| Unit for extended APN-AMBR for uplink |  |  |  |  |  |  |  | octet 6 |
| Extended APN-AMBR for uplink |  |  |  |  |  |  |  | octet 7 |
| Extended APN-AMBR for uplink (continued) |  |  |  |  |  |  |  | octet 8 |

# Figure 9.9.4.2a.1: Extended APN aggregate maximum bit rate information element

Table 9.9.4.2a.1: Extended APN aggregate maximum bit rate information element

Unit for for extended APN aggregate maximum bit rate (octet 3) 0 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 0 1 value is incremented in multiples of 100 kbps 0 0 0 0 0 0 0 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 0 0 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 0 0 1 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 0 0 1 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 0 0 1 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 0 0 1 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 0 1 0 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 0 1 0 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 0 1 0 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 0 1 0 1 1 value is incremented in multiples of 256 Gbps 0 0 0 0 0 1 1 0 0 value is incremented in multiples of 1 Tbps 0 0 0 0 0 1 1 0 1 value is incremented in multiples of 4 Tbps 0 0 0 0 0 1 1 1 0 value is incremented in multiples of 16 Tbps 0 0 0 0 0 1 1 1 1 value is incremented in multiples of 64 Tbps 0 0 0 0 1 0 0 0 0 value is incremented in multiples of 256 Tbps 0 0 0 0 1 0 0 0 1 value is incremented in multiples of 1 Pbps 0 0 0 0 1 0 0 1 0 value is incremented in multiples of 4 Pbps 0 0 0 0 1 0 0 1 1 value is incremented in multiples of 16 Pbps 0 0 0 0 1 0 1 0 0 value is incremented in multiples of 64 Pbps 0 0 0 0 1 0 1 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. APN aggregate maximum bit rate (octets 4 and 5) Octets 4 and 5 represent the binary coded value of APN aggregate maximum bit rate.

* * * Next Change * * *

#### 9.9.4.3a	Extended quality of service

The purpose of the extended quality of service information element is to specify the extended maximum bit rate and the extended guaranteed bit rate for a bearer context.

The extended quality of service information element is coded as shown in figure 9.9.4.3a.1 and table 9.9.4.3a.1.

The extended quality of service is a type 4 information element with a length of 12 octets.

NOTE:	Refer to 3GPP TR 38.913 [52] for the maximum bit rate a NR capable UE can support.

Editor's Note [WI: EDCE5-CT, CR#2891]:	3GPP TR 38.913 [52] will be replaced by the normative RAN specifications when available.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended EPS quality of service IEI |  |  |  |  |  |  |  | octet 1 |
| Length of EPS quality of service contents |  |  |  |  |  |  |  | octet 2 |
| Unit for maximum bit rate |  |  |  |  |  |  |  | octet 3 |
| Maximum bit rate for uplink |  |  |  |  |  |  |  | octet 4 |
| Maximum bit rate for uplink (continued) |  |  |  |  |  |  |  | octet 5 |
| Maximum bit rate for downlink |  |  |  |  |  |  |  | octet 6 |
| Maximum bit rate for downlink (continued) |  |  |  |  |  |  |  | octet 7 |
| Unit for guaranteed bit rate |  |  |  |  |  |  |  | octet 8 |
| Guaranteed bit rate for uplink |  |  |  |  |  |  |  | octet 9 |
| Guaranteed bit rate for uplink (continued) |  |  |  |  |  |  |  | octet 10 |
| Guaranteed bit rate for downlink |  |  |  |  |  |  |  | octet 11 |
| Guaranteed bit rate for downlink (continued) |  |  |  |  |  |  |  | octet 12 |

# Figure 9.9.4.3a.1: Extended quality of service information element

Table 9.9.4.3a.1: Extended quality of service information element

Unit for maximum bit rate (octet 3) 0 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 0 1 value is incremented in multiples of 100 kbps 0 0 0 0 0 0 0 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 0 0 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 0 0 1 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 0 0 1 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 0 0 1 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 0 0 1 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 0 1 0 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 0 1 0 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 0 1 0 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 0 1 0 1 1 value is incremented in multiples of 256 Gbps 0 0 0 0 0 1 1 0 0 value is incremented in multiples of 1 Tbps 0 0 0 0 0 1 1 0 1 value is incremented in multiples of 4 Tbps 0 0 0 0 0 1 1 1 0 value is incremented in multiples of 16 Tbps 0 0 0 0 0 1 1 1 1 value is incremented in multiples of 64 Tbps 0 0 0 0 1 0 0 0 0 value is incremented in multiples of 256 Tbps 0 0 0 0 1 0 0 0 1 value is incremented in multiples of 1 Pbps 0 0 0 0 1 0 0 1 0 value is incremented in multiples of 4 Pbps 0 0 0 0 1 0 0 1 1 value is incremented in multiples of 16 Pbps 0 0 0 0 1 0 1 0 0 value is incremented in multiples of 64 Pbps 0 0 0 0 1 0 1 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Maximum bit rate for uplink (octets 4 and 5) Octets 4 and 5 represent the binary coded value of maximum bit rate for uplink in units defined by octet 3. Maximum bit rate for downlink (octets 6 and 7) Octets 6 and 7 represent the binary coded value of maximum bit rate for downlink in units defined by octet 3. Unit for guaranteed bit rate (octet 8) The coding is identical to that of the unit for maximum bit rate (octet 3). Guaranteed bit rate for uplink (octets 9 and 10) Octets 9 and 10 represent the binary coded value of guaranteed bit rate for uplink in units defined by octet 8. Guaranteed bit rate for downlink (octets 11 and 12) Octets 11 and 12 represent the binary coded value of guaranteed bit rate for downlink in units defined by octet 8.