3GPP TSG CT Meeting #77	CP-172155

# Sapporo, JP; 11th – 12th September 2017								(rev of CP-172150) (rev of C1-173784)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 2891 | rev | 6 | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extending maximum bitrate in EPS QoS IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Vodafone, AT&T, MediaTek Inc., NTT DOCOMO, ZTE, Ericsson, Deutsche Telekom, Qualcomm, Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | EDCE5-CT |  |  |  |  |  |  | Date: |  | 2017-09-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | [RfC for CP-172155 Rev6] It was requested to remove the NOTE and modify the EN. [RfC for CP-172150 Rev5] Adding supporting companies. [RfC for CP-172138 Rev4] Cover sheet update. [RfC for CP-172130 Rev3] SA2 requested CT1 to specify the encoding scheme in support of the maximum bit rate in a future proof manner. The following text added in C1-173784 is not future proof if RAN revisites the max bit rate that the NR can support. “In this version of the protocol, the UE and the network shall interpret - any maximum bit rate value and guaranteed bit rate value for uplink larger than 10 Gbps as 10 Gbps; and - any maximum bit rate value and guaranteed bit rate value for the downlink larger than 20 Gbps as 20 Gbps.” [RfC for C1-173784 Rev2] NR supports 20Gbps for downlink and 10Gbps for uplink per UE according to the requirements in TR38.913 in Rel-15. The maximum bit rate defined in the NAS protocol has to be updated in order to support 20 Gbps data rate. Furthermore, the incoming LS from SA2 in S2-173685 indicated that GTP-C can signal data rates up to 4 Tbps today and asks CT1 whetheran alignment is desirable. In June meeting, RAN3 have agreed to increase signalling support on S1 and X2 to 4Tbps. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [SoC for CP-172155 Rev6] remove the NOTE and the RAN reference specification, and modify the EN. [SoC for CP-172150 Rev5] add Qualcomm, Nokia and Alcatel-Lucent Shanghai Bell as co-signers. [SoC for CP-172138 Rev4] update the cover sheet. [SoC for CP-172130 Rev3] The above mentioned text is replaced by a NOTE. This note points to the reference of the RAN specification in which the maximum bit rate a NR capable UE can support is given. [SoC for C1-173784 Rev2] An extended Quality of Service information element is defined to support bit rates of 20 Gbps. Additinally, an extended APN-AMBR is aldo defined. It is also proposed to entend the maximum bitrate encoding to support high bit rates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Supporting of 20 Gpbs with the NR cannot be met. The coding in support of the maximum bit rate is not future proof. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.1,8.3.3.11 (new), 8.3.6.1, 8.3.6.17 (new), 8.3.6.18 (new), 8.3.8.1, 8.3.8.6 (new), 8.3.10.1, 8.3.10.9 (new), 8.3.18.1, 8.3.18.14 (new), 8.3.18.15 (new) 9.9.4.2a (new), 9.9.4.3a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * 1s Change * * *

#### 8.3.3.1	Message definition

This message is sent by the network to the UE to request activation of a dedicated EPS bearer context associated with the same PDN address(es) and APN as an already active default EPS bearer context. See table 8.3.3.1.

Message type:	ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST

Significance:		dual

Direction:			network to UE

# Table 8.3.3.1: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | EPS bearer identity | EPS bearer identity 9.3.2 | M | V | 1/2 |
|  | Procedure transaction identity | Procedure transaction identity 9.4 | M | V | 1 |
|  | Activate dedicated EPS bearer context request message identity | Message type 9.8 | M | V | 1 |
|  | Linked EPS bearer identity | Linked EPS bearer identity 9.9.4.6 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.9.2.9 | M | V | 1/2 |
|  | EPS QoS | EPS quality of service 9.9.4.3 | M | LV | 2-14 |
|  | TFT | Traffic flow template 9.9.4.16 | M | LV | 2-256 |
| 5D | Transaction identifier | Transaction identifier 9.9.4.17 | O | TLV | 3-4 |
| 30 | Negotiated QoS | Quality of service 9.9.4.12 | O | TLV | 14-22 |
| 32 | Negotiated LLC SAPI | LLC service access point identifier 9.9.4.7 | O | TV | 2 |
| 8- | Radio priority | Radio priority 9.9.4.13 | O | TV | 1 |
| 34 | Packet flow Identifier | Packet flow Identifier 9.9.4.8 | O | TLV | 3 |
| 27 | Protocol configuration options | Protocol configuration options 9.9.4.11 | O | TLV | 3-253 |
| C- | WLAN offload indication | WLAN offload acceptability 9.9.4.18 | O | TV | 1 |
| 33 | NBIFOM container | NBIFOM container 9.9.4.19 | O | TLV | 3-257 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.9.4.26 | O | TLV-E | 4-65538 |
| xx | Extended QoS | Extended quality of service 9.9.4.3a | O | TLV | 12 |

#### 8.3.3.2	Transaction identifier

If the UE supports A/Gb mode or Iu mode or both, a network supporting mobility from S1 mode to A/Gb mode or Iu mode or both shall include this IE

#### 8.3.3.3	Negotiated QoS

If the UE supports A/Gb mode or Iu mode or both, a network supporting mobility from S1 mode to A/Gb mode or Iu mode or both shall include the corresponding R99 QoS parameter values of a PDP context.

#### 8.3.3.4	Negotiated LLC SAPI

If the UE supports A/Gb mode, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.3.5	Radio priority

If the UE supports A/Gb mode, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.3.6	Packet flow identifier

If the UE supports A/Gb mode and BSS packet flow procedures, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.3.7	Protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.3.8	WLAN offload indication

This IE shall be included in the message when the network wishes to indicate if the UE is allowed to offload the traffic of the associated PDN connection to WLAN(s), as specified in subclause 9.9.4.18.

#### 8.3.3.9	NBIFOM container

This information element is used to transfer information associated with network-based IP flow mobility, see 3GPP TS 24.161 [36].

#### 8.3.3.10	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.3.11	Extended quality of service

This IE is included in the message when the network wishes to use the extended maximum and guaranteed bit rate.

* * * Next Change * * *

#### 8.3.6.1	Message definition

This message is sent by the network to the UE to request activation of a default EPS bearer context. See table 8.3.6.1.

Message type:	ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST

Significance:		dual

Direction:			network to UE

# Table 8.3.6.1: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message content

| IEI | Information Element |  | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator |  | Protocol discriminator 9.2 | M | V | 1/2 |
|  | EPS bearer identity |  | EPS bearer identity 9.3.2 | M | V | 1/2 |
|  | Procedure transaction identity |  | Procedure transaction identity 9.4 | M | V | 1 |
|  | Activate default EPS bearer context request message identity |  | Message type 9.8 | M | V | 1 |
|  | EPS QoS |  | EPS quality of service 9.9.4.3 | M | LV | 2-14 |
|  | Access point name | Access point name 9.9.4.1 |  | M | LV | 2-101 |
|  | PDN address | PDN address 9.9.4.9 |  | M | LV | 6-14 |
| 5D | Transaction identifier | Transaction identifier 9.9.4.17 |  | O | TLV | 3-4 |
| 30 | Negotiated QoS | Quality of service 9.9.4.12 |  | O | TLV | 14-22 |
| 32 | Negotiated LLC SAPI |  | LLC service access point identifier 9.9.4.7 | O | TV | 2 |
| 8- | Radio priority | Radio priority 9.9.4.13 |  | O | TV | 1 |
| 34 | Packet flow Identifier | Packet flow Identifier 9.9.4.8 |  | O | TLV | 3 |
| 5E | APN-AMBR | APN aggregate maximum bit rate 9.9.4.2 |  | O | TLV | 4-8 |
| 58 | ESM cause | ESM cause 9.9.4.4 |  | O | TV | 2 |
| 27 | Protocol configuration options | Protocol configuration options 9.9.4.11 |  | O | TLV | 3-253 |
| B- | Connectivity type |  | Connectivity type 9.9.4.2A | O | TV | 1 |
| C- | WLAN offload indication |  | WLAN offload acceptability 9.9.4.18 | O | TV | 1 |
| 33 | NBIFOM container |  | NBIFOM container 9.9.4.19 | O | TLV | 3-257 |
| 66 | Header compression configuration |  | Header compression configuration 9.9.4.22 | O | TLV | 5-257 |
| 9- | Control plane only indication |  | Control plane only indication 9.9.4.23 | O | TV | 1 |
| 7B | Extended protocol configuration options |  | Extended protocol configuration options 9.9.4.26 | O | TLV-E | 4-65538 |
| 6E | Serving PLMN rate control |  | Serving PLMN rate control 9.9.4.28 | O | TLV | 4 |
| xx | Extended APN-AMBR |  | Extended APN aggregate maximum bit rate 9.9.4.2a | O | TLV | 8 |
| yy | Extended QoS |  | Extended quality of service 9.9.4.3a | O | TLV | 12 |

#### 8.3.6.2	Transaction identifier

If the UE supports A/Gb mode or Iu mode or both, a network supporting mobility from S1 mode to A/Gb mode or Iu mode or both shall include this IE.

#### 8.3.6.3	Negotiated QoS

If the UE supports A/Gb mode or Iu mode or both, a network supporting mobility from S1 mode to A/Gb mode or Iu mode or both shall include the corresponding R99 QoS parameter values of a PDP context.

#### 8.3.6.4	Negotiated LLC SAPI

If the UE supports A/Gb mode, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.6.5	Radio priority

If the UE supports A/Gb mode, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.6.6	Packet flow identifier

If the UE supports A/Gb mode and BSS packet flow procedures, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.6.7	APN-AMBR

This IE is included in the message when the network wishes to transmit the APN-AMBR to the UE for possible uplink policy enforcement.

#### 8.3.6.8	ESM cause

The network shall include this IE, if the network allocated a PDN address of a PDN type which is different from the PDN type requested by the UE.

#### 8.3.6.9	Protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

This IE shall be included if the network supports Local IP address in the traffic flow aggregate description and TFT filter, the PDN Type is different from Non-IP, and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.6.10	Connectivity type

The network shall include the Connectivity type IE if:

-	the network is configured to indicate when a PDN connection is a LIPA PDN connection; and

-	the present PDN connection is a LIPA PDN connection.

#### 8.3.6.11	WLAN offload indication

This IE shall be included in the message when the network wishes to indicate if the UE is allowed to offload the traffic of the associated PDN connection to WLAN(s), as specified in subclause 9.9.4.18.

#### 8.3.6.12	NBIFOM container

This information element is used to transfer information associated with network-based IP flow mobility, see 3GPP TS 24.161 [36].

#### 8.3.6.13	Header compression configuration

The network may include the Header compression configuration IE if:

-	the network accepts an IP PDN type; and

-	control plane CIoT EPS optimization is selected.

#### 8.3.6.14	Control plane only indication

The network shall include the Control plane only indication IE if the associated PDN connection is only for control plane CIoT EPS optimization.

#### 8.3.6.15	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

This IE shall be included if the network supports Local IP address in the traffic flow aggregate description and TFT filter, the PDN Type is different from Non-IP, and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.6.16	Serving PLMN rate control

This IE shall be included when the network wishes to indicate the maximum number of uplink ESM DATA TRANSPORT messages including User data container IEs the UE is allowed to send per 6 minute interval.

#### 8.3.6.17	Extended APN aggregate maximum bit rate

This IE is included in the message when the network wishes to transmit the extended APN-AMBR to the UE for possible uplink policy enforcement.

#### 8.3.6.18	Extended quality of service

#### This IE is included in the message when the network wishes to use the extended maximum and guaranteed bit rate.

* * * Next Change * * *

#### 8.3.8.1	Message definition

This message is sent by the UE to the network to request the allocation of a dedicated bearer resource. See table 8.3.8.1.

Message type:	BEARER RESOURCE ALLOCATION REQUEST

Significance:		dual

Direction:			UE to network

# Table 8.3.8.1: BEARER RESOURCE ALLOCATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | EPS bearer identity | EPS bearer identity 9.3.2 | M | V | 1/2 |
|  | Procedure transaction identity | Procedure transaction identity 9.4 | M | V | 1 |
|  | Bearer resource allocation request message identity | Message type 9.8 | M | V | 1 |
|  | Linked EPS bearer identity | Linked EPS bearer identity 9.9.4.6 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.9.2.9 | M | V | 1/2 |
|  | Traffic flow aggregate | Traffic flow aggregate description 9.9.4.15 | M | LV | 2-256 |
|  | Required traffic flow QoS | EPS quality of service 9.9.4.3 | M | LV | 2-14 |
| 27 | Protocol configuration options | Protocol configuration options 9.9.4.11 | O | TLV | 3-253 |
| C- | Device properties | Device properties 9.9.2.0A | O | TV | 1 |
| 33 | NBIFOM container | NBIFOM container 9.9.4.19 | O | TLV | 3-257 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.9.4.26 | O | TLV-E | 4-65538 |
| xx | Extended QoS | Extended quality of service 9.9.4.3a | O | TLV | 12 |

#### 8.3.8.2	Protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.8.3	Device properties

The UE shall include this IE if the UE is configured for NAS signalling low priority.

#### 8.3.8.4	NBIFOM container

This information element is used to transfer information associated with network-based IP flow mobility, see 3GPP TS 24.161 [36].

#### 8.3.8.5	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.8.6	Extended quality of service

This IE is included in the message when the UE wishes to use the extended maximum and guaranteed bit rate.

* * * Next Change * * *

#### 8.3.10.1	Message definition

This message is sent by the UE to the network to request the modification of a dedicated bearer resource, or to request re-negotiation of header compression configuration associated to an EPS bearer context if the network has previously indicated support of Control plane CIoT EPS optimization and Header compression for control plane CIoT EPS optimization. See table 8.3.10.1.

Message type:	BEARER RESOURCE MODIFICATION REQUEST

Significance:		dual

Direction:			UE to network

# Table 8.3.10.1: BEARER RESOURCE MODIFICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | EPS bearer identity | EPS bearer identity 9.3.2 | M | V | 1/2 |
|  | Procedure transaction identity | Procedure transaction identity 9.4 | M | V | 1 |
|  | Bearer resource modification request message identity | Message type 9.8 | M | V | 1 |
|  | EPS bearer identity for packet filter | Linked EPS bearer identity 9.9.4.6 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.9.2.9 | M | V | 1/2 |
|  | Traffic flow aggregate | Traffic flow aggregate description 9.9.4.15 | M | LV | 2-256 |
| 5B | Required traffic flow QoS | EPS quality of service 9.9.4.3 | O | TLV | 3-15 |
| 58 | ESM cause | ESM cause 9.9.4.4 | O | TV | 2 |
| 27 | Protocol configuration options | Protocol configuration options 9.9.4.11 | O | TLV | 3-253 |
| C- | Device properties | Device properties 9.9.2.0A | O | TV | 1 |
| 33 | NBIFOM container | NBIFOM container 9.9.4.19 | O | TLV | 3-257 |
| 66 | Header compression configuration | Header compression configuration 9.9.4.22 | O | TLV | 5-257 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.9.4.26 | O | TLV-E | 4-65538 |
| xx | Extended QoS | Extended quality of service 9.9.4.3a | O | TLV | 12 |

#### 8.3.10.2	Required traffic flow QoS

This IE is included in the message when the UE requests a change of QoS for the indicated traffic flows and does not request to release all bearer resources for the EPS bearer context.

#### 8.3.10.3	ESM cause

This IE is included in the message when the UE requests the release of a dedicated bearer resource.

#### 8.3.10.4	Protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.10.5	Device properties

The UE shall include this IE if the UE is configured for NAS signalling low priority.

#### 8.3.10.6	NBIFOM container

This information element is used to transfer information associated with network-based IP flow mobility, see 3GPP TS 24.161 [36].

#### 8.3.10.7	Header compression configuration

This IE is included in the message if the UE wishes to re-negotiate header compression configuration associated to an EPS bearer context and both the UE and the network supports Control plane CIoT EPS optimization and header compression.

#### 8.3.10.8	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.10.9	Extended quality of service

This IE is included in the message when the UE wishes to use the extended maximum and guaranteed bit rate.

* * * Next Change * * *

#### 8.3.18.1	Message definition

This message is sent by the network to the UE to request modification of an active EPS bearer context, or to request re-negotiation of header compression configuration associated to an EPS bearer context if the UE has previously indicated support of Control plane CIoT EPS optimization and Header compression for control plane CIoT EPS optimization. See table 8.3.18.1.

Message type:	MODIFY EPS BEARER CONTEXT REQUEST

Significance:		dual

Direction:			network to UE

# Table 8.3.18.1: MODIFY EPS BEARER CONTEXT REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | EPS bearer identity | EPS bearer identity 9.3.2 | M | V | 1/2 |
|  | Procedure transaction identity | Procedure transaction identity 9.4 | M | V | 1 |
|  | Modify EPS bearer context request message identity | Message type 9.8 | M | V | 1 |
| 5B | New EPS QoS | EPS quality of service 9.9.4.3 | O | TLV | 3-15 |
| 36 | TFT | Traffic flow template 9.9.4.16 | O | TLV | 3-257 |
| 30 | New QoS | Quality of service 9.9.4.12 | O | TLV | 14-22 |
| 32 | Negotiated LLC SAPI | LLC service access point identifier 9.9.4.7 | O | TV | 2 |
| 8- | Radio priority | Radio priority 9.9.4.13 | O | TV | 1 |
| 34 | Packet flow Identifier | Packet flow Identifier 9.9.4.8 | O | TLV | 3 |
| 5E | APN-AMBR | APN aggregate maximum bit rate 9.9.4.2 | O | TLV | 4-8 |
| 27 | Protocol configuration options | Protocol configuration options 9.9.4.11 | O | TLV | 3-253 |
| C- | WLAN offload indication | WLAN offload acceptability 9.9.4.18 | O | TV | 1 |
| 33 | NBIFOM container | NBIFOM container 9.9.4.19 | O | TLV | 3-257 |
| 66 | Header compression configuration | Header compression configuration 9.9.4.22 | O | TLV | 5-257 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.9.4.26 | O | TLV-E | 4-65538 |
| xx | Extended APN-AMBR | Extended APN aggregate maximum bit rate 9.9.4.2a | O | TLV | 8 |
| yy | Extended QoS | Extended quality of service 9.9.4.3a | O | TLV | 12 |

#### 8.3.18.2	New EPS QoS

When the EPS QoS of the EPS bearer context is modified, the network shall include the modified EPS QoS assigned to the EPS bearer context.

#### 8.3.18.3	TFT

This IE provides the UE with packet filters.

#### 8.3.18.4	New QoS

If the UE supports A/Gb mode or Iu mode or both and when the corresponding R99 QoS of the EPS bearer context is modified, a network supporting mobility from S1 mode to A/Gb mode or Iu mode or both shall include the corresponding R99 QoS parameter values of a PDP context.

#### 8.3.18.5	Negotiated LLC SAPI

If the UE supports A/Gb mode and when the negotiated LLC SAPI is modified, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.18.6	Radio priority

If the UE supports A/Gb mode and when the radio priority is modified, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.18.7	Packet flow identifier

If the UE supports A/Gb mode and BSS packet flow procedures, a network supporting mobility from S1 mode to A/Gb mode shall include this IE.

#### 8.3.18.8	APN-AMBR

This IE is included when the APN-AMBR has been changed by the network.

#### 8.3.18.9	Protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is not supported by the UE or the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.18.10	WLAN offload indication

This IE shall be included in the message when the network wishes to indicate if the UE is allowed to offload the traffic of the associated PDN connection to WLAN(s), as specified in subclause 9.9.4.18.

#### 8.3.18.11	NBIFOM container

This information element is used to transfer information associated with network-based IP flow mobility, see 3GPP TS 24.161 [36].

#### 8.3.18.12	Header compression configuration

This IE is included in the message if the network wishes to re-negotiate header compression configuration associated to an EPS bearer context and both the UE and the network support Control plane CIoT EPS optimization and header compression.

#### 8.3.18.13	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE and the extended protocol configuration options is supported by both the UE and the network end-to-end for the PDN connection (see subclause 6.6.1.1).

#### 8.3.18.14	Extended APN aggregate maximum bit rate

This IE is included in the message when the extended APN-AMBR has been changed by the network.

#### 8.3.18.15	Extended quality of service

This IE is included in the message when the network wishes to use the extended maximum and guaranteed bit rate.

* * * Next Change * * *

#### 9.9.4.2a	Extended APN aggregate maximum bit rate

The purpose of the extended APN aggregate maximum bit rate information element is to indicate the initial subscribed APN-AMBR when the UE establishes a PDN connection or to indicate the new APN-AMBR if it is changed by the network.

The extended APN aggregate maximum bit rate information element is coded as shown in figure 9.9.4.2a.1 and table 9.9.4.2a.1.

The extended APN aggregate maximum bit rate is a type 4 information element with a length of 8 octets.

Editor's Note [WI: EDCE5-CT, CR#2891]:	It is FFS whether it is needed to specify the maximum bit rate value a NR capable UE can support, and if needed how to specify it.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended APN aggregate maximum bit rate IEI |  |  |  |  |  |  |  | octet 1 |
| Length of extended APN aggregate maximum bit rate contents |  |  |  |  |  |  |  | octet 2 |
| Unit for extended APN-AMBR for downlink |  |  |  |  |  |  |  | octet 3 |
| Extended APN-AMBR for downlink |  |  |  |  |  |  |  | octet 4 |
| Extended APN-AMBR for downlink (continued) |  |  |  |  |  |  |  | octet 5 |
| Unit for extended APN-AMBR for uplink |  |  |  |  |  |  |  | octet 6 |
| Extended APN-AMBR for uplink |  |  |  |  |  |  |  | octet 7 |
| Extended APN-AMBR for uplink (continued) |  |  |  |  |  |  |  | octet 8 |

# Figure 9.9.4.2a.1: Extended APN aggregate maximum bit rate information element

Table 9.9.4.2a.1: Extended APN aggregate maximum bit rate information element

Unit for for extended APN aggregate maximum bit rate (octet 3) 0 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 0 1 value is incremented in multiples of 100 kbps 0 0 0 0 0 0 0 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 0 0 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 0 0 1 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 0 0 1 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 0 0 1 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 0 0 1 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 0 1 0 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 0 1 0 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 0 1 0 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 0 1 0 1 1 value is incremented in multiples of 256 Gbps 0 0 0 0 0 1 1 0 0 value is incremented in multiples of 1 Tbps 0 0 0 0 0 1 1 0 1 value is incremented in multiples of 4 Tbps 0 0 0 0 0 1 1 1 0 value is incremented in multiples of 16 Tbps 0 0 0 0 0 1 1 1 1 value is incremented in multiples of 64 Tbps 0 0 0 0 1 0 0 0 0 value is incremented in multiples of 256 Tbps 0 0 0 0 1 0 0 0 1 value is incremented in multiples of 1 Pbps 0 0 0 0 1 0 0 1 0 value is incremented in multiples of 4 Pbps 0 0 0 0 1 0 0 1 1 value is incremented in multiples of 16 Pbps 0 0 0 0 1 0 1 0 0 value is incremented in multiples of 64 Pbps 0 0 0 0 1 0 1 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. APN aggregate maximum bit rate (octets 4 and 5) Octets 4 and 5 represent the binary coded value of APN aggregate maximum bit rate.

* * * Next Change * * *

#### 9.9.4.3a	Extended quality of service

The purpose of the extended quality of service information element is to specify the extended maximum bit rate and the extended guaranteed bit rate for a bearer context.

The extended quality of service information element is coded as shown in figure 9.9.4.3a.1 and table 9.9.4.3a.1.

The extended quality of service is a type 4 information element with a length of 12 octets.

Editor's Note [WI: EDCE5-CT, CR#2891]:	It is FFS whether it is needed to specify the maximum bit rate value a NR capable UE can support, and if needed how to specify it.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended EPS quality of service IEI |  |  |  |  |  |  |  | octet 1 |
| Length of EPS quality of service contents |  |  |  |  |  |  |  | octet 2 |
| Unit for maximum bit rate |  |  |  |  |  |  |  | octet 3 |
| Maximum bit rate for uplink |  |  |  |  |  |  |  | octet 4 |
| Maximum bit rate for uplink (continued) |  |  |  |  |  |  |  | octet 5 |
| Maximum bit rate for downlink |  |  |  |  |  |  |  | octet 6 |
| Maximum bit rate for downlink (continued) |  |  |  |  |  |  |  | octet 7 |
| Unit for guaranteed bit rate |  |  |  |  |  |  |  | octet 8 |
| Guaranteed bit rate for uplink |  |  |  |  |  |  |  | octet 9 |
| Guaranteed bit rate for uplink (continued) |  |  |  |  |  |  |  | octet 10 |
| Guaranteed bit rate for downlink |  |  |  |  |  |  |  | octet 11 |
| Guaranteed bit rate for downlink (continued) |  |  |  |  |  |  |  | octet 12 |

# Figure 9.9.4.3a.1: Extended quality of service information element

Table 9.9.4.3a.1: Extended quality of service information element

Unit for maximum bit rate (octet 3) 0 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 0 1 value is incremented in multiples of 100 kbps 0 0 0 0 0 0 0 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 0 0 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 0 0 1 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 0 0 1 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 0 0 1 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 0 0 1 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 0 1 0 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 0 1 0 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 0 1 0 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 0 1 0 1 1 value is incremented in multiples of 256 Gbps 0 0 0 0 0 1 1 0 0 value is incremented in multiples of 1 Tbps 0 0 0 0 0 1 1 0 1 value is incremented in multiples of 4 Tbps 0 0 0 0 0 1 1 1 0 value is incremented in multiples of 16 Tbps 0 0 0 0 0 1 1 1 1 value is incremented in multiples of 64 Tbps 0 0 0 0 1 0 0 0 0 value is incremented in multiples of 256 Tbps 0 0 0 0 1 0 0 0 1 value is incremented in multiples of 1 Pbps 0 0 0 0 1 0 0 1 0 value is incremented in multiples of 4 Pbps 0 0 0 0 1 0 0 1 1 value is incremented in multiples of 16 Pbps 0 0 0 0 1 0 1 0 0 value is incremented in multiples of 64 Pbps 0 0 0 0 1 0 1 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Maximum bit rate for uplink (octets 4 and 5) Octets 4 and 5 represent the binary coded value of maximum bit rate for uplink in units defined by octet 3. Maximum bit rate for downlink (octets 6 and 7) Octets 6 and 7 represent the binary coded value of maximum bit rate for downlink in units defined by octet 3. Unit for guaranteed bit rate (octet 8) The coding is identical to that of the unit for maximum bit rate (octet 3). Guaranteed bit rate for uplink (octets 9 and 10) Octets 9 and 10 represent the binary coded value of guaranteed bit rate for uplink in units defined by octet 8. Guaranteed bit rate for downlink (octets 11 and 12) Octets 11 and 12 represent the binary coded value of guaranteed bit rate for downlink in units defined by octet 8.