3GPP TSG-CT6 Meeting #88	C6-180149

# Kunming (P.R. of China), 17th-20th April 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.890 | CR | 0010 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the section regarding usage of IMSI as SUPI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2018-04-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarification is needed regarding usage of IMSI as SUPI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification is added regarding usage of IMSI as SUPI in TS 31.890. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Text in TS 31.890 might cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[17]	3GPP TS 38.211: "NR Physical channels and modulation"

[yy]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3"

…

### 6.2.4	Subscriber Permanent Identifier

According to TS 23.501 [9] clause 5.9.1, each subscriber in the 5G system shall be allocated one 5G Subscription Permanent Identifier (SUPI). As per TS 24.501 [yy] clause 5.3.2, the SUPI can be the IMSI or a Network Access Identifier (NAI). Though non-IMSI based SUPIs are possible by using NAI, the IMSI can be contained within the NAI for the SUPI.

A new USIM service table bit may be needed to indicate if the USIM supports SUPI types other than IMSI. If the USIM supports SUPI types other than IMSI, CT6 needs to create a new EF for that.

According to TR 33.899 [6] clause E.7.2.1.2, SA3 agreed that privacy for the 5G permanent subscription identifier is part of 5G system Phase 1. Furthermore, SA3 also agreed that subscription identifier privacy shall be based upon HN asymmetric key solution, as indicated in TR 33.899 [6] clause E.7.2.6.2. SA3 has also agreed in TS 33.501 [8] clause 5.1.5 that the home network public key shall be stored on the tamper resistant secure hardware component. Anyway, SA3 has not concluded in which entities (e.g. ME, USIM) the calculation of SUCI is done.

CT6 needs to expand the USIM capabilities depending on the SA3 decision about the calculation of SUCI:

- if the calculation is done on the USIMUICC, then a new mechanism to calculate the SUCI is required, either extending one of the existing commands or with the introduction of a new command. Moreover, CT6 may decide to create a new EF that is not accessible by the ME to store the home network public key, in order to allow a standardized way to manage it;

- if the calculation is performed in the ME, then CT6 needs to add a new EF for the storage of the home network public key.

Editor’s Note:	This section needs to be revised considering the SA3 decisions made in TS 33.501 [8].