3GPP TSG-CT Meeting #81	CP-182011

Gold Coast, Australia, 10th Sep 2018 - 11th Sep 2018	was C4-186598

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.571 | CR | 0032 | rev | 4 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding age of location, geographic information and other missing ones in the UserLocation type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Orange |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2018-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some important information are missing in the EutraLocation and NrLocation types: - Geographical-Information - Geodetic-Information - Age-Of-Location-Information - Location timestamp - RAN Node-ID Rev 4: Correct syntax error Remove useless "()" in pattern definition in Open API file annex. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Listed information are added to the mentionned types. Rev 4 changes: - Corrected syntax error in open API file. ":" character added at the end of "ueLocationTimestamp" property name declaration in EutraLocation and NrLocation objects. - Removed useless "()" in GeographicalInformation and geodeticInformation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The location informaiton is incomplete and services using this information such as emergency one will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 5.4.2; 5.4.3, 5.4.4; A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[3]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[4]	IETF RFC 1166: "Internet Numbers".

[5]	IETF RFC 5952: "A recommendation for IPv6 address text representation".

[6]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[7]	3GPP TS 23.003: "Numbering, addressing and identification".

[8]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[9]	IETF RFC 7807: "Problem Details for HTTP APIs".

[10]	IETF RFC 3339: "Date and Time on the Internet: Timestamps".

[11]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP) ".

[12]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

[13]	3GPP TS 24.007: " Mobile radio interface signalling layer 3; General aspects".

[14]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[15]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace"

[16]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[17]	IETF RFC 7042: "IANA Considerations and IETF Protocol and Documentation Usage for IEEE 802 Parameters".

[18]	IETF RFC 6733: "Diameter Base Protocol".

[X]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[Y]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[Y1]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[Y2]	ITU-T Recommendation Q.763 (1999): "Specifications of Signalling System No.7; Formats and codes".

* * * Next Change * * * *

#### 5.4.4.8	Type: EutraLocation

Table 5.4.4.8-1: Definition of type EutraLocation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tai | Tai | M | 1 | Tracking Area Identity |
| ecgi | Ecgi | M | 1 | E-UTRA Cell Identity |
| ageOfLocationInformation | integer | O | 0 1 | The value represents the elapsed time in minutes since the last network contact of the mobile station. Value "0" indicates that the location information was obtained after a successful paging procedure for Active Location Retrieval when the UE is in idle mode or after a successful NG-RAN location reporting procedure with the eNB when the UE is in connected mode. Any other value than "0" indicates that the location information is the last known one. See 3GPP TS 29.002 [X] subclause 17.7.8. |
| ueLocationTimestamp | DateTime | O | 0..1 | The value represents the UTC time when the UeLocation information was acquired. |
| geographicalInformation | string | O | 0..1 | Refer to geographical Information. See 3GPP TS 23.032 [Y1] subclause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F; |
| geodeticInformation | string | O | 0..1 | Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) [Y2] subclause 3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F. |
| globalNgenbId | GlobalRanNodeId | O | 0..1 | It indicates the global identity of the ng-eNodeB in which the UE is currently located. See 3GPP TS 38.413 [Y] subclause 9.3.1.8. |

* * * Next Change * * * *

#### 5.4.4.9	Type: NrLocation

Table 5.4.4.9-1: Definition of type NrLocation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tai | Tai | M | 1 | Tracking Area Identity |
| ncgi | Ncgi | M | 1 | NR Cell Identity |
| ageOfLocationInformation | integer | O | 0 1 | The value represents the elapsed time in minutes since the last network contact of the mobile station. Value "0" indicates that the location information was obtained after a successful paging procedure for Active Location Retrieval when the UE is in idle mode or after a successful NG-RAN location reporting procedure with the gNB when the UE is in connected mode. Any other value than "0" indicates that the location information is the last known one. See 3GPP TS 29.002 [X] subclause 17.7.8. |
| ueLocationTimestamp | DateTime | O | 0..1 | The value represents the UTC time when the UeLocation information was acquired. |
| geographicalInformation | string | O | 0..1 | Refer to geographical Information. See 3GPP TS 23.032 [Y1] subclause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F; |
| geodeticInformation | string | O | 0..1 | Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) [Y2] subclause 3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F. |
| globalGnbId | GlobalRanNodeId | O | 0..1 | It indicates the global identity of the gNodeB in which the UE is currently located. See 3GPP TS 38.413 [Y] subclause 9.3.1.6. |

* * * Next Change * * * *

## A.2	Data related to Common Data Types

openapi: 3.0.0

info:

version: '1.R15.0.0'

title: 'Common Data Types'

description: 'Common Data Types'

paths: {}

#

# Definition based on 3GPP TS 29.571 version 0.7.0

#

components:

schemas:

#

# Common Data Types for  Generic usage definitiones as defined in subclause 5.2

#

# COMMON SIMPLE DATA TYPES

#

Binary:

format: binary

type: string

Bytes:

format: byte

type: string

Date:

format: date

type: string

DateTime:

format: date-time

type: string

DiameterIdentity:

type: string

pattern: '^([A-Za-z0-9]+(-[A-Za-z0-9]+).)+[a-z]{2,}$'

Double:

format: double

type: number

DurationSec:

type: integer

Float:

format: float

type: number

Int32:

format: int32

type: integer

Int64:

type: integer

format: int64

Ipv4Addr:

type: string

Ipv6Addr:

type: string

Ipv6Prefix:

type: string

MacAddr48:

type: string

pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$'

SupportedFeatures:

type: string

pattern: '^[A-Fa-f0-9]*$'

Uinteger:

type: integer

minimum: 0

Uint16:

type: integer

minimum: 0

maximum: 65535

Uint32:

format: int32

type: integer

minimum: 0

Uint64:

format: int64

type: integer

minimum: 0

Uri:

type: string

TimeZone:

format: TimeZone

type: string

#

#   ENUMERATED

#

PatchOperation:

anyOf:

- type: string

enum:

- "add"

- "copy"

- "move"

- "remove"

- "replace"

- "test"

- type: string

#

#  COMMON STRUCTURED DATA TYPES

#

ProblemDetails:

type: object

properties:

type:

$ref: '#/components/schemas/Uri'

title:

type: string

status:

type: integer

instance:

$ref: '#/components/schemas/Uri'

cause:

type: string

invalidParams:

type: array

items:

$ref: '#/components/schemas/InvalidParam'

minItems: 0

Link:

type: object

properties:

href:

$ref: '#/components/schemas/Uri'

PatchItem:

type: object

properties:

op:

$ref: '#/components/schemas/PatchOperation'

path:

type: string

from:

type: string

value:

nullable: true

required:

- op

- path

LinksValueSchema:

oneOf:

- type: array

items:

$ref: '#/components/schemas/Link'

minItems: 1

- $ref: '#/components/schemas/Link'

SelfLink:

type: object

properties:

self:

$ref: '#/components/schemas/Link'

required:

- self

InvalidParam:

type: object

properties:

param:

type: string

reason:

type: string

required:

- param

#

# Data Types related to Subscription, Identification and Numbering as defined in subclause 5.3

#

# SIMPLE DATA TYPES

#

Dnn:

type: string

Gpsi:

type: string

pattern: '^(msisdn-[0-9]{5,15}|extid-.+|.+)$'

GroupId:

type: string

pattern: '^(groupid-[A-Fa-f0-9]{14,34}|.+)$'

Pei:

type: string

pattern: '^(imei-[0-9]{15}|imeisv-[0-9]{16}|.+)$'

Supi:

format: Supi

type: string

pattern: '^(imsi-[0-9]{5,15}|nai-.+|.+)$'

NfInstanceId:

type: string

format: uuid

AmfId:

type: string

pattern: '^[A-Fa-f0-9]{6}$'

RfspIndex:

type: integer

minimum: 1

maximum: 256

#

# STRUCTURED DATA TYPES

#

Guami:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

amfId:

$ref: '#/components/schemas/AmfId'

required:

- plmnId

- amfId

NetworkId:

type: object

properties:

mnc:

$ref: '#/components/schemas/Mnc'

mcc:

$ref: '#/components/schemas/Mcc'

minProperties: 1

#

# Data Types related to 5G Network as defined in subclause 5.4

#

# SIMPLE DATA TYPES

#

ApplicationId:

type: string

PduSessionId:

type: integer

minimum: 0

maximum: 255

Mcc:

type: string

pattern: '^\d{3}$'

Mnc:

type: string

pattern: '^\d{2,3}$'

Tac:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

EutraCellId:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

NrCellId:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

#

# Enumerations

#

AccessType:

type: string

enum:

- 3GPP_ACCESS

- NON_3GPP_ACCESS

RatType:

anyOf:

- type: string

enum:

- NR

- EUTRA

- WLAN

- VIRTUAL

- type: string

PduSessionType:

anyOf:

- type: string

enum:

- IPV4V6

- IPV4

- IPV6

- UNSTR

- ETHER

- type: string

UpIntegrity:

anyOf:

- type: string

enum:

- "REQUIRED"

- "PREFERRED"

- "NOT_NEEDED"

- type: string

UpConfidentiality:

anyOf:

- type: string

enum:

- "REQUIRED"

- "PREFERRED"

- "NOT_NEEDED"

- type: string

SscMode:

anyOf:

- type: string

enum:

- "SSC_MODE_1"

- "SSC_MODE_2"

- "SSC_MODE_3"

- type: string

#

# Structured Data Types

#

# the following data type is FFS

DefaultQosInformation:

type: string #FFS

Snssai:

type: object

properties:

sst:

$ref: '#/components/schemas/Uinteger'

minimum: 0

maximum: 255

sd:

type: string

required:

- sst

PlmnId:

type: object

properties:

mcc:

$ref: '#/components/schemas/Mcc'

mnc:

$ref: '#/components/schemas/Mnc'

required:

- mcc

- mnc

Tai:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

tac:

$ref: '#/components/schemas/Tac'

required:

- plmnId

- tac

Ecgi:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

# PLMN Identity

eutraCellId:

$ref: '#/components/schemas/EutraCellId'

required:

- plmnId

- eutraCellId

Ncgi:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

nrCellId:

$ref: '#/components/schemas/NrCellId'

required:

- plmnId

- nrCellId

UserLocation:

type: object

properties:

eutraLocation:

$ref: '#/components/schemas/EutraLocation'

nrLocation:

$ref: '#/components/schemas/NrLocation'

n3gaLocation:

$ref: '#/components/schemas/N3gaLocation'

EutraLocation:

type: object

properties:

tai:

$ref: '#/components/schemas/Tai'

ecgi:

$ref: '#/components/schemas/Ecgi'

ageOfLocationInformation:

type: integer

minimum: 0

maximum: 32767

ueLocationTimestamp:

$ref: '#/components/schemas/DateTime'

geographicalInformation:

type: string

pattern: '^[0-9A-F]{16}$'

geodeticInformation:

type: string

pattern: '^[0-9A-F]{20}$'

globalNgenbId:

$ref: '#/components/schemas/GlobalRanNodeId'

required:

- tai

- ecgi

NrLocation:

type: object

properties:

tai:

$ref: '#/components/schemas/Tai'

ncgi:

$ref: '#/components/schemas/Ncgi'

ageOfLocationInformation:

type: integer

minimum: 0

maximum: 32767

ueLocationTimestamp:

$ref: '#/components/schemas/DateTime'

geographicalInformation:

type: string

pattern: '^[0-9A-F]{16}$'

geodeticInformation:

type: string

pattern: '^[0-9A-F]{20}$'

globalGnbId:

$ref: '#/components/schemas/GlobalRanNodeId'

required:

- tai

- ncgi

N3gaLocation:

type: object

properties:

n3gppTai:

$ref: '#/components/schemas/Tai'

n3IwfId:

type: string

pattern: '^[A-Fa-f0-9]+$'

ueIpv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

ueIpv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

portNumber:

$ref: '#/components/schemas/Uinteger'

required:

- n3gppTai

- n3IwfId

UpSecurity:

type: object

properties:

upIntegr:

$ref: '#/components/schemas/UpIntegrity'

upConfid:

$ref: '#/components/schemas/UpConfidentiality'

required:

- upIntegr

- upConfid

RefToBinaryData:

type: object

properties:

contentId:

type: string

required:

- contentId

#

# Data related to Data Types related to 5G QoS as defined in subclause 5.5

#

#

# SIMPLE DATA TYPES

#

#

Qfi:

type: integer

minimum: 0

maximum: 63

5qi:

type: integer

minimum: 0

maximum: 255

BitRate:

type: string

pattern: '^\d+(\.\d+)? (bps|Kbps|Mbps|Gbps|Tbps)$'

ArpPriorityLevel:

type: integer

minimum: 1

maximum: 15

5qiPriorityLevel:

type: integer

minimum: 1

maximum: 127

PacketDelBudget:

type: integer

minimum: 1

PacketErrRate:

type: integer

PacketLossRate:

type: integer

minimum: 0

maximum: 1000

AverWindow:

type: string #FFS

MaxDataBurstVol:

type: integer

minimum: 1

# Enumerations

#

PreemptionCapability:

anyOf:

- type: string

enum:

- NOT_PREEMPT

- MAY_PREEMPT

- type: string

DelayCritical:

anyOf:

- type: string

enum:

- CRITICAL

- NON_CRITICAL

- type: string

PreemptionVulnerability:

anyOf:

- type: string

enum:

- NOT_PREEMPTABLE

- PREEMPTABLE

- type: string

ReflectiveQoSAttribute:

anyOf:

- type: string

enum:

- RQOS

- NO_RQOS

- type: string

NotificationControl:

anyOf:

- type: string

enum:

- REQUESTED

- NOT_REQUESTED

- type: string

#

# Structured Data Types

#

Arp:

type: object

properties:

priorityLevel:

$ref: '#/components/schemas/ArpPriorityLevel'

preemptCap:

$ref: '#/components/schemas/PreemptionCapability'

preemptVuln:

$ref: '#/components/schemas/PreemptionVulnerability'

required:

- priorityLevel

- preemptCap

- preemptVuln

Ambr:

type: object

properties:

uplink:

$ref: '#/components/schemas/BitRate'

downlink:

$ref: '#/components/schemas/BitRate'

required:

- uplink

- downlink

#

# HTTP responses

#

responses:

'400':

description: Bad request

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'401':

description: Unauthorized

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'405':

description: Method Not Allowed

'408':

description: Request Timeout

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'409':

description: Conflict

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'410':

description: Gone

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'411':

description: Length Required

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'413':

description: Payload Too Large

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'414':

description: URI Too Long

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'415':

description: Unsupported Media Type

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'501':

description: Not Implemented

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'504':

description: Gateway Timeout

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

default:

description: Generic Error

* * * End of Changes * * * *