3GPP TSG-CT Meeting #81	CP-182012

Gold Coast, Australia, 10th Sep 2018 - 11th Sep 2018	was C4-185373

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 0001 | rev | 2 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Implementing the Indirect Delivery method for the GET method to retrieve NF instances |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Orange |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2018-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 29.501 sub-clause 4.9 defines 4 different possible methods that an API may implement when a NF Service Producer needs to return the representations of multiple resources to a NF Service Consumer, e.g. during the query of a large collection of resources. TS 29.510 defines a method for an NF to query the NRF to get some of the instances of a certain nf-type registered in the NRF. The current data structure supported by the GET Response Body on the {apiRoot}/nnrf-nfm/v1/nf-instances/{nfInstanceID} resource (an array of URI) does not comply with any of the 4 solutions described. Rev 2: there is a clash with Ericsson CR C4-186289. The later allows returning an empty object in response to a GET to retrieve NF instances whereas Orange C4-185373 doesn't. The following CR resolves the clash by allowing returning empty object. Reference section for UriList data type in Table 6.1.6.1-1 is also incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to use the mechanism and data structure defined in TS 29.501 sub-clause 4.9.4 Indirect Delivery. Reference to JSON IETF RFC 8259 is added. 3GPP HAL hypermedia is added to the list of media of the API Rev 2 changes: The presence of attribute "_links" in Table 6.1.6.2.X-1 defining the data type UriList is made optional to allow returning an empty object and the OpenAPI file reflects the change. _Links in the text of table 6.1.3.2.3.1-3 describing the UriList data type is changed to "_links". Reference section for UriList data type in Table 6.1.6.1-1 is changed to 6.1.6.2.X. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The API design does not follow the recommendations given in the TS 29.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2.3.1; 6.1.6.1; 6.1.6.2.X; 6.1.6.2.Y; Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	GET

This method retrieves a list of all NF instances currently registered in the NRF. This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nf-type | NFType | O | 0..1 | The type of NF to restrict the list of returned NF Instances. |
| limit | integer | O | 0..1 | Maximum number of items to be returned in this query. |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UriList | M | 1 | 200 OK | The response body contains an "_links" object containing the URI of each registered NF in the NRF. |
| ProblemDetails | M | 1 | 404 Not Found | This status code shall be returned if the queried NF type does not have any NF instances registered with the NRF. |

* * * Next Change * * * *

### 6.1.6	Data Model

#### 6.1.6.1	General

This subclause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nnrf service based interface protocol.

Table 6.1.6.1-1: Nnrf_NFManagement specific Data Types

| Data type | Section defined | Description |
| --- | --- | --- |
| NFProfile | 6.1.6.2.2 |  |
| NFService | 6.1.6.2.3 |  |
| DefaultNotificationSubscription | 6.1.6.2.4 | Data structure for specifying the notifications the NF service subscribes by default along with callback URI. |
| IpEndPoint | 6.1.6.2.5 |  |
| UdrInfo | 6.1.6.2.6 |  |
| UdmInfo | 6.1.6.2.7 |  |
| AusfInfo | 6.1.6.2.8 |  |
| SupiRange | 6.1.6.2.9 |  |
| IdentityRange | 6.1.6.2.10 |  |
| AmfInfo | 6.1.6.2.11 |  |
| SmfInfo | 6.1.6.2.12 |  |
| UpfInfo | 6.1.6.2.13 | Information related to UPF |
| SnssaiUpfInfoItem | 6.1.6.2.14 |  |
| DnnUpfInfoItem | 6.1.6.2.15 |  |
| SubscriptionData |  | 6.1.6.2.16 |  |  |  |
| NotificationData |  | 6.1.6.2.17 |  |  |  |
| NFRegistrationData |  | 6.1.6.2.18 |  |  |  |
| NFServiceVersion |  | 6.1.6.2.19 |  | Contains the version details of an NF service. |  |
| PcfInfo |  | 6.1.6.2.20 |  |  |  |
| BsfInfo |  | 6.1.6.2.21 |  |  |  |
| Ipv4AddressRange |  | 6.1.6.2.22 |  |  |  |
| Ipv6PrefixRange |  | 6.1.6.2.23 |  |  |  |
| InterfaceUpfInfoItem |  | 6.1.6.2.24 |  |  |  |
| UriList |  | 6.1.6.2.X |  |  |  |
| Fqdn |  | 6.1.6.3.2 |  |  |  |
| NFType |  | 6.1.6.3.3 |  |  |  |
| NotificationType |  | 6.1.6.3.4 |  |  |  |
| TransportProtocol |  | 6.1.6.3.5 |  |  |  |
| NotificationEventType |  | 6.1.6.3.6 |  |  |  |
| NFStatus |  | 6.1.6.3.7 |  |  |  |
| DataSetId |  | 6.1.6.3.8 |  |  |  |
| UPInterfaceType |  | 6.1.6.3.9 |  |  |  |

Table 6.1.6.1-2 specifies data types re-used by the Nnrf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnrf service based interface.

Table 6.1.6.1-2: Nnrf_NFManagement re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| N1MessageClass | 3GPP TS 29.518 [6] | The N1 message type |
| N2InformationClass | 3GPP TS 29.518 [6] | The N2 information type |
| IPv4Addr | 3GPP TS 29.571 [7] |  |
| IPv6Addr | 3GPP TS 29.571 [7] |  |
| IPv6Prefix | 3GPP TS 29.571 [7] |  |
| Uri | 3GPP TS 29.571 [7] |  |
| Dnn | 3GPP TS 29.571 [7] |  |
| SupportedFeatures | 3GPP TS 29.571 [7] |  |
| Snssai | 3GPP TS 29.571 [7] |  |
| PlmnId | 3GPP TS 29.571 [7] |  |
| Guami | 3GPP TS 29.571 [7] |  |
| Tai | 3GPP TS 29.571 [7] |  |
| Ecgi | 3GPP TS 29.571 [7] |  |
| NfInstanceId | 3GPP TS 29.571 [7] |  |
| LinksValueSchema | 3GPP TS 29.571 [7] | 3GPP Hypermedia link |

* * * Next Change * * * *

##### 6.1.6.2.X	Type: UriList

Table 6.1.6.2.X-1: Definition of type UriList

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| _links | map(LinksValueSchema) | O | 1..N | See sub-clause 4.9.4 of 3GPP TS 29.501 [5] for the description of the members. |

* * * Next Change * * * *

##### 6.1.6.3.Y	Relation Types

###### 6.1.6.3.Y.1	General

This clause describes the possible relation types defined within NRF API. See sub-clause 4.7.5.2 of 3GPP TS 29.501 [5] for the description of the relation types.

Table 6.1.6.3.Y.1-1: supported registered relation types

Relation Name

self

item

* * * Next Change * * * *

Annex A (normative):	OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the Nnrf Service API(s). It consists of OpenAPI 3.0.0 specifications, in YAML format.

## A.2	Nnrf_NFManagement API

openapi: 3.0.0

info:

version: '1.PreR15.0.0'

title: 'NRF NFManagement Service'

description: 'NRF NFManagement Service'

security:

- oAuth2Clientcredentials: []

paths:

/nf-instances:

get:

summary: Retrieves a collection of NF Instances

operationId: GetNFInstances

tags:

- NF Instances (Store)

parameters:

- name: nf-type

in: query

description: Type of NF

required: false

schema:

$ref: '#/components/schemas/NFType'

- name: limit

in: query

description: How many items to return at one time

required: false

schema:

type: integer

responses:

'200':

description: Expected response to a valid request

content:

application/3gppHal+json:

schema:

type: object

properties:

_links:

type: object

description: 'list of the URI of NF instances. It has two members whose names are item and self. The item one contains an array of URIs.'

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LinksValueSchema'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

/nf-instances/{nfInstanceID}:

get:

summary: Read the profile of a given NF Instance

operationId: GetNFInstance

tags:

- NF Instance ID (Document)

parameters:

- name: nfInstanceID

in: path

description: Unique ID of the NF Instance

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/NFProfile'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

put:

summary: Register a new NF Instance

operationId: RegisterNFInstance

tags:

- NF Instance ID (Document)

parameters:

- name: nfInstanceID

in: path

required: true

description: Unique ID of the NF Instance to register

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/NFProfile'

required: true

responses:

'200':

description: OK (Profile Replacement)

content:

application/json:

schema:

$ref: '#/components/schemas/NFProfile'

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/NFRegistrationData'

'400':

description: Bad request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

patch:

summary: Update NF Instance profile

operationId: UpdateNFInstance

tags:

- NF Instance ID (Document)

parameters:

- name: nfInstanceID

in: path

required: true

description: Unique ID of the NF Instance to update

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/NFProfile'

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

delete:

summary: Deregisters a given NF Instance

operationId: DeregisterNFInstance

tags:

- NF Instance ID (Document)

parameters:

- name: nfInstanceID

in: path

required: true

description: Unique ID of the NF Instance to deregister

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

responses:

'204':

description: Expected response to a successful deregistration

/subscriptions:

post:

summary: Create a new subscription

operationId: CreateSubscription

tags:

- Subscriptions (Collection)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/SubscriptionData'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SubscriptionData'

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

callbacks:

onNFStatusEvent:

'{$request.body#/nfStatusNotificationUri}':

post:

requestBody:

description: Notification Payload

content:

application/json:

schema:

$ref: '#/components/schemas/NotificationData'

responses:

'200':

description: Expected response to a successful callback processing

/subscriptions/{subscriptionID}:

delete:

summary: Deletes a subscription

operationId: RemoveSubscription

tags:

- Subscription ID (Document)

parameters:

- name: subscriptionID

in: path

required: true

description: Unique ID of the subscription to remove

schema:

type: string

responses:

'204':

description: Expected response to a successful subscription removal

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '/oauth2/token'

scopes: {}

schemas:

NFProfile:

type: object

required:

- nfInstanceId

- nfType

- nfStatus

properties:

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nfType:

$ref: '#/components/schemas/NFType'

nfStatus:

$ref: '#/components/schemas/NFStatus'

plmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

sNssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

nsiList:

type: array

items:

type: string

fqdn:

$ref: '#/components/schemas/Fqdn'

interPlmnFqdn:

$ref: '#/components/schemas/Fqdn'

ipv4Addresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

ipv6Prefixes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

capacity:

type: integer

load:

type: integer

minimum: 0

maximum: 100

locality:

type: string

udrInfo:

$ref: '#/components/schemas/UdrInfo'

udmInfo:

$ref: '#/components/schemas/UdmInfo'

ausfInfo:

$ref: '#/components/schemas/AusfInfo'

amfInfo:

$ref: '#/components/schemas/AmfInfo'

smfInfo:

$ref: '#/components/schemas/SmfInfo'

upfInfo:

$ref: '#/components/schemas/UpfInfo'

pcfInfo:

$ref: '#/components/schemas/PcfInfo'

bsfInfo:

$ref: '#/components/schemas/BsfInfo'

nfServices:

type: array

items:

$ref: '#/components/schemas/NFService'

NFService:

type: object

required:

- serviceInstanceId

- serviceName

- version

- schema

properties:

serviceInstanceId:

type: string

serviceName:

type: string

version:

type: array

items:

$ref: '#/components/schemas/NFServiceVersion'

schema:

type: string

fqdn:

$ref: '#/components/schemas/Fqdn'

interPlmnFqdn:

$ref: '#/components/schemas/Fqdn'

ipEndPoints:

type: array

items:

$ref: '#/components/schemas/IpEndPoint'

apiPrefix:

type: string

defaultNotificationSubscriptions:

type: array

items:

$ref: '#/components/schemas/DefaultNotificationSubscription'

allowedPlmns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

allowedNfTypes:

type: array

items:

$ref: '#/components/schemas/NFType'

allowedNfDomains:

type: array

items:

type: string

allowedNssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

capacity:

type: integer

load:

type: integer

minimum: 0

maximum: 100

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

NFType:

anyOf:

- type: string

enum:

- NRF

- UDM

- AMF

- SMF

- AUSF

- NEF

- PCF

- SMSF

- NSSF

- UDR

- LMF

- GMLC

- 5G_EIR

- SEPP

- UPF

- N3IWF

- AF

- UDSF

- BSF

- CHF

- type: string

Fqdn:

type: string

IpEndPoint:

type: object

properties:

ipv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

ipv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

transport:

$ref: '#/components/schemas/TransportProtocol'

port:

type: integer

SubscriptionData:

type: object

required:

- nfStatusNotificationUri

properties:

nfStatusNotificationUri:

type: string

reqNotifEvents:

type: array

items:

$ref: '#/components/schemas/NotificationEventType'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

subscrCondition:

oneOf:

- type: object

properties:

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

- type: object

properties:

nfType:

$ref: '#/components/schemas/NFType'

serviceName:

type: string

nfSetId:

type: string

UdrInfo:

type: object

properties:

groupId:

type: string

supiRanges:

type: array

items:

$ref: '#/components/schemas/SupiRange'

gpsiRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

externalGroupIdentityfiersRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

supportedDataSets:

type: array

items:

$ref: '#/components/schemas/DataSetId'

SupiRange:

type: object

properties:

start:

type: string

end:

type: string

pattern:

type: string

IdentityRange:

type: object

properties:

start:

type: string

end:

type: string

pattern:

type: string

DataSetId:

anyOf:

- type: string

enum:

- SUBSCRIPTION

- POLICY

- EXPOSURE

- APPLICATION

- type: string

UdmInfo:

type: object

properties:

groupId:

type: string

supiRanges:

type: array

items:

$ref: '#/components/schemas/SupiRange'

gpsiRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

externalGroupIdentityfiersRanges:

type: array

items:

$ref: '#/components/schemas/IdentityRange'

routingIndicator:

type: string

AusfInfo:

type: object

properties:

groupId:

type: string

supiRanges:

type: array

items:

$ref: '#/components/schemas/SupiRange'

routingIndicator:

type: string

AmfInfo:

type: object

required:

- amfSetId

- amfRegionId

- guamiList

properties:

amfSetId:

type: string

amfRegionId:

type: string

guamiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

backupInfoAmfFailure:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

backupInfoAmfRemoval:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

SmfInfo:

type: object

required:

- dnnList

properties:

dnnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

pgwFqdn:

$ref: '#/components/schemas/Fqdn'

UpfInfo:

type: object

required:

- sNssaiUpfInfoList

properties:

sNssaiUpfInfoList:

type: array

items:

$ref: '#/components/schemas/SnssaiUpfInfoItem'

minItems: 1

smfServingArea:

type: array

items:

type: string

interfaceUpfInfoList:

type: array

items:

$ref: '#/components/schemas/InterfaceUpfInfoItem'

SnssaiUpfInfoItem:

type: object

required:

- sNssai

- dnnUpfInfoList

properties:

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnnUpfInfoList:

type: array

items:

$ref: '#/components/schemas/DnnUpfInfoItem'

minItems: 1

DnnUpfInfoItem:

type: object

required:

- dnn

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

InterfaceUpfInfoItem:

type: object

required:

- interfaceType

properties:

interfaceType:

$ref: '#/components/schemas/UPInterfaceType'

ipv4EndpointAddress:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6EndpointAddress:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

ipv6EndpointPrefix:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

endpointFqdn:

$ref: '#/components/schemas/Fqdn'

networkInstance:

type: string

UPInterfaceType:

anyOf:

- type: string

enum:

- N3

- N6

- N9

- type: string

PcfInfo:

type: object

properties:

dnnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

BsfInfo:

type: object

properties:

ipv4AdddressRanges:

type: array

items:

$ref: '#/components/schemas/Ipv4AddressRange'

Ipv6PrefixRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6PrefixRange'

Ipv4AddressRange:

type: object

properties:

start:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

end:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

Ipv6PrefixRange:

type: object

properties:

start:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

end:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

DefaultNotificationSubscription:

type: object

required:

- notificationType

- callbackUri

properties:

notificationType:

$ref: '#/components/schemas/NotificationType'

callbackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

n1MessageClass:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N1MessageClass'

n2InformationClass:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InformationClass'

NotificationType:

anyOf:

- type: string

enum:

- N1_MESSAGES

- N2_INFORMATION

- LOCATION_NOTIFICATION

- type: string

TransportProtocol:

anyOf:

- type: string

enum:

- TCP

- type: string

NotificationEventType:

anyOf:

- type: string

enum:

- NF_REGISTERED

- NF_DEREGISTERED

- NF_PROFILE_CHANGED

- type: string

NotificationData:

type: object

required:

- event

- nfInstanceUri

properties:

event:

$ref: '#/components/schemas/NotificationEventType'

nfInstanceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

newProfile:

$ref: '#/components/schemas/NFProfile'

NFStatus:

anyOf:

- type: string

enum:

- REGISTERED

- SUSPENDED

- type: string

NFRegistrationData:

type: object

required:

- heartBeatTimer

- nfProfile

properties:

heartBeatTimer:

type: integer

nfProfile:

$ref: '#/components/schemas/NFProfile'

NFServiceVersion:

type: object

required:

- apiVersionInUri

- apiFullVersion

properties:

apiVersionInUri:

type: string

apiFullVersion:

type: string

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

externalDocs:

description: Documentation

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.510/29510-070.zip'

* * * End of Changes * * * *