3GPP TSG-CT Meeting #82	CP-183079

# Sorrento, Italy, 10 - 11 December 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0828 | rev |  | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extend Get Identity command to provide non-IMSI based SUPI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Gemalto |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2018-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As defined in 3GPP TS 23.003 and TS 33.501 that - a SUPI can be either an IMSI or a non-IMSI based Network Specific Identifier; - and “The ME shall read the SUCI calculation information from the USIM, including the SUPI, the Routing Indicator, the Home Network Public Key, the Home Network Public Key Identifier and the list of protection scheme identifiers.” Consequently, the current specification of 3GPP TS 31.102 need be aligned with this new requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the Get_Identity command to return a SUPI that is a non-IMSI based Network Specific Identity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 3GPP TS 31.102 is not aligned with 3GPP TS 33.501 and 3GPP TS 23.003 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.8, 4.4.11.1, 7.5.2, 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Begin change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 21.111: "USIM and IC Card Requirements".

[2]	3GPP TS 22.011: "Service accessibility".

…

[104]		3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[105]		3GPP TS 33.501: "Security architecture and procedures for 5G System".

[106]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[xx]	IETF RFC 7542: "The Network Access Identifier".

***** Next change *****

### 4.2.8	EFUST (USIM Service Table)

This EF indicates which services are available. If a service is not indicated as available in the USIM, the ME shall not select this service.

| Identifier: '6F38' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '04' |  |  |  |  |  |  |
| File size: X bytes, (X ≥ 1) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n1 to n8 |  |  | M |  | 1 byte |
| 2 | Services n9 to n16 |  |  | O |  | 1 byte |
| 3 | Services n17 to n24 |  |  | O |  | 1 byte |
| 4 | Services n25 to n32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services n(8X-7) to n(8X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1: | Local Phone Book |
|  | Service n°2: | Fixed Dialling Numbers (FDN) |
|  | […] |  |
|  | Service n°122 | 5GS Mobility Management Information |
|  | Service n°123 | 5G Security Parameters |
|  | Service n°124 | Subscription identifier privacy support |
|  | Service n°125 | SUCI calculation by the USIM |
|  | Service n°126 | UAC Access Identities support |
|  | Service n°127 | Expect control plane-based Steering of Roaming information during initial registration in VPLMN |
|  | Service n°128 | Call control on PDU Session by USIM |
|  | Service n°xxx | Non-IMSI based SUPI support |

The EF shall contain at least one byte. Further bytes may be included, but if the EF includes an optional byte, then it is mandatory for the EF to also contain all bytes before that byte. Other services are possible in the future and will be coded on further bytes in the EF. The coding falls under the responsibility of the 3GPP.

Service n°46 can only be declared "available" if service n°45 is declared "available".

Service n°95, n°99 and n°115 shall not be declared "available" if an ISIM application is present on the UICC.

Service n°125 shall only be taken into account if Service n°xxx is declared "available". If Service n°124 and Service n°125 are declared "available", the "SUCI calculation is to be performed by the USIM". If Service n°124 is declared "available" and Service n°125 is not declared "available", the "SUCI calculation is to be performed by the ME".

Coding:

1 bit is used to code each service:
bit = 1: service available;
bit = 0: service not available.

-	Service available means that the USIM has the capability to support the service and that the service is available for the user of the USIM unless the service is identified as "disabled" in EFEST.
Service not available means that the service shall not be used by the USIM user, even if the USIM has the capability to support the service.

First byte:

|  |  | b8 |  | b7 |  | b6 |  | B5 |  | B4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°3 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°4 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°5 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°6 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°7 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°8 |

Second byte:

|  |  | b8 |  | b7 |  | b6 |  | B5 |  | B4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°9 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°10 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°11 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°12 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°13 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°14 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°16 |

etc.

***** Next change *****

### 4.4.11	Contents of files at the DF5GS level

#### 4.4.11.1	Introduction

This clause describes the files that are specific for 5GS.

DF5GS shall be present at the ADFUSIM level if any of the following services are "available" in EFUST (USIM Service Table):

| Service n°122 | 5GS Mobility Management Information |
| --- | --- |
| Service n°123 | 5G Security Parameters |
| Service n°124 | Subscription identifier privacy support |
| Service n°125 | SUCI calculation by the USIM |
| Service n°126 | UAC Access Identities support |
| Service n°127 | Steering of UE in VPLMN |
| Service n°xxx | Non-IMSI based SUPI support |

***** Next change *****

## 7.5	GET IDENTITY

### 7.5.1	Command description

The function can be used in the following contexts:

-	a SUCI context, to retrieve the SUCI when "SUCI calculation is to be performed by the USIM".

-	a Non-IMSI based SUPI context, to retrieve the Non-IMSI based SUPI when "Non-IMSI based SUPI support" service is available.

The function is related to a particular USIM and shall not be executable unless the USIM application has been selected and activated, and the current directory is the USIM ADF or any subdirectory under this ADF and a successful PIN verification procedure has been performed (see clause 5).

If GET IDENTITY command is not supported by the UICC, then the status word '6D00' (Instruction code not supported or invalid) shall be returned.

#### 7.5.1.1	SUCI context

SUCI context shall be supported if "SUCI calculation is to be performed by the USIM" (i.e. Service n°124 and Service n°125 are "available").

The command returns the SUCI which is a privacy preserving identifier containing the concealed SUPI. The function is used in 5GS in the specific cases described in 3GPP TS 33.501 [105] prior to mutual authentication between the UE and the SN.

The SUCI returned is calculated as described in 3GPP TS 33.501 [105].

For the execution of the command, the following information shall be available in the USIM:

- Home network identifier (i.e. MCC and MNC) (see Note).

- Routing indicator (see Note).

- Home network public key (see Note).

- Home network public key identifier (see Note).

- Protection scheme identifier (see Note).

- SUPI.

NOTE:	Provision and storage of the information in the USIM when "SUCI calculation is to be performed by the USIM" (i.e. Service n°124 and Service n°125 are "available") is out of the scope of the specification.

The SUCI is designed for one-time use, however, the freshness and randomness of SUCI returned upon each call of the command depends on the protection scheme configured. There is the special case where the protection scheme used is null-scheme, in such case SUCI contains the non concealed SUPI.

If the home network public key is not provisioned in the USIM, the SUCI shall be calculated using the null-scheme irrespective of the protection scheme stored in the USIM.

The returned SUCI consists of the concatenation of the following information as described in 3GPP TS 23.003 [25]:

Editor’s Note: Coding of the following information in SUCI is not yet defined in 3GPP TS 24.501 [104]

-	SUPI Type

-	Home network identifier (i.e. MCC and MNC).

-	Routing indicator.

-	Protection scheme identifier.

-	Home network public key identifier.

-	Home network identifier.

-	Scheme output, resulting from the protection scheme profile, identified by the protection scheme identifier. The protection scheme profile shall be one of those defined in Annex C of 3GPP TS 33.501 [105] or one of those specified by the Home network.

If SUCI context is supported and:

-	Service n°124 is not "available" or:

-	"SUCI calculation is to be performed by the ME” (i.e. Service n°124 is "available", and Service n°125 is not "available")

the status word '6985' (Conditions of use not satisfied) shall be returned

Editor’s Note: It is FFS to specify the behavior in case other parameters (e.g. Home network public key identifier, some necessary data is not provisioned) are not correctly configured

### 7.5.2	Command parameters and data

| Code | Value |
| --- | --- |
| CLA | As specified in TS 31.101 [11] |
| INS | '78' |
| P1 | '00' |
| P2 | Identity context, see Table X below' |
| Lc | Length of the subsequent data field or not present, see below |
| Data | See below |
| Le | '00', or maximum length of data expected in response |

Parameter P2 specifies the identity context as follows:

# Table 8: Coding of the reference control P2

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - | X | X | X | X | X | X | X | Identity Context (See below) |
| - | 0 | 0 | 0 | 0 | 0 | 0 | 1 | SUCI |
|  | 0 | 0 | 0 | 0 | 0 | 1 | X | Non-IMSI based SUPI |

All other codings are RFU.

#### 7.5.2.1	SUCI context

Command parameters/data: None

Response parameters/data:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 to Le | SUCI TLV data object | Le |

Subscription Concealed Identifier TLV data object:

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| SUCI TLV data object tag | 'A1' | M | 1 |
| Length | X | M | Note |
| SUCI value | -- | M | X |
| NOTE: The length is coded according to ISO/IEC 8825-1 [35] |  |  |  |

-  SUCI

For contents and coding of the SUCI data object TLV value see 3GPP TS 23.003 [25] and 3GPP TS 24.501 [104]. The most significant bit of SUCI is the most significant bit of the 1st byte of this TLV value field. The least significant bit of SUCI is the least significant bit of the last byte of this TLV value field.

Editor’s Note: Format of SUCI value is still to be completed in 3GPP TS 24.501 [104]..

#### 7.5.2.2	Non-IMSI based SUPI context

Command parameters/data: None

Response parameters/data:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 to Le | Non-IMSI based SUPI TLV data object | Le |

Non-IMSI based SUPI TLV data object:

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| Non-IMSI based SUPI TLV data object tag | 'A1' | C | 1 |
| Length | X | C | Note |
| Non-IMSI based SUPI value | -- | C | X |
| NOTE: The length is coded according to ISO/IEC 8825-1 [35] |  |  |  |

This data object is only present if and only if the Non-IMSI based SUPI is supported by the USIM (i.e. Service n°xxx is "available").

- Non-IMSI based SUPI

Contents and coding of the Non-IMSI based SUPI data object TLV value is in the form of a NAI as defined in NAI RFC 7542 [xx]

***** Next change *****

### 5.3.xx	Non-IMSI based SUPI procedure

Requirement:	"Non-IMSI based SUPI support" (i.e. Service n°xxx is "available").

Request:		The ME uses the GET IDENTITY command in Non-IMSI based SUPI context to retrieve the SUPI from the USIM.

***** End of change *****