3GPP TSG-CT WG4 Meeting #89	C4-190618

# Montreal, Canada; 25th Feb - 1st March                                          revision of C4-190588

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.509 | CR | 0046 | rev | 6 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE parameters update support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the agreed papers for updating UDM-related parameters of UE, S2-1811541 in SA2, the UDM services shall also be updated to support UE parameters update. Nausf_UPUProtection shall be extended in case of updating UE parameters update data after the UE has been successfully authenticated and registered to the 5G system. The detail content of the UE parameters update and the related NAS procedures are also agreed in C1-188946. SA3 agreed a corresponding security procedure in CR 0484 to TS 33.501 (S3-183742). This contribution proposes to modify the related description of the service operation for AUSF, and the resource and the data type definitions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the related description of the AUSF service operation, and the resource and the data type definitions to support UE parameters update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE parameters update in 5GS is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.1, 5.x, 6.x, A.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS.29.503. CR .0143. |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a new OpenAPI specification file TS29509_ Nausf_UPUProtection with addition of API Nausf_UPUProtection in 3GPP TS 29.509. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AMF	Access and Mobility Management Function

API	Application Programming Interface

AUSF	Authentication Server Function

MAC	Message Authentication Code

NF	Network Function

SEAF	SEcurity Anchor Function

SoR	Steering of Roaming

URI	Uniform Resource Identifier

UPU	UE Parameters Update

* * * Next Change * * * *

5.1	Introduction

The AUSF offers to NF Service Consumers (e.g. AMF) the following services:

-	Nausf_UEAuthentication

-	Nausf_SoRProtection

-	Nausf_UPUProtection

* * * Next Change * * * *

5.3.2.2.1	General

The Protect service operation is used in the following procedures:

-	Procedure for steering of UE in VPLMN during registration (see subclause 6.14.2.1 of 3GPP TS 33.501 [8]);

-	Procedure for steering of UE in VPLMN after registration (see subclause 6.14.2.2 of 3GPP TS 33.501 [8]).

The NF Service Consumer (e.g. UDM) uses this service operation to request the AUSF to compute the SOR-MAC-IAUSF and the CounterSoR by providing Steering Information List. The NF Service Consumer (e.g. UDM) may also request the AUSF to compute the SoR-XMAC-IUE by providing the indication that an acknowledgement is requested from the UE.

Figure 5.3.2.2-1: Steering of UE in VPLMN

1.	The NF Service Consumer (e.g. UDM) shall send a POST request to the AUSF that was used to authenticate the UE. The payload of the body shall contain the Steering Information List and the acknowledge indication.

2a.	On success, "200 OK" shall be returned. The payload body shall contain the requested security material necessary to protect the Steering of Roaming procedure.

2b.	On failure, one of the HTTP status code listed in table 6.2.7.3-1 shall be returned with the message body containing a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.2.7.3-1. If the CounterSoR associated with the KAUSF of the UE, is about to wrap around, the AUSF shall use the "COUNTER-WRAP" cause.

* * * Next Change * * * *

5.x	Nausf_UPUProtection Service

5.x.1	Service Description

The AUSF is acting as NF Service Producer. It provides UPUProtection service to the NF Service Consumer.

This service permits to provide the NF Service Consumer (e.g. UDM) with the UPU-MAC-IAUSF and CounterUPU to protect the UE Parameters Update Data from being tampered with or removed by the VPLMN.

In option this service also allows to provide the NF Service Consumer (e.g. UDM) with the UPU-XMAC-IUE that allows the NF Service Consumer (e.g. UDM) to verify that the UE received UE Parameters Update Data correctly.

5.x.2	Service Operations

5.x.2.1	Introduction

The service operation defined for the Nausf_UPUProtection is as follows:

-	Protect

5.x.2.2	Protect

5.x.2.2.1	General

The Protect service operation is used in the following procedures:

-	Procedure for UE Parameters Update (see subclause 6.15.2.1 of 3GPP TS 33.501 [8]).

The NF Service Consumer (e.g. UDM) uses this service operation to request the AUSF to compute the UPU-MAC-IAUSF and CounterUPU by providing the UE Parameters Update Data (UPU Data). The NF Service Consumer (e.g. UDM) may also request the AUSF to compute the UPU-XMAC-IUE by providing the indication that an acknowledgement is requested from the UE.

Figure 5.x.2.2-1: UE Parameters Update in VPLMN

1.	The NF Service Consumer (e.g. UDM) shall send a POST request to the AUSF that was used to authenticate the UE. The payload of the body shall contain the UE Parameters Update Data (UPU Data) and the acknowledge indication.

2a.	On success, "200 OK" shall be returned. The payload body shall contain the requested security material necessary to protect the UE Parameters Update procedure.

2b.	On failure, one of the HTTP status code listed in table 6.x.7.3-1 shall be returned with the message body containing a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.x.7.3-1. If the CounterUPU associated with the KAUSF of the UE, is about to wrap around, the AUSF shall use the "COUNTER-WRAP" cause.

* * * Next Change * * * *

6.x	Nausf_UPUProtection Service API

6.x.1	API URI

URIs of this API shall have the following root:

{apiRoot}/{apiName}/{apiVersion}/

where "apiRoot" is defined in subclause 4.4.1 of 3GPP TS 29.501 [5], the "apiName" shall be set to "nausf-upuprotection" and the "apiVersion" shall be set to "v1" for the current version of this specification.

6.x.2	Usage of HTTP

6.x.2.1	General

HTTP/2, as defined in IETF RFC 7540 [6], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

6.x.2.2	HTTP standard headers

6.x.2.2.1	General

The usage of HTTP standard headers is specified in subclause 5.2.2 of 3GPP TS 29.500 [4].

6.x.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [7], shall be used as content type of the HTTP bodies specified in the present specification as indicated in subclause 5.4 of 3GPP TS 29.500 [4].

-	The Problem Details JSON Object (IETF RFC 7807 [11]. The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json"

6.x.2.3	HTTP custom headers

6.x.2.3.1	General

In this version of the API, no specific custom headers are defined for the "Nausf_UPUProtection" service.

For 3GPP specific HTTP custom headers used across all service based interfaces, see subclause 5.2.3 of 3GPP TS 29.500 [4].

6.x.3	Resources

6.x.3.1	Overview

The structure of the Resource URIs of the UPUProtection service is shown in Figure 6.x.3.1-1

Figure 6.x.3.1-1: Resource URI structure of the UPUProtection API

Table 6.x.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.x.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| ue-upu (Custom operation) | {apiRoot}/nausf-upuprotection/v1/{supi}/ue-upu/generate-upu-data | generate-upu-data (POST) | Resource for UPU security material computation |

6.x.3.2	Resource: ue-upu

6.x.3.2.1	Description

It is the resource to which the custom operation used to generate the UPU security material is associated with.

6.x.3.2.2	Resource Definition

Resource URI: {apiRoot}/nausf-upuprotection/v1/supi/ue-upu

This resource shall support the resource URI variables defined in table 6.x.3.2.2-1.

Table 6.x.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.x.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] subclause 5.9.2) pattern: '^(imsi-[0-9]{5,15}¦nai-.+¦.+)$' |

6.x.3.2.3	Resource Standard Methods

No Standard Methods are supported for this resource.

6.x.3.2.4	Resource Custom Operations

6.x.3.2.4.1	Overview

Table 6.x.3.2.4.1-1: Custom operations

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| /generate-upu-data | POST | The AUSF calculates the UPU-MAC-IAUSF and the CounterUPU to protect the UE Parameters Update Data provided. It may also calculate the UPU-XMAC-IUE to verify that the UE received the UE Parameters Update Data if the indication that an acknowledgement is requested from the UE is provided. |

6.x.3.2.4.2	Operation: generate-upu-data

6.x.3.2.4.2.1	Description

This custom operation is used by the NF service consumer (e.g. UDM) to request the AUSF to compute the security material (UPU-MAC-IAUSF, CounterUPU and UPU-XMAC-IUE) needed to ensure the protection of the UPU procedure (see 3GPP TS 33.501 [8]).

6.x.3.2.4.2.2	Operation Definition

This method shall support the request data structures specified in table 6.x.3.2.4.2.2-1 and the response data structures and response codes specified in table 6.x.3.2.4.2.2-2.

Table 6.x.3.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UpuInfo | M | 1 | Contains the UE Parameters Update Data and may contain the indication that an acknowledgement is requested from the UE (as specified in 3GPP TS 33.501 [8]). |

Table 6.x.3.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UpuSecurityInfo | M | 1 | 200 OK | Upon success, the response body will contain UPU-MAC-IAUSF and CounterUPU and may contain the UPU-XMAC-IUE. |
| ProblemDetails | M | 1 | 503 Service Unavailable | The "cause" attribute shall be set to one of the following application error: - COUNTER_WRAP See table 6.x.7.3-1 for the description of these errors. |

6.x.4	Custom Operations without associated resources

6.x.4.1	Overview

There is no Custom Operation in the current version of this API.

6.x.5	Notifications

6.x.5.1	General

There is no use of notification in the current version of this API.

6.x.6	Data Model

6.x.6.1	General

This subclause specifies the application data model supported by the API.

Table 6.x.6.1-1 specifies the data types defined for the Nausf-UPUProtection service based interface protocol.

Table 6.x.6.1-1: Nausf specific Data Types

| Data type | Section defined | Description |
| --- | --- | --- |
| UpuInfo | 6.x.6.2.2 | Contains the UE parameters update Information |
| UpuSecurityInfo | 6.x.6.2.3 | Contains the material generated for securing of UPU. It contains at least the UPU-MAC-IAUSF and CounterUPU. |
| UpuData | 6.x.6.2.4 | Contains UE parameters update data set (e.g., the updated Routing ID Data or the Default configured NSSAI). |
| UpuMac | 6.x.6.3.2 | MAC value for protecting UPU procedure (UPU-MAC-IAUSF and UPU-MAC-IUE) |
| CounterUpu | 6.x.6.3.2 | CounterUPU |
| AckInd | 6.x.6.3.2 | Contains the indication of whether the acknowledgement from UE is needed |

Table 6.x.6.1-2 specifies data types re-used by the Nausf-UPUProtection service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nausf service based interface.

Table 6.x.6.1-2: Nausf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Snssai | 3GPP TS 29.571 [10] | Default configured NSSAI |

6.x.6.2	Structured data types

6.x.6.2.1	Introduction

The following subclauses define the structures to be used in resource representations.

6.x.6.2.2	Type: UpuInfo

Table 6.x.6.2.2-1: Definition of type UpuInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upuDataList | array(UpuData) | M | 1..N | This information defines the UE Parameters Update (UPU). A secured packed with the Routing indicator update data and/or the Default configured NSSAI update data are included. See subclause 6.x.6.2.4. |
| upuAckInd | UpuAckInd | M | 1 | Contains the indication of whether the acknowledgement from UE is needed. |

6.x.6.2.3	Type: UpuSecurityInfo

Table 6.x.6.2.3-1: Definition of type UpuSecurityInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upuMacIausf | UpuMac | M | 1 | Contains the UPU-MAC-IAUSF. |
| counterUpu | CounterUpu | M | 1 | Contains the CounterUPU. |
| upuXmacIue | UpuMac | O | 0..1 | When present, contains the UPU-XMAC-IUE. It shall be included, if the UDM requests the acknowledgement from the UE. |

6.x.6.2.4	Type: UpuData

Table 6.x.6.2.4-1: Definition of type UpuData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| secPacket | SecuredPacket | C | 0..1 | Presents if the Routing indicator update data is required to be updated, and contains a secured packet with the Routing indicator to be updated. |
| defaultConfNssai | array(Snssai) | C | 0..N | Presents if the Default configured NSSAI is required to be updated, and contains the Default configured NSSAI to be updated. |

6.x.6.3	Simple data types and enumerations

6.x.6.3.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

6.x.6.3.2	Simple data types

Table 6.x.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| UpuMac | string | pattern: "^[A-Fa-f0-9]{32}$" |
| CounterUpu | string | pattern: "^[A-Fa-f0-9]{4}$" |
| UpuAckInd | boolean | true indicates that the UPU-XMAC-IUE shall be computed and returned in the response |

6.x.6.3.3	Enumeration: UpuDataType

Table 6.2.6.3.3-1: Enumeration UpuDataType

| Enumeration value | Description |
| --- | --- |
| "ROUT_INDICATOR" | Routing indicator update data |
| "DEFAULT_CONF_NSSAI" | Default configured NSSAI update data |

6.x.7	Error Handling

6.x.7.1	General

HTTP error handling shall be supported as specified in subclause 5.2.4 of 3GPP TS 29.500 [4].

6.x.7.2	Protocol Errors

Protocol Error Handling shall be supported as specified in subclause 5.2.7.2 of 3GPP TS 29.500 [4].

6.x.7.3	Application Errors

The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nausf_UPUProtection service. The following application errors listed in Table 6..7.3-1 are specific for the Nausf_UPUProtection service.

Table 6.x.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| COUNTER_WRAP | 503 Service Unavailable | The CounterUPU associated with the KAUSF of the UE is about to wrap around. The AUSF suspends the UPU protection service for the UE until a new KAUSF is generated. |

6.x.8	Security

As indicated in 3GPP TS 33.501 [8], the access to the Nausf_UPUProtection API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [13]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [14]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nausf_UPUProtection API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [14], subclause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nausf_UPUProtection service.

The Nausf_UPUProtection Service API does not define any scopes for OAuth2 authorization as specified in 3GPP TS 33.501 [8]; it defines a single scope consisting on the name of the service (i.e., "nausf-upuprotection"), and it does not define any additional scopes at resource or operation level.

* * * Next Change * * * *

A.x	Nausf_UPUProtection API

openapi: 3.0.0

info:

version: 1.0.0

title: Nausf_UPUProtection Service

description: AUSF UPU Protection Service

servers:

- url: '{apiRoot}/nausf-upuprotection/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nausf-upuprotection

paths:

/{supi}/ue-upu:

post:

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/UpuInfo'

required: true

responses:

'200':

description: UpuSecurityInfo

content:

application/json:

schema:

$ref: '#/components/schemas/UpuSecurityInfo'

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nausf-upuprotection: Access to the Nausf_UPUProtection API

schemas:

UpuInfo:

type: object

properties:

upuDataList:

type: array

items:

$ref: '#/components/schemas/UpuData'

minItems: 1

ackInd:

$ref: '#/components/schemas/AckInd'

required:

- upuDataList

- ackInd

UpuSecurityInfo:

type: object

properties:

upuMacIausf:

$ref: '#/components/schemas/UpuMac'

counterUpu:

$ref: '#/components/schemas/CounterUpu'

upuXmacIue:

$ref: '#/components/schemas/UpuMac'

required:

- upuMacIausf

- counterUpu

UpuData:

type: object

properties:

secPacket:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/SecuredPacket'

defaultConfNssai

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

oneOf:

- required: [secPacket]

- required: [defaultConfNssai]

UpuMac:

type: string

pattern: '^[A-Fa-f0-9]{32}$'

CounterUpu:

type: string

pattern: '^[A-Fa-f0-9]{4}$'

AckInd:

type: boolean

externalDocs:

description: 3GPP TS 29.509 V15.3.0; 5G System; Authentication Server Services

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.509'

* * * End of Changes * * * *