3GPP TSG CT WG1 Meeting #115	C1-191352

# Montreal (Canada), 25 Feb - 1 March 2019	(was C1-190537)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 0815 | rev | 2 | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution on editor's notes on whether explicit start and stop indications for SMS over NAS is needed |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification contains n editor’s note on whether explicit start and stop indications for SMS over NAS are needed, quote of 4.5.5: Editor's note: Whether explicit start and stop indications for SMS over NAS need to be defined is FFS. The case of whether explicit start and stop indications for SMS over NAS are needed was heavily discussed during the study work of 5GS in CT1 (in the TR). The majority of companies did not see the need of explicit start and stop indications to be standardized for SMS over NAS. As the Rel-15 of 3GPP is frozen and no progress has been achieved in this area, it is therefore propose to remove the editor’s notes so UE implementations based in TS 24.501 can be complete (i.e., there is not need of explicit start and stop indications for SMS over NAS). Changes from the agreed version of the CR at CT1#114 Only editorial changes to the cover sheet in order to correct several typos and grammatical mistakes so that the text has now become fully clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The editor’s note is removed. Interoperability impact analysis This CR does not result in any interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s note remains in the specification on whether explicit start and stop indications for SMS over NAS is needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Next change ***

### 4.5.5	Exception handling and avoiding double barring

There are several services for which the NAS needs to be informed when the service starts and stops,

-	because, while the service is ongoing, the mapping of other access attempts to a specific access category can be affected; and

-	in order to avoid double barring at the start of these services.

These services are:

a)	emergency service;

b)	MMTEL voice;

c)	MMTEL video;

d)	SMSoIP; and

e)	SMS over NAS.

The UE considers an emergency service a) as started when 5GMM receives a request from upper layers to register for emergency services or to establish a PDU session with request type = "initial emergency request" or "existing emergency PDU session". It considers the emergency service as stopped when this PDU session is released.

In addition, the UE considers an emergency service a) as started when the 5GMM receives a request for emergency services fallback from the upper layer and performs emergency services fallback as specified in subclause 4.13.4.2 of 3GPP TS 23.502 [9]. In this case, the UE considers the emergency service as stopped when:

-	the emergency PDU session established during the emergency services fallback is released if the UE has moved to an E-UTRA cell connected to 5GCN; or

-	the service request procedure involved in the emergency services fallback is completed otherwise.

While an emergency service a) is ongoing, any access attempt triggered by the initiation of a registration, de-registration or service request procedure or by an uplink user data packet to be sent for a PDU session with suspended user-plane resources is mapped to access category 2 = emergency.

Once the emergency service has successfully passed access control, then as long as the service is ongoing, the following access attempts are allowed to proceed without further access control checking in order to avoid double barring:

-	any service request procedure related to the PDU session associated with request type = "initial emergency request" or "existing emergency PDU session";

-	any uplink user data packet to be sent for a PDU session with suspended user-plane resources associated with request type = "initial emergency request" or "existing emergency PDU session"; and

-	any:

1)	service request procedure; or

2)	registration procedure;

initiated in 5GMM-IDLE mode for the purpose of NAS signalling connection recovery or following a fallback indication from the lower layers (see subclauses 5.3.1.1 and 5.3.1.2).

NOTE 1:	Although the access control checking is skipped, the mapping is performed in order to derive an RRC establishment cause.

For services b) to e) the 5GMM receives explicit start and stop indications from the upper layers.

Once the service has successfully passed access control, then as long as the service is ongoing, the following access attempts are allowed to proceed without further access control checking in order to avoid double barring:

-	for services b), c) and d):

1)	any service request procedure related to the PDU session established for DNN = "IMS"; and

2)	any uplink user data packet to be sent for a PDU session with suspended user-plane resources established for DNN = "IMS";

-	for service d), if the upper layers have indicated a DNN used for SMSoIP and the indicated DNN used for SMSoIP is different from "IMS":

1)	any service request procedure related to the PDU session established for the DNN used for SMSoIP; and

2)	any uplink user data packet to be sent for a PDU session with suspended user-plane resources established for the DNN used for SMSoIP; and

-	for services b), c), d) and e), any:

1)	service request procedure; or

2)	registration procedure;

initiated in 5GMM-IDLE mode for the purpose of NAS signalling connection recovery or following a fallback indication from the lower layers (see subclause 5.3.1.1 and 5.3.1.2).

While an MMTEL voice call is ongoing:

-	any service request procedure related to the PDU session established for DNN = "IMS" is mapped to access category 4;

-	any uplink user data packet to be sent for a PDU session with suspended user-plane resources established for DNN = "IMS" is mapped to access category 4; and

-	any:

1)	service request procedure; or

2)	registration procedure;

initiated in 5GMM-IDLE mode for the purpose of NAS signalling connection recovery or following a fallback indication from the lower layers (see subclause 5.3.1.1 and 5.3.1.2) is mapped to access category 4.

While an MMTEL video call is ongoing and no MMTEL voice call is ongoing:

-	any service request procedure related to the PDU session established for DNN = "IMS" is mapped to access category 5;

-	any uplink user data packet to be sent for a PDU session with suspended user-plane resources established for DNN = "IMS" is mapped to access category 5; and

-	any:

1)	service request procedure; or

2)	registration procedure;

initiated in 5GMM-IDLE mode for the purpose of NAS signalling connection recovery or following a fallback indication from the lower layers (see subclause 5.3.1.1 and 5.3.1.2) is mapped to access category 5.

While an SMSoIP is ongoing, no MMTEL video call is ongoing and no MMTEL voice call is ongoing:

-	any service request procedure related to the PDU session established:

1)	for DNN = "IMS"; or

2)	for the DNN used for SMSoIP, if the upper layers have indicated a DNN used for SMSoIP and the indicated DNN used for SMSoIP is different from "IMS";

is mapped to access category 6; and

-	any uplink user data packet to be sent for a PDU session with suspended user-plane resources established:

1)	for DNN = "IMS"; or

2)	for the DNN used for SMSoIP, if the upper layers have indicated a DNN used for SMSoIP and the indicated DNN used for SMSoIP is different from "IMS";

is mapped to access category 6; and

-	any:

1)	service request procedure; or

2)	registration procedure;

initiated in 5GMM-IDLE mode for the purpose of NAS signalling connection recovery or following a fallback indication from the lower layers (see subclause 5.3.1.1 and 5.3.1.2) is mapped to access category 6.

While an SMS over NAS is ongoing, no SMSoIP is ongoing, no MMTEL video call is ongoing and no MMTEL voice call is ongoing:

-	any:

1)	service request procedure; or

2)	registration procedure;

initiated in 5GMM-IDLE mode for the purpose of NAS signalling connection recovery or following a fallback indication from the lower layers (see subclause 5.3.1.1 and 5.3.1.2) is mapped to access category 6.

NOTE 2:	Although the access control checking is skipped, the mapping is performed in order to derive an RRC establishment cause.

If an access category is determined and the access control checking is skipped, the NAS shall determine the RRC establishment cause from one or more determined access identities and the access category as specified in subclause 4.5.6, the NAS shall initiate the procedure to send the initial NAS message for the access attempt and shall provide the RRC establishment cause to lower layers.