3GPP TSG-CT Meeting #84	CP-191203

# Newport Beach, US; 03rd – 04th June 2019

# Source:	3GPP TSG CT WG4

# Title:	New SID: Study on Nudsf Service Based Interface

# Document for:	Approval

Agenda Item:	16.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	 Study on Nudsf Service Based Interface

## Acronym: FS_NUDSF

## Unique identifier: 	 840029

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 750025 | CT aspects on 5G System - Phase 1 | 5G Phase 1 Stage 3 Work - Parent |

## 3	Justification

With the Service Based Architecture that is the basis of the 5GC, operators are looking at cloud native deployments where NF services may operate in a stateless and dataless manner. In such deployments, the Unstructured Data Storage Function (UDSF) is a critical component where stateless/dataless NFs may store state, context and other data related to its function.

Such deployments will benefit from allowing a highly optimized storage mechanism to hold information for all NFs regardless of vendor or type simplifying and optimizing the network topology.

Although the objective of the UDSF is to provide Unstructured data storage as defined in 3GPP TS23.501, clauses 4.2.5, and 6.2.12, describes that Unstructured data refers to data for which the structure is not defined in 3GPP specifications.

It does not however mean that a set of procedures can't be specified by 3GPP or that data structures/models can't be defined by operators themselves.

## 4	Objective

The objective with this study is to continue the study of the UDSF that was started in rel-15 and documented in 3GPP TR 29.891 clause 6.9. Specifically, the study should focus on:

- Study the Nudsf service as a service based interface and as defined in 3GPP  TS 23.502

- Study the required operations

- Study what resources (if any) will be required

- Study a Subscribe to notifications of data change

- Study a Notification of a data change

- Evaluate  the latency/performance of an SBI based Nudsf

- Optionally, study non-SBI based alternatives (if any)

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| TR | 29.8ab | Study on Nudsf Service Based Interface | CT#85, Sep 2019 | CT#86, Dec 2019 | Rapporteur: Askerup, Anders, Hewlett Packard Enterprise |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {E.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {E.g. "TSG#89"} | {Free text} |

## 6	Work item Rapporteur(s)

Askerup, Anders, Hewlett Packard Enterprise, Anders.Askerup@hpe.com.

## 7	Work item leadership

CT4

## 8	Aspects that involve other WGs

The stage 2, especially as documented in 3GPP  TS 23.502, may require updates depending on the outcome of the study.

## 9	Supporting Individual Members

Supporting IM name

Hewlett Packard Enterprise

Openet Telecom

China Mobile

Verizon

Telecom Italia

Deutsche Telekom

Orange

Vodafone