3GPP TSG-CT Meeting #84	CP-191226

Newport Beach California, United States, 3rd Jun 2019 - 3rd Jun 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0116 | rev | 4 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to CreateSMContext for eNS Support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | ZTE |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-05-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During EPS to 5GS mobility procedures, the S-NSSAI shall be returned in Response message of CreateSMContext service operation. In Home Routed roaming case, S-NSSAI of home PLMN shall be returned. Such statement is not clearly described in existing procedure description in TS29.502, and needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are made to support eNS: - In 5.2.2.2.2 / 5.2.2.2.3, list the S-NSSAI as returned parameter in Response message; - In 6.16.2.3, clarify that which S-NSSAI shall be included in CreateSMContext Response message, for Home-Routed roaming case; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current procedure description remains unclear on return S-NSSAI of home PLMN in Home Routed roaming case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2, 5.2.2.2.3, 6.1.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  |  |  |  |  | TS23.502 CR#1217 |  |  |
| affected: |  |  | X |  |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X |  |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not require a version update of any OpenAPI specification file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev3: In 5.2.2.2.3, move the new bullet mistakenly added in step 1 (request) to step 2 (response). Rev4: correction on cover page |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.2.2.2.2	EPS to 5GS Idle mode mobility using N26 interface

The NF Service Consumer (e.g. AMF) shall request the SMF to move a UE EPS PDN connection to 5GS using N26 interface, as follows.

Figure 5.2.2.2.2-1: EPS to 5GS Idle mode mobility using N26 interface

1.	The NF Service Consumer shall send a POST request towards the SMF (+PGW-C) of each UE EPS PDN connection, as specified in subclause 5.2.2.2.1, with the following additional information:

-	UE EPS PDN connection, including the EPS bearer contexts, received from the MME, representing the individual SM context resource to be created;

-	pduSessionsActivateList, including the PDU Session ID of all the PDU session(s) to be re-activated.

2a.	Upon receipt of such a request, if a corresponding PDU session is found based on the EPS bearer contexts (after invoking a Create service operation towards the H-SMF, for a Home Routed PDU session) and if it is possible to proceed with moving the PDN connection to 5GS, the SMF shall return a 201 Created response including the following information:

-	PDU Session ID corresponding to the default EPS bearer ID of the EPS PDN connection;

-	S-NSSAI assigned to the PDU session; in home routed roaming case, the S-NSSAI for home PLMN shall be returned;

-	allocatedEbiList, containing the EBI(s) allocated to the PDU session;

and, if the PDU session that is derived by the SMF based on the EPS bearer contexts was requested to be re-activated, i.e. if the PDU Session ID was present in the pduSessionsActivateList:

-	upCnxState attribute set to ACTIVATING;

-	N2 SM information to request the 5G-AN to assign resources to the PDU session (see PDU Session Resource Setup Request Transfer IE in subclause 9.3.4.1 of 3GPP TS 38.413 [9]), including (among others) the transport layer address and tunnel endpoint of the uplink termination point for the user plane data for this PDU session (i.e. UPF's GTP-U F-TEID for uplink traffic).

The "Location" header shall be present in the POST response and shall contain the URI of the created SM context resource.

The NF Service Consumer (e.g. AMF) shall store the association of the PDU Session ID and the SMF ID, and store the allocated EBI(s) associated to the PDU Session ID.

NOTE:	The behaviour specified in this step also applies if the POST request collides with an existing SM context, i.e. if the POST request includes the same SUPI, or PEI for an emergency registered UE without a UICC or without an authenticated SUPI, and the default EPS bearer ID received in the UE EPS PDN connection is the same as in the existing SM context.

2b.	Same as step 2b of figure 5.2.2.2.1-1. Steps 3 to 4 are skipped in this case.

If the SMF determines that seamless session continuity from EPS to 5GS is not supported for the PDU session, the SMF shall set the "cause" attribute in the ProblemDetails structure to "NO_EPS_5GS_CONTINUITY".

3.	Same as step 3 of figure 5.2.2.3.2.2-1, if the SMF returned a 201 Created response with the upConnectionState set to ACTIVATING and N2 SM Information,

4.	Same as step 4 of figure 5.2.2.3.2.2-1.

* * * Next Change * * * *

5.2.2.2.3	EPS to 5GS Handover Preparation using N26 interface

The NF Service Consumer (e.g. AMF) shall request the SMF to handover a UE EPS PDN connection to 5GS using N26 interface, as follows.

Figure 5.2.2.2.3-1: EPS to 5GS handover using N26 interface

1.	The NF Service Consumer shall send a POST request, as specified in subclause 5.2.2.2.1, with the following additional information:

-	UE EPS PDN connection, including the EPS bearer contexts, representing the individual SM context resource to be created;

-	hoState attribute set to PREPARING (see subclause 5.2.2.3.4.1);

-	targetId identifying the target RAN Node ID and TAI received in the Forward Relocation Request from the source MME.

2a.	Upon receipt of such a request, if a corresponding PDU session is found based on the EPS bearer contexts (after invoking a Create service operation towards the H-SMF, for a Home Routed PDU session) and it is possible to proceed with handing over the PDN connection to 5GS, the SMF shall return a 201 Created response including the following information:

-	hoState attribute set to PREPARING and N2 SM information to request the target 5G-AN to assign resources to the PDU session, as specified in step 2 of Figure 5.2.2.3.4.2-1;

-	PDU Session ID corresponding to the default EPS bearer ID of the EPS PDN connection;

-	S-NSSAI assigned to the PDU session; in home routed roaming case, the S-NSSAI for home PLMN shall be returned;

-	allocatedEbiList, containing the EBI(s) allocated to the PDU session.

The "Location" header shall be present in the POST response and shall contain the URI of the created SM context resource.

The NF Service Consumer (e.g. AMF) shall store the association of the PDU Session ID and the SMF ID, and store the allocated EBI(s) associated to the PDU Session ID.

NOTE:	The behaviour specified in this step also applies if the POST request collides with an existing SM context, i.e. if the POST request includes the same SUPI, or PEI for an emergency registered UE without a UICC or without an authenticated SUPI, and the default EPS bearer ID received in the UE EPS PDN connection is the same as in the existing SM context.

2b.	Same as step 2b of figure 5.2.2.2.1-1 with the following additions. Steps 3 to 4' are skipped in this case.

The error response shall include the following additional information:

-	hoState attribute set to NONE.

If the SMF determines that seamless session continuity from EPS to 5GS is not supported for the PDU session, the SMF shall set the "cause" attribute in the ProblemDetails structure to "NO_EPS_5GS_CONTINUITY".

* * * Next Change * * * *

6.1.6.2.3	Type: SMContextCreatedData

Table 6.1.6.2.3-1: Definition of type SmContextCreatedData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hsmfUri | Uri | C | 0..1 | This IE shall be present in HR roaming scenarios if the additionalHsmfUri IE was received in the request and the V-SMF established the PDU session towards an alternative SMF listed in the additionalHsmfUri IE. When present, it shall contain the URI of the H-SMF towards which the PDU session was established. |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall be present, during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall be set to the PDU Session ID. |
| sNssai | Snssai | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain the S-NSSAI assigned to the PDU session. In Home-Routed roaming case, this IE shall contain the S-NSSAI for home PLMN. |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present if the SMF was requested to activate the user plane connection of the PDU session in the corresponding request. When present, it shall be set as specified in subclause 5.2.2.2.2. |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if N2 SM Information needs to be sent to the AN. |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. |
| allocatedEbiList | array(EbiArpmapping) | C | 1..N | This IE shall be present if the consumer NF is an AMF and Inter-system mobility happens. When present, it shall contain an array of EBI to ARP mappings currently allocated to the PDU session. |
| hoState | HoState | C | 0..1 | This IE shall be present if the SMF was requested to prepare an EPS to 5GS handover for the PDU session in the corresponding request. When present, it shall be set as specified in subclause 5.2.2.2.3. |
| smfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the SMF service instance serving the PDU session Context. This IE may be used by the AMF to identify PDU session contexts affected by a failure or restart of the SMF service instance (see subclause 6.2 of 3GPP TS 23.527 [24]). |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the SMF service instance serving the PDU session was (re)started (see subclause 6.3 of 3GPP TS 23.527 [24]). |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in subclause 6.1.8 is supported. |

* * * End Of Change * * * *