3GPP TSG-CT Meeting #85	CP-192029

# Newport Beach, US; 16th – 17th September 2019  					        Revision of C4-193550

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0160 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Insertion of a PSA and UL CL/BP into the data path of a PDU session with an I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell, ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 2 requirements for inserting an UL CL or BP into the data path of a PDU session with an I-SMF are specified in clause 5.34.6 of TS 23.501 and clause 4.23.9 of TS 23.502, and in CR 23.502 #1520 (S2-1907214) agreed at SA2#134. The I-SMF sends an Update Request to the SMF (Indication of UL CL or BP Insertion, IPv6 prefix@PSA2, DNAIs supported by PSA2, DL tunnel info of UL CL/BP). The SMF sends Update Response (set of N4 info including DNAI, Indication of no DNAI change, Indication of no local PSA change). N4 info for local traffic corresponding to N4 rules (PDR, FAR, URR, QER, etc) related with the support of a DNAI. N4 info may indicate info (e.g. 5G AN Tunnel Info) that the SMF does not know and that the I-SMF needs to determine itself to build the actual rules sent to the UPF(s). Each element of N4 information for local traffic handling provided to the I-SMF includes the DNAI it corresponds to. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update SM Context request is not used for inserting a PSA and UL/CL or BP. So related changes specified in an earlier CT4 meeting are reverted. Update Request is extended with new parameters to report the insertion of a PSA and UL CL or BP. Update Response is extended to report N4 information payloads related to DNAI(s) and the new Indications of no DNAI change and no local PSA change. Each N4 information payload is sent in a binary part using PFCP encoding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 5.2.2.3.1, 5.2.2.3.13, 5.2.2.8.1, 5.2.2.8.2.x (new), 6.1.2.2.2, 6.1.2.4, 6.1.2.4.x (new), 6.1.6.1, 6.1.6.2.11, 6.1.6.2.12, 6.1.6.2.x (new), 6.1.6.2.y (new), 6.1.6.2.z (new), 6.1.6.3.x (new), 6.1.6.3.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features in the OpenAPI specification file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 2: incorrect psaInd definition in the OpenAPI specification file PsaInformation: type: object properties: psaInd: type: PsaIndication dnaiList: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai' minItems: 1 ueIpv6Prefix: $ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix' corrected to psaInd: $ref: '#/components/schemas/PsaIndication' |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[7]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[8]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[9]	3GPP TS 38.413: "NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[10]	IETF RFC 2387: "The MIME Multipart/Related Content-type".

[11]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[12]	IETF RFC 2045: "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[13]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[14]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[15]	OpenAPI Initiative, "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md

[16]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[17]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[18]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[19]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[20]	3GPP TS 29.518: "5G System; Access and Mobility Management Service; Stage 3".

[21]	3GPP TS 23.380: "IMS Restoration Procedures".

[22]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[23]	IETF RFC 7807: "Problem Details for HTTP APIs".

[24]	3GPP TS 23.527: "5G System; Restoration Procedures".

[25]	3GPP TS 32.255: "Charging management; 5G data connectivity domain charging; stage 2".

[26]	3GPP TS 32.291: "Charging management; 5G system, charging service; Stage 3".

[27]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[28]	3GPP TR 21.900: "Technical Specification Group working methods".

[x]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; stage 3".

* * * Next Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BP	Branching Point

DNN	Data Network Name

HR	Home Routed

H-SMF	Home SMF

I-SMF	Intermediate SMF

JSON	Javascript Object NotationNAS	Non-Access Stratum

LADN	Local Area Data Network

PSA	PDU Session Anchor

SM	Session Management

SMF	Session Management Function

UL CL	Uplink Classifier

UPF	User Plane Function

V-SMF	Visited SMF

* * * Next Change * * * *

#### 5.2.2.3	Update SM Context service operation

##### 5.2.2.3.1	General

The Update SM Context service operation shall be used to update an individual SM context and/or provide N1 or N2 SM information received from the UE or the AN, for a given PDU session, towards the SMF, or the V-SMF for HR roaming scenarios.

It is used in the following procedures:

-	PDU Session modification (see clause 4.3.3 of 3GPP TS 23.502 [3]);

-	UE or network requested PDU session release (see clause 4.3.4.2 and clause 4.3.4.3 of 3GPP TS 23.502 [3]);

-	UE or network-initiated MA PDU session release over a single access (see clause 4.22 of 3GPP TS 23.502 [3]);

-	Activation or Deactivation of the User Plane connection of an existing PDU session, i.e. establishment or release of the N3 tunnel between the AN and serving CN (see clause 5.6.8 of 3GPP TS 23.501 [2] and clauses 4.2.2.2, 4.2.3 and 4.2.6 of 3GPP TS 23.502 [3]);

-	Xn and N2 Handover procedures (see clauses 4.9.1 of 3GPP TS 23.502 [3]);

-	Handover between 3GPP and untrusted non-3GPP access procedures (see clause 4.9.2 of 3GPP TS 23.502 [3]);

-	Inter-AMF change due to AMF planned maintenance or AMF failure (see clause 5.21.2 of 3GPP TS 23.501 [2]), or inter-AMF mobility in CM-IDLE mode (see clause 4.2.2.2 of 3GPP TS 23.502 [3]);

-	RAN Initiated QoS Flow Mobility (see clause 4.14.1 of 3GPP TS 23.502 [3] and clause 8.2.5 of 3GPP TS 38.413 [9]);

-	All procedures requiring to provide N1 or N2 SM information to the SMF, e.g. UE requested PDU Session Establishment procedure (see clause 4.3.2.2 of 3GPP TS 23.502 [3]), session continuity procedure (see clause 4.3.5 of 3GPP TS 23.502 [3]);

-	EPS to 5GS Idle mode mobility or handover using N26 interface (see clause 4.11 of 3GPP TS 23.502 [3]);

-	5GS to EPS Handover using N26 interface (see clause 4.11.1.2 of 3GPP TS 23.502 [3]);

-	PDU Session Reactivation during P-CSCF Restoration procedure via AMF (see clause 5.8.4.3 of 3GPP TS 23.380 [21]);

-	AMF requested PDU session release due to a change of the set of network slices for a UE where a network slice instance is no longer available (see clause 4.3.4.2 of 3GPP TS 23.502 [3]);

-	AMF receives an "initial request" with PDU Session Id which already exists in PDU session context of the UE (see clause 5.4.5.2.5 of 3GPP TS 24.501 [7]);

-	Secondary RAT Usage Data Reporting (see clause 4.21 of 3GPP TS 23.502 [3]).

The NF Service Consumer (e.g. AMF) shall update an individual SM context and/or provide N1 or N2 SM information to the SMF by using the HTTP POST method (modify custom operation) as shown in Figure 5.2.2.3.1-1.

Figure 5.2.2.3.1-1: SM context update

1.	The NF Service Consumer shall send a POST request to the resource representing the individual SM context resource in the SMF. The payload body of the POST request shall contain the modification instructions and/or the N1 or N2 SM information. If the request contains EBI(s) to revoke, then the SMF shall disassociate the EBI(s) with the QFI(s) with which they are associated.

2a.	On success, "204 No Content" or "200 OK" shall be returned; in the latter case, the payload body of the POST response shall contain the representation describing the status of the request and/or N1 or N2 SM information.

If the ExemptionInd IE is included in the request message, indicating that the NAS SM message included in the request was exempted from NAS congestion control by the AMF, the SMF shall verify that the included 5G SM message can be exempted from a NAS SM congestion control activated in the AMF as specified in clause 5.19.7 of 3GPP TS 23.501[2].

The SMF may indicate to the NF Service Consumer that it shall release EBI(s) that were assigned to the PDU session by including the releaseEbiList IE, e.g. when a QoS flow is released.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned.  For a 4xx/5xx response, the message body shall contain an SmContextUpdateError structure, including:

-	a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.3.3.2-3;

-	N1 SM information, if the SMF needs and can return a response to the UE;

-	N2 SM information, if the SMF needs and can return a response to the NG-RAN.

The following clauses specify additional requirements applicable to specific scenarios.

* * * Next Change * * * *

##### 5.2.2.3.13	Void

* * * Next Change * * * *

#### 5.2.2.8	Update service operation

##### 5.2.2.8.1	General

The Update service operation shall be used in HR roaming scenarios to:

-	update an individual PDU session in the H-SMF and/or provide the H-SMF with information received by the V-SMF in N1 SM signalling from the UE;

-	release a MA PDU session over a single access in the H-SMF;

-	update an individual PDU session in the V-SMF and/or provide information necessary for the V-SMF to send N1 SM signalling to the UE.

It is invoked by the V-SMF in the following procedures:

-	UE or visited network requested PDU session modification (see clause 4.3.3.3 of 3GPP TS 23.502 [3]);

-	UE requested PDU session release (see clause 4.3.4.3 of 3GPP TS 23.502 [3]);

-	UE or visited network (e.g. AMF, V-SMF) initiated MA PDU session release over a single access (see clause 4.22 of 3GPP TS 23.502 [3]);

-	EPS to 5GS handover execution using N26 interface (see clause 4.11 of 3GPP TS 23.502 [3]);

-	Handover between 3GPP and untrusted non-3GPP access procedures (see clause 4.9.2 of 3GPP TS 23.502 [3]), for a Home Routed PDU session, without AMF change or with target AMF in same PLMN;

-	All procedures requiring to provide the H-SMF with information received by the V-SMF in N1 SM signalling from the UE to the H-SMF.

It is invoked by the I-SMF in the following procedures:

-	Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF (see clause 4.23.9.1 of 3GPP TS 23.502 [3]).

It is invoked by the H-SMFin the following procedures:

-	Home network requested PDU session modification (see clause 4.3.3.3 of 3GPP TS 23.502 [3]);

-	Home network requested PDU session release (see clause 4.3.4.3 of 3GPP TS 23.502 [3]);

-	Home network (e.g. H-SMF) initiated MA PDU session release over a single access (see clause 4.22 of 3GPP TS 23.502 [3]);

-	All procedures requiring to provide information necessary for the V-SMF to send N1 SM signalling to the UE;

-	EPS Bearer ID allocation or revocation (see clauses 4.11.1.4.1 and 4.11.1.4.3 of 3GPP TS 23.502 [3]);

-	Secondary authorization/authentication by an DN-AAA server (see clause 4.3.2.3 of of 3GPP TS 23.502 [3]).

* * * Next Change * * * *

###### 5.2.2.8.2.x	Addition of PSA and BP or UL CL controlled by I-SMF

This clause applies only in case of non-roaming or LBO roaming as control of an UL CL or BP in VPLMN is not supported in HR roaming case (see clause 5.34.4 of 3GPP TS 23.501 [2]).

An I-SMF and I-UPF have already been inserted for the PDU session.

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications:

The POST request shall contain:

-	the requestIndication IE set to NW_REQ_PDU_SES_MOD;

-	the indication that an UL CL or BP has been inserted into the data path of the PDU session;

-	the list of DNAIs supported by the inserted PSA;

-	the new UE IPv6 prefix at the PSA, assigned by the I-SMF or by the UPF supporting the PSA, if IPv6 multi-homing applies to the PDU session;

-	the icnTunnelInfo with the N9 tunnel information of the UL CL or BP for the downlink traffic, if a UPF different from the earlier I-UPF is selected for the UL CL or BP.

2a.	Same as step 2a of Figure 5.2.2.8.2-1, with the following modifications:

The payload body of the POST response shall contain:

-	N4 information for the handling of the local traffic that is offloaded at the PSA;

-	the DNAI related to the N4 information targetting the PSA;

-	the indication that the DNAI shall not change, if applicable;

-	the indication that the local PSA shall not change, if applicable.

* * * Next Change * * * *

##### 6.1.2.2.2	Content type

The following content types shall be supported:

-	the JSON format (IETF RFC 8259 [11]). The use of the JSON format shall be signalled by the content type "application/json". See also clause 5.4 of 3GPP TS 29.500 [4].

-	the Problem Details JSON Object (IETF RFC 7807 [23]). The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json".

NOTE:	"application/json" is used in a response that includes a payload body containing an application-specific data structure, see clause 4.8 of 3GPP TS 29.501 [5].

Multipart messages shall also be supported (see clause 6.1.2.4) using the content type "multipart/related", comprising:

-	one JSON body part with the "application/json" content type; and

-	one or two binary body parts with 3gpp vendor specific content subtypes.

The 3gpp vendor specific content subtypes defined in Table 6.1.2.2.2-1 shall be supported.

Table 6.1.2.2.2-1: 3GPP vendor specific content subtypes

| content subtype | Description |
| --- | --- |
| vnd.3gpp.ngap | Binary encoded payload, encoding NG Application Protocol (NGAP) IEs, as specified in clause 9.3 of 3GPP TS 38.413 [9] (ASN.1 encoded). |
| vnd.3gpp.5gnas | Binary encoded payload, encoding a 5GS NAS message or 5G NAS IEs, as specified in 3GPP TS 24.501 [7]. |
| vnd.3gpp.pfcp | Binary encoded payload, encoding a PFCP message, as specified in 3GPP TS 29.244 [x]. |
| NOTE: Using 3GPP vendor content subtypes allows to describe the nature of the opaque payload (e.g. NGAP or 5GS NAS information) without having to rely on metadata in the JSON payload. |  |

See clause 6.1.2.4 for the binary payloads supported in the binary body part of multipart messages.

* * * Next Change * * * *

#### 6.1.2.4	HTTP multipart messages

HTTP multipart messages shall be supported, to transfer opaque N1 and/or N2 SM payloads or N4 information, in the following service operations (and HTTP messages):

-	Create SM Context Request and Response (POST);

-	Update SM Context Request and Response (POST);

-	Release SM Context Request (POST);

-	Create Request and Response (POST);

-	Update Request and Response (POST (modify)).

HTTP multipart messages shall include one JSON body part and one or two binary body parts comprising:

-	an N1 SM payload, an N2 SM payload or both, over N11 (see clause 6.1.6.4);

-	one or two N1 SM payloads, over N16 (see clause 6.1.6.4);

-	one or two N2 SM payloads over N11 (see clause 5.2.2.3.3);

-	one, two or three N4 payloads over N16a (see clause 6.1.6.4.x).

The JSON body part shall be the "root" body part of the multipart message. It shall be encoded as the first body part of the multipart message. The "Start" parameter does not need to be included.

The multipart message shall include a "type" parameter (see IETF RFC 2387 [10]) specifying the media type of the root body part, i.e. "application/json".

NOTE:	The "root" body part (or "root" object) is the first body part the application processes when receiving a multipart/related message, see IETF RFC 2387 [10]. The default root is the first body within the multipart/related message. The "Start" parameter indicates the root body part, e.g. when this is not the first body part in the message.

For each binary body part in a HTTP multipart message, the binary body part shall include a Content-ID header (see IETF RFC 2045 [12]), and the JSON body part shall include an attribute, defined with the RefToBinaryData type, that contains the value of the Content-ID header field of the referenced binary body part.

Examples of multipart/related messages can be found in Annex B.

* * * Next Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsmf service based interface protocol.

Table 6.1.6.1-1: Nsmf specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SmContextCreateData | 6.1.6.2.2 | Information within Create SM Context Request |  |
| SmContextCreatedData | 6.1.6.2.3 | Information within Create SM Context Response |  |
| SmContextUpdateData | 6.1.6.2.4 | Information within Update SM Context Request |  |
| SmContextUpdatedData | 6.1.6.2.5 | Information within Update SM Context Response |  |
| SmContextReleaseData | 6.1.6.2.6 | Information within Release SM Context Request |  |
| SMContextRetrieveData | 6.1.6.2.7 | Information within Retrieve SM Context Request |  |
| SmContextStatusNotification | 6.1.6.2.8 | Information within Notify SM Context Status Request |  |
| PduSessionCreateData | 6.1.6.2.9 | Information within Create Request |  |
| PduSessionCreatedData | 6.1.6.2.10 | Information within Create Response |  |
| HsmfUpdateData | 6.1.6.2.11 | Information within Update Request towards H-SMF |  |
| HsmfUpdatedData | 6.1.6.2.12 | Information within Update Response from H-SMF |  |
| ReleaseData | 6.1.6.2.13 | Information within Release Request |  |
| HsmfUpdateError | 6.1.6.2.14 | Error within Update Response from H-SMF |  |
| VsmfUpdateData | 6.1.6.2.15 | Information within Update Request towards V-SMF |  |
| VsmfUpdatedData | 6.1.6.2.16 | Information within Update Response from V-SMF |  |
| StatusNotification | 6.1.6.2.17 | Information within Notify Status Request |  |
| QosFlowItem | 6.1.6.2.18 | Individual QoS flow |  |
| QosFlowSetupItem | 6.1.6.2.19 | Individual QoS flow to setup |  |
| QosFlowAddModifyRequestItem | 6.1.6.2.20 | Individual QoS flow requested to be created or modified |  |
| QosFlowReleaseRequestItem | 6.1.6.2.21 | Individual QoS flow requested to be released |  |
| QosFlowProfile | 6.1.6.2.22 | QoS flow profile |  |
| GbrQosFlowInformation | 6.1.6.2.23 | GBR QoS flow information |  |
| QosFlowNotifyItem | 6.1.6.2.24 | Notification related to a QoS flow |  |
| SMContextRetrievedData | 6.1.6.2.27 | Information within Retrieve SM Context Response |  |
| TunnelInfo | 6.1.6.2.28 | Tunnel Information |  |
| StatusInfo | 6.1.6.2.29 | Status of SM context or of PDU session |  |
| VsmfUpdateError | 6.1.6.2.30 | Error within Update Response from V-SMF |  |
| EpsPdnCnxInfo | 6.1.6.2.31 | EPS PDN Connection Information from H-SMF to V-SMF |  |
| EpsBearerInfo | 6.1.6.2.32 | EPS Bearer Information from H-SMF to V-SMF |  |
| PduSessionNotifyItem | 6.1.6.2.33 | Notification related to a PDU session |  |
| EbiArpMapping | 6.1.6.2.34 | EBI to ARP mapping |  |
| SmContextCreateError | 6.1.6.2.35 | Error within Create SM Context Response |  |
| SmContextUpdateError | 6.1.6.2.36 | Error within Update SM Context Response |  |
| PduSessionCreateError | 6.1.6.2.37 | Error within Create Response |  |
| MmeCapabilities | 6.1.6.2.38 | MME capabilities |  |
| SmContext | 6.1.6.2.39 | Complete SM Context | DTSSA |
| ExemptionInd | 6.1.6.2.40 | Exemption Indication |  |
| PsaInformation | 6.1.6.2.x | PSA Information | DTSSA |
| DnaiInformation | 6.1.6.2.y | DNAI Information | DTSSA |
| N4Information | 6.1.6.2.z | N4 Information | DTSSA |
| GtpTeid | 6.1.6.3.2 | GTP Tunnel Endpoint Identifier |  |
| ProcedureTransactionId | 6.1.6.3.2 | Procedure Transaction Identifier |  |
| EpsPdnCnxContainer | 6.1.6.3.2 | UE EPS PDN Connection container from SMF to AMF |  |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Id |  |
| EpsBearerContainer | 6.1.6.3.2 | EPS Bearer container from SMF to AMF |  |
| UpCnxState | 6.1.6.3.3 | User Plane Connection State |  |
| HoState | 6.1.6.3.4 | Handover State |  |
| RequestType | 6.1.6.3.5 | Request Type in Create (SM context) service operation. |  |
| RequestIndication | 6.1.6.3.6 | Request Indication in Update (SM context) service operation. |  |
| NotificationCause | 6.1.6.3.7 | Cause for generating a notification |  |
| Cause | 6.1.6.3.8 | Cause information |  |
| ResourceStatus | 6.1.6.3.9 | Status of SM context or PDU session resource |  |
| DnnSelectionMode | 6.1.6.3.10 | DNN Selection Mode |  |
| EpsInterworkingIndication | 6.1.6.3.11 | EPS Interworking Indication |  |
| N2SmInfoType | 6.1.6.3.12 | N2 SM Information Type |  |
| MaxIntegrityProtectedDataRate | 6.1.6.3.13 | Maximum Integrity Protected Data Rate |  |
| MaReleaseIndication | 6.1.6.3.14 | Multi-Access PDU session release Indication | MAPDU |
| SmContextType | 6.1.6.3.15 | Type of SM Context information | DTSSA |
| PsaIndication | 6.1.6.3.x | Indication of whether a PSA is inserted or removed | DTSSA |
| N4MessageType | 6.1.6.3.y | N4 Message Type | DTSSA |

Table 6.1.6.1-2 specifies data types re-used by the Nsmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmf service based interface.

Table 6.1.6.1-2: Nsmf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uint32 | 3GPP TS 29.571 [13] | Unsigned 32-bit integers |  |
| Ipv4Addr | 3GPP TS 29.571 [13] | IPv4 Address |  |
| Ipv6Prefix | 3GPP TS 29.571 [13] | IPv6 Prefix |  |
| Uri | 3GPP TS 29.571 [13] | Uniform Resource Identifier |  |
| Supi | 3GPP TS 29.571 [13] | Subscription Permanent Identifier |  |
| Pei | 3GPP TS 29.571 [13] | Permanent Equipment Identifier |  |
| Gpsi | 3GPP TS 29.571 [13] | General Public Subscription Identifier |  |
| AccessType | 3GPP TS 29.571 [13] | Access Type (3GPP or non-3GPP access) |  |
| SupportedFeatures | 3GPP TS 29.571 [13] | Supported features |  |
| Qfi | 3GPP TS 29.571 [13] | QoS Flow Identifier |  |
| pduSessionId | 3GPP TS 29.571 [13] | PDU Session Identifier |  |
| pduSessionType | 3GPP TS 29.571 [13] | PDU Session Type |  |
| Ambr | 3GPP TS 29.571 [13] | PDU Session Aggregate Maximum Bit Rate |  |
| 5Qi | 3GPP TS 29.571 [13] | 5G QoS Identifier |  |
| Arp | 3GPP TS 29.571 [13] | Allocation and Retention Priority |  |
| ReflectiveQoSAttribute | 3GPP TS 29.571 [13] | Reflective QoS Attribute |  |
| Dynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics for a 5QI that is neither standardized nor pre-configured. |  |
| NonDynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics that replace the default QoS characteristics for a standardized or pre-configured 5QI. |  |
| PacketLossRate | 3GPP TS 29.571 [13] | Packet Loss Rate |  |
| NotificationControl | 3GPP TS 29.571 [13] | Notification Control |  |
| Dnn | 3GPP TS 29.571 [13] | Data Network Name |  |
| Snssai | 3GPP TS 29.571 [13] | Single Network Slice Selection Assistance Information |  |
| NfInstanceId | 3GPP TS 29.571 [13] | NF Instance Identifier |  |
| UserLocation | 3GPP TS 29.571 [13] | User Location |  |
| TimeZone | 3GPP TS 29.571 [13] | Time Zone |  |
| ProblemDetails | 3GPP TS 29.571 [13] | Error description |  |
| UpSecurity | 3GPP TS 29.571 [13] | User Plane Security Policy Enforcement information |  |
| RefToBinaryData | 3GPP TS 29.571 [13] | Cross-Reference to binary data encoded within a binary body part in an HTTP multipart message. |  |
| Guami | 3GPP TS 29.571 [13] | Globally Unique AMF ID |  |
| BackupAmfInfo | 3GPP TS 29.571 [13] | Backup AMF Information |  |
| PresenceState | 3GPP TS 29.571 [13] | Indicates the UE presence in or out of a LADN service area |  |
| TraceData | 3GPP TS 29.571 [13] | Trace control and configuration parameters |  |
| PlmnId | 3GPP TS 29.571 [13] | PLMN Identity |  |
| RatType | 3GPP TS 29.571 [13] | RAT Type |  |
| NgApCause | 3GPP TS 29.571 [13] | NGAP Cause |  |
| 5GMmCause | 3GPP TS 29.571 [13] | 5G MM Cause |  |
| DurationSec | 3GPP TS 29.571 [13] | Duration in units of seconds |  |
| AdditionalQosFlowInfo | 3GPP TS 29.571 [13] | Additional QoS Flow Information |  |
| NfGroupId | 3GPP TS 29.571 [13] | Network Function Group Id |  |
| SecondaryRatUsageReport | 3GPP TS 29.571 [13] | Secondary RAT Usage Report |  |
| SecondaryRatUsageInfo | 3GPP TS 29.571 [13] | Secondary RAT Usage Information |  |
| ServiceName | 3GPP TS 29.510 [19] | Service Name |  |
| NgRanTargetId | 3GPP TS 29.518 [20] | NG-RAN Target Id |  |
| RoamingChargingProfile | 3GPP TS 32.291 [26] | Roaming Charging Profile |  |

* * * Next Change * * * *

##### 6.1.6.2.11	Type: HsmfUpdateData

Table 6.1.6.2.11-1: Definition of type HsmfUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestIndication | RequestIndication | M | 1 | This IE shall indicate the request type. |  |
| pei | Pei | C | 0..1 | This IE shall be present if it is available and has not been provided earlier to the H-SMF. When present, this IE shall contain the permanent equipment identifier. |  |
| vcnTunnelInfo | TunnelInfo | C | 1 | This IE shall be present if the N9 tunnel information on the visited CN side provided earlier to the H-SMF has changed. When present, this IE shall contain the new N9 tunnel information on the visited CN side. |  |
| servingNetwork | PlmnId | C | 0..1 | This IE shall contain the serving core network operator PLMN ID, if it has changed. |  |
| anType | AccessType | C | 0..1 | This IE shall be present if the Access Network Type provided earlier to the H-SMF has changed, e.g. during a handover of the PDU session between 3GPP access and untrusted non-3GPP access (see clause 5.2.2.8.2.5). When present, this IE shall indicate the new Access Network Type to which the PDU session is to be associated. |  |
| secondAnType | AccessType | C | 0..1 | This IE shall indicate the additional Access Network Type to which the PDU session is to be associated. This IE shall be present when the UE registers over an additional access type. | MAPDU |
| ratType | RatType | C | 0..1 | This IE shall be present and indicate the RAT Type used by the UE, if available, upon a change of RAT Type. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall be present if it is available, the UE Location has changed and needs to be reported to the H-SMF. When present, this IE shall contain: - the new UE location information; and - the timestamp, if available, indicating the UTC time when the UeLocation information was acquired. (NOTE 1) |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall be present if it is available, the UE Time Zone has changed and needs to be reported to the H-SMF. When present, this IE shall contain the new UE Time Zone. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType indicates a non-3GPP access and a valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location; and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. (NOTE 1) |  |
| pauseCharging | boolean | C | 0..1 | This IE shall be present if the H-SMF enabled the use of Pause Pause of Charging for the PDU session during the PDU session establishment and Pause of Charging needs to be started or stopped (see clause 4.4.4 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: to Start Pause of Charging; - false: to Stop Pause of Charging. |  |
| pti | ProcedureTransactionId | C | 0..1 | This IE shall be present if the requestIndication indicates a UE requested PDU session modification or release. When present, it shall contain the PTI value received from the UE. |  |
| n1SmInfoFromUe | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF has received known N1 SM information from the UE that does not need to be interpreted by the V-SMF. When present, this IE shall reference the n1SmInfoFromUe binary data (see clause 6.1.6.4.4). |  |
| unknownN1SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF has received unknown N1 SM information from the UE. When present, this IE shall reference the unknownN1SmInfo binary data (see clause 6.1.6.4.4). |  |
| qosFlowsRelNotifyList | array(QosFlowItem) | C | 1..N | This IE shall be present if QoS flows have been released. |  |
| qosFlowsNotifyList | array(QosFlowNotifyItem) | C | 1..N | This IE shall be present if the QoS targets for GBR QoS flow(s) are not fulfilled anymore or when they are fulfilled again. |  |
| NotifyList | array(PduSessionNotifyItem) | C | 1..N | Description of notifications related to the PDU session. This IE shall be present if the NG-RAN has established user plane resources for the PDU session that do not fulfil the User Plane Security Enforcement with a value Preferred, or when the user plane security enforcement is fulfilled again. |  |
| epsBearerId | array(EpsBearerId) | C | 0..N | This IE shall be present during an EPS to 5GS handover execution using the N26 interface. When present, it shall contain the list of EPS bearer Id(s) successfully handed over to 5GS. The array shall be empty if no resource was successfully allocated in 5GS for any PDU session. |  |
| hoPreparationIndication | boolean | C | 0..1 | This IE shall be present during an EPS to 5GS handover preparation and handover execution using the N26 interface. When present, it shall be set as follows: - true: an EPS to 5GS handover preparation is in progress; the PGW-C/SMF shall not switch the DL user plane of the PDU session yet. - false: there is no on-going EPS to 5GS handover preparation in progress. If a handover preparation was in progress, the handover has been completed. The PGW-C/SMF shall switch the DL user plane of the PDU session using the N9 tunnel information that has been received in the vcnTunnelInfo. It shall be set to "true" during an EPS to 5GS handover preparation using the N26 interface. It shall be set to "false" during an EPS to 5GS handover execution using the N26 interface. |  |
| revokeEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present to request the H-SMF to revoke some EBIs (see clause 4.11.1.4.1 of 3GPP TS 23.502 [3]). When present, it shall contain the EBIs to revoke. |  |
| cause | Cause | C | 0..1 | This IE shall be present and set as specified in clause 5.2.2.8.2.6 during P-CSCF restoration procedure for home-routed PDU session. When present, this IE shall indicate the NF Service Consumer cause of the requested modification. |  |
| ngApCause | NgApCause | C | 0..1 | This IE shall be present, if the information is available and if this information is permitted to be sent to the H-SMF operator according to the V-SMF operator's policy. When present, this IE shall indicate the NGAP cause for the requested modification. |  |
| 5gMmCauseValue | 5GMmCause | C | 0..1 | The V-SMF shall include this IE if it received it from the AMF and if this information is permitted to be sent to the H-SMF operator according to the V-SMF operator's policy. |  |
| alwaysOnRequested | boolean | C | 0..1 | This IE shall be present and set to true if the UE requests to change the PDU session to an always-on PDU session and this is allowed by local policy in the V-SMF. When present, it shall be set as follows: - true: request for an always-on PDU session - false (default): not a request for an always-on PDU session |  |
| epsInterworkingInd | EpsInterworkingIndication | O | 0..1 | This IE may be present if the indication has been received from AMF and is allowed to be forwarded to H-SMF by operator configuration. When present, this IE shall indicate whether the PDU session may possibly be moved to EPS and whether N26 interface to be used during EPS interworking procedures. |  |
| secondaryRatUsageReport | array(SecondaryRatUsageReport) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows. (NOTE 2) |  |
| secondaryRatUsageInfo | array(SecondaryRatUsageInfo) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows and/or the whole PDU session. |  |
| anTypeCanBeChanged | boolean | C | 0..1 | This IE shall be present and set to true to indicate that the Access Network Type associated to the PDU session can be changed (see clause 5.2.2.8.2.2), during a Service Request procedure (see clauses 4.2.3.2 and 4.3.3.3 of 3GPP TS 23.502 [3])), in response to paging or NAS notification indicating non-3GPP access, when the PDU Session for which the UE was paged or notified is in the List Of Allowed PDU Sessions provided by the UE, and the AMF received N2 SM Information only or N1 SM Container and N2 SM Information from the SMF in step 3a of clause 4.2.3.3 of 3GPP TS 23.502 [3]. When present, it shall be set as follows: - true: the access type of the PDU session can be changed. - false (default): the access type of the PDU session cannot be changed. |  |
| maReleaseInd | MaReleaseIndication | C | 0..1 | This IE shall be present if a MA PDU session is requested to be released over a single access. When present, it shall indicate the access to be released. | MAPDU |
| psaInfo | array(PsaInformation) | C | 1..N | This IE shall be present, for a PDU session with an I-SMF, if one or more PSAs (and UL CL or BP) are inserted and/or removed by the I-SMF. | DTSSA |
| NOTE 1: In shared networks, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. NOTE 2: An SMF complying with this version of the specification should report secondary RAT usage using the secondaryRatUsageInfo attribute that replaces the secondaryRatUsageReport attribute. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.12	Type: HsmfUpdatedData

Table 6.1.6.2.12-1: Definition of type HsmfUpdatedData

| Attribute name | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1SmInfoToUe | RefToBinaryData |  | C |  | 0..1 |  | This IE shall be present if the H-SMF needs to send N1 SM information to the UE that does not need to be interpreted by the V-SMF. When present, this IE shall reference the n1SmInfoToUe binary data (see clause 6.1.6.4.4). |  |  |  |  |
| n4Info | N4Information |  | O |  | 0..1 |  | This IE may be present if the SMF needs to send N4 information to the I-SMF for the control of traffic offloaded at a PSA controlled by an I-SMF. |  | DTSSA |  |  |
| n4InfoExt1 | N4Information |  | O |  | 0..1 |  | This IE may be present if the SMF needs to send additional N4 information to the I-SMF for the control of traffic offloaded at a PSA controlled by an I-SMF (e.g. during a change of PSA). |  | DTSSA |  |  |
| n4InfoExt2 | N4Information | O | 0..1 | This IE may be present if the SMF needs to send additional N4 information to the I-SMF for the control of traffic offloaded at a PSA controlled by an I-SMF (e.g. during a change of PSA). | DTSSA |

* * * Next Change * * * *

##### 6.1.6.2.x	Type: PsaInformation

Table 6.1.6.2.x-1: Definition of type PsaInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| psaInd | PsaIndication | M | 1 | This IE shall indicate, for a PDU session with an I-SMF, if a PSA and UL CL or BP is inserted or removed by the I-SMF. |
| dnaiList | array(Dnai) | M | 1.N | This IE shall indicate the DNAI(s) supported by the PSA that is inserted or removed. |
| ueIpv6Prefix | Ipv6Prefix | C | 0..1 | This IE shall be present if a PSA and UL CL or BP is inserted or removed, and IPv6 multi-homing applies to the PDU session. |

##### 6.1.6.2.y	Type: DnaiInformation

Table 6.1.6.2.y-1: Definition of type DnaiInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dnai | Dnai | M | 1 |  |
| noDnaiChangeInd | boolean | C | 0..1 | This IE shall be sent by the SMF to the I-SMF during the insertion of a PSA and BP/UL CL controlled by I-SMF. When present, it shall be set as follows: - true: DNAI shall not be changed; - false: DNAI may be changed. |
| noLocalPsaChangeInd | boolean | C | 0..1 | This IE shall be sent by the SMF to the I-SMF during the insertion of a PSA and BP/UL CL controlled by I-SMF. When present, it shall be set as follows: - true: local PSA shall not be changed; - false: local PSA may be changed. |

##### 6.1.6.2.z	Type: N4Information

Table 6.1.6.2.z-1: Definition of type N4Information

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n4MessageType | N4MessageType | M | 1 | This IE shall indicate the PFCP message signalled in the n4MessagePayload. |
| n4MessagePayload | RefToBinaryData | M | 1 | This IE shall reference the N4 Information binary data (for the n4Info attribute) or the N4 Information Ext1 binary data (for the n4InfoExt1 attribute), see clause 6.1.6.4.x. |
| n4DnaiInfo | DnaiInformation | C | 0..1 | This IE shall be present if the N4 information relates to a PSA. When present, it shall indicate the DNAI related to the N4 Information. |

##### 6.1.6.3.x	Enumeration: PsaIndication

The enumeration PsaIndication indicates whether a PSA and an UL CL or BP has been inserted or removed to/from the data path of a PDU session by an I-SMF. It shall comply with the provisions defined in table 6.1.6.3.x-1.

Table 6.1.6.3.x-1: Enumeration PsaIndication

| Enumeration value | Description |
| --- | --- |
| "PSA_INSERTED" | A PSA and UL CL or BP has been inserted into the data path of the PDU session. |
| "PSA_REMOVED" | A PSA and UL CL or BP has been removed from the data path of the PDU session. |

##### 6.1.6.3.y	Enumeration: N4MessageType

The enumeration N4MessageType indicates the PFCP message type sent within a N4 Message Payload. It shall comply with the provisions defined in table 6.1.6.3.x-1.

Table 6.1.6.3.x-1: Enumeration N4MessageType

| Enumeration value | Description |
| --- | --- |
| "PFCP_SES_EST_REQ" | PFCP Session Establishment Request |
| "PFCP_SES_EST_RSP" | PFCP Session Establishment Response |
| "PFCP_SES_MOD_REQ" | PFCP Session Modification Request |
| "PFCP_SES_MOD_RSP" | PFCP Session Modification Response |
| "PFCP_SES_DEL_REQ" | PFCP Session Deletion Request |
| "PFCP_SES_DEL_RSP" | PFCP Session Deletion Response |
| "PFCP_SES_REP_REQ" | PFCP Session Report Request |
| "PFCP_SES_REP_RSP" | PFCP Session Report Response |

* * * Next Change * * * *

##### 6.1.6.4.x	N4 Message Payload

The N4 Message Payload shall encode a PFCP session related message as specified in 3GPP TS 29.244 [x], using the vnd.3gpp.pfcp content-type.

* * * Next Change * * * *

## A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.1.0.alpha-1'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

/pdu-sessions/{pduSessionRef}/modify:

post:

summary:  Update (initiated by V-SMF or I-SMF)

tags:

- Individual PDU session (H-SMF or SMF)

operationId: UpdatePduSession

parameters:

- name: pduSessionRef

in: path

description:  PDU session reference

required: true

schema:

type: string

requestBody:

description: representation of the updates to apply to the PDU session

required: true

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/HsmfUpdateData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/HsmfUpdateData'

binaryDataN1SmInfoFromUe:

type: string

format: binary

binaryDataUnknownN1SmInfo:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoFromUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataUnknownN1SmInfo:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

responses:

'200':

description: successful update of a PDU session with content in the response

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/HsmfUpdatedData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/HsmfUpdatedData'

binaryDataN1SmInfoToUe:

type: string

format: binary

binaryDataN4Information:

type: string

format: binary

binaryDataN4InformationExt1:

type: string

format: binary

binaryDataN4InformationExt2:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoToUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN4Information:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt1:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt2:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

'204':

description: successful update of a PDU session without content in the response

'400':

$ref: '#/components/responses/HsmfUpdateError'

'403':

$ref: '#/components/responses/HsmfUpdateError'

'404':

$ref: '#/components/responses/HsmfUpdateError'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: '#/components/responses/HsmfUpdateError'

'503':

$ref: '#/components/responses/HsmfUpdateError'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

[…]

HsmfUpdateData:

type: object

properties:

requestIndication:

$ref: '#/components/schemas/RequestIndication'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

vcnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

secondAnType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

pauseCharging:

type: boolean

pti:

$ref: '#/components/schemas/ProcedureTransactionId'

n1SmInfoFromUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

unknownN1SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

qosFlowsRelNotifyList:

type: array

items:

$ref: '#/components/schemas/QosFlowItem'

minItems: 1

qosFlowsNotifyList:

type: array

items:

$ref: '#/components/schemas/QosFlowNotifyItem'

minItems: 1

NotifyList:

type: array

items:

$ref: '#/components/schemas/PduSessionNotifyItem'

minItems: 1

epsBearerId:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 0

hoPreparationIndication:

type: boolean

revokeEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

cause:

$ref: '#/components/schemas/Cause'

ngApCause:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NgApCause'

5gMmCauseValue:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5GMmCause'

alwaysOnRequested:

type: boolean

default: false

epsInterworkingInd:

$ref: '#/components/schemas/EpsInterworkingIndication'

secondaryRatUsageReport:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageReport'

minItems: 1

secondaryRatUsageInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageInfo'

minItems: 1

anTypeCanBeChanged:

type: boolean

default: false

maReleaseInd:

$ref: '#/components/schemas/MaReleaseIndication'

psaInfo:

$ref: '#/components/schemas/PsaInformation'

required:

- requestIndication

HsmfUpdatedData:

type: object

properties:

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

[…]

ExemptionInd:

type: object

properties:

dnnCongestion:

type: boolean

default: false

snssaiOnlyCongestion:

type: boolean

default: false

snssaiDnnCongestion:

type: boolean

default: false

PsaInformation:

type: object

properties:

psaInd:

$ref: '#/components/schemas/PsaIndication'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

DnaiInformation:

type: object

properties:

dnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

noDnaiChangeInd:

type: boolean

noLocalPsaChangeInd:

type: boolean

required:

- dnai

N4Information:

type: object

properties:

n4MessageType:

$ref: '#/components/schemas/N4MessageType'

n4MessagePayload:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n4DnaiInfo:

$ref: '#/components/schemas/DnaiInformation'

required:

- n4MessageType

- n4MessagePayload

[…]

SmContextType:

anyOf:

- type: string

enum:

- EPS_PDN_CONNECTION

- SM_CONTEXT

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- EPS_PDN_CONNECTION

- SM_CONTEXT

PsaIndication:

anyOf:

- type: string

enum:

- PSA_INSERTED

- PSA_REMOVED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- PSA_INSERTED

- PSA_REMOVED

N4MessageType:

anyOf:

- type: string

enum:

- PFCP_SES_EST_REQ

- PFCP_SES_EST_RSP

- PFCP_SES_MOD_REQ

- PFCP_SES_MOD_RSP

- PFCP_SES_DEL_REQ

- PFCP_SES_DEL_RSP

- PFCP_SES_REP_REQ

- PFCP_SES_REP_RSP

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- PFCP_SES_EST_REQ

- PFCP_SES_EST_RSP

- PFCP_SES_MOD_REQ

- PFCP_SES_MOD_RSP

- PFCP_SES_DEL_REQ

- PFCP_SES_DEL_RSP

- PFCP_SES_REP_REQ

- PFCP_SES_REP_RSP

[…]

* * * End of Changes * * * *