3GPP TSG-CT Meeting #85	CP-192080

# Newport Beach, US; 16th – 17th September 2019          (Revision of C4-193785, C4-193333)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.573 | CR | 0020 | rev | 2 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Exchange IPX security information lists |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc16 |  |  |  |  |  | Date: |  |  | 2019-09-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 13.2.2.2, step 3 of Procedure for Key agreement and Parameter exchange in TS 33.501, the two SEPPs shall exchange IPX security information lists. In section 13.2.2.4.3, the IPX security information list includes the IPX provider identifier and the List of raw public keys or certificates for that IPX. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support to exchange the IPX security information lists between SEPPs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.x (new), 6.1.4.1, 6.1.4.y (new), 6.1.5.1, 6.1.5.2.a (new), 6.1.5.2.b (new), 6.1.5.2.c (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible new feature to the OpenAPI file for N32 Handshake API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1: Reuse the current parameter exchange operation; Add FFS for the pattern of rawPublicKeyList and certificateList. Rev.2: (for CT#85): Fixes to OpenAPI: rawPublicKeyList shall be rawPublicKeyList: certificateList shall be certificateList: |  |  |  |  |  |  |  |  |

* * * * Begin of Change * * * *

5.2.3.x	Parameter Exchange Procedure for Security Information list Exchange

The initiating SEPP shall initiate a Security Information list exchange procedure towards the responding SEPP to exchange the Security Information lists that contain information on IPX public keys or certificates that are needed to verify IPX modifications at the receiving SEPP as specified in clause 13.2.2.2 of 3GPP TS 33.501 [6].

The procedure is described in Figure 5.2.3.x-1 below.

Figure 5.2.3.x-1: Parameter Exchange Procedure for Security Information List exchange

1.	The initiating SEPP issues a HTTP POST request towards the responding SEPP with the request body containing the "SecParamExchReqData" IE carrying the following information:

-	IPX provider identifier connected to the initiating SEPP;

-	List of raw public keys or certificates for that IPX.

2a.	On successful processing of the request, the responding SEPP shall respond to the initiating SEPP with a "200 OK" status code and a POST response body that contains the "SecParamExchRspData" IE carrying the following information:

-	IPX provider identifier connected to the responding SEPP;

-	List of raw public keys or certificates for that IPX.

2b.	On failure, the responding SEPP shall respond to the initiating SEPP with an appropriate 4xx/5xx status code as specified in clause 6.1.4.3.

* * * * Next Change * * * *

6.1.4.3.1	Description

This custom operation is used between the SEPPs to exchange the parameters for the N32-f connection. The HTTP method POST shall be used on the following URI:

URI: {apiRoot}/n32c-handshake/v1/exchange-params

This operation shall support the resource URI variables defined in table 6.1.4.3.1-1.

Table 6.1.4.3.1-1: URI variables for this Operation

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1. |

6.1.4.3.2	Operation Definition

This operation shall support the request data structures and response codes specified in tables 6.1.4.3.2-1 and 6.1.4.3.2-2.

Table 6.1.4.3.2-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SecParamExchReqData | M | 1 | The IE shall contain the parameters requested by the requesting SEPP. |

Table 6.1.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SecParamExchRspData | M | 1 | 200 OK | This represents the successful processing of the requested parameters. The SEPP shall provide the selected parameters (i.e selected cipher suite and/or selected protection policy) depending on what was requested by the requesting SEPP and what is supported by the responding SEPP, or the SEPP shall provide the security information lists of the connected IPXs. |
| ProblemDetails | M | 1 | 4xx / 5xx | All the mandatory to support 4xx and 5xx status codes as specified in clause 5.2.7.1 and their corresponding application errors specified in clause 5.2.7.2 of 3GPP TS 29.500 [4] shall be supported. |

* * * * Next Change * * * *

6.1.5.1	General

This clause specifies the application data model supported by the API.

Table 6.1.5.1-1 specifies the data types defined for the N32 interface.

Table 6.1.5.1-1: N32 specific Data Types

| Data type | Section defined | Description |
| --- | --- | --- |
| SecNegotiateReqData | 6.1.5.2.2 | Defines the security capabilities of a SEPP sent to a receiving SEPP. |
| SecNegotiateRspData | 6.1.5.2.3 | Defines the selected security capabilities by a SEPP. |
| SecurityCapability | 6.1.5.3.3 | Enumeration of security capabilities. |
| SecParamExchReqData | 6.1.5.2.4 | Request data structure for parameter exchange |
| SecParamExchRspData | 6.1.5.2.5 | Response data structure for parameter exchange |
| ProtectionPolicy | 6.1.5.2.6 | The protection policy to be negotiated between the SEPPs. |
| ApiIeMapping | 6.1.5.2.7 | API URI to IE mapping on which the protection policy needs to be applied. |
| IeInfo | 6.1.5.2.8 |  |
| ApiSignature | 6.1.5.2.9 |  |
| N32fContextInfo | 6.1.5.2.10 | N32-f context information |
| N32fErrorInfo | 6.1.5.2.11 | N32-f error information. |
| FailedModificationInfo | 6.1.5.2.12 | Information on N32-f modifications block that failed to process. |
| N32fErrorDetail | 6.1.5.2.13 | Details about the N32f error. |
| CallbackName | 6.1.5.2.14 | Callback Name. |
| IpxProviderSecInfo | 6.1.5.2.a | Defines the security information list of an IPX. |
| HttpMethod | 6.1.5.3.4 | Enumeration of HTTP methods. |
| IeType | 6.1.5.3.5 | Enumeration of types of IEs (i.e kind of IE) to specify the protection policy. |
| IeLocation | 6.1.5.3.6 | Location of the IE in a HTTP message. |
| N32fErrorType | 6.1.5.3.7 | Type of error while processing N32-f message. |
| FailureReason | 6.1.5.3.8 | Reason for failure to reconstruct a HTTP/2 message from N32-f message. |

Table 6.1.5.1-2 specifies data types re-used by the N32 interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Namf service based interface.

Table 6.1.5.1-2: N32 re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Fqdn | 3GPP TS 29. 510 [18] |  |

* * * * Next Change * * * *

6.1.5.2.4	Type: SecParamExchReqData

Table 6.1.5.2.4-1: Definition of type SecParamExchReqData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n32fContextId | string | M | 1 | This IE shall contain the context identifier to be used by the responding SEPP for subsequent JOSE protected message forwarding procedure over N32-f towards the initiating SEPP. The initiating SEPP shall use this context identifier to locate the cipher suite and protection policy exchanged and agreed to be used with the responding SEPP, for the message forwarding procedure over N32-f. |
| jweCipherSuiteList | array(string) | C | 1..N | This IE shall be present during the parameter exchange procedure for cipher suite negotiation (see clause 5.2.3.2). When present, this IE shall contain the ordered list of JWE cipher suites supported by the requesting SEPP. Valid values for the string are as specified in clause 5.1 of IETF RFC 7518 [13]. |
| jwsCipherSuiteList | array(string) | C | 1..N | This IE shall be present during the parameter exchange procedure for cipher suite negotiation (see clause 5.2.3.2). When present, this IE shall contain the ordered list of JWS cipher suites supported by the requesting SEPP. Valid values for the string are as specified in clause 3.1 of IETF RFC 7518 [13]. |
| protectionPolicyInfo | ProtectionPolicy | C | 0..1 | This IE shall be present during the parameter exchange procedure for protection policy exchange (see clause 5.2.3.3). When present, this IE shall contain the protection policy requested by the requesting SEPP. |
| ipxProviderSecInfoList | array(IpxProviderSecInfo) | C | 1..N | This IE includes the list of IPX security information. |

6.1.5.2.5	Type: SecParamExchRspData

Table 6.1.5.2.5-1: Definition of type SecParamExchRspData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n32fContextId | string | M | 1 | This IE shall contain the context identifier to be used by the initiating SEPP for subsequent JOSE protected message forwarding procedure over N32-f towards the responding SEPP. The responding SEPP shall use this context identifier to locate the cipher suite and protection policy exchanged and agreed to be used with the initiating SEPP, for the message forwarding procedure over N32-f. |
| selectedJweCipherSuite | string | C | 1 | This IE shall be present during the parameter exchange procedure for cipher suite negotiation (see clause 5.2.3.2). When present, this IE shall contain the JWE cipher suite selected by the responding SEPP. |
| selectedJwsCipherSuite | string | C | 1 | This IE shall be present during the parameter exchange procedure for cipher suite negotiation (see clause 5.2.3.2). When present, this IE shall contain the JWS cipher suite selected by the responding SEPP. |
| selProtectionPolicyInfo | ProtectionPolicy | C | 0..1 | This IE shall be present during the parameter exchange procedure for protection policy exchange (see clause 5.2.3.3). When present, this IE shall contain the protection policy selected by the responding SEPP. |
| ipxProviderSecInfoList | array(IpxProviderSecInfo) | C | 1..N | This IE includes the list of IPX security information. |

* * * * Next Change * * * *

6.1.5.2.a	Type: IpxProviderSecInfo

Table 6.1.5.2.a-1: Definition of type IpxProviderSecInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipxProviderId | Fqdn | M | 1 | This IE shall uniquely identify the IPX. |
| rawPublicKeyList | array(string) | C | 1..N | This IE includes the list of raw public keys for the IPX. |
| certificateList | array(string) | C | 1..N | This IE includes the list of certificates for the IPX. |
| NOTE: Either the rawPublicKeyList attribute, or the certificateList attribute, shall be present. |  |  |  |  |

Editor's Note: How to define the pattern of rawPublicKeyList and certificateList is FFS.

* * * * Next Change * * * *

A.2	N32 Handshake API

openapi: 3.0.0

info:

version: '1.0.1'

title: 'N32 Handshake API'

description: |

N32-c Handshake Service.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

servers:

- url: '{apiRoot}/n32c-handshake/v1'

variables:

apiRoot:

default: https://example.com

description:  apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

externalDocs:

description: 3GPP TS 29.573 V15.2.0; 5G System; Public Land Mobile Network (PLMN) Interconnection; Stage 3

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.573/

paths:

/exchange-capability:

post:

summary:  Security Capability Negotiation

tags:

- Security Capability Negotiation

operationId: PostExchangeCapability

requestBody:

description: Custom operation for security capability negotiation

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SecNegotiateReqData'

responses:

'200':

description: OK (Successful negitiation of security capabilities)

content:

application/json:

schema:

$ref: '#/components/schemas/SecNegotiateRspData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/exchange-params:

post:

summary: Parameter Exchange

tags:

- Parameter Exchange

operationId: PostExchangeParams

requestBody:

description: Custom operation for parameter exchange

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SecParamExchReqData'

responses:

'200':

description: OK (Successful exchange of parameters)

content:

application/json:

schema:

$ref: '#/components/schemas/SecParamExchRspData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/n32f-terminate:

post:

summary: N32-f Context Terminate

tags:

- N32-f Context Terminate

operationId: PostN32fTerminate

requestBody:

description: Custom operation for n32-f context termination

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/N32fContextInfo'

responses:

'200':

description: OK (Successful exchange of parameters)

content:

application/json:

schema:

$ref: '#/components/schemas/N32fContextInfo'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/n32f-error:

post:

summary: N32-f Error Reporting Procedure

tags:

- N32-f Error Report

operationId: PostN32fError

requestBody:

description: Custom operation for n32-f error reporting procedure

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/N32fErrorInfo'

responses:

'204':

description: successful error reporting

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected errorcomponents:

schemas:

SecurityCapability:

anyOf:

- type: string

enum:

- TLS

- PRINS

- type: string

ApiSignature:

oneOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

- $ref: '#/components/schemas/CallbackName'

HttpMethod:

anyOf:

- type: string

enum:

- GET

- PUT

- POST

- DELETE

- PATCH

- HEAD

- OPTIONS

- CONNECT

- TRACE

- type: string

IeType:

anyOf:

- type: string

enum:

- UEID

- LOCATION

- KEY_MATERIAL

- AUTHENTICATION_MATERIAL

- AUTHORIZATION_TOKEN

- OTHER

- NONSENSITIVE

- type: string

IeLocation:

anyOf:

- type: string

enum:

- URI_PARAM

- HEADER

- BODY

- MULTIPART_BINARY

- type: string

IeInfo:

type: object

required:

- ieLoc

- ieType

properties:

ieLoc:

$ref: '#/components/schemas/IeLocation'

ieType:

$ref: '#/components/schemas/IeType'

reqIe:

type: string

rspIe:

type: string

isModifiable:

type: boolean

ApiIeMapping:

type: object

required:

- apiSignature

- apiMethod

- IeList

properties:

apiSignature:

$ref: '#/components/schemas/ApiSignature'

apiMethod:

$ref: '#/components/schemas/HttpMethod'

IeList:

type: array

items:

$ref: '#/components/schemas/IeInfo'

minItems: 1

ProtectionPolicy:

type: object

required:

- apiIeMappingList

properties:

apiIeMappingList:

type: array

items:

$ref: '#/components/schemas/ApiIeMapping'

minItems: 1

dataTypeEncPolicy:

type: array

items:

$ref: '#/components/schemas/IeType'

minItems: 1

SecNegotiateReqData:

type: object

required:

- sender

- supportedSecCapabilityList

properties:

sender:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

supportedSecCapabilityList:

type: array

items:

$ref: '#/components/schemas/SecurityCapability'

minItems: 1

SecNegotiateRspData:

type: object

required:

- sender

- selectedSecCapability

properties:

sender:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

selectedSecCapability:

$ref: '#/components/schemas/SecurityCapability'

SecParamExchReqData:

type: object

required:

- n32fContextId

properties:

n32fContextId:

type: string

jweCipherSuiteList:

type: array

items:

type: string

minItems: 1

jwsCipherSuiteList:

type: array

items:

type: string

minItems: 1

protectionPolicyInfo:

$ref: '#/components/schemas/ProtectionPolicy'

ipxProviderSecInfoList:

type: array

items:

$ref: '#/components/schemas/IpxProviderSecInfo'

minItems: 1

SecParamExchRspData:

type: object

required:

- n32fContextId

properties:

n32fContextId:

type: string

selectedJweCipherSuite:

type: string

selectedJwsCipherSuite:

type: string

selProtectionPolicyInfo:

$ref: '#/components/schemas/ProtectionPolicy'

ipxProviderSecInfoList:

type: array

items:

$ref: '#/components/schemas/IpxProviderSecInfo'

minItems: 1

N32fContextInfo:

type: object

required:

- n32fContextId

properties:

n32fContextId:

type: string

CallbackName:

type: object

required:

- callbackType

properties:

callbackType:

type: string

N32fErrorInfo:

type: object

required:

- n32fMessageId

- n32fErrorType

properties:

n32fMessageId:

type: string

n32fErrorType:

$ref: '#/components/schemas/N32fErrorType'

failedModificationList:

type: array

items:

$ref: '#/components/schemas/FailedModificationInfo'

minItems: 1

errorDetailsList:

type: array

items:

$ref: '#/components/schemas/N32fErrorDetail'

minItems: 1

FailedModificationInfo:

type: object

required:

- ipxId

- n32fErrorType

properties:

ipxId:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

n32fErrorType:

$ref: '#/components/schemas/N32fErrorType'

N32fErrorDetail:

type: object

required:

- attribute

- msgReconstructFailReason

properties:

attribute:

type: string

msgReconstructFailReason:

$ref: '#/components/schemas/FailureReason'

IpxProviderSecInfo:

type: object

required:

- ipxProviderId

properties:

ipxProviderId:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

rawPublicKeyList:

type: array

items:

type: string

minItems: 1

certificateList:

type: array

items:

type: string

minItems: 1

N32fErrorType:

anyOf:

- type: string

enum:

- INTEGRITY_CHECK_FAILED

- INTEGRITY_CHECK_ON_MODIFICATIONS_FAILED

- MODIFICATIONS_INSTRUCTIONS_FAILED

- DECIPHERING_FAILED

- MESSAGE_RECONSTRUCTION_FAILED

- type: string

FailureReason:

anyOf:

- type: string

enum:

- INVALID_JSON_POINTER

- INVALID_INDEX_TO_ENCRYPTED_BLOCK

- INVALID_HTTP_HEADER

- type: string

* * * * End of Change * * * *