3GPP TSG-CT Meeting #85	CP-192195

# Newport Beach, US; 16th – 17th September 2019          (Revision of C4-193637, C4-193394)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.571 | CR | 0121 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Small Data Rate Control Status |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-09-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Alginment with Stage 2, For small data rate control 1) during PDU session establishment: a) if available, the AMF provides the small data rate control status to the SMF (in Nsmf_PDUSession_CreateSMContext ) b) for home-routed sessions, the V-SMF provides the small data rate control status to the H-SMF if received from the AMF which is then stored in the UE context c) the small data rate control parameters are provided by the SMF to the UPF/NEF 2) during PDU session release: a) the UPF/NEF provides the current small data rate status to the SMF, and b) the SMF provides the current small data rate status to the AMF which is then stored in the UE context 3) The Namf_Communication_UEContextTransfer to contain a List of Small Data Rate Control Statuses And we can read the details about small data rate control Status in stage 2 specification 23.501 clause 5.31.14.3. Based on the information above small data rate control Status will be transferred from AMF to SMF, AMF to AMF as UE context, SMF to AMF, SMF to NEF, and 29.518, 29.502, 29.541 will be impacted, therefore small data rate control Status should be defined as a common component in 29.571. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define structured type SmallDataRateStatus which stand for the small data Rate Control Status. - Revsion of Yaml file accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Small data rate control won’t be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.x(new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features in the OpenAPI specification file of CommonData.yaml. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1: additional exception reports is only for uplink; update the name of the attributes. Rev.2: (for CT#85): Fixes to OpenAPI: type shall be $ref |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

5.4.4.x	Type: SmallDataRateStatus

Table 5.4.4.x-1: Definition of type SmallDataRateStatus

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| remainPacketsUl | integer | M | 1 | This IE shall contain the number of packets which are still allowed to send in uplink in the given time unit for uplink small data rate control, see clause 5.31.14.3 of 3GPP TS 23.501 [8]. |
| remainPacketsDl | integer | M | 1 | This IE shall contain the number of packets which are still allowed to send in downlink in the given time unit for donwlink small data rate control, see clause 5.31.14.3 of 3GPP TS 23.501 [8]. |
| validityTime | DateTime | M | 1 | This IE shall indicate the point in time at which the small data rate control status will be invalid, see clause 5.31.14.3 of 3GPP TS 23.501 [8]. |
| remainExReportsUl | integer | O | 0..1 | When present, indicates number of additional exception reports which are still allowed to send in uplink in the given time unit for uplink small data rate control, see clause 5.31.14.3 of 3GPP TS 23.501 [8]. |

* * * Next Change * * * *

A.2	Data related to Common Data Types

openapi: 3.0.0

**********skipped for clarity**********

MaPduCapability:

type: object

properties:

atsssLL:

type: boolean

default: false

mptcp:

type: boolean

default: boolean

AtsssCapability:

type: object

properties:

atsssLL:

type: boolean

default: false

mptcp:

type: boolean

default: false

SmallDataRateStatus:

type: object

required:

- remainPacketsUl

- remainPacketsDl

- validityTime

properties:

remainPacketsUl:

type: integer

minimum: 0

remainPacketsDl:

type: integer

minimum: 0

validityTime:

$ref: '#/components/schemas/DateTime'

remainExReportsUl:

type: integer

minimum: 0

**********skipped for clarity**********

* * * End of Changes * * * *