3GPP TSG-CT Meeting #85	CP-192228

# Newport Beach, US; 16th – 17th September 2019	(revision of C4-193056)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.505 | CR | 0093 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of OperatorSpecificDataContainer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-09-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | C4-184411 modified the definition of OperatorSpecificDataContainer data type, but only in the normative table in section 5.4.2.3. However, the OpenAPI part was not modified accordingly, leaving the API specification inconsistent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align the OpenAPI specification with the description of the data type in section 5.4.2.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incorrect, due to contradictory data type definitions, and cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.3, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the OpenAPI specification file for the Nudr_DataRepository API for Subscription Data. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1 (CT#85): In OpenAPI, replace the response of the GET operation with a map of OperatorSpecificDataContainer, in alignment with Table 5.2.10.3.1-3, by adding the lines: type: object additionalProperties: |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.4.2.3	Type: OperatorSpecificDataContainer

# Table 5.4.2.3-1: Definition of type OperatorSpecificDataContainer

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dataType | string | M | 1 | This attribute value must be a string equal to the name of the actual data type of the "value" attribute. |
| value | any type | M | 1 |  |

NOTE:	In this release of the specification, if dataType attribute takes value "object", it is not possible to indicate the schema of the JSON object included in the "value" attribute, and therefore the usage of the "object" type is not recommended.

* * * Next Change * * * *

## A.2	Nudr_DataRepository API for Subscription Data

(… text not shown for clarity …)

/subscription-data/{ueId}/operator-specific-data:

get:

summary: Retrieves the operator specific data of a UE

operationId: QueryOperSpecData

tags:

- Operator-Specific Data Container (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: object

additionalProperties:

$ref: '#/components/schemas/OperatorSpecificDataContainer'

(… text not shown for clarity …)

OperatorSpecificDataContainer:

type: object

required:

- dataType

- value

properties:

dataType:

type: string

enum:

- string

- integer

- number

- boolean

- object

value:

oneOf:

- type: string

- type: integer

- type: number

- type: boolean

- type: object

(… text not shown for clarity …)

* * * End of Changes * * * *