3GPP TSG-CT Meeting #85	CP-192232

# Newport Beach, US; 16th – 17th September 2019

3GPP TSG-CT WG4 Meeting #93	C4-193775

# Wroclaw, Poland; 26th – 30th August 2019											   was C4-193153

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.003 | CR | 0540 | rev | 3 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections about the Routing Indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | MCC |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When the AMF gets a SUCI from the UE, it uses the SUCI as UE identifier to retrieve authentication data from the UDM. In this case, the AMF shall translate the SUCI from NAS message (BCD encoded) to the SUCI used on SBI interface (UTF-8 string encoded). In current TS23.003, the Routing Indicator is described as following: Routing Indicator, consisting of 1 to 4 decimal digits assigned by the home network operator and provisioned in the USIM...If no Routing Indicator is configured on the USIM, this data field shall be set to the value 0. The above description of Routing Indicator clearly shows that the Routing Indicator is with flexible length from 1 to 4. Meanwhile, if Routing Indicator is configured, each digit shall be regarded as meaningful, i.e. value "012" is not the same as value "12". It does not give clear instruction on how to encode the UTF based SUCI string used on SBI interface if the Routing Indicator is not configured, i.e. whether encode the value 0 (can be regarded as default value) to UTF-8 string "0" (only one decimal digit "0") or "00" / "000" / "0000". CT1 has clarified the default value of the Routing Indicator in case there is no Routing Indicator configuration in the USIM(C1-192713 in C1#116 meeting): If no Routing Indicator is configured in the USIM, the UE shall code bits 1 to 4 of octet 8 of the Routing Indicator as "0000" and the remaining digits as "1111". In this scenario, it means the default value of Routing Indicator provided by the UE should be 0FFF (in hexadecimal), but not 0000 (in hexadecimal). Meanwhile, it is not clear for CT1 on how to encode the Routing Indicator with default value 0 to UTF-8 string: "0", or "00" / "000" / "0000"? To avoid such mis-interpretation, it is proposed to further improve the description of Routing Indicator. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the description of Routing Indicator: - Routing Indicator is with flexible length, and if it is configured, each digital shall be regarded as meaningful; - The default value 0 (in the case the Routing Indicator is not configured in the USIM) shall be encoded as "0" (only one decimal digit). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of Routing Indicator may lead mis-interpretation when AMF transcode the SUCI from UE to the SUCI used on SBI level (i.e. as UE identifier to retrieve the authentication data from UDM), especially in the case if no Routing Indicator configured in the USIM. And such mis-interpretation may cause potential interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2.2B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2: - Update the coversheet, to provide clear description for reason for change; - Clarify that each digit in Routing Indicator shall be regarded as meaningful, and the default value (if no Routing Indicator configured) shall be "0" (only one decimal digit), Rev3: - CR database corrected in the 3GU. This CR was marked as Rel-15 instead of Rel-16. |  |  |  |  |  |  |  |  |

## 2.2B	Subscription Concealed Identifier (SUCI)

The SUCI is a privacy preserving identifier containing the concealed SUPI. It is defined in clause 6.12.2 of 3GPP TS 33.501 [124].

Figure 2.2B-1: Structure of SUCI

The SUCI is composed of the following parts:

1)	SUPI Type, consisting in a value in the range 0 to 7. It identifies the type of the SUPI concealed in the SUCI. The following values are defined:

-	0: IMSI

-	1: Network Specific Identifier

-	2 to 7: spare values for future use.

2)	Home Network Identifier, identifying the home network of the subscriber.

When the SUPI Type is an IMSI, the Home Network Identifier is composed of two parts:

-	Mobile Country Code (MCC), consisting of three decimal digits. The MCC identifies uniquely the country of domicile of the mobile subscription;

-	Mobile Network Code (MNC), consisting of two or three decimal digits. The MNC identifies the home PLMN of the mobile subscription.

When the SUPI type is a Network Specific Identifier, the Home Network Identifier consists of a string of characters with a variable length representing a domain name as specified in clause 2.2 of IETF RFC 7542 [126].

3)	Routing Indicator, consisting of 1 to 4 decimal digits assigned by the home network operator and provisioned in the USIM, that allow together with the Home Network Identifier to route network signalling with SUCI to AUSF and UDM instances capable to serve the subscriber.

Each decimal digit present in the Routing Indicator shall be regarded as meaningful (e.g. value "012" is not the same as value "12"). If no Routing Indicator is configured on the USIM, this data field shall be set to the value 0 (i.e. only consist of one decimal digit of "0").

4)	Protection Scheme Identifier, consisting in a value in the range of 0 to 15 (see Annex C.1 of 3GPP TS 33.501 [124]). It represents the null-scheme or a non-null-scheme specified in Annex C of 3GPP TS 33.501 [124] or a protection scheme specified by the HPLMN;

5)	Home Network Public Key Identifier, consisting in a value in the range 0 to 255. It represents a public key provisioned by the HPLMN and it is used to identify the key used for SUPI protection. In case of null-scheme being used, this data field shall be set to the value 0;

6)	Scheme Output, consisting of a string of characters with a variable length or hexadecimal digits, dependent on the used protection scheme, as defined below. It represents the output of a public key protection scheme specified in Annex C of 3GPP TS 33.501 [124] or the output of a protection scheme specified by the HPLMN.

Figure 2.2B-2 defines the scheme output for the null-scheme.

Figure 2.2B-2: Scheme Output for the null-scheme

The Mobile Subscriber Identification Number (MSIN) as defined in clause 2.2 or the username identifies the mobile subscription within the Home Network. The scheme output is formatted as a variable length of characters as specified for the username in clause 2.2 of IETF RFC 7542 [126].

NOTE:	If the null-scheme is used, the NFs can derive SUPI from SUCI when needed. The AMF derives SUPI used for AUSF discovery from SUCI when the Routing-Indicator is zero and the Protection Scheme is null-scheme.

Figure 2.2B-3 defines the scheme output for the Elliptic Curve Integrated Encryption Scheme Profile A.

Figure 2.2B-3: Scheme Output for Elliptic Curve Integrated Encryption Scheme Profile A

The ECC ephemeral public key is formatted as 64 hexadecimal digits, which allows to encode 256 bits.

The ciphertext value is formatted as a variable length of hexadecimal digits.

The MAC tag value is formatted as 16 hexadecimal digits, which allows to encode 64 bits.

Editor's Note: clause C.3.2 of TS 33.501 specifies that the scheme output may contain other parameters (not further defined in the specification). It is FFS how to format these parameters.

Figure 2.2B-4 defines the scheme output for the Elliptic Curve Integrated Encryption Scheme Profile B.

Figure 2.2B-4: Scheme Output for Elliptic Curve Integrated Encryption Scheme Profile B

The ECC ephemeral public key is formatted as 66 hexadecimal digits, which allows to encode 264 bits.

The ciphertext value is formatted as a variable length of hexadecimal digits.

The MAC tag value is formatted as 16 hexadecimal digits, which allows to encode 64 bits.

Editor's Note: clause C.3.2 of TS 33.501 specifies that the scheme output may contain other parameters (not further defined in the specification). It is FFS how to format these parameters.

Figure 2.2B-5 defines the scheme output for HPLMN proprietary protection schemes.

Figure 2.2B-5: Scheme Output for HPLMN proprietary protection schemes

The HPLMN defined scheme output is formatted as a variable length of hexadecimal digits. Its format is not further defined in 3GPP specifications.

As examples, assuming the IMSI 234150999999999, where MCC=234, MNC=15 and MSISN=0999999999, the Routing Indicator 678, and a Home Network Public Key Identifier of 27:

-	the SUCI for the null-scheme is composed of: 0, 234, 15, 678, 0, 0 and 0999999999

-	the SUCI for the Profile <A> protection scheme is composed of: 0, 234, 15, 678, 1, 27, <EEC ephemeral public key value>, <encryption of 0999999999> and <MAC tag value>