TSG-CT WG Meeting #85	CP-192250

Newport Beach California, USA, 16 - 17 September 2019				(Revision of CP-192091)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.520 | CR | 0049 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhance the Nnwdaf_AnalyticsInfo service to support service experience |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  |  | Date: |  | 2019-08-19 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 23.288, service experience analytics was defined as a feature of NWDAF, and was updated by S2-1908454 in SA2#134 meeting. The functionality shall be defined as stage 3 specification in the Nnwdaf_AnalyticsInfo service. However, it is not supported by Nnwadaf_AnalyticsInfo service (eNA stage 3 specification in TS 29.520). In subclause 4.3.1.1, the description of service experience need to be added; In subclause 4.3.2.2.2, the parameters of service experience need to be added in service operation; In subclause 5.2.6, the data models need to be enhanced to support service experience; In subclause 5.2.8, service experience need to be added as a new feature; In subclause A.3, the open API need to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the Nnwdaf_AnalyticsInfo service to support service experience in the corresponding clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | feature missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1; 4.3.2.2.2; 5.2.6.1; 5.2.6.2.2; 5.2.6.2.3; 5.2.6.3.3; 5.2.8; A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 23.288 CR#0001 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR introduces backward compatible feature into OpenAPI file for Nnwdaf_AnalyticsInfo API. |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev 3: In subclause A.3, the line for dnais has an extra space before, which should be corrected. Rev 4: Deletion of new line is removed in clause 4.3.2.2.2. |
| --- | --- |

# ]Additional discussion(if needed):

…

# Proposed changes:

# *** 1st Change ***

#### 4.3.1.1	Overview

The Nnwdaf_AnalyticsInfo Service as defined in 3GPP TS 23.501 [2], 3GPP TS 23.288 [17] and 3GPP TS 23.503 [4], is provided by the Network Data Analytics Function (NWDAF).

This service:

-	allows NF consumers to request and get different type of analytic event information.

The types of observed events include:

-	Load level of network slice instance;

-	Service experience for an application or for a network slice;

-	NF load analytics information for a specific NF;

-	Network load performance in an area of interest;

-	Expected behaviour information for a group of UEs or a specific UE;

-	Abnormal behaviour information for a group of UEs or a specific UE;

-	Mobility related information for a group of UEs or a specific UE;

-	Communication pattern for a specific UE;

-	Congestion information of user data in a specific location.

# *** Next Change ***

##### 4.3.2.2.2	Request and get from NWDAF Analytics information

Figure 4.3.2.2.2-1 shows a scenario where the NF service consumer (e.g. PCF) sends a request to the NWDAF to request and get from NWDAF analytics information (as shown in 3GPP TS 23.288 [17]).

Figure 4.3.2.2.2-1: Requesting a NWDAF Analytics information

The NF service consumer (e.g. PCF) shall invoke the Nnwdaf_AnalyticsInfo_Request service operation when represent the NWDAFanalyticsinformation of particular network slice instance. The NF service consumer shall send an HTTP GET request on the resource URI "{apiRoot}/nnwdaf-analyticsinfo/v1/analytics" representing the "NWDAF Analytics" (as shown in figure 4.3.2.2.2-1, step 1), to request analytics data according to the query parameter value of the "event-id" attribute. The EventFilter data structure provided in the request body shall include:

-	identification(s) of network slice when the eventId is "LOAD_LEVEL_INFORMATION" via:

1)	identification of network slice(s) by "snssais" attribute; or

2)	any slices indication via the "anySlice" attribute.

-	if the feature "ServiceExperience" is supported and the event is "SERVICE_EXPERIENCE", it shall provide:

- identification of application via identification of application(s) by "appIds" attribute;

- identification of target UE(s) to which the subscription applies by "supi", "intGroupId" or "anyUeInd" attribute in the "tgt-ue" attribute;

and may provide:

- identification of network slice(s) by "snssais" attribute;

- identification of DNN via identification of Dnn(s) by "dnns" attribute;

- identification of user plane accesses to one or more DN(s) where applications are deployed via "dnais" attribute;

- event specific filter information in the "event-filter" attribute:

- identification of network area to which the subscription applies via identification of network area by "networkArea" attribute;

Upon the the reception of the HTTP GET request, the NWDAF shall:

-	analyse the requested analytic data. For eventId of "LOAD_LEVEL_INFORMATION", the NWDAF shall analyse the load level information of corresponding network slice instance(s).

If the HTTP request message from the NF service consumer is accepted, the NWDAF shall respond with "200 OK" status code with the message body containing the analytics with parameters as relevant for the requesting NF service consumer. The AnalyticsData data structure in the response body shall include:

-	analytics with parameters indicated in the request. For eventId of "LOAD_LEVEL_INFORMATION ", the analytics shall include the load level information of corresponding network slice instance.

If the request NWDAF Analytics data does not exist, the NWDAF shall respond with "204 No Content ".

# *** Next Change ***

#### 5.2.6.1	General

This subclause specifies the application data model supported by the API.

Table 5.2.6.1-1 specifies the data types defined for the Nnwdaf_AnalyticsInfo service based interface protocol.

Table 5.2.6.1-1: Nnwdaf_AnalyticsInfo specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EventId | 5.2.6.3.3 | Describes the type of analytics. |  |
| AnalyticsData | 5.2.6.2.2 | Describes analytics with parameters indicated in the request |  |

Table 5.2.6.1-2 specifies data types re-used by the Nnwdaf_AnalyticsInfo service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnwdaf service based interface.

Table 5.2.6.1-2: Nnwdaf_AnalyticsInfo re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnySlice | 5.1.6.3.2 |  |  |
| ProblemDetails | 3GPP TS 29.571 [8] | Used in error responses to provide more detailed information about an error. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.2.8-1. |  |
| Snssai | 3GPP TS 29.571 [8] |  | ServiceExperience SliceLoadLevel |
| SliceLoadLevelInformation | 5.1.6.2.6 |  |  |
| ApplicationId | 3GPP TS 29.571 [8] | Identifies the application. | ServiceExperience |
| Dnn | 3GPP TS 29.571 [8] | Identifies the DNN. | ServiceExperience |
| Dnai | 3GPP TS 29.571 [8] | Identifies a user plane access to one or more DN(s) | ServiceExperience |

# *** Next Change ***

##### 5.2.6.2.2	Type AnalyticsData

Table 5.2.6.2.2-1: Definition of type AnalyticsData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sliceLoadLevelInfos | array(SliceLoadLevelInformation) | M | 1..N | The slices and there load level information. |  |
| svcExpInfo | ServiceExperienceInfo(FFS) | C | 1..N | The service experience information. | ServiceExperience |
| supportedFeatures | SupportedFeatures | M | 1 | List of Supported features used as described in subclause 5.1.8. This parameter shall be supplied by NWDAF in the reply of GET request that request the analytics resource. |  |

# *** Next Change ***

##### 5.2.6.2.3	Type EventFilter

Table 5.2.6.2.3-1: Definition of type EventFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anySlice | AnySlice | C | 0..1 | Default is "FALSE". (NOTE) |  |
| snssais | array(Snssai) | C | 1..N | Identification(s) of network slice to which the subscription belongs. (NOTE) |  |
| appIds | array(ApplicationId) | C | 1..N | Identification(s) of application. It shall be included when eventId is "SERVICE_EXPERIENCE". | ServiceExperience |
| dnns | array(Dnn) | C | 1..N | Identification(s) of DNN. It may be included when eventId is "SERVICE_EXPERIENCE". | ServiceExperience |
| dnais | array(Dnai) | C | 1..N | Identification(s) of user plane accesses to DN(s) which the subscription applies. It may be included when eventId is "SERVICE_EXPERIENCE". | ServiceExperience |
| NOTE: When event-id in the request is "LOAD_LEVEL_INFORMATION ", either information about slice(s) identified by snssais, or anySlice set to "TRUE" shall be included. |  |  |  |  |  |

# *** Next Change ***

##### 5.2.6.3.3	Enumeration: EventId

Table 5.2.6.3.3-1: Enumeration EventId

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LOAD_LEVEL_INFORMATION | Represent the analytics of load level information of corresponding network slice instance. |  |
| SERVICE_EXPERIENCE | Represent the analytics of service experience of corresponding application. | ServiceExperience |

# *** Next Change ***

### 5.2.8	Feature negotiation

The optional features in table 5.2.8-1 are defined for the Nnwdaf_AnalyticsInfo API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [6].

Table 5.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| X | ServiceExperience | This feature indicates support for the event realted to service experience. |

# *** Next Change ***

# A.3	Nnwdaf_AnalyticsInfo API

openapi: 3.0.0

info:

version: 1.0.2

title: Nnwdaf_AnalyticsInfo

description: |

Nnwdaf_AnalyticsInfo Service API.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.520 V15.4.0; 5G System; Network Data Analytics Services.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.520/'

(… text not shown for clarity …)

AnalyticsData:

type: object

properties:

sliceLoadLevelInfos:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/SliceLoadLevelInformation'

minItems: 1

description: The slices and their load level information.

svcExpInfo:

type: string

EventFilter:

type: object

properties:

anySlice:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/AnySlice'

snssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

description: Identification(s) of network slice.

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

dnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

dnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

not:

required: [anySlice, snssais]

EventId:

anyOf:

- type: string

enum:

- LOAD_LEVEL_INFORMATION

- SERVICE_EXPERIENCE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- LOAD_LEVEL_INFORMATION: Represent the analytics of load level information of corresponding network slice instance.

- SERVICE_EXPERIENCE: Represent the analytics of service experience information of the specific applications.

*** End of Changes ***