3GPP TSG-CT1 Meeting #121	C1-198855

# Reno, Nevada, United States, 11th Nov 2019 - 15th Nov 2019   Revision of C1-198652

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0097 | rev | 3 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Location information to SDS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HOME OFFICE |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData2 |  |  |  |  |  | Date: |  |  | 2019-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Location information is important for Emergency Services in order to manage the resources in the field. To reduce the amount of voice traffic the Emergency Services are using more data services to manage users. The addition of the Location information allows the Control Room to manage the resources on the ground more efficiently. The increasing use of data, SDS, will also require Location Information. The Location Information is also used for auditing purposes and may be required by a court of law in the future. SA6 have introduced the following SA1 requirements into Stage 2: From 22.282 requirements [R-5.2.2-012] The MCData SDS shall provide the option to add a field indicating location of the sending user/UE. [R-5.2.2-013] The MCData SDS shall allow empty messages including only a field indicating location of the sending user/UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of User location as an optional parameter as part of the SDS to align with updates made to Stage 2 (CR 0174r2 to TS 23.282). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reduces the capabilities of a Control Room to manage it’s resources efficiently, also the Location information is important for audit trails. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.2.1, 15.1.2.1, 15.1.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ========== First Change =============

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2";

[3]	3GPP TS 23.280:" Common functional architecture to support mission critical services; Stage 2";

[4]	IETF RFC 3261 (June 2002): "SIP: Session Initiation Protocol".

[5]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[6]	IETF RFC 3428 (December 2002): "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[7]	IETF RFC 6050 (November 2010): "A Session Initiation Protocol (SIP) Extension for the Identification of Services".

[8]	IETF RFC 3841 (August 2004): "Caller Preferences for the Session Initiation Protocol (SIP)".

[9]	IETF RFC 4826 (May 2007): "Extensible Markup Language (XML) Formats for Representing Resource Lists".

[10]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control Protocol specification".

[11]	3GPP TS 24.481: "Mission Critical Services (MCS) group management Protocol specification".

[12]	3GPP TS 24.484: "Mission Critical Services (MCS) configuration management Protocol specification".

[13]	IETF RFC 4483 (May 2006): "A Mechanism for Content Indirection in Session Initiation Protocol (SIP) Messages.

[14]	IETF RFC 4122 (July 2005): "A Universally Unique IDentifier (UUID) URN Namespace".

[15]	3GPP TS 24.582: "Mission Critical Data (MCData) media plane control Protocol specification";

[16]	IETF RFC 3840 (August 2004): "Indicating User Agent Capabilities in the Session Initiation Protocol (SIP)".

[17]	IETF RFC 4975 (September 2007): "The Message Session Relay Protocol (MSRP)".

[18]	IETF RFC 5366 (October 2008): "Conference Establishment Using Request-Contained Lists in the Session Initiation Protocol (SIP)".

[19]	IETF RFC 6135 (February 2011): "An Alternative Connection Model for the Message Session Relay Protocol (MSRP) ".

[20]	IETF RFC 6714 (August 2012): "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)".

[21]	IETF RFC 6086 (January 2011): "Session Initiation Protocol (SIP) INFO Method and Package Framework".

[22]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[23]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[24]	3GPP TS 24.482: "Mission Critical Services (MCS) identity management Protocol specification.

[25]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to Proximity-services (ProSe) Function Protocol aspects; Stage 3".

[26]	3GPP TS 33.180: "Security of the Mission Critical Service".

[27]	IETF RFC 3856 (August 2004): "A Presence Event Package for the Session Initiation Protocol (SIP)".

[28]	W3C: "XML Encryption Syntax and Processing Version 1.1", https://www.w3.org/TR/xmlenc-core1/.

[29]	W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core/.

[30]	IETF RFC 4648 (October 2006): "The Base16, Base32, and Base64 Data Encodings".

[31]	3GPP TS 23.003: "Numbering, addressing and identification".

[32]	IETF RFC 2045 (November 1996): "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[33]	IETF RFC 2392 (August 1998): "Content-ID and Message-ID Uniform Resource Locators".

[34]	IETF RFC 3903 (October 2004): "Session Initiation Protocol (SIP) Extension for Event State Publication".

[35]	IETF RFC 4354 (January 2006): "A Session Initiation Protocol (SIP) Event Package and Data Format for Various Settings in Support for the Push-to-Talk over Cellular (PoC) Service".

[36]	IETF RFC 6665 (July 2012): "SIP-Specific Event Notification".

[37]	3GPP TS 29.283: "Diameter Data Management Applications".

[38]	IETF RFC 4028 (April 2005): "Session Timers in the Session Initiation Protocol (SIP)".

[39]	IETF RFC 3856 (August 2004): "A Presence Event Package for the Session Initiation Protocol (SIP)".

[40]	IETF RFC 3863 (August 2004): "Presence Information Data Format (PIDF)".

[41]	IETF RFC 4661 (September 2006): "An Extensible Markup Language (XML)-Based Format for Event Notification Filtering".

[42]	3GPP TS 24.483: "Mission Critical Services (MCS) Management Object (MO)".

[43]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[44]	IETF RFC 5627 (October 2009): "Obtaining and Using Globally Routable User Agent URIs (GRUUs) in the Session Initiation Protocol (SIP)".

[45]	IETF RFC 4567 (July 2006): "Key Management Extensions for Session Description Protocol (SDP) and Real Time Streaming Protocol (RTSP)".

[46]	IETF RFC 3986 (January 2005): "Uniform Resource Identifier (URI): Generic Syntax".

[47]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[XX]	3GPP TS 29.199-09: "Open Service Access (OSA); Parlay X web services; Part 9: Terminal location".

### ========== Second Change =============

#### 6.2.2.1	Generating an SDS Message

In order to generate an SDS message, the MCData client:

1)	shall generate an SDS SIGNALLING PAYLOAD message as specified in subclause 15.1.2;

2)	shall generate a DATA PAYLOAD message as specified in subclause 15.1.4;

3)	shall include in the SIP request, the SDS SIGNALLING PAYLOAD message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1; and

4)	shall include in the SIP request, the DATA PAYLOAD message in an application/vnd.3gpp.mcdata-payload MIME body as specified in subclause E.2.

When generating an SDS SIGNALLING PAYLOAD message as specified in subclause 15.1.2, the MCData client:

1)	shall set the Date and time IE to the current time as specified in subclause 15.2.8;

2)	if the SDS message starts a new conversation, shall set the Conversation ID IE to a newly generated Conversation ID value as specified in subclause 15.2.9;

3)	if the SDS message continues an existing unfinished conversation, shall set the Conversation ID IE to the Conversation ID value of the existing conversation as specified in subclause 15.2.9;

4)	shall set the Message ID IE to a newly generated Message ID value as specified in subclause 15.2.10;

5)	if the SDS message is in reply to a previously received SDS message, shall include the InReplyTo message ID IE with the Message ID value in the previously received SDS message;

6)	if the SDS message is for user consumption, shall not include an Application ID IE as specified in subclause 15.2.7and shall not include an Extended application ID IE as specified in subclause 15.2.24;

7)	if the SDS message is intended for an application on the terminating MCData client, shall include:

a)	an Application ID IE with a Application ID value representing the intended application as specified in subclause 15.2.7; or

b)	an Extended application ID IE with an Extended application ID value representing the intended application as specified in subclause 15.2.24;

NOTE:	The value chosen for the Application ID value is decided by the mission critical organisation.

8)	if only a delivery disposition notification is required shall include a SDS disposition request type IE set to "DELIVERY" as specified in subclause 15.2.3;

9)	if only a read disposition notification is required shall include a SDS disposition request type IE set to "READ" as specified in subclause 15.2.3;

10)	if both a delivery and read disposition notification is required shall include a SDS disposition request type IE set to "DELIVERY AND READ" as specified in subclause 15.2.3; and

xx)	may set the User location IE to the current location of the UE as specified in clause 15.2.x.

When generating an DATA PAYLOAD message for SDS as specified in subclause 15.1.4, the MCData client:

1)	shall set the Number of payloads IE to the number of Payload IEs that needs to be encoded, as specified in subclause 15.2.12;

2)	if end-to-end security is required for a one-to-one communication, shall include the Security parameters and Payload IE with security parameters as described in 3GPP TS 33.180 [26]. Otherwise, if end-to-end security is not required for a one-to-one communication, shall include the Payload IE as specified in subclause 15.1.4; and

3)	for each Payload IE included:

a)	if the payload is text, shall set the Payload content type as "TEXT" as specified in subclause 15.2.13;

b)	if the payload is binary data, shall set the Payload content type as "BINARY" as specified in subclause 15.2.13;

c)	if the payload is hyperlinks, shall set the Payload content type as "HYPERLINKS" as specified in subclause 15.2.13;

d)	if the payload is location, shall set the Payload content type as "LOCATION" as specified in subclause 15.2.13;

e)	if payload is enhanced status for a group, shall set the Payload content type as “ENHANCED STATUS” as specified in subclase 15.2.13; and

f)	shall include the data to be sent in the Payload data.

### ========== Third Change =============

### 15.1.2	SDS SIGNALLING PAYLOAD message

#### 15.1.2.1	Message definition

This message is sent by the UE to other UEs when sending an SDS data payload. This message provides the signalling content related to the SDS data payload. For the contents of the message see Table 15.1.2.1-1.

Message type:	SDS SIGNALLING PAYLOAD

Direction:			UE to other UEs (can be via network)

# Table 15.1.2.1-1: SDS SIGNALLING PAYLOAD message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | SDS signalling payload message identity | Message type 15.2.2 | M | V | 1 |
|  | Date and time | Date and time 15.2.8 | M | V | 5 |
|  | Conversation ID | Conversation ID 15.2.9 | M | V | 16 |
|  | Message ID | Message ID 15.2.10 | M | V | 16 |
| 21 | InReplyTo message ID | InReplyTo message ID 15.2.11 | O | TV | 17 |
| 22 | Application ID | Application ID 15.2.7 | O | TV | 2 |
| 8- | SDS disposition request type | SDS disposition request type 15.2.3 | O | TV | 1 |
| 7D | Extended application ID | Extended application ID 15.2.24 | O | TLV-E | 3-x |
| 7E | User location | User location 15.2.x | O | TLV-E | 4-x |

### ========== Fourth Change =============

### 15.1.7	SDS OFF-NETWORK MESSAGE message

#### 15.1.7.1	Message definition

This message is sent by the UE to other Ues to share application or user payload in a SDS message. For contents of the message see Table 15.1.7.1-1.

Message type:	SDS OFF-NETWORK MESSAGE

Direction:			UE to other Ues

Table 15.1.7.1-1: SDS OFF-NETWORK MESSAGE message content

| IEI | Information Element |  | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- | --- |
|  | SDS off-network message message identity |  | Message Type 15.2.2 | M | V | 1 |
|  | Date and time |  | Date and time 15.2.8 | M | V | 5 |
|  | Number of payloads |  | Number of payloads 15.2.12 | M | V | 1 |
|  | Conversation ID |  | Conversation ID 15.2.9 | M | V | 16 |
|  | Message ID |  | Message ID 15.2.10 | M | V | 16 |
|  | Sender MCData user ID |  | MCData user ID 15.2.15 | M | LV-E | 3-x |
| 21 | InReplyTo message ID |  | InReplyTo message ID 15.2.11 | O | TV | 17 |
| 22 | Application ID |  | Application ID 15.2.7 | O | TV | 2 |
| 8- | SDS disposition request type |  | SDS disposition request type 15.2.3 | O | TV | 1 |
| 23 |  | Security parameters | MCData Protected Payload message 3GPP TS 33.180 [26] | O | TV | 32 |
| 7B | MCData group ID |  | MCData group ID 15.2.14 | O | TLV-E | 4-x |
| 7C | Recipient MCData user ID |  | MCData user ID 15.2.15 | O | TLV-E | 4-x |
| 78 | Payload |  | Payload 15.2.13 | O | TLV-E | 4-x |
| 7D | Extended application ID |  | Extended application ID 15.2.24 | O | TLV-E | 3-x |
| 7E | User location |  | User location 15.2.x | O | TLV-E | 4-x |

### ========== End of Changes =============