3GPP TSG-CT WG1 Meeting #120	C1-196632

# Portoroz (Slovenia), 7-11 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0077 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of LMR Message as a value for MCData Payload content type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Hytera Communications Corp, Sepura |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCCI_CT |  |  |  |  |  | Date: |  |  | 2019-10-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 3GPP TR 24.883 describes transporting a native LMR message type for interworking purposes. Even if a MCData system does not make active use of this payload content type, the value should not be used for other purposes to ensure interoperability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ‘The value used in 3GPP TS 29.582’is added as an assigned Payload content type with a reference to that specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability risk in connected systems where LMR interworking is permitted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 15.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | C1-196113 |  |  |  |  |  |  |  |  |

***** First change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2";

[3]	3GPP TS 23.280:" Common functional architecture to support mission critical services; Stage 2";

[4]	IETF RFC 3261 (June 2002): "SIP: Session Initiation Protocol".

[5]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[6]	IETF RFC 3428 (December 2002): "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[7]	IETF RFC 6050 (November 2010): "A Session Initiation Protocol (SIP) Extension for the Identification of Services".

[8]	IETF RFC 3841 (August 2004): "Caller Preferences for the Session Initiation Protocol (SIP)".

[9]	IETF RFC 4826 (May 2007): "Extensible Markup Language (XML) Formats for Representing Resource Lists".

[10]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control Protocol specification".

[11]	3GPP TS 24.481: "Mission Critical Services (MCS) group management Protocol specification".

[12]	3GPP TS 24.484: "Mission Critical Services (MCS) configuration management Protocol specification".

[13]	IETF RFC 4483 (May 2006): "A Mechanism for Content Indirection in Session Initiation Protocol (SIP) Messages.

[14]	IETF RFC 4122 (July 2005): "A Universally Unique IDentifier (UUID) URN Namespace".[15]	3GPP TS 24.582: "Mission Critical Data (MCData) media plane control Protocol specification";

[16]	IETF RFC 3840 (August 2004): "Indicating User Agent Capabilities in the Session Initiation Protocol (SIP)".

[17]	IETF RFC 4975 (September 2007): "The Message Session Relay Protocol (MSRP)".

[18]	IETF RFC 5366 (October 2008): "Conference Establishment Using Request-Contained Lists in the Session Initiation Protocol (SIP)".

[19]	IETF RFC 6135 (February 2011): "An Alternative Connection Model for the Message Session Relay Protocol (MSRP) ".

[20]	IETF RFC 6714 (August 2012): "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)".

[x]	3GPP TS 29.582: "Mission Critical Data (MCData) signalling control interworking with LMR systems; Protocol specification".

***** Next change *****

### 15.2.13	Payload

The Payload information element contains the payload intended for the recipient user or application;

The Payload information element is coded as shown in Figure 15.2.13-1, Table 15.2.13-1, Table 15.2.13-2 and Table 15.2.13-3.

The Payload information element is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Payload IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Payload contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet 4 |
| Payload contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 15.2.13-1: Payload information element

Table 15.2.13-1: Payload contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Payload content type |  |  |  |  |  |  |  | octet 4 |
|  |  |  |  |  |  |  |  | octet 5 |
| Payload data |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Table 15.2.13-2: Payload content type

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | TEXT |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | BINARY |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | HYPERLINKS |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | FILEURL |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |  | LOCATION |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | ENHANCED STATUS |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | Value allocated for use in interworking (NOTE) |
| All other values are reserved.: |  |  |  |  |  |  |  |  |  |
| NOTE: Usage of this value is described in 3GPP TS 29.582 [x]. |  |  |  |  |  |  |  |  |  |

Table 15.2.13-3: Payload data

Payload data is included in octet 5 to octet n; Max value of 65535 octets. Payload data contains the payload destined for the user or application. A file URL is encoded as specified in IETF RFC 1738 [rfc1738]. The length of location information payload content is 6 bytes. First 3 bytes contain the latitude information and next 3 bytes contain the longitude information.

***** End changes *****