3GPP TSG-CT WG1 Meeting #120	C1-196690

# Portoroz (Slovenia), 7-11 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0091 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct target of error response |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet, L3Harris Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16 |  |  |  |  |  | Date: |  |  | 2019-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SIP error responses are indicated to be sent to the MCData server, when they should be sent to the MCData client. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the target of SIP error responses in clauses 7.3.3 and 7.3.4 from MCData server to MCData client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MCData client will not receive a response to a SIP PUBLISH message it has sent when there is an error, possibly leading to faulty operation of the MCData client. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3, 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

= = = = = = = = FIRST CHANGE = = = = = = = =

### 7.3.3	SIP PUBLISH request for service authorisation and service settings

The MCData server shall support obtaining service authorization specific information from a SIP PUBLISH request for MCData server settings.

Upon receiving a SIP PUBLISH request containing:

1)	an Event header field set to the "poc-settings" value;

2)	an application/poc-settings+xml MIME body; and

3)	an application/vnd.3gpp.mcdata-info+xml MIME body containing an <mcdata-access-token> element and an <mcdata-client-id> element;

the MCData server:

1)	shall identify the IMS public user identity from the P-Asserted-Identity header field;

2)	shall perform the procedures in subclause 7.3.1A;

3)	if the procedures in subclause 7.3.1A were not successful shall send a SIP 403 (Forbidden) response towards the MCData client with the warning text set to: "140 unable to decrypt XML content " in a Warning header field as specified in subclause 4.9, and not continue with the rest of the steps in this subclause;

4)	shall perform service authorization for the identified MCData ID as described in 3GPP TS 33.180 [26];

5)	if service authorization was successful shall bind the MCData ID to the IMS public user identity;

NOTE 1:	The MCData server will store the binding MCData ID, IMS public user identity and an identifier addressing the MCData server in an external database.

6)	if service authorization was not successful, shall send a SIP 403 (Forbidden) response towards the MCData client with the warning text set to: "101 service authorisation failed" in a Warning header field as specified in subclause 4.9, and not continue with the rest of the steps in this subclause;

7)	shall process the SIP PUBLISH request according to rules and procedures of IETF RFC 3903 [34] and if processing of the SIP request was not successful, do not continue with the rest of the steps;

8)	shall cache the received MCData service settings until the MCData service settings expiration timer expires;

9)	shall send a SIP 200 (OK) response according 3GPP TS 24.229 [5];

10)	shall download the MCData user profile from the MCData user database as defined in 3GPP TS 29.283 [37] if not already stored at the MCData server and use the <selected-user-profile-index> element of the poc-settings event package if included to identify the active MCData user profile for the MCData client;

NOTE 2:	If the <selected-user-profile-index> element of the poc-settings event package is included then only that MCData user profile is needed to be downloaded from the MCData user database.

11)	if there is no <selected-user-profile-index> element included in the poc-settings event package then if multiple MCData user profiles are stored at the MCData server or downloaded for the MCData user from the MCData user database, shall determine the pre-selected MCData user profile to be used as the active MCData user profile by identifying the MCData user profile (see the MCData user profile document in 3GPP TS 24.484 [12]) in the collection of MCData user profiles that contains a <Pre-selected-indication> element; and

NOTE 3:	If only one MCData user profile is stored at the MCData server or only one MCData user profile is downloaded from the MCData user database, then by default this MCData user profile is the pre-selected MCData user profile.

12)	if an <ImplicitAffiliations> element is contained in the <OnNetwork> element of the MCData user profile document with one or more <entry> elements containing an MCData group ID (see the MCData user profile document in 3GPP TS 24.484 [12]) for the served MCData ID, shall perform implicit affiliation as specified in subclause 8.2.2.2.15

= = = = = = = = NEXT CHANGE = = = = = = = =

### 7.3.4	Receiving SIP PUBLISH request for MCData service settings only

Upon receiving a SIP PUBLISH request containing:

1)	an Event header field set to the "poc-settings" value;

2)	an application/poc-settings+xml MIME body; and

3)	an application/vnd.3gpp.mcdata-info+xml MIME body containing an <mcdata-request-uri> element and an <mcdata-client-id> element;

The MCData server:

1)	shall identify the IMS public user identity from the P-Asserted-Identity header field;

2)	shall perform the procedures in subclause 7.3.1A;

3)	if the procedures in subclause 7.3.1A were not successful, shall send a SIP 403 (Forbidden) response towards the MCData client with the warning text set to: "140 unable to decrypt XML content" in a Warning header field as specified in subclause 4.9, and not continue with the rest of the steps in this subclause;

4)	shall verify that a binding between the IMS public user identity in the Request-URI and the MCData ID in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml exists at the MCData server;

5)	if a binding exists between the IMS public user identity and the MCData ID in the request and the validity period of the binding has not expired shall download the MCData user profile from the MCData user database as defined in 3GPP TS 29.283 [37] if not already stored at the MCData server;

6)	if a binding does not exist between the IMS public user identity and the MCData ID in the request or the binding exists, but the validity period of the binding has expired, shall reject the SIP PUBLISH request with a SIP 404 (Not Found) response and not continue with any of the remaining steps;

7)	shall process the SIP PUBLISH request according to rules and procedures of IETF RFC 3903 [34] and if processing of the SIP request was not successful, do not continue with the rest of the steps;

8)	shall cache the received MCData service settings until the MCData service settings expiration timer expires;

9)	shall send a SIP 200 (OK) response according 3GPP TS 24.229 [5];

10)	shall download the MCData user profile from the MCData user database as defined in 3GPP TS 29.283 [37] if not already stored at the MCData server and use the <selected-user-profile-index> element of the poc-settings event package if included to identify the active MCData user profile for the MCData client;

NOTE 1:	If the <selected-user-profile-index> element of the poc-settings event package is included then only that MCData user profile is needed to be downloaded from the MCData user database.

11)	if there is no <selected-user-profile-index> element included in the poc-settings event package then if multiple MCData user profiles are stored at the MCData server or downloaded for the MCData user from the MCData user database, shall determine the pre-selected MCData user profile to be used as the active MCData user profile by identifying the MCData user profile (see the MCData user profile document in 3GPP TS 24.484 [12]) in the collection of MCData user profiles that contains a <Pre-selected-indication> element; and

NOTE 2:	If only one MCData user profile is stored at the MCData server or only one MCData user profile is downloaded from the MCData user database, then by default this MCData user profile is the pre-selected MCData user profile.

12)	if an <ImplicitAffiliations> element is contained in the <OnNetwork> element of the MCData user profile document with one or more <entry> elements containing an MCData group ID (see the MCData user profile document in 3GPP TS 24.484 [12]) for the served MCData ID, shall perform implicit affiliation as specified in subclause 8.2.2.2.15.

= = = = = = = = END CHANGES = = = = = = = =