3GPP TSG-CT WG Meeting #86	CP-193129

# Itges, SPAIN; 09th – 10th December 2019   Revision of C4-195178, C4-195485, C4-195584, C4-195608

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0232 | rev | 4 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMF derived CN assisted RAN parameters tuning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In clause 4.3.2 PDU session establishment in TS 23.502: The SMF may provide the SMF derived CN assisted RAN parameters tuning to the AMF by invoking Nsmf_PDUSession_SMContextStatusNotify (SMF derived CN assisted RAN parameters tuning) service. The AMF stores the SMF derived CN assisted RAN parameters tuning in the associated PDU Session context for this UE. If the SMF derived CN assisted RAN parameters tuning are stored for the activated PDU Session(s), the AMF may derive updated CN assisted RAN parameters tuning and provide them the (R)AN. In clause 4.3.3.2 PDU session modification in TS 23.502: 3c. For SMF requested modification due to updated SMF-Associated parameters from the UDM, the SMF may provide the SMF derived CN assisted RAN parameters tuning to the AMF. The SMF invokes Nsmf_PDUSession_SMContextStatusNotify (SMF derived CN assisted RAN parameters tuning) towards the AMF. The AMF stores the SMF derived CN assisted RAN parameters tuning in the associated PDU Session context for this UE. In clause 5.4.6.2 of TS 23.501, the definition of the Core Network assisted RAN parameters tuning: The SMF uses the SMF-Associated parameters (e.g. Expected UE Behaviour parameters or Network Configuration parameters of the UE) to derive the SMF derived CN assisted RAN parameters tuning. In clause 4.15.6.3 of TS 23.502 for the Expected UE Behaviour parameters: The Expected UE Moving Trajectory may be used by the AMF. All other parameters may be used by the AMF and by the SMF. It is proposed to send all the parameters in the Expected UE Behaviour parameters except the Expected UE Moving Trajectory to the AMF. In clause 4.15.6.3a of TS 23.502 for the Network Configuration parameters: The Suggested Number of Downlink Packets is classified as SMF-Associated parameter. It is proposed to send the Suggested Number of Downlink Packets to the AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support the SMF derived CN assisted RAN parameters tuning in Nsmf_PDUSession_SMContextStatusNotify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.5.1, 6.1.6, 6.1.6.2.29, 6.1.6.2.x(new), 6.1.6.3.8, 6.1.6.3.9, 6.1.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features to the OpenAPI file for Nsmf_PDUSession API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - New feature added to Table 6.1.8-1 - New Cause value was added to Table 6.1.6.3.8-1 - New enumeration value name changed from "NORMAL" to "UNCHANGED" - In 5.2.2.5.1, "e.g." was removed before "to provide the SMF derived CN assisted RAN parameters tuning" - In 6.1.6.2, suggestedNumberOfDlPackets attribute was removed from Table 6.1.6.2.x-1 - Table 6.1.6.2.29-1, new statement in the description was removed, plus editorials Rev2: - In 5.2.2.5.1 the very first change was adjusted and the existing statement under step 1 was amended (when the notification is triggered by PDU session handover) - In Table 6.1.6.2.29-1, cnAssistedRanPara became C and the condition changed as follows. Before the attribute precence was dependent on the resourceStatus value. Now it depend on the cause value - In Table 6.1.8-1 new statement was added to the description of the CARPT flag Rev3: Editorial fix in Table 6.1.6.2.29-1 and suggestedNumberOfDlPackets was removed also from A.2 Rev4 (for plenary): - Applicabilty column added to Table 6.1.6.2.29-1. New cnAssistedRanPara is applicable for the CARPT - The new enumeration Cause value added to the OpenAPI |  |  |  |  |  |  |  |  |

* * * * Begin of Change * * * *

#### 5.2.2.5	Notify SM Context Status service operation

##### 5.2.2.5.1	General

The Notify SM Context Status service operation shall be used by the SMF to notify the NF Service Consumer about the status of an SM context related to a PDU session (e.g. when the SM context is released and the release is not triggered by a Release SM Context Request, or when the SM context is moved to another system) in the SMF, or the V-SMF for HR roaming scenarios.

The Notify SM Context Status service operation may also be used by the SMF to provide the SMF derived CN assisted RAN parameters tuning to the NF Service Consumer (e.g. AMF), if the NF Service Consumer has indicated support of the CARPT (CN Assisted RAN Parameters Tuning) feature.

It is used in the following procedures:

-	UE requested PDU Session Establishment procedure, when the PDU session establishment fails after the Create SM Context response or to provide the SMF derived CN assisted RAN parameters tuning (see clause 4.3.2.2 of 3GPP TS 23.502 [3]);

-	UE or Network requested PDU session Modification (see clause 4.3.3.2 of 3GPP TS 23.502 [3]) to provide the SMF derived CN assisted RAN parameters tuning;

-	UE or Network requested PDU session release (see clause 4.3.4.2 of 3GPP TS 23.502 [3]), e.g. SMF initiated release;

-	Handover of a PDU Session procedure between untrusted non-3GPP to 3GPP access (see clauses 4.9.2.3.2 and 4.9.2.4.2 of 3GPP TS 23.502 [3]);

-	Interworking procedures without N26 interface, e.g. 5GS to EPS Mobility (see clause 4.11.2.2 of 3GPP TS 23.502 [3]);

-	Handover from 5GC-N3IWF to EPS (see clause 4.11.3.2 of 3GPP TS 23.502 [3]);

-	Handover from 5GS to EPC/ePDG (see clause 4.11.4.2 of 3GPP TS 23.502 [3]).

The SMF shall notify the NF Service Consumer by using the HTTP POST method as shown in Figure 5.2.2.5.1-1.

Figure 5.2.2.5.1-1: SM context status notification

1.	The SMF shall send a POST request to the SM Context Status callback reference provided by the NF Service Consumer during the subscription to this notification. The payload body of the POST request shall contain the notification payload.

If the notification is triggered by PDU session handover, the notification payload shall contain the resourceStatus IE with value "RELEASED" and the Cause IE with the value "PDU_SESSION_HANDED_OVER".

If the notification is triggered by the SMF derived CN assisted RAN parameters tuning, notification payload shall contain the resourceStatus IE with value "UNCHANGED" and the Cause IE with the value "CN_ASSISTED_RAN_PARAMETER_TUNING".

2a.	On success, "204 No Content" shall be returned and the payload body of the POST response shall be empty.

If the SMF indicated in the request that the SM context resource is released, the NF Service Consumer shall release its association with the SMF for the PDU session and release the EBI(s) that were assigned to the PDU session.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.7.3.1-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.7.3.1-2.

If the NF Service Consumer (e.g. AMF) is not able to handle the notification but knows by implementation specific means that another NF Service Consumer (e.g. AMF) is able to handle the notification (e.g. AMF deployment with Backup AMF), it shall reply with an HTTP "307 temporary redirect" error response pointing to the URI of the new NF Service Consumer. If the NF Service Consumer is not able to handle the notification but another unknown NF Service Consumer could possibly handle the notification (e.g. AMF deployment with UDSF), it shall reply with an HTTP "404 Not found" error response.

If the SMF receives a "307 temporary redirect" response, the SMF shall use this URI as Notification URI in subsequent communication and shall resend the notification to that URI.

If the SMF becomes aware that a new NF Service Consumer (e.g. AMF) is requiring notifications (e.g. via the "404 Not found" response or via Namf_Communication service AMFStatusChange Notifications, or via link level failures, see clause 6.5.2 of 3GPP TS 29.500 [4]), and the SMF knows alternate or backup Address(es) where to send Notifications (e.g. via the GUAMI and/or backupAmfInfo received when the SM context was established or via AMFStatusChange Notifications, or via the Nnrf_NFDiscovery service specified in 3GPP TS 29.510 [19] using the service name and GUAMI or backupAMFInfo obtained during the creation of the SM context, see clause 6.5.2.2 of 3GPP TS 29.500 [4]), the SMF shall exchange the authority part of the corresponding Notification URI with one of those addresses and shall use that URI in subsequent communication; the SMF shall resend the notification to that URI.

* * * * Next Change * * * *

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsmf service based interface protocol.

Table 6.1.6.1-1: Nsmf specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| SmContextCreateData | 6.1.6.2.2 | Information within Create SM Context Request |  |
| SmContextCreatedData | 6.1.6.2.3 | Information within Create SM Context Response |  |
| SmContextUpdateData | 6.1.6.2.4 | Information within Update SM Context Request |  |
| SmContextUpdatedData | 6.1.6.2.5 | Information within Update SM Context Response |  |
| SmContextReleaseData | 6.1.6.2.6 | Information within Release SM Context Request |  |
| SMContextRetrieveData | 6.1.6.2.7 | Information within Retrieve SM Context Request |  |
| SmContextStatusNotification | 6.1.6.2.8 | Information within Notify SM Context Status Request |  |
| PduSessionCreateData | 6.1.6.2.9 | Information within Create Request |  |
| PduSessionCreatedData | 6.1.6.2.10 | Information within Create Response |  |
| HsmfUpdateData | 6.1.6.2.11 | Information within Update Request towards H-SMF, or from I-SMF to SMF |  |
| HsmfUpdatedData | 6.1.6.2.12 | Information within Update Response from H-SMF, or from SMF to I-SMF |  |
| ReleaseData | 6.1.6.2.13 | Information within Release Request |  |
| HsmfUpdateError | 6.1.6.2.14 | Error within Update Response from H-SMF |  |
| VsmfUpdateData | 6.1.6.2.15 | Information within Update Request towards V-SMF, or from SMF to I-SMF |  |
| VsmfUpdatedData | 6.1.6.2.16 | Information within Update Response from V-SMF, or from I-SMF to SMF |  |
| StatusNotification | 6.1.6.2.17 | Information within Notify Status Request |  |
| QosFlowItem | 6.1.6.2.18 | Individual QoS flow |  |
| QosFlowSetupItem | 6.1.6.2.19 | Individual QoS flow to setup |  |
| QosFlowAddModifyRequestItem | 6.1.6.2.20 | Individual QoS flow requested to be created or modified |  |
| QosFlowReleaseRequestItem | 6.1.6.2.21 | Individual QoS flow requested to be released |  |
| QosFlowProfile | 6.1.6.2.22 | QoS flow profile |  |
| GbrQosFlowInformation | 6.1.6.2.23 | GBR QoS flow information |  |
| QosFlowNotifyItem | 6.1.6.2.24 | Notification related to a QoS flow |  |
| SMContextRetrievedData | 6.1.6.2.27 | Information within Retrieve SM Context Response |  |
| TunnelInfo | 6.1.6.2.28 | Tunnel Information |  |
| StatusInfo | 6.1.6.2.29 | Status of SM context or of PDU session |  |
| VsmfUpdateError | 6.1.6.2.30 | Error within Update Response from V-SMF |  |
| EpsPdnCnxInfo | 6.1.6.2.31 | EPS PDN Connection Information from H-SMF to V-SMF |  |
| EpsBearerInfo | 6.1.6.2.32 | EPS Bearer Information from H-SMF to V-SMF |  |
| PduSessionNotifyItem | 6.1.6.2.33 | Notification related to a PDU session |  |
| EbiArpMapping | 6.1.6.2.34 | EBI to ARP mapping |  |
| SmContextCreateError | 6.1.6.2.35 | Error within Create SM Context Response |  |
| SmContextUpdateError | 6.1.6.2.36 | Error within Update SM Context Response |  |
| PduSessionCreateError | 6.1.6.2.37 | Error within Create Response |  |
| MmeCapabilities | 6.1.6.2.38 | MME capabilities |  |
| SmContext | 6.1.6.2.39 | Complete SM Context | DTSSA |
| ExemptionInd | 6.1.6.2.40 | Exemption Indication |  |
| PsaInformation | 6.1.6.2.41 | PSA Information | DTSSA |
| DnaiInformation | 6.1.6.2.42 | DNAI Information | DTSSA |
| N4Information | 6.1.6.2.43 | N4 Information | DTSSA |
| CnAssistedRanPara | 6.1.6.2.x | SMF derived CN assisted RAN parameters tuning | CARPT |
| GtpTeid | 6.1.6.3.2 | GTP Tunnel Endpoint Identifier |  |
| ProcedureTransactionId | 6.1.6.3.2 | Procedure Transaction Identifier |  |
| EpsPdnCnxContainer | 6.1.6.3.2 | UE EPS PDN Connection container from SMF to AMF |  |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Id |  |
| EpsBearerContainer | 6.1.6.3.2 | EPS Bearer container from SMF to AMF |  |
| UpCnxState | 6.1.6.3.3 | User Plane Connection State |  |
| HoState | 6.1.6.3.4 | Handover State |  |
| RequestType | 6.1.6.3.5 | Request Type in Create (SM context) service operation. |  |
| RequestIndication | 6.1.6.3.6 | Request Indication in Update (SM context) service operation. |  |
| NotificationCause | 6.1.6.3.7 | Cause for generating a notification |  |
| Cause | 6.1.6.3.8 | Cause information |  |
| ResourceStatus | 6.1.6.3.9 | Status of SM context or PDU session resource |  |
| DnnSelectionMode | 6.1.6.3.10 | DNN Selection Mode |  |
| EpsInterworkingIndication | 6.1.6.3.11 | EPS Interworking Indication |  |
| N2SmInfoType | 6.1.6.3.12 | N2 SM Information Type |  |
| MaxIntegrityProtectedDataRate | 6.1.6.3.13 | Maximum Integrity Protected Data Rate |  |
| MaReleaseIndication | 6.1.6.3.14 | Multi-Access PDU session release Indication | MAPDU |
| SmContextType | 6.1.6.3.15 | Type of SM Context information | DTSSA |
| PsaIndication | 6.1.6.3.16 | Indication of whether a PSA is inserted or removed | DTSSA |
| N4MessageType | 6.1.6.3.17 | N4 Message Type | DTSSA |

Table 6.1.6.1-2 specifies data types re-used by the Nsmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmf service based interface.

Table 6.1.6.1-2: Nsmf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uint32 | 3GPP TS 29.571 [13] | Unsigned 32-bit integers |  |
| Ipv4Addr | 3GPP TS 29.571 [13] | IPv4 Address |  |
| Ipv6Prefix | 3GPP TS 29.571 [13] | IPv6 Prefix |  |
| Uri | 3GPP TS 29.571 [13] | Uniform Resource Identifier |  |
| Supi | 3GPP TS 29.571 [13] | Subscription Permanent Identifier |  |
| Pei | 3GPP TS 29.571 [13] | Permanent Equipment Identifier |  |
| Gpsi | 3GPP TS 29.571 [13] | General Public Subscription Identifier |  |
| AccessType | 3GPP TS 29.571 [13] | Access Type (3GPP or non-3GPP access) |  |
| SupportedFeatures | 3GPP TS 29.571 [13] | Supported features |  |
| Qfi | 3GPP TS 29.571 [13] | QoS Flow Identifier |  |
| pduSessionId | 3GPP TS 29.571 [13] | PDU Session Identifier |  |
| pduSessionType | 3GPP TS 29.571 [13] | PDU Session Type |  |
| Ambr | 3GPP TS 29.571 [13] | PDU Session Aggregate Maximum Bit Rate |  |
| 5Qi | 3GPP TS 29.571 [13] | 5G QoS Identifier |  |
| Arp | 3GPP TS 29.571 [13] | Allocation and Retention Priority |  |
| ReflectiveQoSAttribute | 3GPP TS 29.571 [13] | Reflective QoS Attribute |  |
| Dynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics for a 5QI that is neither standardized nor pre-configured. |  |
| NonDynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics that replace the default QoS characteristics for a standardized or pre-configured 5QI. |  |
| PacketLossRate | 3GPP TS 29.571 [13] | Packet Loss Rate |  |
| NotificationControl | 3GPP TS 29.571 [13] | Notification Control |  |
| Dnn | 3GPP TS 29.571 [13] | Data Network Name |  |
| Snssai | 3GPP TS 29.571 [13] | Single Network Slice Selection Assistance Information |  |
| NfInstanceId | 3GPP TS 29.571 [13] | NF Instance Identifier |  |
| UserLocation | 3GPP TS 29.571 [13] | User Location |  |
| TimeZone | 3GPP TS 29.571 [13] | Time Zone |  |
| ProblemDetails | 3GPP TS 29.571 [13] | Error description |  |
| UpSecurity | 3GPP TS 29.571 [13] | User Plane Security Policy Enforcement information |  |
| RefToBinaryData | 3GPP TS 29.571 [13] | Cross-Reference to binary data encoded within a binary body part in an HTTP multipart message. |  |
| Guami | 3GPP TS 29.571 [13] | Globally Unique AMF ID |  |
| BackupAmfInfo | 3GPP TS 29.571 [13] | Backup AMF Information |  |
| PresenceState | 3GPP TS 29.571 [13] | Indicates the UE presence in or out of a LADN service area |  |
| TraceData | 3GPP TS 29.571 [13] | Trace control and configuration parameters |  |
| PlmnId | 3GPP TS 29.571 [13] | PLMN Identity |  |
| RatType | 3GPP TS 29.571 [13] | RAT Type |  |
| NgApCause | 3GPP TS 29.571 [13] | NGAP Cause |  |
| 5GMmCause | 3GPP TS 29.571 [13] | 5G MM Cause |  |
| DurationSec | 3GPP TS 29.571 [13] | Duration in units of seconds |  |
| AdditionalQosFlowInfo | 3GPP TS 29.571 [13] | Additional QoS Flow Information |  |
| NfGroupId | 3GPP TS 29.571 [13] | Network Function Group Id |  |
| SecondaryRatUsageReport | 3GPP TS 29.571 [13] | Secondary RAT Usage Report |  |
| SecondaryRatUsageInfo | 3GPP TS 29.571 [13] | Secondary RAT Usage Information |  |
| Dnai | 3GPP TS 29.571 [13] | Data Network Access Identifier | DTSSA |
| PlmnIdNid | 3GPP TS 29.571 [13] | PLMN Identity and, for SNPN, Network Identity |  |
| SmallDataRateStatus | 3GPP TS 29.571 [13] | Small Data Rate Control Status | CPCIOT |
| ServiceName | 3GPP TS 29.510 [19] | Service Name |  |
| NgRanTargetId | 3GPP TS 29.518 [20] | NG-RAN Target Id |  |
| RoamingChargingProfile | 3GPP TS 32.291 [26] | Roaming Charging Profile |  |
| StationaryIndication | 3GPP TS 29.571 [13] | Stationary Indication |  |
| ScheduledCommunicationTime | 3GPP TS 29.571 [13] | Scheduled Communication Time |  |
| ScheduledCommunicationType | 3GPP TS 29.571 [13] | Scheduled Communication Type |  |
| TrafficProfile | 3GPP TS 29.571 [13] | Traffic Profile |  |
| BatteryIndication | 3GPP TS 29.571 [13] | Battery Indication |  |

* * * * Next Change * * * *

##### 6.1.6.2.29	Type: StatusInfo

Table 6.1.6.2.29-1: Definition of type StatusInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resourceStatus | ResourceStatus | M | 1 | This IE shall indicate the status of the SM context or PDU session resource. |  |
| cause | Cause | O | 0..1 | When present, this IE shall indicate the cause for the resource status change. |  |
| cnAssistedRanPara | CnAssistedRanPara | C | 0..1 | This attribute shall be present when the cause value is "CN_ASSISTED_RAN_PARAMETER_TUNING". When present, this IE shall include the SMF derived CN assisted RAN parameters tuning. | CARPT |

* * * * Next Change * * * *

##### 6.1.6.3.8	Enumeration: Cause

The enumeration Cause indicates a cause information. It shall comply with the provisions defined in table 6.1.6.3.8-1.

Table 6.1.6.3.8-1: Enumeration Cause

| Enumeration value | Description |
| --- | --- |
| "REL_DUE_TO_HO" | Release due to Handover |
| "EPS_FALLBACK" | Mobility due to EPS fallback for IMS voice is on-going. |
| "REL_DUE_TO_UP_SEC" | Release due to user plane Security requirements that cannot be fulfilled. |
| "DNN_CONGESTION" | Release due to the DNN based congestion control. |
| "S_NSSAI_CONGESTION" | Release due to the S-NSSAI based congestion control. |
| "REL_DUE_TO_REACTIVATION" | Release due to PDU session reactivation. |
| "5G_AN_NOT_RESPONDING" | The 5G AN did not respond to the request initiated by the network. |
| "REL_DUE_TO_SLICE_NOT_AVAILABLE" | Release due to the associated S-NSSAI becomes no longer available. |
| "REL_DUE_TO_DUPLICATE_SESSION_ID" | Release due to a UE request to establish a new PDU session with an identical PDU session Id. |
| "PDU_SESSION_STATUS_MISMATCH" | Release due to mismatch of PDU Session status between UE and AMF. |
| "HO_FAILURE" | Handover preparation failure |
| "INSUFFICIENT_UP_RESOURCES" | Failure to activate the User Plane connection of a PDU session due to insufficient user plane resources. |
| "PDU_SESSION_HANDED_OVER" | The PDU session is handed over to another system or access. |
| "CN_ASSISTED_RAN_PARAMETER_TUNING" | SMF derived CN assisted RAN parameters tuning. |

* * * * Next Change * * * *

##### 6.1.6.2.x	Type: CnAssistedRanPara

Table 6.1.6.2.x-1: Definition of type CnAssistedRanPara

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| stationaryIndication | StationaryIndication | O | 0..1 | Identifies whether the UE is stationary or mobile (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| communicationDurationTime | DurationSec | O | 0..1 | Indicates for how long the UE will normally stay in CM-Connected for data transmission (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| periodicTime | DurationSec | O | 0..1 | Identifies interval time of periodic communication (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| scheduledCommunicationTime | ScheduledCommunicationTime | O | 0..1 | Identifies time and day of the week when the UE is available for communication (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| scheduledCommunicationType | ScheduledCommunicationType | O | 0..1 | Indicates that the Scheduled Communication Type (see 3GPP TS 23.502 [3] clause 4.15.6.3). (NOTE 2) |
| trafficProfile | TrafficProfile | O | 0..1 | Identifies the type of data transmission: single packet transmission (UL or DL), dual packet transmission (UL with subsequent DL or DL with subsequent UL), and multiple packets transmission (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| batteryIndication | BatteryIndication | O | 0..1 | Indicates the power consumption type(s) of the UE (see 3GPP TS 23.502 [3] clause 4.15.6.3). |
| NOTE 1: At least one of optional parameters above shall be present. NOTE 2: The value of attribute "scheduledCommunicationType" shall be used together with the value of "scheduledCommunicationTime". |  |  |  |  |

* * * * Next Change * * * *

##### 6.1.6.3.9	Enumeration: ResourceStatus

The enumeration ResourceStatus indicates the status of an SM context or PDU session resource. It shall comply with the provisions defined in table 6.1.6.3.9-1.

Table 6.1.6.3.9-1: Enumeration ResourceStatus

| Enumeration value | Description |
| --- | --- |
| "RELEASED" | The SM context or PDU session resource is released. |
| "UNCHANGED" | The status of SM context or PDU session resource is not changed. |

* * * * Next Change * * * *

### 6.1.8	Feature Negotiation

The feature negotiation mechanism specified in clause 6.6 of 3GPP TS 29.500 [4] shall be used to negotiate the optional features applicable between the SMF and the NF Service Consumer, for the Nsmf_PDUSession service, if any.

The NF Service Consumer shall indicate the optional features it supports for the Nsmf_PDUSession service, if any, by including the supportedFeatures attribute in the HTTP POST request when requesting the SMF to create an SM context or a PDU session resource.

The SMF shall determine the supported features for the created SM context or PDU session resource as specified in clause 6.6 of 3GPP TS 29.500 [4] and shall indicate the supported features by including the supportedFeatures attribute in the representation of the SM context or PDU session resource it returns in the HTTP response confirming the creation of the resource.

The syntax of the supportedFeatures attribute is defined in clause 5.2.2 of 3GPP TS 29.571 [13].

The following features are defined for the Nsmf_PDUSession service.

Table 6.1.8-1: Features of supportedFeatures attribute used by Nsmf_PDUSession service

| Feature Number | Feature | M/O | Description |
| --- | --- | --- | --- |
| 1 | CPCIOT | O | Control Plane CIoT 5GS Optimisation This optional feature allows transport of small user data via AMF and SMF by encapsulating them in NAS, reducing the total number of control plane messages for infrequent small data transactions. The SMF shall indicate its support of this feature in supportedFeatures attribute in its profile registered in NRF. An NF service consumer (e.g. AMF) shall only select SMF(s) that supports this feature for PDU sessions with Control Plane CIoT 5GS Optimisation enabled. |
| 2 | MAPDU | O | This feature bit indicates whether the SMF supports Multi-Access PDU session procedures related to Access Traffic Steering, Switching and Splitting (see clauses 4.2.10 and 5.32 of 3GPP TS 23.501 [2]). |
| 3 | DTSSA | O | Deployments Topologies with specific SMF Service Areas An SMF that supports this feature shall support the procedures specified in clause 5.34 of 3GPP TS 23.501 [2] and in clause 4.23 of 3GPP TS 23.502 [3]. |
| x | CARPT | O | SMF derived CN Assisted RAN parameters Tuning. NF Consumer (e.g. AMF) and SMF that support this feature shall support exchanging SMF derived CN assisted RAN parameters in Notify SM Context Status service operation (see clause 5.2.2.5.1). |
| Feature number: The order number of the feature within the supportedFeatures attribute (starting with 1). Feature: A short name that can be used to refer to the bit and to the feature. M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

* * * * Next Change * * * *

## A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.1.0.alpha-2'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

StatusInfo:

type: object

properties:

resourceStatus:

$ref: '#/components/schemas/ResourceStatus'

cause:

$ref: '#/components/schemas/Cause'

cnAssistedRanPara:

$ref: '#/components/schemas/CnAssistedRanPara'

required:

- resourceStatus

[…]

N4Information:

type: object

properties:

n4MessageType:

$ref: '#/components/schemas/N4MessageType'

n4MessagePayload:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n4DnaiInfo:

$ref: '#/components/schemas/DnaiInformation'

required:

- n4MessageType

- n4MessagePayload

CnAssistedRanPara:

type: object

properties:

stationaryIndication:

$ref: 'TS29571_CommonData.yaml#/components/schemas/StationaryIndication'

communicationDurationTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

periodicTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

scheduledCommunicationTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

scheduledCommunicationType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationType'

trafficProfile:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TrafficProfile'

batteryIndication:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BatteryIndication'

[…]

ResourceStatus:

anyOf:

- type: string

enum:

- RELEASED

- UNCHANGED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- RELEASED

- UNCHANGED

[…]

Cause:

anyOf:

- type: string

enum:

- REL_DUE_TO_HO

- EPS_FALLBACK

- REL_DUE_TO_UP_SEC

- DNN_CONGESTION

- S_NSSAI_CONGESTION

- REL_DUE_TO_REACTIVATION

- 5G_AN_NOT_RESPONDING

- REL_DUE_TO_SLICE_NOT_AVAILABLE

- REL_DUE_TO_DUPLICATE_SESSION_ID

- PDU_SESSION_STATUS_MISMATCH

- HO_FAILURE

- INSUFFICIENT_UP_RESOURCES

- PDU_SESSION_HANDED_OVER

- CN_ASSISTED_RAN_PARAMETER_TUNING

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- REL_DUE_TO_HO

- EPS_FALLBACK

- REL_DUE_TO_UP_SEC

- DNN_CONGESTION

- S_NSSAI_CONGESTION

- REL_DUE_TO_REACTIVATION

- 5G_AN_NOT_RESPONDING

- REL_DUE_TO_SLICE_NOT_AVAILABLE

- REL_DUE_TO_DUPLICATE_SESSION_ID

- PDU_SESSION_STATUS_MISMATCH

- HO_FAILURE

- INSUFFICIENT_UP_RESOURCES

- PDU_SESSION_HANDED_OVER

- CN_ASSISTED_RAN_PARAMETER_TUNING

* * * * End of Change * * * *