3GPP TSG-CT Meeting #86	CP-193168

# Sitges, SPAIN; 09th – 10th December 2019								           Revision of C4-195479

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0228 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobile Originated Data Transfer for Control Plane CIoT 5GS Optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Cisco, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell, Cisco, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-10-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With Control Plane CIoT 5GS Optimisation, the AMF transfers mobile originated data received over NAS to the SMF using a new Nsmf_PDUSession_MessageTransfer service operation. See step 4 of Figure 4.24.1 of TS 23.502 and step 1 of Figure 4.25.4-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new Send MO Data service operation is defined to enable the AMF to request the SMF to transfer small data received from the UE over NAS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.1, 5.2.1, 5.2.2.x (new), 6.1.3.1, 6.1.3.3.4.1, 6.1.3.3.4.x (new), 6.1.6.1, 6.1.6.2.x (new), 6.1.6.4.x (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features to the OpenAPI specification file. This CR corresponds to the Key Issue "CP CIoT Optimization" , raw 4, Item 2 of the 5G_CIoT work plan (C4-194075). Rev 2: in A.2, reference below is corrected as shown in red requestBody: description: representation of the payload of Send MO Data Request required: true content: multipart/related: # message with a binary body part schema: type: object properties: jsonData: $ref: '#/components/schemas/SendMoDataReqData' binaryMoData: type: string format: binary Table 6.1.3.3.4.1-1 (overview of Custom operations) was missed to be updated with a new entry for the new service operation. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BP	Branching Point

DNAI	Data Network Access Identifier

DNN	Data Network Name

HR	Home Routed

H-SMF	Home SMF

I-SMF	Intermediate SMF

JSON	Javascript Object NotationNAS	Non-Access Stratum

LADN	Local Area Data Network

MO	Mobile Originated

PSA	PDU Session Anchor

SM	Session Management

SMF	Session Management Function

SNPN	Stand-alone Non-Public Network

UL CL	Uplink Classifier

UPF	User Plane Function

V-SMF	Visited SMF

* * * Next Change * * * *

## 5.1	Introduction

The SMF supports the following services.

Table 5.1-1: NF Services provided by SMF

| Service Name | Description | Example Consumer |
| --- | --- | --- |
| Nsmf_PDUSession | This service manages the PDU sessions and uses the policy and charging rules received from the PCF. The service operations exposed by this NF service allows the consumer NFs to establish, modify and delete the PDU sessions, and to send mobile originated data. | V-SMF, H-SMF, AMF |
| Nsmf_EventExposure | This service exposes the events happening on the PDU sessions to the consumer NFs. | PCF, NEF, AMF |

The Nsmf_EventExposure service is specified in 3GPP TS 29.508 [6].

* * * Next Change * * * *

## 5.2	Nsmf_PDUSession Service

### 5.2.1	Service Description

The Nsmf_PDUSession service operates on the PDU Sessions. The service operations exposed by this service allow other NFs to establish, modify and release the PDU Sessions. The following are the key functionalities of this NF service:

-	Creation, modification and deletion of SM contexts for PDU Sessions; an SM context represents an association between the NF Service Consumer (e.g. AMF) and the SMF for a PDU session;

-	Retrieval of SM contexts of PDU sessions (i.e. UE EPS PDN connection or complete SM context), e.g. to move PDU sessions towards the EPC using the N26 interface or to transfer SM contexts between I-SMFs or V-SMFs over the N38 interface;

-	Creation, modification and deletion of PDU sessions between the V-SMF and H-SMF in HR roaming scenarios, or between the I-SMF and SMF for PDU sessions involving an I-SMF;

-	Sending of mobile originated data (received over NAS) for a PDU session to the SMF, V-SMF in HR roaming scenarios, or I-SMF for PDU sessions involving an I-SMF;

-	Association of policy and charging rules with PDU Sessions and binding the policy and charging rules to flows;

-	Interacting with the UPF over N4 for creating, modifying and releasing user plane sessions;

-	Process user plane events from the UPF and apply the corresponding policy and charging rules.

The Nsmf_PDUSession service supports the following service operations.

Table 5.2.1-1: Service operations supported by the Nsmf_PDUSession service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Create SM Context | Create an SM context in SMF, or in V-SMF in HR roaming scenarios, or in I-SMF during the I-SMF insertion and change scenarios, for a PDU session. | Request/Response | AMF |
| Update SM Context | Update the SM context of a PDU session and/or provide the SMF with N1 or N2 SM information received from the UE or from the AN. | Request/Response | AMF, I-SMF |
| Release SM Context | Release the SM context of a PDU session when the PDU session has been released. | Request/Response | AMF |
| Notify SM Context Status (NOTE 1) | Notify the NF Service Consumer about the status of an SM Context of a PDU session (e.g. the SM Context is released within the SMF). | Subscribe/Notify | AMF |
| Retrieve SM Context (NOTE 2) | Retrieve an SM context of a PDU session: - from SMF, or from V-SMF in HR roaming scenarios, for 5GS to EPS mobility; - from SMF during I-SMF insertion or from I-SMF during I-SMF change/removal; - from V-SMF during change of V-SMF. | Request/Response | AMF, I-SMF, V-SMF, SMF |
| Create | Create a PDU session in the H-SMF in HR roaming scenarios, or in the SMF for PDU sessions involving an I-SMF. | Request/Response | V-SMF, I-SMF |
| Update | Update a PDU session in the H-SMF or V-SMF in HR roaming scenarios, or in the I-SMF or SMF for PDU sessions involving an I-SMF. | Request/Response | V-SMF, H-SMF, I-SMF, SMF |
| Release | Release a PDU session in the H-SMF in HR roaming scenarios, or in the SMF for PDU sessions involving an I-SMF. | Request/Response | V-SMF, I-SMF |
| Notify Status (NOTE 3) | Notify the NF Service Consumer about the status of a PDU session (e.g. the PDU session is released due to local reasons within the SMF). | Subscribe/Notify | V-SMF, I-SMF |
| Send MO Data (NOTE x) | Send mobile originated data received over NAS for a PDU session | Request/Response | AMF |
| NOTE 1: This corresponds to the SMContextStatusNotify service operation defined in 3GPP TS 23.502 [3]. NOTE 2: This corresponds to the ContextRequest service operation defined in 3GPP TS 23.502 [3]. NOTE 3: This corresponds to the StatusNotify service operation defined in 3GPP TS 23.502 [3]. NOTE x: This corresponds to the MessageTransfer service operation defined in 3GPP TS 23.502 [3]. |  |  |  |

* * * Next Change * * * *

#### 5.2.2.x	Send MO Data service operation

##### 5.2.2.x.1	General

The Send MO Data service operation shall be used to send mobile originated data received over NAS, for a given PDU session, towards the SMF, or the V-SMF for HR roaming scenarios, or the I-SMF for a PDU session with an I-SMF.

It is used in the following procedures:

-	UPF anchored Mobile Originated Data Transport in Control Plane CIoT 5GS Optimisation (see clause 4.24.1 of 3GPP TS 23.502 [3]);

-	NEF anchored Mobile Originated Data Transport (see clause 4.25.4 of 3GPP TS 23.502 [3]).

The NF Service Consumer (e.g. AMF) shall send mobile originated data to the SMF by using the HTTP POST method (send-mo-data custom operation) as shown in Figure 5.2.2.x.1-1.

Figure 5.2.2.x.1-1: Send MO Data

1.	The NF Service Consumer shall send a POST request to the resource representing the individual SM context resource in the SMF. The payload body of the POST request shall contain the mobile originated data to send.

2a.	On success, "204 No Content" shall be returned.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned.  For a 4xx/5xx response, the message body shall contain a ProblemDetails, with the "cause" attribute indicating the cause of the failure.

* * * Next Change * * * *

### 6.1.3	Resources

#### 6.1.3.1	Overview

Figure 6.1.3.1-1 describes the resource URI structure of the Nsmf_PDUSession API.

Figure 6.1.3.1-1: Resource URI structure of the Nsmf_PDUSession API

NOTE:	The sm-contexts and pdu-sessions collection resources can be distributed on different processing instances or hosts. Thus, the authority and/or deployment-specific string of the apiRoot of the created individual sm context and pdu-session resources' URIs can differ from the authority and/or deployment-specific string of the apiRoot of the sm-contexts and pdu-sessions distributed collections' URIs.

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| SM contexts collection | {apiRoot}/nsmf-pdusession/{apiVersion}/sm-contexts | POST | Create SM Context |
| Individual SM context | {apiRoot}/nsmf-pdusession/{apiVersion}/sm-contexts/{smContextRef}/retrieve | retrieve (POST) | Retrieve SM Context |
|  | {apiRoot}/nsmf-pdusession/{apiVersion}/sm-contexts/{smContextRef}/modify | modify (POST) | Update SM Context |
|  | {apiRoot}/nsmf-pdusession/{apiVersion}/sm-contexts/{smContextRef}/release | release (POST) | Release SM Context |
|  | {apiRoot}/nsmf-pdusession/{apiVersion}/sm-contexts/{smContextRef}/send-mo-data | send-mo-data (POST) | Send MO Data |
| PDU sessions collection (H-SMF or SMF) | {apiRoot}/nsmf-pdusession/{apiVersion}/pdu-sessions | POST | Create |
| Individual PDU session (H-SMF or SMF) | {apiRoot}/nsmf-pdusession/{apiVersion}/pdu-sessions/{pduSessionRef}/modify | modify (POST) | Update (initiated by V-SMF or I-SMF) |
|  | {apiRoot}/nsmf-pdusession/{apiVersion}/pdu-sessions/{pduSessionRef}/release | release (POST) | Release |
| Individual PDU session (V-SMF or I-SMF) | {vsmfPduSessionUri}/modify or {ismfPduSessionUri}/modify | modify (POST) | Updated (initiated by H-SMF or SMF) |
|  | {vsmfPduSessionUri} or {ismfPduSessionUri} | POST | Notify Status |

* * * Next Change * * * *

###### 6.1.3.3.4.1	Overview

Table 6.1.3.3.4.1-1: Custom operations

| Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {resourceUri}/modify | POST | Update SM Context service operation. |
| {resourceUri}/release | POST | Release SM Context service operation. |
| {resourceUri}/retrieve | POST | Retrieve SM Context service operation |
| {resourceUri}/send-mo-data | POST | Send MO Data service operation |

* * * Next Change * * * *

###### 6.1.3.3.4.x	Operation: send-mo-data

6.1.3.3.4.x.1	Description

6.1.3.3.4.x.2	Operation Definition

This custom operation enables to send mobile originated data received over NAS, for a given PDU session, towards the SMF, or the V-SMF for HR roaming scenarios, or the I-SMF for a PDU session with an I-SMF.

This operation shall support the request data structures specified in table 6.1.3.3.4.x.2-1 and the response data structure and response codes specified in table 6.1.3.3.4.x.2-2.

Table 6.1.3.3.4.x.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SendMoDataReqData | M | 1 | Representation of the payload of a Send MO Data Request |

Table 6.1.3.3.4.x.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
|  |  |  | 204 No Content | Successful MO data transfer |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

* * * Next Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsmf service based interface protocol.

Table 6.1.6.1-1: Nsmf specific Data Types

| Data type | Clause defined | Description |  |
| --- | --- | --- | --- |
| SmContextCreateData | 6.1.6.2.2 | Information within Create SM Context Request |  |
| SmContextCreatedData | 6.1.6.2.3 | Information within Create SM Context Response |  |
| SmContextUpdateData | 6.1.6.2.4 | Information within Update SM Context Request |  |
| SmContextUpdatedData | 6.1.6.2.5 | Information within Update SM Context Response |  |
| SmContextReleaseData | 6.1.6.2.6 | Information within Release SM Context Request |  |
| SMContextRetrieveData | 6.1.6.2.7 | Information within Retrieve SM Context Request |  |
| SmContextStatusNotification | 6.1.6.2.8 | Information within Notify SM Context Status Request |  |
| PduSessionCreateData | 6.1.6.2.9 | Information within Create Request |  |
| PduSessionCreatedData | 6.1.6.2.10 | Information within Create Response |  |
| HsmfUpdateData | 6.1.6.2.11 | Information within Update Request towards H-SMF, or from I-SMF to SMF |  |
| HsmfUpdatedData | 6.1.6.2.12 | Information within Update Response from H-SMF, or from SMF to I-SMF |  |
| ReleaseData | 6.1.6.2.13 | Information within Release Request |  |
| HsmfUpdateError | 6.1.6.2.14 | Error within Update Response from H-SMF |  |
| VsmfUpdateData | 6.1.6.2.15 | Information within Update Request towards V-SMF, or from SMF to I-SMF |  |
| VsmfUpdatedData | 6.1.6.2.16 | Information within Update Response from V-SMF, or from I-SMF to SMF |  |
| StatusNotification | 6.1.6.2.17 | Information within Notify Status Request |  |
| QosFlowItem | 6.1.6.2.18 | Individual QoS flow |  |
| QosFlowSetupItem | 6.1.6.2.19 | Individual QoS flow to setup |  |
| QosFlowAddModifyRequestItem | 6.1.6.2.20 | Individual QoS flow requested to be created or modified |  |
| QosFlowReleaseRequestItem | 6.1.6.2.21 | Individual QoS flow requested to be released |  |
| QosFlowProfile | 6.1.6.2.22 | QoS flow profile |  |
| GbrQosFlowInformation | 6.1.6.2.23 | GBR QoS flow information |  |
| QosFlowNotifyItem | 6.1.6.2.24 | Notification related to a QoS flow |  |
| SMContextRetrievedData | 6.1.6.2.27 | Information within Retrieve SM Context Response |  |
| TunnelInfo | 6.1.6.2.28 | Tunnel Information |  |
| StatusInfo | 6.1.6.2.29 | Status of SM context or of PDU session |  |
| VsmfUpdateError | 6.1.6.2.30 | Error within Update Response from V-SMF |  |
| EpsPdnCnxInfo | 6.1.6.2.31 | EPS PDN Connection Information from H-SMF to V-SMF |  |
| EpsBearerInfo | 6.1.6.2.32 | EPS Bearer Information from H-SMF to V-SMF |  |
| PduSessionNotifyItem | 6.1.6.2.33 | Notification related to a PDU session |  |
| EbiArpMapping | 6.1.6.2.34 | EBI to ARP mapping |  |
| SmContextCreateError | 6.1.6.2.35 | Error within Create SM Context Response |  |
| SmContextUpdateError | 6.1.6.2.36 | Error within Update SM Context Response |  |
| PduSessionCreateError | 6.1.6.2.37 | Error within Create Response |  |
| MmeCapabilities | 6.1.6.2.38 | MME capabilities |  |
| SmContext | 6.1.6.2.39 | Complete SM Context |  |
| ExemptionInd | 6.1.6.2.40 | Exemption Indication |  |
| PsaInformation | 6.1.6.2.41 | PSA Information |  |
| DnaiInformation | 6.1.6.2.42 | DNAI Information |  |
| N4Information | 6.1.6.2.43 | N4 Information |  |
| SendMoDataReqData | 6.1.6.2.x | Information within Send MO Data Request |  |
| GtpTeid | 6.1.6.3.2 | GTP Tunnel Endpoint Identifier |  |
| ProcedureTransactionId | 6.1.6.3.2 | Procedure Transaction Identifier |  |
| EpsPdnCnxContainer | 6.1.6.3.2 | UE EPS PDN Connection container from SMF to AMF |  |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Id |  |
| EpsBearerContainer | 6.1.6.3.2 | EPS Bearer container from SMF to AMF |  |
| UpCnxState | 6.1.6.3.3 | User Plane Connection State |  |
| HoState | 6.1.6.3.4 | Handover State |  |
| RequestType | 6.1.6.3.5 | Request Type in Create (SM context) service operation. |  |
| RequestIndication | 6.1.6.3.6 | Request Indication in Update (SM context) service operation. |  |
| NotificationCause | 6.1.6.3.7 | Cause for generating a notification |  |
| Cause | 6.1.6.3.8 | Cause information |  |
| ResourceStatus | 6.1.6.3.9 | Status of SM context or PDU session resource |  |
| DnnSelectionMode | 6.1.6.3.10 | DNN Selection Mode |  |
| EpsInterworkingIndication | 6.1.6.3.11 | EPS Interworking Indication |  |
| N2SmInfoType | 6.1.6.3.12 | N2 SM Information Type |  |
| MaxIntegrityProtectedDataRate | 6.1.6.3.13 | Maximum Integrity Protected Data Rate |  |
| MaReleaseIndication | 6.1.6.3.14 | Multi-Access PDU session release Indication |  |
| SmContextType | 6.1.6.3.15 | Type of SM Context information |  |
| PsaIndication | 6.1.6.3.16 | Indication of whether a PSA is inserted or removed |  |
| N4MessageType | 6.1.6.3.17 | N4 Message Type |  |

Table 6.1.6.1-2 specifies data types re-used by the Nsmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmf service based interface.

Table 6.1.6.1-2: Nsmf re-used Data Types

| Data type | Reference | Comments |  |
| --- | --- | --- | --- |
| Uint32 | 3GPP TS 29.571 [13] | Unsigned 32-bit integers |  |
| Ipv4Addr | 3GPP TS 29.571 [13] | IPv4 Address |  |
| Ipv6Prefix | 3GPP TS 29.571 [13] | IPv6 Prefix |  |
| Uri | 3GPP TS 29.571 [13] | Uniform Resource Identifier |  |
| Supi | 3GPP TS 29.571 [13] | Subscription Permanent Identifier |  |
| Pei | 3GPP TS 29.571 [13] | Permanent Equipment Identifier |  |
| Gpsi | 3GPP TS 29.571 [13] | General Public Subscription Identifier |  |
| AccessType | 3GPP TS 29.571 [13] | Access Type (3GPP or non-3GPP access) |  |
| SupportedFeatures | 3GPP TS 29.571 [13] | Supported features |  |
| Qfi | 3GPP TS 29.571 [13] | QoS Flow Identifier |  |
| pduSessionId | 3GPP TS 29.571 [13] | PDU Session Identifier |  |
| pduSessionType | 3GPP TS 29.571 [13] | PDU Session Type |  |
| Ambr | 3GPP TS 29.571 [13] | PDU Session Aggregate Maximum Bit Rate |  |
| 5Qi | 3GPP TS 29.571 [13] | 5G QoS Identifier |  |
| Arp | 3GPP TS 29.571 [13] | Allocation and Retention Priority |  |
| ReflectiveQoSAttribute | 3GPP TS 29.571 [13] | Reflective QoS Attribute |  |
| Dynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics for a 5QI that is neither standardized nor pre-configured. |  |
| NonDynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics that replace the default QoS characteristics for a standardized or pre-configured 5QI. |  |
| PacketLossRate | 3GPP TS 29.571 [13] | Packet Loss Rate |  |
| NotificationControl | 3GPP TS 29.571 [13] | Notification Control |  |
| Dnn | 3GPP TS 29.571 [13] | Data Network Name |  |
| Snssai | 3GPP TS 29.571 [13] | Single Network Slice Selection Assistance Information |  |
| NfInstanceId | 3GPP TS 29.571 [13] | NF Instance Identifier |  |
| UserLocation | 3GPP TS 29.571 [13] | User Location |  |
| TimeZone | 3GPP TS 29.571 [13] | Time Zone |  |
| ProblemDetails | 3GPP TS 29.571 [13] | Error description |  |
| UpSecurity | 3GPP TS 29.571 [13] | User Plane Security Policy Enforcement information |  |
| RefToBinaryData | 3GPP TS 29.571 [13] | Cross-Reference to binary data encoded within a binary body part in an HTTP multipart message. |  |
| Guami | 3GPP TS 29.571 [13] | Globally Unique AMF ID |  |
| BackupAmfInfo | 3GPP TS 29.571 [13] | Backup AMF Information |  |
| PresenceState | 3GPP TS 29.571 [13] | Indicates the UE presence in or out of a LADN service area |  |
| TraceData | 3GPP TS 29.571 [13] | Trace control and configuration parameters |  |
| PlmnId | 3GPP TS 29.571 [13] | PLMN Identity |  |
| RatType | 3GPP TS 29.571 [13] | RAT Type |  |
| NgApCause | 3GPP TS 29.571 [13] | NGAP Cause |  |
| 5GMmCause | 3GPP TS 29.571 [13] | 5G MM Cause |  |
| DurationSec | 3GPP TS 29.571 [13] | Duration in units of seconds |  |
| AdditionalQosFlowInfo | 3GPP TS 29.571 [13] | Additional QoS Flow Information |  |
| NfGroupId | 3GPP TS 29.571 [13] | Network Function Group Id |  |
| SecondaryRatUsageReport | 3GPP TS 29.571 [13] | Secondary RAT Usage Report |  |
| SecondaryRatUsageInfo | 3GPP TS 29.571 [13] | Secondary RAT Usage Information |  |
| Dnai | 3GPP TS 29.571 [13] | Data Network Access Identifier |  |
| PlmnIdNid | 3GPP TS 29.571 [13] | PLMN Identity and, for SNPN, Network Identity |  |
| SmallDataRateStatus | 3GPP TS 29.571 [13] | Small Data Rate Control Status |  |
| ServiceName | 3GPP TS 29.510 [19] | Service Name |  |
| NgRanTargetId | 3GPP TS 29.518 [20] | NG-RAN Target Id |  |
| RoamingChargingProfile | 3GPP TS 32.291 [26] | Roaming Charging Profile |  |

* * * Next Change * * * *

##### 6.1.6.2.x	Type: SendMoDataReqData

Table 6.1.6.2.x-1: Definition of type SendMoDataReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| moData | RefToBinaryData | M | 1 | This IE shall reference the mobile originated data (see clause 6.1.6.4.x). | CPCIOT |
| ueLocation | UserLocation | O | 0..1 | When present, this IE shall contain the user location. | CPCIOT |

* * * Next Change * * * *

##### 6.1.6.4.x	Mobile Originated Data

Mobile Originated Data shall encode the Data Contents of the CIoT small data container or Payload Container specified in 3GPP TS 24.501 [7], using the vnd.3gpp.5gnas content-type, as summarized in Table 6.1.6.4.x-1.

Table 6.1.6.4.x-1: Mobile Originated Data

| Mobile Originated Data | Reference (3GPP TS 24.501 [7]) | Related NAS SM message |
| --- | --- | --- |
| Data contents in octet 4 to octet up to 257 of CIoT small data container contents. | 9.11.3.67 | Control Plane Service Request |
| Payload container contents in octets 4 to n | 9.11.3.39 | UL NAS Transport |

* * * Next Change * * * *

## A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.1.0.alpha-2'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

paths:

/sm-contexts:

[…]

/sm-contexts/{smContextRef}/send-mo-data:

post:

summary:  Send MO Data

tags:

- Individual SM context

operationId: SendMoData

parameters:

- name: smContextRef

in: path

description:  SM context reference

required: true

schema:

type: string

requestBody:

description: representation of the payload of Send MO Data Request

required: true

content:

multipart/related:  # message with a binary body part

schema:

type: object

properties:

jsonData:

$ref: '#/components/schemas/SendMoDataReqData'

binaryMoData:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryMoData:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

responses:

'204':

description: successful sending of MO data

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

[…]

N4Information:

type: object

properties:

n4MessageType:

$ref: '#/components/schemas/N4MessageType'

n4MessagePayload:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n4DnaiInfo:

$ref: '#/components/schemas/DnaiInformation'

required:

- n4MessageType

- n4MessagePayload

SendMoDataReqData:

type: object

properties:

moData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

required:

- moData

[…]

* * * End of Change * * * *