3GPP TSG-CT Meeting #86	CP-193284

Sitges, SPAIN; 09th – 10th December 2019	(was C1-198854, CP-193266)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.379 | CR | 0529 | rev | 5 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Preconfigured regroup – XML schema for regroup using preconfigured group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Work item code: | enh2MCPTT-CT |  |  |  |  |  | Date: |  |  | 2019-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA6 has concluded work on user/group regroup based on a preconfigured group. Based on TS 23.379, this feature needs to be brought into stage 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This Tdoc creates the XML schema for regroup using preconfigured group. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The regroup based on a preconfigured group feature will not be completely implemented in Rel-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.7, F.7.1, F.7.2, F.7.3, F.7.4 (all new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 4 is created for the CT Plenary #86 using exactly revision 3 as agreed in CT1 and adding the "mcpttregroup.xsd" file that was missing. |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

# F.7	XML schema for regroup using preconfigured group

## F.7.1	General

This subclause defines the XML schema and MIME type for regroup using preconfigured group.

## F.7.2	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:preconfiguredRegroup:1.0"

xmlns:mcpttrgrp="urn:3gpp:ns:preconfiguredRegroup:1.0"

attributeFormDefault="unqualified" elementFormDefault="qualified">

<!-- root XML element -->

<xs:element name="mcpttregroup" type="mcpttrgrp:mcpttregroup-Type" id="info"/>

<xs:complexType name="mcpttregroup-Type">

<xs:sequence>

<xs:element name="mcpttregroup-Params" type="mcpttrgrp:mcpttregroup-ParamsType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttrgrp:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="mcpttregroup-ParamsType">

<xs:sequence>

<xs:element name="preconfig-group-id" type="mcpttrgrp:preconfig-group-Type"/>

<xs:element name="mcptt-regroup-id" type="mcpttrgrp:mcptt-regroup-uri-Type"/>

<xs:element name="groups-for-regroup" type="mcpttrgrp:groups-for-regroup-Type"/>

<xs:element name="users-for-regroup" type="mcpttrgrp:users-for-regroup-Type"/>

<xs:element name="regroup-action" type="xs:string"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttrgrp:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="preconfig-group-Type">

<xs:sequence>

<xs:element type="xs:anyURI" name="preconfigured-group" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttrgrp:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="mcptt-regroup-uri-Type">

<xs:sequence>

<xs:element type="xs:anyURI" name="mcptt-regroup-uri" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttrgrp:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="groups-for-regroup-Type">

<xs:sequence>

<xs:element type="xs:anyURI" name="group" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttrgrp:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="users-for-regroup-Type">

<xs:sequence>

<xs:element type="xs:anyURI" name="user" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttrgrp:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

* * * * * * * NEXT CHANGE * * * * * * *

## F.7.3	Semantic

The <preconfigured-group> element shall contain a URI identifying an MCPTT group that is to be used as the template for the regroup.

The <mcptt-regroup-uri> element shall shall contain a URI identifying the regroup.

The < groups-for-regroup> element shall contain a list of <group> elements that shall each contain a URI of a group that is to be a constituent group of the regroup.

The <users-for-regroup> element shall contain a list of <user> elements that shall each contain an MCPTT ID of a user that is to be affiliated to the regroup.

The XML document shall have either a <groups-for-regroup> element or a <users-for-regroup> element, but not both.

If the <regroup-action> element contains the string "create" then:

1)	if a <groups-for-regroup> element exists in the received XML, then:

a)	if the recipient is the controlling MCPTT function for the MCPTT group identified in the <preconfigured-group> element the recipient shall follow the procedures to create a group regroup with identity equal to the value contained in the <mcptt-regroup-uri> element based on the configuration of the preconfigured MCPTT group identified in the <preconfigured-group> element;

b)	if the recipient is a non-controlling MCPTT function, the recipient shall follow the procedures to affiliate users belonging to any constituent groups of the group regroup with identity equal to the value contained in the <mcptt-regroup-uri> element based on the configuration of the preconfigured MCPTT group identified in the <preconfigured-group> element; and

c)	if the recipient is the terminating participating MCPTT function for one or more MCPTT users affiliated to a constituent group of the group regroup, the recipient shall follow the procedures to notify each MCPTT user in the list of users in the <users-for-regroup> element that it serves of the group regroup and affiliate those users to the group regroup; and

2)	if a <users-for-regroup> element exists in the received XML, then:

a)	if the recipient is the controlling MCPTT function for the MCPTT group identified in the <preconfigured-group> element, the recipient shall follow the procedures to create a user regroup with identity equal to the value contained in the <mcptt-regroup-uri> element based on the configuration of the preconfigured MCPTT group identified in the <preconfigured-group> element; and

b)	if the recipient is the terminating participating MCPTT function for one or more MCPTT users identified in the <users-for-regroup> element, the recipient shall follow the procedures to notify each MCPTT user in the list of users in the <users-for-regroup> element that it serves of the user regroup and affiliate those users to the user regroup.

If the <regroup-action> element contains the string "remove" then:

1)	the recipient shall follow the procedures to remove the regroup identified in the <mcptt-regroup-uri> element.

The recipient of the XML ignores any unknown element and any unknown attribute.

* * * * * * * NEXT CHANGE * * * * * * *

## F.7.4	IANA registration template

Editor's Note:	[enh2MCPTT-CT, CR 0529] MCC is requested to submit the IANA registration for this media type after the completion of 3GPP release 16.

Your Name:

<MCC name>

Your Email Address:

<MCC email address>

Media Type Name:

Application

Subtype name:

vnd.3gpp.mcptt-regroup+xml

Required parameters:

# None

Optional parameters:

"charset"	the parameter has identical semantics to the charset parameter of the "application/xml" media type as specified in section 9.1 of IETF RFC 7303.

Encoding considerations:

binary.

Security considerations:

Same as general security considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. In addition, this media type provides a format for exchanging information in SIP, so the security considerations from IETF RFC 3261 apply.

The information transported in this media type does not include active or executable content.

Mechanisms for privacy and integrity protection of protocol parameters exist. Those mechanisms as well as authentication and further security mechanisms are described in 3GPP TS 24.229.

This media type does not include provisions for directives that institute actions on a recipient's files or other resources.

This media type does not include provisions for directives that institute actions that, while not directly harmful to the recipient, may result in disclosure of information that either facilitates a subsequent attack or else violates a recipient's privacy in any way.

This media type does not employ compression.

Interoperability considerations:

Same as general interoperability considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. Any unknown XML elements and any unknown XML attributes are to be ignored by recipient of the MIME body.

Published specification:

3GPP TS 24.379 "Mission Critical Push To Talk (MCPTT) call control" version 16.2.0, available via http://www.3gpp.org/specs/numbering.htm.

Applications which use this media type:

Applications supporting the mission critical push to talk as described in the published specification.

Fragment identifier considerations:

The handling in section 5 of IETF RFC 7303 applies.

Restrictions on usage:

None

Provisional registration? (standards tree only):

N/A

Additional information:

1.	Deprecated alias names for this type: none

2.	Magic number(s): none

3.	File extension(s): none

4.	Macintosh File Type Code(s): none

5.	Object Identifier(s) or OID(s): none

Intended usage:

Common

Person to contact for further information:

-	Name: <MCC name>

-	Email: <MCC email address>

-	Author/Change controller:

i)	Author: 3GPP CT1 Working Group/3GPP_TSG_CT_WG1@LIST.ETSI.ORG

ii)	Change controller: <MCC name>/<MCC email address>

* * * * * * * END CHANGES * * * * * * *