3GPP TSG-CT WG1 Meeting #124-e	C1-203927

Electronic meeting, 2-10 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.380 | CR | 0245 | rev | - | Current version: | 13.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SSRC handling for implicit floor request case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCPTT-CT |  |  |  |  |  | Date: |  |  | 2020-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No procedure is defined to handle the SSRC in case of implicit floor request with/without implicit grant support in 3GPP TS 24.380. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced the new media fmtp attribute to support the ssrc value in case of implicit floor request with/without implicit grant support. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The talker will not know the SSRC value to use for voice packets when the floor is granted for an implicit floor request, and SSRC value cannot be conveyed to listeners to identify the sender of voice packet. (e.g Multi talker scenario in a group call). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.2.2, 12.1.2.3 and 14.3.6(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

#### 12.1.2.2	Semantics

In an SDP offer and answer, the "mc_queueing" fmtp attribute is used to indicate support of the Floor Request message queueing mechanism, as defined in the present specification.

In an SDP offer, the "mc_priority" fmtp attribute indicates (using an integer value between '1' and '255') the maximum floor priority that the offerer requests to be used with Floor Request messages sent by the offerer. In an SDP answer, the attribute parameter indicates the maximum priority level that the answerer has granted to the offerer. The value must be equal or less than the value provided in the associated SDP offer.

NOTE 1: If the "mc_priority" fmtp attribute is not used within an SDP offer or answer, a default priority value is assumed.

In an SDP offer, the "mc_granted" fmtp attribute parameter indicates that the offerer supports the procedure where the answerer indicates, using the fmtp attribute in the associated SDP answer, that the floor has been granted to the offerer.

NOTE 2: When the "mc_granted" fmtp attribute is used in an SDP offer, it does not indicate an actual request for the floor. The SDP "mc_implicit_request" fmtp attribute can be used to request the floor. In an SDP answer, the attribute indicates that the floor has been granted to the offerer.

NOTE 3: Once the offerer has been granted the floor, the offerer has the floor until it receives a Floor Revoke message, or until the offerer itself releases the floor by sending a Floor Release message, as described in the present specification.

In an SDP offer, the "mc_implicit_request" fmtp attribute indicates that the offerer implicitly requests the floor (without the need to send a Floor Request message). In an SDP answer, the attribute parameter indicates that the answerer has accepted the implicit floor request. Once the answerer grants the floor to the offerer, the answerer will send a Floor Granted message.

NOTE 4: The usage of the "mc_implicit_request" fmtp attribute in an SDP answer does not mean that the answerer has granted the floor to the offerer, only that the answerer has accepted the implicit floor request.

In an SDP answer, the "mc_ssrc" fmtp attribute indicates ssrc value of the offerer. The value may be equal to the value provided in the associated SDP offer or different if the collision is detected. If the associated SDP offer doesn’t indicate the ssrc value then server can determine an appropriate value. This value is returned in an SDP answer only if the answerer accepts the implicit floor request offered in SDP.

* * * * * * * NEXT CHANGE * * * * * * *

#### 12.1.2.3	Syntax

Table 12.1.2.3-1: SDP "fmtp" attribute for the MCPTT media plane control channel

fmtp-attr-mpcp      = "a=fmtp:" "MCPTT" SP attr-param-list

attr-param-list     =  attr-param *(COLON attr-param)

attr-param          =  mc_queueing / mc_priority / mc_granted / mc_implicit_request / mc_ssrc

mc_queueing          =  "mc_queueing"

mc_priority         =  "mc_priority=" 1*2(DIGIT)

mc_granted          =  "mc_granted"

mc_implicit_request =  "mc_implicit_request"

mc_ssrc		        =  "mc_ssrc=" 1*(DIGIT)

* * * * * * * NEXT CHANGE * * * * * * *

### 14.3.6	"mc_ssrc" fmtp attribute

If the “ssrc” media attribute is included in an SDP offer and the SIP request shall be interpreted as an implicit floor request (“mc_implicit_request” fmtp attribute is included), the MCPTT server shall return the value included in the SDP offer or new ssrc value if collision is detected or if the associated SDP offer doesn’t indicate the ssrc value then server can determine an appropriate value in the SDP answer of "mc_ssrc" fmtp attribute “mc_granted” fmtp attribute is present in the application m-line.

* * * * * * * END CHANGES * * * * * * *