3GPP TSG-CT WG1 Meeting #123-e	C1-202760

Electronic meeting, 16-24 April February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.588 | CR | 0002 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on the V2X policies regarding QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. SA2 approved CR0087 against TS 23.287 to update “SLRB configuration” to “AS layer configuration” and “SLRB” to “radio bearer” with the following reason for change: “AS layer configurations could include more than the SLRB configuration, and the actual parameters of AS layer configuration shall be defined by RAN.” So this paper would like propose to define “AS configuration” included in the configuration parameter for PC5, which includes SLRB mapping rules for future use. The term “SLRB” itself referring SL radio bearer remains same since RAN2 still uses the same term in their specifications. 2. There’s an editorial error in table 5.3.1.49 regarding Maximum data burst value: Maximum data burst volume: The averaging window field indicates a binary representation of the maximum data burst volume for both sending and receiving in octets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define “AS configuration” included in the configuration parameter for PC5, which includes SLRB mapping rules for future use Fixed wrong parameter name regarding maximum data burst volume. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with stage 2, and the configuration is limited to be used for SLRB mapping only, which is not futureproof. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1 - More co-sourcing companies; - Reason for change is updated; - Instead of updating SLRB mappinr rules to “AS configuration mapping rules”, define “AS configuration” which includes SLRB mapping rules. |  |  |  |  |  |  |  |  |

***** First change *****

5.3.1	General

The UE policies for V2X communication over PC5 are coded as shown in figures 5.3.1.1 and table 5.3.1.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | V2XP info type = {UE policies for V2X communication over PC5} |  |  |  | octet k |
| Spare |  |  |  |  |  |  |  |  |
| Length of V2XP info contents |  |  |  |  |  |  |  | octet k+1 octet k+2 |
| Validity timer |  |  |  |  |  |  |  | octet k+3 octet k+TBD |
| VSITPMRI | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet k+TBD+1 |
| Served by E-UTRA or served by NR |  |  |  |  |  |  |  | octet k+TBD+2 octet o1 |
| Not served by E-UTRA and not served by NR |  |  |  |  |  |  |  | octet o1+1 octet o2 |
| V2X service identifier to Tx profiles mapping rules |  |  |  |  |  |  |  | octet (o2+1)* octet o3* |
| Privacy config |  |  |  |  |  |  |  | octet o3+1 octet o4 |
| V2X communication over PC5 in E-UTRA |  |  |  |  |  |  |  | octet o4+1 octet o5 |
| V2X communication over PC5 in NR |  |  |  |  |  |  |  | octet o5+1 octet l |

Figure 5.3.1.1: V2XP Info = {UE policies for V2X communication over PC5}

Table 5.3.1.1: V2XP Info = {UE policies for V2X communication over PC5}

V2XP info type (bit 1 to 4 of octet k) shall be set to "0001" (UE policies for V2X communication over PC5)

Length of Length of V2XP info contents (octets k+1 to k+2) indicates the length of V2XP info contents.

Validity timer:

V2X service identifier to Tx profiles mapping rules indicator (VSITPMRI) The VSITPMRI bit indicates presence of the V2X service identifier to Tx profiles mapping rules field. Bit 8 0 V2X service identifier to Tx profiles mapping rules field is absent 1 V2X service identifier to Tx profiles mapping rules field is present

Served by E-UTRA or served by NR: The served by E-UTRA or served by NR field is coded according to figure 5.3.1.2 and table 5.3.1.2, and contains configuration parameters for V2X communication over PC5 when the UE is served by E-UTRA or served by NR.

Not served by E-UTRA and not served by NR: The not served by E-UTRA and not served by NR field is coded according to figure 5.3.1.6 and table 5.3.1.6, and contains configuration parameters for V2X communication over PC5 when the UE is not served by E-UTRA or NR.

V2X service identifier to Tx profiles mapping rules: The V2X service identifier to Tx profiles mapping rules field is coded according to figure 5.3.1.12 and table 5.3.1.12, and contains a list of V2X service identifier to Tx profiles mapping rules.

Privacy config: The Privacy config field is coded according to figure 5.3.1.15 and table 5.3.1.15, and contains configuration parameters for privacy configuration.

V2X communication over PC5 in E-UTRA: The V2X communication over PC5 in E-UTRA field is coded according to figure 5.3.1.19 and table 5.3.1.19, and contains configuration parameters for V2X communication over PC5 in E-UTRA.

V2X communication over PC5 in NR: The V2X communication over PC5 in NR field is coded according to figure 5.3.1.31 and table 5.3.1.31, and contains configuration parameters for V2X communication over PC5 in NR.

If the length of V2XP info contents field indicates a length bigger than indicated in figure 5.3.1.1, receiving entity shall ignore any superfluous octets located at the end of the V2XP info contents.

Editor's note: exact semantic and length of validity timer field are FFS.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of served by E-UTRA or served by NR contents |  |  |  |  |  |  |  | octet k+TBD+2 octet k+TBD+3 |
| Authorized PLMN and RATs combinations |  |  |  |  |  |  |  | octet k+TBD+4 octet o1 |

Figure 5.3.1.2: Served by E-UTRA or served by NR

Table 5.3.1.2: Served by E-UTRA or served by NR

Authorized PLMN and RATs combinations: The authorized PLMN and RATs combinations field is coded according to figure 5.3.1.3 and table 5.3.1.3.

If the length of served by E-UTRA or served by NR contents field indicates a length bigger than indicated in figure 5.3.1.2, receiving entity shall ignore any superfluous octets located at the end of the served by E-UTRA or served by NR contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of authorized PLMN and RATs combinations contents |  |  |  |  |  |  |  | octet k+TBD+4 octet k+TBD+5 |
| Authorized PLMN and RATs combination 1 |  |  |  |  |  |  |  | octet (k+TBD+6)* octet (k+TBD+9)* |
| Authorized PLMN and RATs combination 2 |  |  |  |  |  |  |  | octet (k+TBD+10)* octet (k+TBD+13)* |
| ... |  |  |  |  |  |  |  | octet (k+TBD+14)* octet (k+TBD+1+n*4)* |
| Authorized PLMN and RATs combination n |  |  |  |  |  |  |  | octet (k+TBD+2+n*4)* octet (k+TBD+5+n*4)* = octet o1* |

Figure 5.3.1.3: Authorized PLMN and RATs combinations

Table 5.3.1.3: Authorized PLMN and RATs combinations

Authorized PLMN and RATs combination: The authorized PLMN and RATs combination field is coded according to figure 5.3.1.4 and table 5.3.1.4.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PLMN ID |  |  |  |  |  |  |  | octet k+TBD+10 octet k+TBD+12 |
| PEIEN | PNIEN | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet k+TBD+13 |

Figure 5.3.1.4: Authorized PLMN and RATs combination

Table 5.3.1.4: Authorized PLMN and RATs combination

PLMN ID: The PLMN ID field is coded according to figure 5.3.1.5 and table 5.3.1.5.

PC5 E-UTRA indicator when served by E-UTRA or served by NR (PEIEN): The PEIEN bit indicates whether the UE is authorized to use V2X communication over PC5 E-UTRA in the PLMN indicated by the PLMN ID field when served by E-UTRA or served by NR. Bit 8 0 Not authorized 1 Authorized

PC5 NR indicator when served by E-UTRA or served by NR (PNIEN): The PNIEN bit indicates whether the UE is authorized to use V2X communication over PC5 NR in the PLMN indicated by the PLMN ID field when served by E-UTRA or served by NR. Bit 7 0 Not authorized 1 Authorized

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet k+TBD+10 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet k+TBD+11 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet k+TBD+12 |

Figure 5.3.1.5: PLMN ID

Table 5.3.1.5: PLMN ID

Mobile country code (MCC): The MCC field is coded as in ITU-T Recommendation E.212 [6], annex A.

Mobile network code (MNC): The coding of MNC field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, MNC digit 3 shall be coded as "1111".

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of not served by E-UTRA and not served by NR contents |  |  |  |  |  |  |  | octet o1+1 octet o1+2 |
| PEINENN | PNINENN | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | VPNENNI | octet o1+3 |
| Radio parameters per geographical area list |  |  |  |  |  |  |  | octet o1+4 octet o2 |

Figure 5.3.1.6: Not served by E-UTRA and not served by NR

Table 5.3.1.6: Not served by E-UTRA and not served by NR

V2X communication over PC5 when not served by E-UTRA and not served by NR indicator (VPNENNI): The VPNENNI bit indicates whether the UE is authorized to use V2X communication over PC5 when not served by E-UTRA and not served by NR. Bit 1 0 Not authorized 1 Authorized

PC5 E-UTRA indicator when not served by E-UTRA and not served by NR (PEINENN): The PEINENN bit indicates whether the UE is authorized to use V2X communication over PC5 E-UTRA when not served by E-UTRA and not served by NR. Bit 8 0 Not authorized 1 Authorized

PC5 NR indicator when not served by E-UTRA and not served by NR (PNINENN): The PNINENN bit indicates whether the UE is authorized to use V2X communication over PC5 NR when not served by E-UTRA and not served by NR. Bit 7 0 Not authorized 1 Authorized

Radio parameters per geographical area list: The radio parameters per geographical area list field is coded according to figure 5.3.1.7 and table 5.3.1.7.

If the length of not served by E-UTRA and not served by NR contents field indicates a length bigger than indicated in figure 5.3.1.6, receiving entity shall ignore any superfluous octets located at the end of the not served by E-UTRA and not served by NR contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters per geographical area list contents |  |  |  |  |  |  |  | octet o1+4 octet o1+5 |
| Radio parameters per geographical area info 1 |  |  |  |  |  |  |  | octet (o1+6)* octet o6* |
| Radio parameters per geographical area info 2 |  |  |  |  |  |  |  | octet (o6+1)* octet o7* |
| ... |  |  |  |  |  |  |  | octet (o7+1)* octet o8* |
| Radio parameters per geographical area info n |  |  |  |  |  |  |  | octet (o8+1)* octet o2* |

Figure 5.3.1.7: Radio parameters per geographical area list

Table 5.3.1.7: Radio parameters per geographical area list

Radio parameters per geographical area info: The radio parameters per geographical area info field is coded according to figure 5.3.1.8 and table 5.3.1.8.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters per geographical area contents |  |  |  |  |  |  |  | octet o6+1 octet o6+2 |
| Geographical area |  |  |  |  |  |  |  | octet o6+3 octet o9 |
| Radio parameters |  |  |  |  |  |  |  | octet o9+1 octet o7-1 |
| MI | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet o7 |

Figure 5.3.1.8: Radio parameters per geographical area info

Table 5.3.1.8: Radio parameters per geographical area info

Geographical area: The geographical area field is coded according to figure 5.3.1.9 and table 5.3.1.9.

Radio parameters: The radio parameters field is coded according to figure 5.3.1.11 and table 5.3.1.11, applicable in the geographical area indicated by the geographical area field when not served by E-UTRA and not served by NR.

Managed indicator (MI): The Managed indicator indicates how the radio parameters indicated in the radio parameters field in the geographical area indicated by the geographical area field are managed. Bit 8 0 Non-operator managed 1 Operator managed

If the length of radio parameters per geographical area contents field indicates a length bigger than indicated in figure 5.3.1.8, receiving entity shall ignore any superfluous octets located at the end of the radio parameters per geographical area contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of geographical area contents |  |  |  |  |  |  |  | octet o6+3 octet o6+4 |
| Coordinate 1 |  |  |  |  |  |  |  | octet (o6+5)* octet (o6+10)* |
| Coordinate 2 |  |  |  |  |  |  |  | octet (o6+11)* octet (o6+16)* |
| ... |  |  |  |  |  |  |  | octet (o6+17)* octet (o6-2+6*n)* |
| Coordinate n |  |  |  |  |  |  |  | octet (o6-1+6*n)* octet (o6+4+6*n)* = octet o9* |

Figure 5.3.1.9: Geographical area

Table 5.3.1.9: Geographical area

Coordinate: The coordinate field is coded according to figure 5.3.1.10 and table 5.3.1.10.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitude |  |  |  |  |  |  |  | octet o6+11 octet o6+13 |
| Longitude |  |  |  |  |  |  |  | octet o6+14 octet o6+17 |

Figure 5.3.1.10: Coordinate area

Table 5.3.1.10: Coordinate area

Latitude: The latitude field is coded according to subclause 6.1 of 3GPP TS 23.032 [7].

Longitude: The longitude field is coded according to subclause 6.1 of 3GPP TS 23.032 [7].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters contents |  |  |  |  |  |  |  | octet o9+1 octet o9+2 |
| Radio parameters contents |  |  |  |  |  |  |  | octet o9+3 octet o7-1 |

Figure 5.3.1.11: Radio parameters

Table 5.3.1.11: Radio parameters

Radio parameters contents:

Editor's notes: radio parameters contents are FFS.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to Tx profiles mapping rules contents |  |  |  |  |  |  |  | octet o2+1 octet o2+2 |
| V2X service identifier to Tx profiles mapping rule 1 |  |  |  |  |  |  |  | octet (o2+3)* octet o10* |
| V2X service identifier to Tx profiles mapping rule 2 |  |  |  |  |  |  |  | octet (o10+1)* octet o11* |
| ... |  |  |  |  |  |  |  | octet (o11+1)* octet o12* |
| V2X service identifier to Tx profiles mapping rule n |  |  |  |  |  |  |  | octet (o12+1)* octet o3* |

Figure 5.3.1.12: V2X service identifier to Tx profiles mapping rules

Table 5.3.1.12: V2X service identifier to Tx profiles mapping rules

V2X service identifier to Tx profiles mapping rule: The V2X service identifier to Tx profiles mapping rule field is coded according to figure 5.3.1.13 and table 5.3.1.13.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to Tx profiles mapping rule contents |  |  |  |  |  |  |  | octet o10+1 octet o10+2 |
| V2X service identifiers |  |  |  |  |  |  |  | octet o10+3 octet o79 |
| Tx profile |  |  |  |  |  |  |  | octet o79 octet (o79 + TBD) = octet o11 |

Figure 5.3.1.13: V2X service identifier to Tx profiles mapping rule

Table 5.3.1.13: V2X service identifier to Tx profiles mapping rules

V2X service identifiers: The V2X service identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14.

Tx profile:

If the length of V2X service identifier to Tx profiles mapping rule contents field indicates a length bigger than indicated in figure 5.3.1.13, receiving entity shall ignore any superfluous octets located at the end of the V2X service identifier to Tx profiles mapping rule contents.

Editor's note: length and coding of Tx profile is FFS. If of variable length, a new length of Tx profile field might need to be introduced.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifiers contents |  |  |  |  |  |  |  | octet o10+3 octet o10+4 |
| V2X service identifier 1 |  |  |  |  |  |  |  | octet (o10+5)* octet (o10+8)* |
| V2X service identifier 2 |  |  |  |  |  |  |  | octet (o10+9)* octet (o10+12)* |
| ... |  |  |  |  |  |  |  | octet (o10+13)* octet (o10+n*4)* |
| V2X service identifier n |  |  |  |  |  |  |  | octet (o10+1+n*4)* octet (o10+4+n*4)* = octet o11-1* |

Figure 5.3.1.14: V2X service identifiers

Table 5.3.1.14: V2X service identifiers

V2X service identifier: The V2X service identifier field contains a binary coded V2X service identifier as specified in ISO TS 17419 ITS-AID AssignedNumbers [5].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of privacy config contents |  |  |  |  |  |  |  | octet o3+1 octet o3+2 |
| V2X services requiring privacy |  |  |  |  |  |  |  | octet o3+3 octet o4-2 |
| Privacy timer |  |  |  |  |  |  |  | octet o4-1 octet o4 |

Figure 5.3.1.15: Privacy config

Table 5.3.1.15: Privacy config

V2X services requiring privacy: The V2X services requiring privacy field is coded according to figure 5.3.1.16 and table 5.3.1.16.

Privacy timer:

The privacy timer field contains binary encoded duration, in units of seconds, after which the UE shall change the source Layer-2 ID and source IP address (for IP data) self-assigned by the UE while performing transmission of V2X communication over the PC5 when privacy is required.

If the length of privacy config contents field indicates a length bigger than indicated in figure 5.3.1.15, receiving entity shall ignore any superfluous octets located at the end of the privacy config contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X services requiring privacy contents |  |  |  |  |  |  |  | octet o3+3 octet o3+4 |
| V2X service requiring privacy 1 |  |  |  |  |  |  |  | octet (o3+5)* octet o12* |
| V2X service requiring privacy 2 |  |  |  |  |  |  |  | octet (o12+1)* octet o13* |
| ... |  |  |  |  |  |  |  | octet (o13+1)* octet o14* |
| V2X service requiring privacy n |  |  |  |  |  |  |  | octet (o14+1)* octet (o4-2)* |

Figure 5.3.1.16: V2X services requiring privacy

Table 5.3.1.16: V2X services requiring privacy

V2X service requiring privacy: The V2X service requiring privacy field is coded according to figure 5.3.1.17 and table 5.3.1.17.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service requiring privacy contents |  |  |  |  |  |  |  |  | octet o12+1 octet o12+2 |  |
| V2X service identifiers |  |  |  |  |  |  |  |  | octet o12+3 octet o15 |  |
| Geographical areas |  |  |  |  |  |  |  |  | octet o15+1 octet o13 |  |

Figure 5.3.1.17: V2X service requiring privacy

Table 5.3.1.17: V2X service requiring privacy

V2X service identifiers: The V2X service identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14.

Geographical areas: The geographical areas field is coded according to figure 5.3.1.18 and table 5.3.1.18.

If the length of V2X service requiring privacy contents field indicates a length bigger than indicated in figure 5.3.1.17, receiving entity shall ignore any superfluous octets located at the end of the V2X service requiring privacy contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of geographical areas contents |  |  |  |  |  |  |  |  | octet o15+1 octet o15+2 |  |
| Geographical area 1 |  |  |  |  |  |  |  |  | octet (o15+3)* octet o23* |  |
| Geographical area 2 |  |  |  |  |  |  |  |  | octet (o23+1)* octet o24* |  |
| ... |  |  |  |  |  |  |  |  | octet (o24+1)* octet o25* |  |
| Geographical area n |  |  |  |  |  |  |  |  | octet (o25+1)* octet o13* |  |

Figure 5.3.1.18: Geographical areas

Table 5.3.1.18: Geographical areas

Geographical area: The geographical area field is coded according to figure 5.3.1.9 and table 5.3.1.9.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X communication over PC5 in E-UTRA contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o4+1 octet o4+2 |  |
| DDL2II |  | VSIEFMRI |  | VSAPI |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | octet o4+3 |  |
| V2X service identifier to destination layer-2 ID mapping rules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o4+4 octet o26 |  |
| PPPP to PDB mapping rules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o26+1 octet o27 |  |
| V2X service identifier to V2X E-UTRA frequency mapping rules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o27+1)* octet o28* |  |
| V2X services authorized for PPPR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o28+1)* octet o29* |  |
| Default destination layer-2 ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o29+1)* octet (o29+3)* = octet o5* |  |

Figure 5.3.1.19: V2X communication over PC5 in E-UTRA

Table 5.3.1.19: V2X communication over PC5 in E-UTRA

Default destination layer-2 ID indicator (DDL2II): The DDL2II bit indicates presence of the default destination layer-2 ID field. Bit 8 0 Default destination layer-2 ID field is absent 1 Default destination layer-2 ID field is present

V2X service identifier to V2X E-UTRA frequency mapping rules indicator (VSIEFMRI): The VSIEFMRI bit indicates presence of the V2X service identifier to V2X E-UTRA frequency mapping rules field. Bit 7 0 V2X service identifier to V2X E-UTRA frequency mapping rules field is absent 1 V2X service identifier to V2X E-UTRA frequency mapping rules field is present

V2X services authorized for PPPR indicator (VSAPI): The VSAPI bit indicates presence of the V2X services authorized for PPPR field. Bit 6 0 V2X services authorized for PPPR field is absent 1 V2X services authorized for PPPR field is present

V2X service identifier to destination layer-2 ID mapping rules: The V2X service identifier to destination layer-2 ID mapping rules field is coded according to figure 5.3.1.20 and table 5.3.1.20.

PPPP to PDB mapping rules: The PPPP to PDB mapping rules field is coded according to figure 5.3.1.22 and table 5.3.1.22.

V2X service identifier to V2X E-UTRA frequency mapping rules: The V2X service identifier to V2X E-UTRA frequency mapping rules field is coded according to figure 5.3.1.24 and table 5.3.1.24.

V2X services authorized for PPPR: The V2X services authorized for PPPR field is coded according to figure 5.3.1.29 and table 5.3.1.29.

Default destination layer-2 ID: The default destination layer-2 ID field is a binary coded layer 2 identifier.

If the length of V2X communication over PC5 in E-UTRA contents field indicates a length bigger than indicated in figure 5.3.1.19, receiving entity shall ignore any superfluous octets located at the end of the V2X communication over PC5 in E-UTRA contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to destination layer-2 ID mapping rules contents |  |  |  |  |  |  |  |  | octet o4+4 octet o4+5 |  |
| V2X service identifier to destination layer-2 ID mapping rule 1 |  |  |  |  |  |  |  |  | octet o4+6* octet o19* |  |
| V2X service identifier to destination layer-2 ID mapping rule 2 |  |  |  |  |  |  |  |  | octet (o19+1)* octet o20* |  |
| ... |  |  |  |  |  |  |  |  | octet (o20+1)* octet o21* |  |
| V2X service identifier to destination layer-2 ID mapping rule n |  |  |  |  |  |  |  |  | octet (o21+1)* octet o26* |  |

Figure 5.3.1.20: V2X service identifier to destination layer-2 ID mapping rules

Table 5.3.1.20: V2X service identifier to destination layer-2 ID mapping rules

V2X service identifier to destination layer-2 ID mapping rule: The V2X service identifier to destination layer-2 ID mapping rule field is coded according to figure 5.3.1.21 and table 5.3.1.21.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to destination layer-2 ID mapping rule contents |  |  |  |  |  |  |  |  | octet o19+1 octet o19+2 |  |
| V2X service identifiers |  |  |  |  |  |  |  |  | octet o19+3 octet o22 |  |
| Destination layer-2 ID |  |  |  |  |  |  |  |  | octet o22+1 octet (o22+3) = octet o20 |  |

Figure 5.3.1.21: V2X service identifier to destination layer-2 ID mapping rule

Table 5.3.1.21: V2X service identifier to destination layer-2 ID mapping rule

V2X service identifiers: The V2X service identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14.

Destination layer-2 ID: The destination layer-2 ID field is a binary coded layer 2 identifier.

If the length of V2X service identifier to destination layer-2 ID mapping rule contents field indicates a length bigger than indicated in figure 5.3.1.21, receiving entity shall ignore any superfluous octets located at the end of the V2X service identifier to destination layer-2 ID mapping rule contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of PPPP to PDB mapping rules contents |  |  |  |  |  |  |  |  | octet o26+1 octet o26+2 |  |
| PPPP to PDB mapping rule 1 |  |  |  |  |  |  |  |  | octet (o26+3)* octet (o26+5)* |  |
| PPPP to PDB mapping rule 2 |  |  |  |  |  |  |  |  | octet (o26+6)* octet (o26+8)* |  |
| ... |  |  |  |  |  |  |  |  | octet (o26+9)* octet (o26+3*n-1)* |  |
| PPPP to PDB mapping rule n |  |  |  |  |  |  |  |  | octet (o26+3*n)* octet (o26+2+3*n)* = octet o27* |  |

Figure 5.3.1.22: PPPP to PDB mapping rules

Table 5.3.1.22: PPPP to PDB mapping rules

PPPP to PDB mapping rule: The PPPP to PDB mapping rule field is coded according to figure 5.3.1.23 and table 5.3.1.23.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | PPPP |  |  |  | octet o26+6 |  |
| PDB |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o26+7 octet o26+8 |  |

Figure 5.3.1.23: PPPP to PDB mapping rule

Table 5.3.1.23: PPPP to PDB mapping rule

ProSe per-packet priority (PPPP): The PPPP field is a ProSe per-packet priority value. Bits 3 2 1 0 0 0 PPPP value 1 0 0 1 PPPP value 2 0 1 0 PPPP value 3 0 1 1 PPPP value 4 1 0 0 PPPP value 5 1 0 1 PPPP value 6 1 1 0 PPPP value 7 1 1 1 PPPP value 8

Packet delay budget (PDB):

The PDB field indicates binary encoded the packet delay budget value in miliseconds to which the ProSe per-packet priority value indicated by the PPPP field is mapped.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to V2X E-UTRA frequency mapping rules contents |  |  |  |  |  |  |  |  | octet o27+1 octet o27+2 |  |
| V2X service identifier to V2X E-UTRA frequency mapping rule 1 |  |  |  |  |  |  |  |  | octet (o27+3)* octet o33* |  |
| V2X service identifier to V2X E-UTRA frequency mapping rule 2 |  |  |  |  |  |  |  |  | octet (o33+1)* octet o34* |  |
| ... |  |  |  |  |  |  |  |  | octet (o34+1)* octet o35* |  |
| V2X service identifier to V2X E-UTRA frequency mapping rule n |  |  |  |  |  |  |  |  | octet (o35+1)* octet o28* |  |

Figure 5.3.1.24: V2X service identifier to V2X E-UTRA frequency mapping rules

Table 5.3.1.24: V2X service identifier to V2X E-UTRA frequency mapping rules

V2X service identifier to V2X E-UTRA frequency mapping rule: The V2X service identifier to V2X E-UTRA frequency mapping rule is coded according to figure 5.3.1.25 and table 5.3.1.25.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to V2X E-UTRA frequency mapping rule contents |  |  |  |  |  |  |  |  | octet o33+1 octet o33+2 |  |
| V2X service identifiers |  |  |  |  |  |  |  |  | octet o33+3 octet o39 |  |
| V2X E-UTRA frequencies with geographical areas list |  |  |  |  |  |  |  |  | octet o39+1 octet o34 |  |

Figure 5.3.1.25: V2X service identifier to V2X E-UTRA frequency mapping rule

Table 5.3.1.25: V2X service identifier to V2X E-UTRA frequency mapping rule

V2X service identifiers: The V2X service identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14.

V2X E-UTRA frequencies with geographical areas list: The V2X E-UTRA frequencies with geographical areas list field is coded according to figure 5.3.1.26 and table 5.3.1.26.

If the length of V2X service identifier to V2X E-UTRA frequency mapping rule contents field indicates a length bigger than indicated in figure 5.3.1.25, receiving entity shall ignore any superfluous octets located at the end of the V2X service identifier to V2X E-UTRA frequency mapping rule contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X E-UTRA frequencies with geographical areas list contents |  |  |  |  |  |  |  |  | octet o39+1 octet o39+2 |  |
| V2X E-UTRA frequencies with geographical areas info 1 |  |  |  |  |  |  |  |  | octet o39+3* octet o40* |  |
| V2X E-UTRA frequencies with geographical areas info 2 |  |  |  |  |  |  |  |  | octet (o40+1)* octet o41* |  |
| ... |  |  |  |  |  |  |  |  | octet (o41+1)* octet o42* |  |
| V2X E-UTRA frequencies with geographical areas info n |  |  |  |  |  |  |  |  | octet (o42+1)* octet o34* |  |

Figure 5.3.1.26: V2X E-UTRA frequencies with geographical areas list

Table 5.3.1.26: V2X E-UTRA frequencies with geographical areas list

V2X E-UTRA frequencies with geographical areas info: The V2X E-UTRA frequencies with geographical areas info field is coded according to figure 5.3.1.27 and table 5.3.1.27.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X E-UTRA frequencies with geographical areas info contents |  |  |  |  |  |  |  |  | octet o40+1 octet o40+2 |  |
| V2X E-UTRA frequencies |  |  |  |  |  |  |  |  | octet o40+3 octet o43 |  |
| Geographical areas |  |  |  |  |  |  |  |  | octet o43+1 octet o41 |  |

Figure 5.3.1.27: V2X E-UTRA frequencies with geographical areas info

Table 5.3.1.27: V2X E-UTRA frequencies with geographical areas info

V2X E-UTRA frequencies: The V2X E-UTRA frequencies field is coded according to figure 5.3.1.28 and table 5.3.1.28.

Geographical areas: The geographical areas field is coded according to figure 5.3.1.18 and table 5.3.1.18.

If the length of V2X E-UTRA frequencies with geographical areas info contents field indicates a length bigger than indicated in figure 5.3.1.27, receiving entity shall ignore any superfluous octets located at the end of the V2X E-UTRA frequencies with geographical areas info contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X E-UTRA frequencies contents |  |  |  |  |  |  |  |  | octet o40+3 octet o40+4 |  |
| V2X E-UTRA frequency 1 |  |  |  |  |  |  |  |  | octet (o40+5)* octet (o40+4+TBD)* |  |
| V2X E-UTRA frequency 2 |  |  |  |  |  |  |  |  | octet (o40+5+TBD)* octet (o40+4+2*TBD)* |  |
| ... |  |  |  |  |  |  |  |  | octet (o40+5+2*TBD)* octet (o40+4+(n-1)*TBD)* |  |
| V2X E-UTRA frequency n |  |  |  |  |  |  |  |  | octet (o40+5+(n-1)*TBD)* octet (o40+4+n*TBD)* = octet o42* |  |

Figure 5.3.1.28: V2X E-UTRA frequencies

Table 5.3.1.28: V2X E-UTRA frequencies

V2X E-UTRA frequency: V2X E-UTRA frequency

Editor's note: length and coding of V2X E-UTRA frequency is FFS.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X services authorized for PPPR contents |  |  |  |  |  |  |  |  | octet o28+1 octet o28+2 |  |
| V2X service authorized for PPPR 1 |  |  |  |  |  |  |  |  | octet (o28+3)* octet o36* |  |
| V2X service authorized for PPPR 2 |  |  |  |  |  |  |  |  | octet (o36+1)* octet o37* |  |
| ... |  |  |  |  |  |  |  |  | octet (o37+1)* octet o38* |  |
| V2X service authorized for PPPR n |  |  |  |  |  |  |  |  | octet (o38+1)* octet o29* |  |

Figure 5.3.1.29: V2X services authorized for PPPR

Table 5.3.1.29: V2X services authorized for PPPR

V2X service authorized for PPPR: The V2X services authorized for PPPR field is coded according to figure 5.3.1.30 and table 5.3.1.30.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service authorized for PPPR contents |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o36+1 octet o36+2 |  |
| V2X service identifiers |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o36+3 octet o37-1 |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | PPPR |  |  |  | octet o37 |  |

Figure 5.3.1.30: V2X service authorized for PPPR

Table 5.3.1.30: V2X service authorized for PPPR

V2X service identifiers: The V2X service identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14.

ProSe per-packet reliability (PPPR): The PPPR field is a ProSe per-packet reliability value. Bits 3 2 1 0 0 0 PPPR value 1 0 0 1 PPPR value 2 0 1 0 PPPR value 3 0 1 1 PPPR value 4 1 0 0 PPPR value 5 1 0 1 PPPR value 6 1 1 0 PPPR value 7 1 1 1 PPPR value 8

If the length of V2X service authorized for PPPR contents field indicates a length bigger than indicated in figure 5.3.1.30, receiving entity shall ignore any superfluous octets located at the end of the V2X service authorized for PPPR contents.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X communication over PC5 in NR contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o5+1 octet o5+2 |  |
| DDL2IBI |  | VSINFMRI |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | octet o5+3 |  |
| V2X service identifier to V2X NR frequency mapping rules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o5+4)* octet o45* |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o45+1 octet o46 |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o46+1 octet o47 |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o47+1 octet o48 |  |
| PC5 QoS mapping rules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o48+1 octet o49 |  |
| AS configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o49+1 octet o50 |  |
| Default destination layer-2 ID for broadcast |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o50+1)* octet (o50+3)* = octet l* |  |

Figure 5.3.1.31: V2X communication over PC5 in NR

Table 5.3.1.31: V2X communication over PC5 in NR

Default destination layer-2 ID for broadcast indicator (DDL2IBI): The DDL2IBI bit indicates presence of the default destination layer-2 ID for broadcast field. Bit 8 0 Default destination layer-2 ID for broadcast field is absent 1 Default destination layer-2 ID for broadcast field is present

V2X service identifier to V2X NR frequency mapping rules indicator (VSINFMRI): The VSINFMRI bit indicates presence of the V2X service identifier to V2X NR frequency mapping rules field. Bit 7 0 V2X service identifier to V2X NR frequency mapping rules field is absent 1 V2X service identifier to V2X NR frequency mapping rules field is present

V2X service identifier to V2X NR frequency mapping rules: The V2X service identifier to V2X NR frequency mapping rules field is coded according to figure 5.3.1.32 and table 5.3.1.32.

V2X service identifier to destination layer-2 ID for broadcast mapping rules: The V2X service identifier to destination layer-2 ID for broadcast mapping rules field is coded according to figure 5.3.1.37 and table 5.3.1.37.

V2X service identifier to destination layer-2 ID for groupcast mapping rules: The V2X service identifier to destination layer-2 ID for groupcast mapping rules field is coded according to figure 5.3.1.39 and table 5.3.1.39.

V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules: The V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules field is coded according to figure 5.3.1.41 and table 5.3.1.41.

PC5 QoS mapping rules: The PC5 QoS mapping rules field is coded according to figure 5.3.1.43 and table 5.3.1.43.

AS configuration: The AS configuration field is coded according to figure 5.3.1.46a and table 5.3.1.46a.

Default destination layer-2 ID for broadcast: The default destination layer-2 ID for broadcast field is a binary coded layer 2 identifier.

If the length of V2X communication over PC5 in NR contents field indicates a length bigger than indicated in figure 5.3.1.31, receiving entity shall ignore any superfluous octets located at the end of the V2X communication over PC5 in NR contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to V2X NR frequency mapping rules contents |  |  |  |  |  |  |  |  | octet o5+4 octet o5+5 |  |
| V2X service identifier to V2X NR frequency mapping rule 1 |  |  |  |  |  |  |  |  | octet (o5+6)* octet o51* |  |
| V2X service identifier to V2X NR frequency mapping rule 2 |  |  |  |  |  |  |  |  | octet (o51+1)* octet o52* |  |
| ... |  |  |  |  |  |  |  |  | octet (o52+1)* octet o53* |  |
| V2X service identifier to V2X NR frequency mapping rule n |  |  |  |  |  |  |  |  | octet (o53+1)* octet o45* |  |

Figure 5.3.1.32: V2X service identifier to V2X NR frequency mapping rules

Table 5.3.1.32: V2X service identifier to V2X NR frequency mapping rules

V2X service identifier to V2X NR frequency mapping rule: The V2X service identifier to V2X NR frequency mapping rule is coded according to figure 5.3.1.33 and table 5.3.1.33.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to V2X NR frequency mapping rule contents |  |  |  |  |  |  |  |  | octet o51+1 octet o51+2 |  |
| V2X service identifiers |  |  |  |  |  |  |  |  | octet o51+3 octet o54 |  |
| V2X NR frequencies with geographical areas list |  |  |  |  |  |  |  |  | octet o54+1 octet o52 |  |

Figure 5.3.1.33: V2X service identifier to V2X NR frequency mapping rule

Table 5.3.1.33: V2X service identifier to V2X NR frequency mapping rule

V2X service identifiers: The V2X service identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14.

V2X NR frequencies with geographical areas list: The V2X NR frequencies with geographical areas list field is coded according to figure 5.3.1.34 and table 5.3.1.34.

If the length of V2X service identifier to V2X NR frequency mapping rule contents field indicates a length bigger than indicated in figure 5.3.1.33, receiving entity shall ignore any superfluous octets located at the end of the V2X service identifier to V2X NR frequency mapping rule contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X NR frequencies with geographical areas list contents |  |  |  |  |  |  |  |  | octet o54+1 octet o54+2 |  |
| V2X NR frequencies with geographical areas info 1 |  |  |  |  |  |  |  |  | octet (o54+3)* octet o55* |  |
| V2X NR frequencies with geographical areas info 2 |  |  |  |  |  |  |  |  | octet (o55+1)* octet o56* |  |
| ... |  |  |  |  |  |  |  |  | octet (o56+1)* octet o57* |  |
| V2X NR frequencies with geographical areas info n |  |  |  |  |  |  |  |  | octet (o57+1)* octet o52* |  |

Figure 5.3.1.34: V2X NR frequencies with geographical areas list

Table 5.3.1.34: V2X NR frequencies with geographical areas list

V2X NR frequencies with geographical areas info: The V2X NR frequencies with geographical areas info field is coded according to figure 5.3.1.35 and table 5.3.1.35.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X NR frequencies with geographical areas info contents |  |  |  |  |  |  |  |  | octet o55+1 octet o55+2 |  |
| V2X NR frequencies |  |  |  |  |  |  |  |  | octet o55+3 octet o58 |  |
| Geographical areas |  |  |  |  |  |  |  |  | octet o58+1 octet o56 |  |

Figure 5.3.1.35: V2X NR frequencies with geographical areas info

Table 5.3.1.35: V2X NR frequencies with geographical areas info

V2X NR frequencies: The V2X NR frequencies field is coded according to figure 5.3.1.36 and table 5.3.1.36.

Geographical areas: The geographical areas field is coded according to figure 5.3.1.18 and table 5.3.1.18.

If the length of V2X NR frequencies with geographical areas info contents field indicates a length bigger than indicated in figure 5.3.1.35, receiving entity shall ignore any superfluous octets located at the end of the V2X NR frequencies with geographical areas info contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X NR frequencies contents |  |  |  |  |  |  |  |  | octet o55+3 octet o55+4 |  |
| V2X NR frequency 1 |  |  |  |  |  |  |  |  | octet (o55+5)* octet (o55+4+TBD)* |  |
| V2X NR frequency 2 |  |  |  |  |  |  |  |  | octet (o55+5+TBD)* octet (o55+4+2*TBD)* |  |
| ... |  |  |  |  |  |  |  |  | octet (o55+5+2*TBD)* octet (o55+4+(n-1)*TBD)* |  |
| V2X NR frequency n |  |  |  |  |  |  |  |  | octet (o55+5+(n-1)*TBD)* octet (o55+4+n*TBD)* = octet o58* |  |

Figure 5.3.1.36: V2X NR frequencies

Table 5.3.1.36: V2X NR frequencies

V2X NR frequency: V2X NR frequency

Editor's note: length and coding of V2X NR frequency is FFS.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to destination layer-2 ID for broadcast mapping rules contents |  |  |  |  |  |  |  |  | octet o45+1 octet o45+2 |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rule 1 |  |  |  |  |  |  |  |  | octet (o45+3)* octet o59* |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rule 2 |  |  |  |  |  |  |  |  | octet (o59+1)* octet o60* |  |
| ... |  |  |  |  |  |  |  |  | octet (o60+1)* octet o61* |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rule n |  |  |  |  |  |  |  |  | octet (o61+1)* octet o46* |  |

Figure 5.3.1.37: V2X service identifier to destination layer-2 ID for broadcast mapping rules

Table 5.3.1.37: V2X service identifier to destination layer-2 ID for broadcast mapping rules

V2X service identifier to destination layer-2 ID for broadcast mapping rule: The V2X service identifier to destination layer-2 ID for broadcast mapping rule field is coded according to figure 5.3.1.38 and table 5.3.1.38.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to destination layer-2 ID for broadcast mapping rule contents |  |  |  |  |  |  |  |  | octet o59+1 octet o59+2 |  |
| V2X service identifiers |  |  |  |  |  |  |  |  | octet o59+3 octet o62 |  |
| Destination layer-2 ID for broadcast |  |  |  |  |  |  |  |  | octet o62+1 octet (o62+3) = octet o60 |  |

Figure 5.3.1.38: V2X service identifier to destination layer-2 ID for broadcast mapping rule

Table 5.3.1.38: V2X service identifier to destination layer-2 ID for broadcast mapping rule

V2X service identifiers: The V2X service identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14.

Destination layer-2 ID for broadcast: The destination layer-2 ID for broadcast field is a binary coded layer 2 identifier.

If the length of V2X service identifier to destination layer-2 ID for broadcast mapping rule contents field indicates a length bigger than indicated in figure 5.3.1.38, receiving entity shall ignore any superfluous octets located at the end of the V2X service identifier to destination layer-2 ID for broadcast mapping rule contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to destination layer-2 ID for groupcast mapping rules contents |  |  |  |  |  |  |  |  | octet o46+1 octet o46+2 |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rule 1 |  |  |  |  |  |  |  |  | octet (o46+3)* octet o63* |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rule 2 |  |  |  |  |  |  |  |  | octet (o63+1)* octet o64* |  |
| ... |  |  |  |  |  |  |  |  | octet (o64+1)* octet o65* |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rule n |  |  |  |  |  |  |  |  | octet (o65+1)* octet o47* |  |

Figure 5.3.1.39: V2X service identifier to destination layer-2 ID for groupcast mapping rules

Table 5.3.1.39: V2X service identifier to destination layer-2 ID for groupcast mapping rules

V2X service identifier to destination layer-2 ID for groupcast mapping rule: The V2X service identifier to destination layer-2 ID for groupcast mapping rule field is coded according to figure 5.3.1.40 and table 5.3.1.40.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to destination layer-2 ID for groupcast mapping rule contents |  |  |  |  |  |  |  |  | octet o63+1 octet o63+2 |  |
| V2X service identifiers |  |  |  |  |  |  |  |  | octet o63+3 octet o80 |  |
| Destination layer-2 ID for groupcast |  |  |  |  |  |  |  |  | octet o80+1 octet (o80+3) = octet o64 |  |

Figure 5.3.1.40: V2X service identifier to destination layer-2 ID for groupcast mapping rule

Table 5.3.1.40: V2X service identifier to destination layer-2 ID for groupcast mapping rule

V2X service identifiers: The V2X service identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14.

Destination layer-2 ID for groupcast: The destination layer-2 ID for groupcast field is a binary coded layer 2 identifier.

If the length of V2X service identifier to destination layer-2 ID for groupcast mapping rule contents field indicates a length bigger than indicated in figure 5.3.1.40, receiving entity shall ignore any superfluous octets located at the end of the V2X service identifier to destination layer-2 ID for groupcast mapping rule contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules contents |  |  |  |  |  |  |  |  | octet o47+1 octet o47+2 |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule 1 |  |  |  |  |  |  |  |  | octet (o47+3)* octet o66* |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule 2 |  |  |  |  |  |  |  |  | octet (o66+1)* octet o67* |  |
| ... |  |  |  |  |  |  |  |  | octet (o67+1)* octet o68* |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule n |  |  |  |  |  |  |  |  | octet (o68+1)* octet o48* |  |

Figure 5.3.1.41: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules

Table 5.3.1.41: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules

V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule: The V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule field is coded according to figure 5.3.1.42 and table 5.3.1.42.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule contents |  |  |  |  |  |  |  |  | octet o66+1 octet o66+2 |  |
| V2X service identifiers |  |  |  |  |  |  |  |  | octet o66+3 octet o81 |  |
| Destination layer-2 ID for unicast initial signalling |  |  |  |  |  |  |  |  | octet o81+1 octet (o81+3) = octet o67 |  |

Figure 5.3.1.42: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule

Table 5.3.1.42: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule

V2X service identifiers: The V2X service identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14.

Destination layer-2 ID for unicast initial signalling: The destination layer-2 ID for unicast initial signalling field is a binary coded layer 2 identifier.

If the length of V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule contents field indicates a length bigger than indicated in figure 5.3.1.42, receiving entity shall ignore any superfluous octets located at the end of the V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of PC5 QoS mapping rules contents |  |  |  |  |  |  |  |  | octet o48+1 octet o48+2 |  |
| PC5 QoS mapping rule 1 |  |  |  |  |  |  |  |  | octet (o48+3)* octet o70* |  |
| PC5 QoS mapping rule 2 |  |  |  |  |  |  |  |  | octet (o70+1)* octet o71* |  |
| ... |  |  |  |  |  |  |  |  | octet (o71+1)* octet o72* |  |
| PC5 QoS mapping rule n |  |  |  |  |  |  |  |  | octet (o72+1)* octet o49* |  |

Figure 5.3.1.43: PC5 QoS mapping rules

Table 5.3.1.43: PC5 QoS mapping rules

PC5 QoS mapping rule: The PC5 QoS mapping rule field is coded according to figure 5.3.1.44 and table 5.3.1.44.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of PC5 QoS mapping rule contents |  |  |  |  |  |  |  |  | octet o70+1 octet o70+2 |  |
| PC5 QoS mapping rule inputs |  |  |  |  |  |  |  |  | octet o70+3 octet o73 |  |
| PC5 QoS mapping rule outputs |  |  |  |  |  |  |  |  | octet o73+1 octet o71 |  |

Figure 5.3.1.44: PC5 QoS mapping rule

Table 5.3.1.44: PC5 QoS mapping rule

PC5 QoS mapping rule inputs: The PC5 QoS mapping rule inputs field is coded according to figure 5.3.1.45 and table 5.3.1.45.

PC5 QoS mapping rule outputs: The PC5 QoS mapping rule outputs field is coded according to figure 5.3.1.46 and table 5.3.1.46.

If the length of PC5 QoS mapping rule contents field indicates a length bigger than indicated in figure 5.3.1.44, receiving entity shall ignore any superfluous octets located at the end of the PC5 QoS mapping rule contents.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of PC5 QoS mapping rule inputs contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o70+3 octet o70+4 |  |
| VARVSI |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | octet o70+5 |  |
| V2X service identifiers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o70+6 octet o74 |  |
| Length of V2X application requirements for V2X service |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o74+1 octet o74+2 |  |
| V2X application requirements for V2X service |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o74+3 octet o73 |  |

Figure 5.3.1.45: PC5 QoS mapping rule inputs

Table 5.3.1.45: PC5 QoS mapping rule inputs

V2X application requirements for V2X service indicator (VARVSI): The VARVSI bit indicates presence of the length of V2X application requirements for V2X service field and the V2X application requirements for V2X service field. Bit 8 0 length of V2X application requirements for V2X service field and V2X application requirements for V2X service field are absent 1 length of V2X application requirements for V2X service field and V2X application requirements for V2X service field are present

V2X service identifiers: The V2X service identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14.

Length of V2X application requirements for V2X service: The length of V2X application requirements for V2X service field indicates length of the V2X application requirements for V2X service field in octets.

V2X application requirements for V2X service: Coding of the V2X application requirements for V2X service is out of scope of the present specification.

If the length of PC5 QoS mapping rule inputs contents field indicates a length bigger than indicated in figure 5.3.1.45, receiving entity shall ignore any superfluous octets located at the end of the PC5 QoS mapping rule inputs contents.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of PC5 QoS mapping rule outputs contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o73+1 octet o73+2 |  |
| GFBRI |  | MFBRI |  | PLAMBRI |  | RI |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | octet o73+3 |  |
| PQI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o70+6 |  |
| Guaranteed flow bit rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o70+7)* octet (o70+9)* |  |
| Maximum flow bit rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o70+10)* octet (o70+12)* |  |
| Per-link aggregate maximum bit rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o70+13)* octet (o70+15)* |  |
| Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o70+16)* octet (o70+17)* = octet o71* |  |

Figure 5.3.1.46: PC5 QoS mapping rule outputs

Table 5.3.1.46: PC5 QoS mapping rule outputs

Guaranteed flow bit rate indicator (GFBRI): The GFBRI bit indicates presence of guaranteed flow bit rate field. Bit 8 0 Guaranteed flow bit rate field is absent 1 Guaranteed flow bit rate field is present

Maximum flow bit rate indicator (MFBRI): The MFBRI bit indicates presence of maximum flow bit rate field. Bit 7 0 Maximum flow bit rate field is absent 1 Maximum flow bit rate field is present

Per-link aggregate maximum bit rate indicator (PLAMBRI): The PLAMBRI bit indicates presence of per-link aggregate maximum bit rate field. Bit 6 0 Per-link aggregate maximum bit rate field is absent 1 Per-link aggregate maximum bit rate field is present

Range indicator (RI): The RI bit indicates presence of range field. Bit 5 0 Range field is absent 1 Range field is present

PQI: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 to Spare 0 0 0 1 0 1 0 0 0 0 0 1 0 1 0 1 PQI 21 0 0 0 1 0 1 1 0 PQI 22 0 0 0 1 0 1 1 1 PQI 23 0 0 0 1 1 0 0 0 to Spare 0 0 1 1 0 1 1 0 0 0 1 1 0 1 1 1 PQI 55 0 0 1 1 1 0 0 0 PQI 56 0 0 1 1 1 0 0 1 PQI 57 0 0 1 1 1 0 1 0 PQI 58 0 0 1 1 1 0 1 1 PQI 59 0 0 1 1 1 1 0 0 to Spare 0 1 0 1 1 0 0 1 0 1 0 1 1 0 1 0 PQI 90 0 1 0 1 1 0 1 1 PQI 91 0 1 0 1 1 1 0 0 to Spare 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 to Operator-specific PQIs 1 1 1 1 1 1 1 0 1 1 1 1 1 1 1 1 Reserved If the UE receives a PQI value (excluding the reserved PQI values) that it does not understand, the UE shall choose a PQI value from the set of PQI values defined in this version of the protocol (see 3GPP TS 23.287 [2]) and associated with: - GBR resource type, if the PC5 QoS mapping rule outputs include the guaranteed flow bit rate field; and - non-GBR resource type, if the PC5 QoS mapping rule outputs do not include the guaranteed flow bit rate field. The UE shall use this chosen PQI value for internal operations only. The UE shall use the received PQI value in subsequent V2X communication over PC5 signalling procedures.

Guaranteed flow bit rate: The guaranteed flow bit rate field indicates guaranteed flow bit rate for both sending and receiving and contains one octet indicating the unit of the guaranteed flow bit rate followed by two octets containing the value of the guaranteed flow bit rate. Unit of the guaranteed flow bit rate: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 Kbps 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Value of the guaranteed flow bit rate is binary coded value of the guaranteed flow bit rate in units defined by the unit of the guaranteed flow bit rate.

Maximum flow bit rate: The maximum flow bit rate field indicates maximum flow bit rate for both sending and receiving and contains one octet indicating the unit of the maximum flow bit rate followed by two octets containing the value of the maximum flow bit rate. Unit of the maximum flow bit rate: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 Kbps 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Value of the maximum flow bit rate is binary coded value of the maximum flow bit rate in units defined by the unit of the maximum flow bit rate.

Per-link aggregate maximum bit rate: The per-link aggregate maximum bit rate field indicates per-link aggregate maximum bit rate for both sending and receiving and contains one octet indicating the unit of the per-link aggregate maximum bit rate followed by two octets containing the value of the per-link aggregate maximum bit rate. Unit of the per-link aggregate maximum bit rate: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 Kbps 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Value of the per-link aggregate maximum bit rate is binary coded value of the per-link aggregate maximum bit rate in units defined by the unit of the per-link aggregate maximum bit rate.

Range The range field indicates a binary encoded value of the range in meters.

If the length of PC5 QoS mapping rule outputs contents field indicates a length bigger than indicated in figure 5.3.1.46, receiving entity shall ignore any superfluous octets located at the end of the PC5 QoS mapping rule outputs contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of AS configuration contents |  |  |  |  |  |  |  |  | octet o49+1 octet o49+2 |  |
| SLRB mapping rules |  |  |  |  |  |  |  |  | octet o49+3 octet o50 |  |

Figure 5.3.1.46a: AS configuration

Table 5.3.1.46a: AS configuration

SLRB mapping rules: The SLRB mapping rules field is coded according to figure 5.3.1.47 and table 5.3.1.47.

If the length of AS configuration contents field indicates a length bigger than indicated in figure 5.3.1.46a, receiving entity shall ignore any superfluous octets located at the end of the AS configuration contents.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of SLRB mapping rules contents |  |  |  |  |  |  |  |  | octet o49+3 octet o49+4 |  |
| SLRB mapping rule 1 |  |  |  |  |  |  |  |  | octet (o49+5)* octet o75* |  |
| SLRB mapping rule 2 |  |  |  |  |  |  |  |  | octet (o75+1)* octet o76* |  |
| ... |  |  |  |  |  |  |  |  | octet (o76+1)* octet o77* |  |
| SLRB mapping rule n |  |  |  |  |  |  |  |  | octet (o77+1)* octet o50* |  |

Figure 5.3.1.47: SLRB mapping rules

Table 5.3.1.47: SLRB mapping rules

SLRB mapping rule: The SLRB mapping rule field is coded according to figure 5.3.1.48 and table 5.3.1.48.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of SLRB mapping rule contents |  |  |  |  |  |  |  |  | octet o75+1 octet o75+2 |  |
| PC5 QoS profile |  |  |  |  |  |  |  |  | octet o75+3 octet o78 |  |
| SLRB |  |  |  |  |  |  |  |  | octet o78+1 octet o76 |  |

Figure 5.3.1.48: SLRB mapping rule

Table 5.3.1.48: SLRB mapping rule

PC5 QoS profile: The PC5 QoS profile field is coded according to figure 5.3.1.49 and table 5.3.1.49.

SLRB

SLRB

If the length of SLRB mapping rule contents field indicates a length bigger than indicated in figure 5.3.1.48, receiving entity shall ignore any superfluous octets located at the end of the SLRB mapping rule contents.

Editor's note: length and coding of SLRB is FFS. If of variable length, a new length of SLRB field might need to be introduced.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of PC5 QoS profile contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o75+3 octet o75+4 |  |
| GFBRI |  | MFBRI |  | PLAMBRI |  | RI |  | PLOI |  | AWI |  | MDBVI |  | 0 Spare |  | octet o73+5 |  |
| PQI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet o75+6 |  |
| Guaranteed flow bit rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o75+7)* octet (o75+9)* |  |
| Maximum flow bit rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o75+10)* octet (o75+12)* |  |
| Per-link aggregate maximum bit rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o75+13)* octet (o75+15)* |  |
| Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o75+7)* octet (o75+8)* |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | Priority level |  |  |  |  |  | octet (o75+9)* |  |
| Averaging window |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o75+10)* octet (o75+11)* |  |
| Maximum data burst volume |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet (o75+12)* octet (o75+13)* = octet o78* |  |

Figure 5.3.1.49:PC5 QoS profile

Table 5.3.1.49:PC5 QoS profile

Guaranteed flow bit rate indicator (GFBRI): The GFBRI bit indicates presence of guaranteed flow bit rate field. Bit 8 0 Guaranteed flow bit rate field is absent 1 Guaranteed flow bit rate field is present

Maximum flow bit rate indicator (MFBRI): The MFBRI bit indicates presence of maximum flow bit rate field. Bit 7 0 Maximum flow bit rate field is absent 1 Maximum flow bit rate field is present

Per-link aggregate maximum bit rate indicator (PLAMBRI): The PLAMBRI bit indicates presence of per-link aggregate maximum bit rate field. Bit 6 0 Per-link aggregate maximum bit rate field is absent 1 Per-link aggregate maximum bit rate field is present

Range indicator (RI): The RI bit indicates presence of range field. Bit 5 0 Range field is absent 1 Range field is present

Priority level octet indicator (OPLI): The OPLI bit indicates presence of the octet of the priority level field. Bit 4 0 The octet of the priority level is absent 1 The octet of the priority level is present

Averaging window indicator (AWI): The AWI bit indicates presence of averaging window field. Bit 3 0 Averaging window field is absent 1 Averaging window field is present

Maximum data burst volume indicator (MDBVI): The MDBVI bit indicates presence of maximum data burst volume field. Bit 2 0 Maximum data burst volume field is absent 1 Maximum data burst volume field is present

PQI: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 to Spare 0 0 0 1 0 1 0 0 0 0 0 1 0 1 0 1 PQI 21 0 0 0 1 0 1 1 0 PQI 22 0 0 0 1 0 1 1 1 PQI 23 0 0 0 1 1 0 0 0 to Spare 0 0 1 1 0 1 1 0 0 0 1 1 0 1 1 1 PQI 55 0 0 1 1 1 0 0 0 PQI 56 0 0 1 1 1 0 0 1 PQI 57 0 0 1 1 1 0 1 0 PQI 58 0 0 1 1 1 0 1 1 PQI 59 0 0 1 1 1 1 0 0 to Spare 0 1 0 1 1 0 0 1 0 1 0 1 1 0 1 0 PQI 90 0 1 0 1 1 0 1 1 PQI 91 0 1 0 1 1 1 0 0 to Spare 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 to Operator-specific PQIs 1 1 1 1 1 1 1 0 1 1 1 1 1 1 1 1 Reserved If the UE receives a PQI value (excluding the reserved PQI values) that it does not understand, the UE shall choose a PQI value from the set of PQI values defined in this version of the protocol (see 3GPP TS 23.287 [2]) and associated with: - GBR resource type, if the PC5 QoS mapping rule outputs include the guaranteed flow bit rate field; and - non-GBR resource type, if the PC5 QoS mapping rule outputs do not include the guaranteed flow bit rate field. The UE shall use this chosen PQI value for internal operations only. The UE shall use the received PQI value in subsequent V2X communication over PC5 signalling procedures.

Guaranteed flow bit rate: The guaranteed flow bit rate field indicates guaranteed flow bit rate for both sending and receiving and contains one octet indicating the unit of the guaranteed flow bit rate followed by two octets containing the value of the guaranteed flow bit rate. Unit of the guaranteed flow bit rate: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 Kbps 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Value of the guaranteed flow bit rate is binary coded value of the guaranteed flow bit rate in units defined by the unit of the guaranteed flow bit rate.

Maximum flow bit rate: The maximum flow bit rate field indicates maximum flow bit rate for both sending and receiving and contains one octet indicating the unit of the maximum flow bit rate followed by two octets containing the value of the maximum flow bit rate. Unit of the maximum flow bit rate: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 Kbps 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Value of the maximum flow bit rate is binary coded value of the maximum flow bit rate in units defined by the unit of the maximum flow bit rate.

Per-link aggregate maximum bit rate: The per-link aggregate maximum bit rate field indicates per-link aggregate maximum bit rate for both sending and receiving and contains one octet indicating the unit of the per-link aggregate maximum bit rate followed by two octets containing the value of the per-link aggregate maximum bit rate. Unit of the per-link aggregate maximum bit rate: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 Kbps 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Value of the per-link aggregate maximum bit rate is binary coded value of the per-link aggregate maximum bit rate in units defined by the unit of the per-link aggregate maximum bit rate.

Range: The range field indicates a binary encoded value of the range in meters.

Priority level: The Priority level field contains a ProSe per-packet priority value. Bits 3 2 1 0 0 0 PPPP value 1 0 0 1 PPPP value 2 0 1 0 PPPP value 3 0 1 1 PPPP value 4 1 0 0 PPPP value 5 1 0 1 PPPP value 6 1 1 0 PPPP value 7 1 1 1 PPPP value 8

Averaging window: The averaging window field indicates a binary representation of the averaging window for both sending and receiving in milliseconds.

Maximum data burst volume: The maximum data burst volume field indicates a binary representation of the maximum data burst volume for both sending and receiving in octets.

If the length of PC5 QoS profile contents field indicates a length bigger than indicated in figure 5.3.1.49, receiving entity shall ignore any superfluous octets located at the end of the PC5 QoS profile contents.

***** End change *****