3GPP TSG-CT Meeting #88e	CP-201184

# E-Meeting, 29th June – 1st July 2020	(was C4-203623)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.505 | CR | 0287 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of SMSoIP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | UDICOM |  |  |  |  |  | Date: |  |  | 2020-05-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the scenarios introduced by CR 0019 to TS 23.632, the UDM needs to store in UDR the address of the IP-SM-GW handling SMS for a given UE, and the Message Waiting Data list. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define resources and service operations in Nudr_DataRepository (SubscriptionData) service to support storage of an IP-SM-GW and the MWD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UDICOM scenarios described in CR 0019 to TS 23.632 (support of SMSoIP in 5G-only deployments) service cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.xx (new), 5.2.yy (new), 5.4.1, 5.4.2.22, 5.4.2.xx (new), 5.4.2.yy (new), 5.4.3.6, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.632 CR 0019 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible changes on the TS29505_Subscription_Data.yaml OpenAPI specification, with impact to the following APIs: - TS29504_Data_Repository.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 2 (to CT#88e): - Use correct TS baseline in the OpenAPI (Annex A.2) definition of "ContextDataSets" - Correction of $ref to IpSmGwRegistration |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.2.1	Overview

# Figure 5.2.1-1: Resource URI sub-level structure for subscription data

# Figure 5.2.1-2: Resource URI sub-level structure for subscription data (cont.)

Table 5.2.1-1 provides an overview of the resources and applicable HTTP methods.

# Table 5.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| AuthenticationSubscription | /subscription-data/{ueId}/authentication-data/authentication-subscription | GET | Retrieve a UE's authentication subscription data |
|  |  | PATCH | Update a UE's authentication subscription data Updates shall be limited to the sequenceNumber attribute. Attempts to patch any other attribute shall be rejected by the UDR. |
| AuthenticationSoR | /subscription-data/{ueId}/ue-update-confirmation-data/sor-data | PUT | Store UE's SOR acknowledgement information (SoR-XMAC-IUE) |
|  |  | GET | Retrieve UE's SoR acknowledgement information |
| AuthenticationUPU | /subscription-data/{ueId}/ue-update-confirmation-data/upu-data | PUT | Store UE Parameter Update acknowledgement information (UPU-XMAC-IUE) |
|  |  | GET | Retrieve UE Parameter Update acknowledgement information |
| SubscribedSNSSAIs | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-snssais | PUT | Store UE-acknowledgement info for change of subscribed S-NSSAIs |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed S-NSSAIs |
| SubscribedCAG | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-cag | PUT | Store UE-acknowledgement info for change of subscribed CAG |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed CAG |
| AuthenticationStatus | /subscription-data/{ueId}/authentication-data/authentication-status | PUT | Store a UE's authentication status |
|  |  | GET | Retrieve a UE's authentication status |
| ProvisionedData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data | GET | Retrieve the UE's subscribed Provisioned Data |
| AccessAndMobilitySubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data /am-data | GET | Retrieve the UE's subscribed Access and Mobility Data |
| SmfSelectionSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/smf-selection-subscription-data | GET | Retrieve the UE's subscribed SMF Selection Data |
| SessionManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sm-data | GET | Retrieve the UE's subscribed SM Subscription Data |
| ContextData | /subscription-data/{ueId}/context-data | GET | Retrieve the UE's context Data |
| Amf3GppAccessRegistration | /subscription-data/{ueId}/context-data/amf-3gpp-access | PUT | Create and Update the AMF registration for 3GPP access |
|  |  | PATCH | Modify the AMF registration for 3GPP access |
|  |  | GET | Retrieve the AMF registration information for 3GPP access |
| AmfNon3GppAccessRegistration | /subscription-data/{ueId}/ context-data/amf-non-3gpp-access | PUT | Update the AMF registration for non 3GPP access |
|  |  | PATCH | Modify the AMF registration for non 3GPP access |
|  |  | GET | Retrieve the AMF registration information for non 3GPP access |
| SmfRegistrations | /subscription-data/{ueId}/context-data/smf-registrations | GET | Retrieve the list of the SMF registrations |
| IndividualSmfRegistration | /subscription-data/{ueId}/context-data /smf-registrations/{pduSessionId} | PUT | Store an individual SMF registration identified by PDU Session Id |
|  |  | DELETE | Delete an individual SMF registration |
|  |  | GET | Retrieve individual SMF registration |
| OperatorSpecificData | /subscription-data/{ueId}/operator-specific-data | GET | retrieve the operator specific subscription data of a UE |
|  |  | PATCH | modify the operator specific subscription data of a UE |
| OperatorDeterminedBarringData | /subscription-data/{ueId}/operator-determined-barring-data | GET | Retrieve the UE's Operator Determined Barring |
| SMSManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-mng-data | GET | Retrieve the UE's subscribed SMS management subscription data. |
| Smsf3GppAccessRegistration | /subscription-data/{ueId}/context-data /smsf-3gpp-access | PUT | Create or Update the SMSF registration |
|  |  | DELETE | Delete the SMSF registration for 3GPP access |
|  |  | GET | Retrieve the SMSF registration information |
| SmsfNon3GppAccessRegistration | /subscription-data/{ueId}/context-data /smsf-non-3gpp-access | PUT | Create or Update the SMSF registration for non 3GPP access |
|  |  | DELETE | Delete the SMSF registration for non 3GPP access |
|  |  | GET | Retrieve the SMSF registration information for non 3GPP access |
| IpSmGwRegistration | /subscription-data/{ueId}/context-data/ip-sm-gw | PUT | Create or Update the IP-SM-GW registration |
|  |  | DELETE | Delete the IP-SM-GW registration |
|  |  | PATCH | Modify the IP-SM-GW registration |
|  |  | GET | Retrieve the IP-SM-GW registration information |
| MessageWaitingData | /subscription-data/{ueId}/context-data/mwd | PUT | Create or Update the SMS Message Waiting Data |
|  |  | DELETE | Delete the SMS Message Waiting Data |
|  |  | PATCH | Modify the SMS Message Waiting Data |
|  |  | GET | Retrieve the SMS Message Waiting Data |
| SdmSubscriptions | /subscription-data/{ueId}/context-data/sdm-subscriptions | GET | Retrieve SDM subscriptions |
|  |  | POST | Create an individual SDM subscription |
| IndividualSdmSubscription | /subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId} | PUT | Update an individual SDM subscription |
|  |  | DELETE | Delete an individual SDM subscription |
|  |  | PATCH | Update an individual SDM Subscription |
| EeSubscriptions | /subscription-data/{ueId}/context-data/ee-subscriptions | GET | Retrieve EE subscriptions |
|  |  | POST | Create an EE subscription |
| IndividualEeSubscription | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription |
|  |  | DELETE | Delete an individual EE subscription |
|  |  | PATCH | Update an individual EE subscription |
| AmfSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/amf-subscriptions | PUT | Store information related the a received Amf-EE-Subscription response |
|  |  | DELETE | Delete the Amf-EE-subscriptions |
|  |  | GET | Retrieve AMF-subscriptions |
|  |  | PATCH | Update AMF-subscriptions |
| EeProfileData | /subscription-data/{ueId}/ ee-profile-data | GET | Retrieve the UE's subscribed EE profile data. |
| ProvisionedParamenterData | /subscription-data/{ueId}/pp-data | PATCH | Update of provisioned parameters. |
|  |  | GET | Retrieves the UE's provisioned parameters. |
| SMSSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-data | GET | Retrieve the UE's subscribed SMS subscription data. |
| SubscriptionDataSubscriptions | /subscription-data/subs-to-notify | GET | Retrieve existing subscriptions |
|  |  | POST | Create a subscription, i.e. subscribe a node to receive notification for change of data |
|  |  | DELETE | Deletes multiple subscriptions for a given UE. |
| IndividualSubscriptionDataSubscription | /subscription-data/subs-to-notify/{subsId} | DELETE | Delete the subscription identified by {subsId}, i.e. unsubscribe a node to receive notification for change of data |
|  |  | PATCH | Update an individual Subscription to notification |
| EeGroupSubscriptions | /subscription-data/group-data/{ueGroupId}/ee-subscriptions | GET | Retrieve EE subscriptions for groups of UEs |
|  |  | POST | Create an EE subscription for groups of UEs |
| IndividualEeGroupSubscription | /subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription for a group of UEs |
|  |  | DELETE | Delete an individual EE subscription for a group of UEs |
|  |  | PATCH | Update an individual EE subscription for a group of UEs |
| TraceData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data /trace-data | GET | Retrieve the UE's trace configuration data |
| IdentityData | /subscription-data/{ueId}/identity-data | GET | Retrieve identity data that corresponds to the provided ueId |
| SharedData (Collection) | /subscription-data/shared-data | GET | Retrieve shared data |
| GroupIdentifiers | /subscription-data/group-data/group-identifiers | GET | Retrieve group identifiers |
| 5GVnGroups | /subscription-data/group-data/5g-vn-groups | GET | Retrieve 5G VN Groups |
| Individual5GVnGroup | /subscription-data/group-data/5g-vn-groups/{externalGroupId} | PUT | Create a 5G VN Group |
|  |  | PATCH | Update a 5G VN Group |
|  |  | DELETE | Delete a 5G VN Group |
|  |  | GET | Retrieve a 5G VN Group |
| LcsPrivacySubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-privacy-data | GET | Retrieve the UE's subscribed LCS privacy Subscription Data |
| LcsMobileOriginatedSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-mo-data | GET | Retrieve the UE's subscribed LCS Mobile Originated Subscription Data |
| NiddAuthorizationData | /subscription-data/{ueId}/nidd-authorization-data | GET | Retrieve the UE's NIDD Authorization Data |

* * * Next Change * * * *

### 5.2.xx	Resource: IpSmGwRegistration

#### 5.2.xx.1	Description

# This resource represents the registered IP-SM-GW.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.xx.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/ip-sm-gw

# This resource shall support the resource URI variables defined in table 5.2.xx.2-1.

# Table 5.2.xx.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: "^(imsi-[0-9]{5,15}¦nai-.+¦msisdn-[0-9]{5,15}¦extid-[^@]+@[^@]+¦.+)$" |

#### 5.2.xx.3	Resource Standard Methods

##### 5.2.xx.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.xx.3.1-1.

# Table 5.2.xx.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.xx.3.1-2 and the response data structures and response codes specified in table 5.2.xx.3.1-3.

# Table 5.2.xx.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| IpSmGwRegistration | M | 1 | The IP-SM-GW registration is created or updated with the received information. |

# Table 5.2.xx.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.xx.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.xx.3.2-1.

# Table 5.2.xx.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.xx.3.2-2 and the response data structures and response codes specified in table 5.2.xx.3.2-3.

# Table 5.2.xx.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

# Table 5.2.xx.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.xx.3.3	PATCH

This method shall support the URI query parameters specified in table 5.2.xx.3.3-1.

# Table 5.2.xx.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.xx.3.3-2 and the response data structures and response codes specified in table 5.2.xx.3.3-3.

# Table 5.2.xx.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the authentication subscription of a UE |

# Table 5.2.xx.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified according to e.g. policy or local configuration. The invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified. The cause attribute may be used to convey any of the following errors: - "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition, common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

##### 5.2.xx.3.4	GET

# This method shall support the URI query parameters specified in table 5.2.xx.3.4-1.

# Table 5.2.xx.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the "fields" query parameter shall be included. The "fields" query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.xx.3.4-2 and the response data structures and response codes specified in table 5.2.xx.3.4-3.

# Table 5.2.xx.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.xx.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IpSmGwRegistration | M | 1 | 200 OK | Upon success, a response body containing the IP-SM-GW Registration Data shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

* * * Next Change * * * *

### 5.2.yy	Resource: MessageWaitingData

#### 5.2.yy.1	Description

# This resource represents the Message Waiting Data list for a given UE.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.yy.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/mwd

# This resource shall support the resource URI variables defined in table 5.2.yy.2-1.

# Table 5.2.yy.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: "^(imsi-[0-9]{5,15}¦nai-.+¦msisdn-[0-9]{5,15}¦extid-[^@]+@[^@]+¦.+)$" |

#### 5.2.yy.3	Resource Standard Methods

##### 5.2.yy.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.yy.3.1-1.

# Table 5.2.yy.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.yy.3.1-2 and the response data structures and response codes specified in table 5.2.yy.3.1-3.

# Table 5.2.yy.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MessageWaitingData | M | 1 | The Message Waiting Data list is created or updated with the received information. |

# Table 5.2.yy.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.yy.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.yy.3.2-1.

# Table 5.2.yy.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.yy.3.2-2 and the response data structures and response codes specified in table 5.2.yy.3.2-3.

# Table 5.2.yy.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

# Table 5.2.yy.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.yy.3.3	PATCH

This method shall support the URI query parameters specified in table 5.2.yy.3.3-1.

# Table 5.2.yy.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.yy.3.3-2 and the response data structures and response codes specified in table 5.2.yy.3.3-3.

# Table 5.2.yy.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the authentication subscription of a UE |

# Table 5.2.yy.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified according to e.g. policy or local configuration. The invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified. The cause attribute may be used to convey any of the following errors: - "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition, common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

##### 5.2.yy.3.4	GET

# This method shall support the URI query parameters specified in table 5.2.yy.3.4-1.

# Table 5.2.yy.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the "fields" query parameter shall be included. The "fields" query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.yy.3.4-2 and the response data structures and response codes specified in table 5.2.yy.3.4-3.

# Table 5.2.yy.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.yy.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageWaitingData | M | 1 | 200 OK | Upon success, a response body containing the Message Waiting Data shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

* * * Next Change * * * *

### 5.4.1	General

This clause specifies the application data model supported by the API.

Table 5.4.1-1 specifies the data types defined for the Nudr service based interface protocol.

# Table 5.4.1-1: Nudr Subscriber Data specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationSubscription | 5.4.2.2 | A UE's authentication data |
| OperatorSpecificDataContainer | 5.4.2.3 | Container for operator specific data |
| SmfRegList | 5.4.2.4 | The list of all the SMF registrations of a UE |
| SubscriptionDataSubscriptions | 5.4.2.5 | A subscription to notifications. |
| DataChangeNotify | 5.4.2.6 | Container for data which have changed and notification was requested when changed. |
| IdentityData | 5.4.2.7 | Identity data corresponds to the provided ueId |
| ProvisionedDataSets | 5.4.2.8 |  |
| SorData | 5.4.2.9 | Used to store the status of the latest SOR data update |
| UpuData | 5.4.2.9A | Used to store the status of the latest UPU data update |
| NssaiAckData | 5.4.2.9B | Used to store the status of the latest NSSAI data update |
| CagAckData | 5.4.2.9C | Used to store the status of the latest CAG data update |
| AmfSubscriptionInfo | 5.4.2.19 | Information the UDR stores and retrieves related to active subscriptions at the AMF(s) |
| EeProfileData | 5.4.2.20 | Event Exposure Profile Data |
| ContextDataSets | 5.4.2.22 |  |
| SequenceNumber | 5.4.2.23 |  |
| MessageWaitingData | 5.4.2.xx |  |
| SmscData | 5.4.2.yy |  |
| AuthMethod | 5.4.3.3 |  |
| DataSetName | 5.4.3.4 |  |
| ContextDataSetName | 5.4.3.6 |  |
| SqnScheme | 5.4.3.7 |  |
| Sign | 5.4.3.8 |  |
| UeUpdateStatus | 5.4.3.9 |  |

Table 5.4.1-2 specifies data types re-used by the Nudr service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr service based interface.

# Table 5.4.1-2: Nudr re-used Data Types

| ` | Reference | Comments |
| --- | --- | --- |
| AccessAndMobilitySubscriptionData | 3GPP TS 29.503 [6] | Access and Mobility Subscription Data |
| SmfSelectionSubscriptionData | 3GPP TS 29.503 [6] | SMF Selection Subscription Data |
| SessionManagementSubscriptionData | 3GPP TS 29.503 [6] | Session Management Subscription Data |
| Amf3GppAccessRegistration | 3GPP TS 29.503 [6] |  |
| AmfNon3GppAccessRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to the AMF where the UE has registered via non 3GPP access. |
| SmfRegistration | 3GPP TS 29.503 [6] |  |
| SmsfRegistration | 3GPP TS 29.503 [6] |  |
| SmsManagementSubscriptionData | 3GPP TS 29.503 [6] |  |
| SupportedFeatures | 3GPP TS 29.571 [3] | Used to negotiate the applicability of the optional features |
| ProblemDetails | 3GPP TS 29.571 [3] | Detailed information about the status code. |
| PpData | 3GPP TS 29.503 [6] |  |
| SmsSubscriptionData | 3GPP TS 29.503 [6] |  |
| PatchItem | 3GPP TS 29.571 [3] | Data structure used for JSON patch. |
| SdmSubscription | 3GPP TS 29.503 [6] |  |
| EeSubscription | 3GPP TS 29.503 [6] |  |
| Uri | 3GPP TS 29.571 [3] |  |
| TraceData | 3GPP TS 29.571 [3] |  |
| Dnn | 3GPP TS 29.571 [3] | Data Network Name |
| Snssai | 3GPP TS 29.571 [3] | Single NSSAI |
| VarUeId | 3GPP TS 29.571 [3] | String represents the SUPI or GPSI. |
| NfInstanceId | 3GPP TS 29.571 [3] |  |
| NotifyItem | 3GPP TS 29.571 [3] | Common data type used for data change notification. |
| OdbData | 3GPP TS 29.571 [3] | Operator Determined Barring Data |
| EventType | 3GPP TS 29.503 [6] |  |
| ExtGroupId | 3GPP TS 29.503 [6] | External Group Identifier |
| PduSessionId | 3GPP TS 29.571 [3] |  |
| DateTime | 3GPP TS 29.571 [3] |  |
| UpuMac | 3GPP TS 29.509 [15] |  |
| 5GVnGroupConfiguration | 3GPP TS 29.503 [6] |  |
| PatchResult | 3GPP TS 29.571 [7] |  |
| SupportedFeatures | 3GPP TS 29.571 [7] |  |
| AppPortId | 3GPP TS 29.503 [6] |  |
| LcsPrivacyData | 3GPP TS 29.503 [6] | LCS Privacy Subscription Data |
| LcsMoData | 3GPP TS 29.503 [6] | LCS Mobile Originated Subscription Data |
| MtcProviderInformation | 3GPP TS 29.571 [3] |  |
| AuthorizationData | 3GPP TS 29.503 [6] |  |
| E164Number | 3GPP TS 29.503 [6] |  |
| NetworkNodeDiameterAddress | 3GPP TS 29.503 [6] |  |
| IpSmGwRegistration | 3GPP TS 29.503 [6] |  |

* * * Next Change * * * *

#### 5.4.2.22	Type: ContextDataSets

# Table 5.4.2.22-1: ContextDataSets

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amf3Gpp | Amf3GppAccessRegistration | O | 0..1 | AMF 3GPP Access registration |
| amfNon3Gpp | AmfNon3GppAccessRegistration | O | 0..1 | AMF Non 3GPP Access registration |
| sdmSubscriptions | array(SdmSubscription) | O | 1..N | SDM-Subscriptions |
| eeSubscriptions | array(EeSubscription) | O | 1..N | Event Exposure Subscriptions |
| smsf3GppAccess | SmsfRegistration | O | 0..1 | SMSF 3GPP Access registration |
| smsfNon3GppAccess | SmsfRegistration | O | 0..1 | SMSF Non 3GPP Access registration |
| subscriptionDataSubscriptions | array(SubscriptionDataSubscriptions) | O | 1..N | This attribute contain the Subscription Data Subscriptions (subs-to-notify) associated with the Notification. |
| smfRegistrations | SmfRegList | O | 0..1 | SMF Registrations |
| ipSmGw | IpSmGwRegistration | O | 0..1 | IP-SM-GW Registration |

* * * Next Change * * * *

#### 5.4.2.xx	Type: MessageWaitingData

# Table 5.4.2.xx-1: Definition of type MessageWaitingData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mwdList | array(SmscData) | O | 1..N | List of addresses of SM-Service Center entities with SMS wating to be delivered to the UE. |

#### 5.4.2.yy	Type: SmscData

# Table 5.4.2.yy-1: Definition of type SmscData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| smscMapAddress | E164Number | C | 0..1 | International E.164 number of the SM-SC; it shall be present if the SM-SC supports the MAP protocol. |
| smscDiameterAddress | NetworkNodeDiameterAddress | C | 0..1 | Diameter Identity of the SM-SC; it shall be present if the SM-SC supports the Diameter protocol. |
| NOTE: At least one of the properties, smscMapAddress or smscDiameterAddress, shall be present. |  |  |  |  |

* * * Next Change * * * *

#### 5.4.3.6	Enumeration: ContextDataSetName

# Table 5.4.3.6-1: Enumeration ContextDataSetName

| Enumeration value | Description |
| --- | --- |
| "AMF_3GPP" | AMF 3GPP Access Registration |
| "AMF_NON_3GPP" | SMF Non 3GPP Access Registration |
| "SDM SUBSCRIPTIONS" | SDM Subscriptions |
| "EE_SUBSCRIPTIONS" | EE Subscriptions |
| "SMSF_3GPP" | SMSF 3GPP Access Registration |
| "SMSF_NON3GPP" | SMSF Non 3GPP Access Registration |
| "SUBS_TO_NOTIFY" | UDR Subscriptions |
| "SMF_REG" | SMF Registrations |
| "IP_SM_GW" | IP-SM-GW Registration |

* * * Next Change * * * *

## A.2	Nudr_DataRepository API for Subscription Data

(… text not shown for clarity …)

/subscription-data/{ueId}/context-data/smsf-non-3gpp-access:

put:

summary: Create the SMSF context data of a UE via non-3GPP access

operationId: CreateSmsfContextNon3gpp

tags:

- SMSF Non-3GPP Registration (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/SmsfRegistration'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

delete:

summary: To remove the SMSF context data of a UE via non-3GPP access

operationId: DeleteSmsfContextNon3gpp

tags:

- SMSF Non-3GPP Registration (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

get:

summary: Retrieves the SMSF context data of a UE using non-3gpp access

operationId: QuerySmsfContextNon3gpp

tags:

- SMSF Non-3GPP Registration (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmsfRegistration'

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

/subscription-data/{ueId}/context-data/ip-sm-gw:

put:

summary: Create the IP-SM-GW context data of a UE

operationId: CreateIpSmGwContext

tags:

- IP-SM-GW Registration (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/IpSmGwRegistration'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

description: Unexpected error

delete:

summary: To remove the IP-SM-GW context data of a UE

operationId: DeleteIpSmGwContext

tags:

- IP-SM-GW Registration (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

description: Unexpected error

patch:

summary: Modify the IP-SM-GW context data of a UE

operationId: ModifyIpSmGwContext

tags:

- IP-SM-GW Registration (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Successful response

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

description: Unexpected error

get:

summary: Retrieves the IP-SM-GW context data of a UE

operationId: QueryIpSmGwContext

tags:

- IP-SM-GW Registration (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/IpSmGwRegistration'

default:

description: Unexpected error

/subscription-data/{ueId}/context-data/mwd:

put:

summary: Create the Message Waiting Data of the UE

operationId: CreateMessageWaitingData

tags:

- Message Waiting Data (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/MessageWaitingData'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

description: Unexpected error

delete:

summary: To remove the Message Waiting Data of the UE

operationId: DeleteMessageWaitingData

tags:

- Message Waiting Data (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

description: Unexpected error

patch:

summary: Modify the Message Waiting Data of the UE

operationId: ModifyMessageWaitingData

tags:

- Message Waiting Data (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Successful response

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

description: Unexpected error

get:

summary: Retrieves the Message Waiting Data of the UE

operationId: QueryMessageWaitingData

tags:

- Message Waiting Data (Document)

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/MessageWaitingData'

default:

description: Unexpected error

(… text not shown for clarity …)

ContextDataSetName:

anyOf:

- type: string

enum:

- AMF_3GPP

- AMF_NON_3GPP

- SDM_SUBSCRIPTIONS

- EE_SUBSCRIPTIONS

- SMSF_3GPP

- SMSF_NON_3GPP

- SUBS_TO_NOTIFY

- SMF_REG

- IP_SM_GW

- type: string

ContextDataSets:

type: object

properties:

amf3Gpp:

$ref: '#/components/schemas/Amf3GppAccessRegistration'

amfNon3Gpp:

$ref: '#/components/schemas/AmfNon3GppAccessRegistration'

sdmSubscriptions:

type: array

items:

$ref: '#/components/schemas/SdmSubscription'

minItems: 1

eeSubscriptions:

type: array

items:

$ref: '#/components/schemas/EeSubscription'

minItems: 1

smsf3GppAccess:

$ref: '#/components/schemas/SmsfRegistration'

smsfNon3GppAccess:

$ref: '#/components/schemas/SmsfRegistration'

subscriptionDataSubscriptions:

type: array

items:

$ref: '#/components/schemas/SubscriptionDataSubscriptions'

minItems: 1

smfRegistrations:

$ref: '#/components/schemas/SmfRegList'

ipSmGw:

$ref: '#/components/schemas/IpSmGwRegistration'

IpSmGwRegistration:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/IpSmGwRegistration'

MessageWaitingData:

type: object

properties:

mwdList:

type: array

items:

$ref: '#/components/schemas/SmscData'

minItems: 1

SmscData:

type: object

anyOf:

- required: [ smscMapAddress ]

- required: [ smscDiameterAddress ]

properties:

smscMapAddress:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/E164Number'

smscDiameterAddress:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/NetworkNodeDiameterAddress'

* * * End of Changes * * * *