3GPP TSG-CT Meeting #89e	CP-202016

# E-Meeting, 14th – 16th September 2020											   Revision of C4-204463

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0373 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Error handling for PDU sessions with an I-SMF or V-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN, TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The AMF cannot determine whether the reason for an error returned during a PDU session establishment or a scenario with I-SMF (or V-SMF) insertion is due to the I-SMF/V-SMF or the SMF/H-SMF, which prevents the AMF to support appropriate error handling e.g. wrt (I/V-)SMF reselection. Examples: An 500 Internal Server Error with the cause INSUFFICIENT_RESOURCES, SYSTEM_FAILURE, NF_FAILOVER, NF_SERVICE_FAILOVER or NF_CONGESTION may be generated by the I-SMF/V-SMF or the SMF/H-SMF and could result in the AMF reselecting an I-SMF/V-SMF or SMF/H-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ProblemDetails data type is extended (following 29.501 guidelines of clause 4.8.3) with a new attribute enabling the receiver of the error (AMF) to determine whether the error was originated by the responding entity (V-SMF/I-SMF) or by the remote entity (H-SMF/SMF). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMF cannot determine the originator of the error for a PDU session with an I-SMF/V-SMF, which prevents proper error handing e.g. SMF reselection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2.3.1, 6.1.3.3.4.2.2, 6.1.3.3.4.5.2, 6.1.6.1, 6.1.6.2.35, 6.1.6.2.36, 6.1.6.2.x (new), 6.1.6.2.y (new), 6.1.7.1, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the OpenAPI specification file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: the solution is modified by extending the ProblemDetails data type. Rev. 2: implements the following corrections identified during the implementation of the CR agreed at CT4#99e: a) in clauses 6.1.3.2.3.1, 6.1.3.3.4.2.2 and 6.1.3.3.4.5.2, all the new status codes with ExtProblemDetails have been corrected to not mandate the presence of ExtProblemDetails; b) Table 6.1.6.1-1 (list of data types) is updated with the two new data types defined by this CR; c) 6.1.7.1: in the second bullet, "true" is corrected to "false". |  |  |  |  |  |  |  |  |

* * * First Change * * * *

###### 6.1.3.2.3.1	POST

This method creates an individual SM context resource in the SMF, or in V-SMF in HR roaming scenarios.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmContextCreateData | M | 1 | Representation of the SM context to be created in the SMF. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmContextCreatedData | M | 1 | 201 Created | Successful creation of an SM context. |
|  |  |  | 307 Temporary Redirect | Temporary redirection, during a UE requested PDU Session Establishment. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the SMF that was selected by the AMF. |
|  |  |  | 308 Permanent Redirect | Permanent redirection, during a UE requested PDU Session Establishment. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the SMF that was selected by the AMF. |
| SmContextCreateError | M | 1 | 400 Bad Request | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| SmContextCreateError | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application error: - N1_SM_ERROR - N2_SM_ERROR - SNSSAI_DENIED - DNN_DENIED - PDUTYPE_DENIED - SSC_DENIED - SUBSCRIPTION_DENIED - DNN_NOT_SUPPORTED - PDUTYPE_NOT_SUPPORTED - SSC_NOT_SUPPORTED - HOME_ROUTED_ROAMING_REQUIRED - OUT_OF_LADN_SERVICE_AREA - NO_EPS_5GS_CONTINUITY - INTEGRITY_PROTECTED_MDR_NOT_ACCEPTABLE - DEFAULT_EPS_BEARER_INACTIVE See table 6.1.7.3-1 for the description of these errors. |
| SmContextCreateError | M | 1 | 404 Not Found | The "cause" attribute shall be set to one of the following application error: - CONTEXT_NOT_FOUND See table 6.1.7.3-1 for the description of these errors. |
| ExtProblemDetails | O | 0..1 | 413 Payload Too Large |  |
| ExtProblemDetails | O | 0..1 | 415 Unsupported Media Type |  |
| ExtProblemDetails | O | 0..1 | 429 Too Many Requests |  |
| SmContextCreateError | M | 1 | 500 Internal Server Error | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4] or to one of the following application errors: - INSUFFICIENT_RESOURCES_SLICE - INSUFFICIENT_RESOURCES_SLICE_DNN See table 6.1.7.3-1 for the description of these errors. |
| SmContextCreateError | M | 1 | 503 Service Unavailable | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4] or to one of the following application errors: - DNN_CONGESTION - S_NSSAI_CONGESTION See table 6.1.7.3-1 for the description of these errors. |
| SmContextCreateError | M | 1 | 504 Gateway Timeout | The "cause" attribute shall be set to one of the following application error: - PEER_NOT_RESPONDING - NETWORK_FAILURE See table 6.1.7.3-1 for the description of these errors. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nsmf-pdusession/{apiVersion}/sm-contexts/{smContextRef} |

Table 6.1.3.2.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the SMF that was selected by the AMF |

Table 6.1.3.2.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the SMF that was selected by the AMF |

* * * Next Change * * * *

###### 6.1.3.3.4.2	Operation: modify

6.1.3.3.4.2.1	Description

6.1.3.3.4.2.2	Operation Definition

This custom operation updates an individual SM context resource and/or provide N1 or N2 SM information received from the UE or the AN, for a given PDU session, towards the SMF, or in V-SMF in HR roaming scenario.

This operation shall support the request data structures specified in table 6.1.3.3.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.3.4.2.2-2.

Table 6.1.3.3.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmContextUpdateData | M | 1 | Representation of the updates to apply to the SM context. |

Table 6.1.3.3.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmContextUpdatedData | C | 0..1 | 200 OK | Successful update of the SM context, when the SMF needs to return information in the response. |
|  |  |  | 204 No Content | Successful update of the SM context, when the SMF does not need to return information in the response. |
| SmContextUpdateError | M | 1 | 400 Bad Request | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| SmContextUpdateError | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application error: - N1_SM_ERROR - N2_SM_ERROR - SUBSCRIPTION_DENIED - OUT_OF_LADN_SERVICE_AREA - PRIORITIZED_SERVICES_ONLY - PDU_SESSION_ANCHOR_CHANGE See table 6.1.7.3-1 for the description of these errors. |
| SmContextUpdateError | M | 1 | 404 Not Found | The "cause" attribute shall be set to one of the following application error: - CONTEXT_NOT_FOUND See table 6.1.7.3-1 for the description of these errors. |
| ExtProblemDetails | O | 0..1 | 413 Payload Too Large |  |
| ExtProblemDetails | O | 0..1 | 415 Unsupported Media Type |  |
| ExtProblemDetails | O | 0..1 | 429 Too Many Requests |  |
| SmContextUpdateError | M | 1 | 500 Internal Server Error | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4]. |
| SmContextUpdateError | M | 1 | 503 Service Unavailable | The "cause" attribute shall be set to one of the errors defined in Table 5.2.7.2-1 of 3GPP TS 29.500 [4] or to one of the following application errors: - DNN_CONGESTION - S-NSSAI_ CONGESTION See table 6.1.7.3-1 for the description of these errors. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

* * * Next Change * * * *

###### 6.1.3.3.4.5	Operation: send-mo-data

6.1.3.3.4.5.1	Description

6.1.3.3.4.5.2	Operation Definition

This custom operation enables to send mobile originated data received over NAS, for a given PDU session, towards the SMF, or the V-SMF for HR roaming scenarios, or the I-SMF for a PDU session with an I-SMF.

This operation shall support the request data structures specified in table 6.1.3.3.4.5.2-1 and the response data structure and response codes specified in table 6.1.3.3.4.5.2-2.

Table 6.1.3.3.4.5.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SendMoDataReqData | M | 1 | Representation of the payload of a Send MO Data Request |

Table 6.1.3.3.4.5.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
|  |  |  | 204 No Content | Successful MO data transfer |
| ExtProblemDetails | O | 0..1 | 400 Bad Request |  |
| ExtProblemDetails | O | 0..1 | 401 Unauthorized |  |
| ExtProblemDetails | O | 0..1 | 403 Forbidden |  |
| ExtProblemDetails | O | 0..1 | 404 Not Found |  |
| ExtProblemDetails | O | 0..1 | 413 Payload Too Large |  |
| ExtProblemDetails | O | 0..1 | 415 Unsupported Media Type |  |
| ExtProblemDetails | O | 0..1 | 429 Too Many Requests |  |
| ExtProblemDetails | O | 0..1 | 500 Internal Server Error |  |
| ExtProblemDetails | O | 0..1 | 503 Service Unavailable |  |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

* * * Next Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsmf service based interface protocol.

Table 6.1.6.1-1: Nsmf specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SmContextCreateData | 6.1.6.2.2 | Information within Create SM Context Request |
| SmContextCreatedData | 6.1.6.2.3 | Information within Create SM Context Response |
| SmContextUpdateData | 6.1.6.2.4 | Information within Update SM Context Request |
| SmContextUpdatedData | 6.1.6.2.5 | Information within Update SM Context Response |
| SmContextReleaseData | 6.1.6.2.6 | Information within Release SM Context Request |
| SmContextRetrieveData | 6.1.6.2.7 | Information within Retrieve SM Context Request |
| SmContextStatusNotification | 6.1.6.2.8 | Information within Notify SM Context Status Request |
| PduSessionCreateData | 6.1.6.2.9 | Information within Create Request |
| PduSessionCreatedData | 6.1.6.2.10 | Information within Create Response |
| HsmfUpdateData | 6.1.6.2.11 | Information within Update Request towards H-SMF, or from I-SMF to SMF |
| HsmfUpdatedData | 6.1.6.2.12 | Information within Update Response from H-SMF, or from SMF to I-SMF |
| ReleaseData | 6.1.6.2.13 | Information within Release Request |
| HsmfUpdateError | 6.1.6.2.14 | Error within Update Response from H-SMF |
| VsmfUpdateData | 6.1.6.2.15 | Information within Update Request towards V-SMF, or from SMF to I-SMF |
| VsmfUpdatedData | 6.1.6.2.16 | Information within Update Response from V-SMF, or from I-SMF to SMF |
| StatusNotification | 6.1.6.2.17 | Information within Notify Status Request |
| QosFlowItem | 6.1.6.2.18 | Individual QoS flow |
| QosFlowSetupItem | 6.1.6.2.19 | Individual QoS flow to setup |
| QosFlowAddModifyRequestItem | 6.1.6.2.20 | Individual QoS flow requested to be created or modified |
| QosFlowReleaseRequestItem | 6.1.6.2.21 | Individual QoS flow requested to be released |
| QosFlowProfile | 6.1.6.2.22 | QoS flow profile |
| GbrQosFlowInformation | 6.1.6.2.23 | GBR QoS flow information |
| QosFlowNotifyItem | 6.1.6.2.24 | Notification related to a QoS flow |
| SmContextRetrievedData | 6.1.6.2.27 | Information within Retrieve SM Context Response |
| TunnelInfo | 6.1.6.2.28 | Tunnel Information |
| StatusInfo | 6.1.6.2.29 | Status of SM context or of PDU session |
| VsmfUpdateError | 6.1.6.2.30 | Error within Update Response from V-SMF |
| EpsPdnCnxInfo | 6.1.6.2.31 | EPS PDN Connection Information from H-SMF to V-SMF |
| EpsBearerInfo | 6.1.6.2.32 | EPS Bearer Information from H-SMF to V-SMF |
| PduSessionNotifyItem | 6.1.6.2.33 | Notification related to a PDU session |
| EbiArpMapping | 6.1.6.2.34 | EBI to ARP mapping |
| SmContextCreateError | 6.1.6.2.35 | Error within Create SM Context Response |
| SmContextUpdateError | 6.1.6.2.36 | Error within Update SM Context Response |
| PduSessionCreateError | 6.1.6.2.37 | Error within Create Response |
| MmeCapabilities | 6.1.6.2.38 | MME capabilities |
| SmContext | 6.1.6.2.39 | Complete SM Context |
| ExemptionInd | 6.1.6.2.40 | Exemption Indication |
| PsaInformation | 6.1.6.2.41 | PSA Information |
| DnaiInformation | 6.1.6.2.42 | DNAI Information |
| N4Information | 6.1.6.2.43 | N4 Information |
| IndirectDataForwardingTunnelInfo | 6.1.6.2.44 | Indirect Data Forwarding Tunnel Information |
| SmContextReleasedData | 6.1.6.2.45 | Information within Release SM Context Response |
| ReleasedData | 6.1.6.2.46 | Information within Release Response |
| SendMoDataReqData | 6.1.6.2.47 | Information within Send MO Data Request |
| CnAssistedRanPara | 6.1.6.2.48 | SMF derived CN assisted RAN parameters tuning |
| UlclBpInformation | 6.1.6.2.49 | UL CL or BP Information |
| TransferMoDataReqData | 6.1.6.2.50 | Information within Transfer MO Data Request |
| TransferMtDataReqData | 6.1.6.2.51 | Information within Transfer MT Data Request |
| TransferMtDataError | 6.1.6.2.52 | Transfer MT Data Error Response |
| TransferMtDataAddInfo | 6.1.6.2.53 | Transfer MT Data Error Response Additional Information |
| VplmnQos | 6.1.6.2.54 | VPLMN QoS |
| DdnFailureSubs | 6.1.6.2.55 | DDN Failure Subscription |
| RetrieveData | 6.1.6.2.56 | Information within Retrieve Request |
| RetrievedData | 6.1.6.2.57 | Information within Retrieve Response |
| SecurityResult | 6.1.6.2.58 | Security Result |
| UpSecurityInfo | 6.1.6.2.59 | User Plane Security Information |
| DdnFailureSubInfo | 6.1.6.2.60 | DDN Failure Subscription Information |
| AlternativeQosProfile | 6.1.6.2.61 | Alternative QoS Profile |
| ProblemDetailsAddInfo | 6.1.6.2.x | Problem Details Additional Information |
| ExtProblemDetails | 6.1.6.2.y | Extended Problem Details |
| Teid | 6.1.6.3.2 | GTP Tunnel Endpoint Identifier |
| ProcedureTransactionId | 6.1.6.3.2 | Procedure Transaction Identifier |
| EpsPdnCnxContainer | 6.1.6.3.2 | UE EPS PDN Connection container from SMF to AMF |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Id |
| EpsBearerContainer | 6.1.6.3.2 | EPS Bearer container from SMF to AMF |
| EpsBearerContextStatus | 6.1.6.3.2 | EPS Bearer context status |
| DrbId | 6.1.6.3.2 | Data Radio Bearer Identifier |
| UpCnxState | 6.1.6.3.3 | User Plane Connection State |
| HoState | 6.1.6.3.4 | Handover State |
| RequestType | 6.1.6.3.5 | Request Type in Create (SM context) service operation. |
| RequestIndication | 6.1.6.3.6 | Request Indication in Update (SM context) service operation. |
| NotificationCause | 6.1.6.3.7 | Cause for generating a notification |
| Cause | 6.1.6.3.8 | Cause information |
| ResourceStatus | 6.1.6.3.9 | Status of SM context or PDU session resource |
| DnnSelectionMode | 6.1.6.3.10 | DNN Selection Mode |
| EpsInterworkingIndication | 6.1.6.3.11 | EPS Interworking Indication |
| N2SmInfoType | 6.1.6.3.12 | N2 SM Information Type |
| MaxIntegrityProtectedDataRate | 6.1.6.3.13 | Maximum Integrity Protected Data Rate |
| MaReleaseIndication | 6.1.6.3.14 | Multi-Access PDU session release Indication |
| SmContextType | 6.1.6.3.15 | Type of SM Context information |
| PsaIndication | 6.1.6.3.16 | Indication of whether a PSA is inserted or removed |
| N4MessageType | 6.1.6.3.17 | N4 Message Type |
| QosFlowAccessType | 6.1.6.3.18 | Access type associated with the QoS Flow |
| UnavailableAccessIndication | 6.1.6.3.19 | Indicates the access type of a MA PDU session that is unavailable |
| ProtectionResult | 6.1.6.3.20 | Protection Result of the security policy indicated as "preferred" |
| QosMonitoringReq | 6.1.6.3.21 | Indicates to measure UL, or DL, or both UL/DL delays. |

Table 6.1.6.1-2 specifies data types re-used by the Nsmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmf service based interface.

Table 6.1.6.1-2: Nsmf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uint32 | 3GPP TS 29.571 [13] | Unsigned 32-bit integers |
| Ipv4Addr | 3GPP TS 29.571 [13] | IPv4 Address |
| Ipv6Prefix | 3GPP TS 29.571 [13] | IPv6 Prefix |
| Uri | 3GPP TS 29.571 [13] | Uniform Resource Identifier |
| Supi | 3GPP TS 29.571 [13] | Subscription Permanent Identifier |
| Pei | 3GPP TS 29.571 [13] | Permanent Equipment Identifier |
| Gpsi | 3GPP TS 29.571 [13] | General Public Subscription Identifier |
| AccessType | 3GPP TS 29.571 [13] | Access Type (3GPP or non-3GPP access) |
| SupportedFeatures | 3GPP TS 29.571 [13] | Supported features |
| Qfi | 3GPP TS 29.571 [13] | QoS Flow Identifier |
| PduSessionId | 3GPP TS 29.571 [13] | PDU Session Identifier |
| PduSessionType | 3GPP TS 29.571 [13] | PDU Session Type |
| Ambr | 3GPP TS 29.571 [13] | PDU Session Aggregate Maximum Bit Rate |
| 5Qi | 3GPP TS 29.571 [13] | 5G QoS Identifier |
| Arp | 3GPP TS 29.571 [13] | Allocation and Retention Priority |
| ReflectiveQoSAttribute | 3GPP TS 29.571 [13] | Reflective QoS Attribute |
| Dynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics for a 5QI that is neither standardized nor pre-configured. |
| NonDynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics that replace the default QoS characteristics for a standardized or pre-configured 5QI. |
| PacketLossRate | 3GPP TS 29.571 [13] | Packet Loss Rate |
| NotificationControl | 3GPP TS 29.571 [13] | Notification Control |
| Dnn | 3GPP TS 29.571 [13] | Data Network Name |
| Snssai | 3GPP TS 29.571 [13] | Single Network Slice Selection Assistance Information |
| NfInstanceId | 3GPP TS 29.571 [13] | NF Instance Identifier |
| UserLocation | 3GPP TS 29.571 [13] | User Location |
| TimeZone | 3GPP TS 29.571 [13] | Time Zone |
| ProblemDetails | 3GPP TS 29.571 [13] | Error description |
| UpSecurity | 3GPP TS 29.571 [13] | User Plane Security Policy Enforcement information |
| RefToBinaryData | 3GPP TS 29.571 [13] | Cross-Reference to binary data encoded within a binary body part in an HTTP multipart message. |
| Guami | 3GPP TS 29.571 [13] | Globally Unique AMF ID |
| BackupAmfInfo | 3GPP TS 29.571 [13] | Backup AMF Information |
| PresenceState | 3GPP TS 29.571 [13] | Indicates the UE presence in or out of a LADN service area |
| TraceData | 3GPP TS 29.571 [13] | Trace control and configuration parameters |
| PlmnId | 3GPP TS 29.571 [13] | PLMN Identity |
| RatType | 3GPP TS 29.571 [13] | RAT Type |
| NgApCause | 3GPP TS 29.571 [13] | NGAP Cause |
| 5GMmCause | 3GPP TS 29.571 [13] | 5G MM Cause |
| DurationSec | 3GPP TS 29.571 [13] | Duration in units of seconds |
| AdditionalQosFlowInfo | 3GPP TS 29.571 [13] | Additional QoS Flow Information |
| NfGroupId | 3GPP TS 29.571 [13] | Network Function Group Id |
| SecondaryRatUsageReport | 3GPP TS 29.571 [13] | Secondary RAT Usage Report |
| SecondaryRatUsageInfo | 3GPP TS 29.571 [13] | Secondary RAT Usage Information |
| Dnai | 3GPP TS 29.571 [13] | Data Network Access Identifier |
| PlmnIdNid | 3GPP TS 29.571 [13] | PLMN Identity and, for SNPN, Network Identity |
| SmallDataRateStatus | 3GPP TS 29.571 [13] | Small Data Rate Control Status |
| ApnRateStatus | 3GPP TS 29.571 [13] | APN Rate Control Status |
| StationaryIndication | 3GPP TS 29.571 [13] | Stationary Indication |
| ScheduledCommunicationTime | 3GPP TS 29.571 [13] | Scheduled Communication Time |
| ScheduledCommunicationType | 3GPP TS 29.571 [13] | Scheduled Communication Type |
| TrafficProfile | 3GPP TS 29.571 [13] | Traffic Profile |
| BatteryIndication | 3GPP TS 29.571 [13] | Battery Indication |
| NfSetId | 3GPP TS 29.571 [13] | NF Set Identifier |
| MoExpDataCounter | 3GPP TS 29.571 [13] | MO Exception Data Counter |
| DddTrafficDescriptor | 3GPP TS 29.571 [13] | Traffic Descriptor |
| ServiceName | 3GPP TS 29.510 [19] | Service Name |
| WAgfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the W-AGF |
| TngfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TNGF |
| TwifInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TWIF |
| ChargingInformation | 3GPP TS 29.512 [30] | CHF addresses |
| NgRanTargetId | 3GPP TS 29.518 [20] | NG-RAN Target Id |
| RoamingChargingProfile | 3GPP TS 32.291 [26] | Roaming Charging Profile |

* * * Next Change * * * *

##### 6.1.6.2.35	Type: SmContextCreateError

Table 6.1.6.2.35-1: Definition of type SmContextCreateError

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| error | ExtProblemDetails | M | 1 | More information on the error shall be provided in the "cause" attribute of the "ProblemDetails" structure. |
| n1SmMsg | RefToBinaryData | C | 0..1 | This IE shall be present, if an N1 SM information is received in the request and the SMF is able to return N1 SM information to the UE. When present, it shall reference the N1 SM Message binary data (see clause 6.1.6.4.2). |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present, if N2 SM information needs to be returned to the NG-RAN during Xn based handover procedure with I-SMF/V-SMF insertion, change or removal. When present, it shall reference the N2 SM Message binary data (see clause 6.1.6.4.3). |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the SMF service instance was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |

* * * Next Change * * * *

##### 6.1.6.2.36	Type: SmContextUpdateError

Table 6.1.6.2.36-1: Definition of type SmContextUpdateError

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| error | ExtProblemDetails | M | 1 | More information on the error shall be provided in the "cause" attribute of the "ProblemDetails" structure. |
| n1SmMsg | RefToBinaryData | C | 0..1 | This IE shall be present, if N1 SM Information needs to be returned to the UE. When present, it shall reference the N1 SM Message binary data (see clause 6.1.6.4.2). |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present, if N2 SM information needs to be returned to the NG-RAN. When present, it shall reference the N2 SM Message binary data (see clause 6.1.6.4.3). |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present if the SMF was requested to activate or deactivate the user plane connection of the PDU session in the corresponding request. When present, it shall be set as specified in clauses 5.2.2.3.2 and 5.2.2.3.16. |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the SMF service instance was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |

* * * Next Change * * * *

##### 6.1.6.2.x	Type: ProblemDetailsAddInfo

Table 6.1.6.2.x-1: Definition of type ProblemDetailsAddInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| remoteError | boolean | O | 0..1 | When present, this IE shall indicate whether the error is originated by the remote entity or by the entity sending the response, as follows: - true: the error is originated by the remote entity (i.e. H-SMF/SMF). - false: the error is originated by the entity sending the response (i.e. V-SMF/I-SMF). This IE shall be present and set to "true" for a HR PDU session or for a PDU session with an I-SMF, when the V-SMF/I-SMF retuns an error response to the AMF that was originated by the H-SMF/SMF. This IE may be present if the error is originated by the V-SMF/I-SMF. |

* * * Next Change * * * *

##### 6.1.6.2.y	Type: ExtProblemDetails

Table 6.1.6.2.y-1: Definition of type ExtProblemDetails as a list of to be combined data types

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| ProblemDetails | 1 | Detail information of the problem |  |
| ProblemDetailsAddInfo | 1 | Additional information to be returned in error response. |  |

* * * Next Change * * * *

#### 6.1.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

The Cause codes mapping performed by AMF between the following HTTP responses returned by the SMF services to the AMF and the 5GMM related values is specified in clause 4.3.2 of 3GPP TS 29.524 [34].

In order to enable the AMF to figure out whether a request (e.g. PDU session establishment request) fails at the V-SMF/I-SMF or at the H-SMF/SMF for a HR PDU session or a PDU session with an I-SMF:

-	a V-SMF/I-SMF that returns an HTTP error response to the AMF that was originated by the H-SMF/SMF shall include the remoteError attribute set to "true" in the ProblemDetails information in the HTTP error response;

-	a V-SMF/I-SMF that originates an error to the AMF may include the remoteError attribute set to "false" in the ProblemDetails information in the HTTP error response.

* * * Next Change * * * *

## A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.1.0'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

paths:

/sm-contexts:

post:

summary:  Create SM Context

tags:

- SM contexts collection

operationId: PostSmContexts

requestBody:

description: representation of the SM context to be created in the SMF

required: true

content:

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateData'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

binaryDataN2SmInformationExt1:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformationExt1:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

callbacks:

smContextStatusNotification:

'{$request.body#/smContextStatusUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SmContextStatusNotification'

responses:

'204':

description: successful notification

'307':

description: temporary redirect

headers:

Location:

required: true

description: 'A URI pointing to the endpoint of another NF service consumer to which the notification should be sent'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

responses:

'201':

description: successful creation of an SM context

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreatedData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreatedData'

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nsmf-pdusession/{apiVersion}/sm-contexts/{smContextRef}'

required: true

schema:

type: string

'307':

description: temporary redirect

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the SMF that was selected by the AMF'

required: true

schema:

type: string

'308':

description: permanent redirect

headers:

Location:

description: 'An alternative URI of the resource located on an alternative service instance within the SMF that was selected by the AMF'

required: true

schema:

type: string

'400':

description: unsuccessful creation of an SM context - bad request

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'403':

description: unsuccessful creation of an SM context - forbidden

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'404':

description: unsuccessful creation of an SM context - not found

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: '#/components/responses/413'

'415':

$ref: '#/components/responses/415'

'429':

$ref: '#/components/responses/429'

'500':

description: unsuccessful creation of an SM context - internal server error

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'503':

description: unsuccessful creation of an SM context - service unavailable

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'504':

description: unsuccessful creation of an SM context - gateway timeout

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextCreateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextCreateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmMessage:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmMessage:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

[…]

/sm-contexts/{smContextRef}/modify:

post:

summary:  Update SM Context

tags:

- Individual SM context

operationId: UpdateSmContext

parameters:

- name: smContextRef

in: path

description:  SM context reference

required: true

schema:

type: string

requestBody:

description: representation of the updates to apply to the SM context

required: true

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateData'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

binaryDataN2SmInformationExt1:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformationExt1:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

responses:

'200':

description: successful update of an SM context with content in the response

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdatedData'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdatedData'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'204':

description: successful update of an SM context without content in the response

'400':

description: unsuccessful update of an SM context - bad request

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'403':

description: unsuccessful update of an SM context - forbidden

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'404':

description: unsuccessful update of an SM context - not found

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: '#/components/responses/413'

'415':

$ref: '#/components/responses/415'

'429':

$ref: '#/components/responses/429'

'500':

description: unsuccessful update of an SM context - Internal server error

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'503':

description: unsuccessful update of an SM context - Service Unavailable

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/SmContextUpdateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/SmContextUpdateError'

binaryDataN1SmMessage:

type: string

format: binary

binaryDataN2SmInformation:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmMessage:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN2SmInformation:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/sm-contexts/{smContextRef}/send-mo-data:

post:

summary:  Send MO Data

tags:

- Individual SM context

operationId: SendMoData

parameters:

- name: smContextRef

in: path

description:  SM context reference

required: true

schema:

type: string

requestBody:

description: representation of the payload of Send MO Data Request

required: true

content:

multipart/related:  # message with a binary body part

schema:

type: object

properties:

jsonData:

$ref: '#/components/schemas/SendMoDataReqData'

binaryMoData:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryMoData:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

responses:

'204':

description: successful sending of MO data

'400':

$ref: '#/components/responses/400'

'401':

$ref: '#/components/responses/401'

'403':

$ref: '#/components/responses/403'

'404':

$ref: '#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: '#/components/responses/413'

'415':

$ref: '#/components/responses/415'

'429':

$ref: '#/components/responses/429'

'500':

$ref: '#/components/responses/500'

'503':

$ref: '#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

[…]

SmContextCreateError:

type: object

properties:

error:

$ref: '#/components/schemas/ExtProblemDetails'

n1SmMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- error

SmContextUpdateError:

type: object

properties:

error:

$ref: '#/components/schemas/ExtProblemDetails'

n1SmMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

upCnxState:

$ref: '#/components/schemas/UpCnxState'

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- error

ExtProblemDetails:

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

- $ref: '#/components/schemas/ProblemDetailsAddInfo'

ProblemDetailsAddInfo:

type: object

properties:

remoteError:

type: boolean

#

# HTTP responses

#

responses:

[…]

'VsmfUpdateError':

description: unsuccessful update of a PDU session

content:

application/json: # message without binary body part

schema:

$ref: '#/components/schemas/VsmfUpdateError'

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/VsmfUpdateError'

binaryDataN1SmInfoFromUe:

type: string

format: binary

binaryDataUnknownN1SmInfo:

type: string

format: binary

binaryDataN4Information:

type: string

format: binary

binaryDataN4InformationExt1:

type: string

format: binary

binaryDataN4InformationExt2:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataN1SmInfoFromUe:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataUnknownN1SmInfo:

contentType:  application/vnd.3gpp.5gnas

headers:

Content-Id:

schema:

type: string

binaryDataN4Information:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt1:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

binaryDataN4InformationExt2:

contentType:  application/vnd.3gpp.pfcp

headers:

Content-Id:

schema:

type: string

'400':

description: Bad request

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'401':

description: Unauthorized

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'413':

description: Payload Too Large

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'415':

description: Unsupported Media Type

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'429':

description: Too Many Requests

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtProblemDetails'

[…]

* * * End of Changes * * * *