3GPP TSG-CT Meeting #89-e	CP-202122

# Online, , 14th Sep 2020 - 16th Sep 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 2552 | rev | 4 | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Type of the N5GC indication information element |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, InterDigital, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon, InterDigital, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-09-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A type 2 information element (IE) was incorrectly introduced to TS 24.501: (1) Firstly, it is inconsistent with the description of the information element in the specification, quote of clause 5.5.1.2.2: If the W-AGF acting on behalf of an N5GC device initiates an initial registration as specified in 3GPP TS 23.316 [6D], the W-AGF acting on behalf of the N5GC device shall include the N5GC indication IE with the N5GC device indication bit set to "N5GC device registration is requested" in the REGISTRATION REQUEST message. Based on the above description, the use of type 2 for the N5GC indication is as matter of fact not possible; (2) IEs of type 2 neither exist in this specification nor in NAS for EPS (TS 24.301), and this is because of the fact that they come with drawbacks as already discussed during CT1#123-e (see discussion on C1-202083 on the 3GPP_TSG_CT_WG1_E_MAIN@LIST.ETSI.ORG mailing list for further details). Furthermore, the use of type 1 IE provides an advantage for the protocol as the sender uses fewer octets. In addition, a type 1 IE provides 4 bits for carrying information, so the unused bits can be used in future releases, if needed, for other purposes than initially thought. This approach therefore provides flexibility and it is future proof. Moreover, note that the specification still can use IEs of type 1, so there is nothing preventing us from using type 1. There is no need of restricting the number IEs of type 1 which can be used in a message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Type 1 is proposed instead for the N5GC indication information element. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency within the specification on the N5GC indication information element remains which leads to problems for implementation in both the UE and the network as the related functionality is actually not implementable. Not appropriate type for the N5GC indication information element remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.1, 9.11.3.72 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Version 4 is submitted direct to plenary and it is same as tdoc C1-205554 submitted to CT1 #125-e. The tdoc was objected by one single company but the technical proposal of the CR was never challenged. Hence, there have been no changes to the content of the CR. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.2.6.1	Message definition

The REGISTRATION REQUEST message is sent by the UE to the AMF. See table 8.2.6.1.1.

Message type:	REGISTRATION REQUEST

Significance:	dual

Direction:		UE to network

Table 8.2.6.1.1: REGISTRATION REQUEST message content

| IEI |  | Information Element |  | Type/Reference |  | Presence |  | Format |  | Length |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Extended protocol discriminator |  | Extended Protocol discriminator 9.2 |  | M |  | V |  | 1 |  |
|  |  | Security header type |  | Security header type 9.3 |  | M |  | V |  | 1/2 |  |
|  |  | Spare half octet |  | Spare half octet 9.5 |  | M |  | V |  | 1/2 |  |
|  |  | Registration request message identity |  | Message type 9.7 |  | M |  | V |  | 1 |  |
|  |  | 5GS registration type |  | 5GS registration type 9.11.3.7 |  | M |  | V |  | 1/2 |  |
|  |  | ngKSI |  | NAS key set identifier 9.11.3.32 |  | M |  | V |  | 1/2 |  |
|  |  | 5GS mobile identity |  | 5GS mobile identity 9.11.3.4 |  | M |  | LV-E |  | 6-n |  |
| C- |  | Non-current native NAS key set identifier |  | NAS key set identifier 9.11.3.32 |  | O |  | TV |  | 1 |  |
| 10 |  | 5GMM capability |  | 5GMM capability 9.11.3.1 |  | O |  | TLV |  | 3-15 |  |
| 2E |  | UE security capability |  | UE security capability 9.11.3.54 |  | O |  | TLV |  | 4-10 |  |
| 2F |  | Requested NSSAI |  | NSSAI 9.11.3.37 |  | O |  | TLV |  | 4-74 |  |
| 52 |  | Last visited registered TAI |  | 5GS tracking area identity 9.11.3.8 |  | O |  | TV |  | 7 |  |
| 17 |  | S1 UE network capability |  | S1 UE network capability 9.11.3.48 |  | O |  | TLV |  | 4-15 |  |
| 40 |  | Uplink data status |  | Uplink data status 9.11.3.57 |  | O |  | TLV |  | 4-34 |  |
| 50 |  | PDU session status |  | PDU session status 9.11.3.44 |  | O |  | TLV |  | 4-34 |  |
| B- |  | MICO indication |  | MICO indication 9.11.3.31 |  | O |  | TV |  | 1 |  |
| 2B |  | UE status |  | UE status 9.11.3.56 |  | O |  | TLV |  | 3 |  |
| 77 |  | Additional GUTI |  | 5GS mobile identity 9.11.3.4 |  | O |  | TLV-E |  | 14 |  |
| 25 |  | Allowed PDU session status |  | Allowed PDU session status 9.11.3.13 |  | O |  | TLV |  | 4-34 |  |
| 18 |  | UE's usage setting |  | UE's usage setting 9.11.3.55 |  | O |  | TLV |  | 3 |  |
| 51 |  | Requested DRX parameters |  | 5GS DRX parameters 9.11.3.2A |  | O |  | TLV |  | 3 |  |
| 70 |  | EPS NAS message container |  | EPS NAS message container 9.11.3.24 |  | O |  | TLV-E |  | 4-n |  |
| 74 |  | LADN indication |  | LADN indication 9.11.3.29 |  | O |  | TLV-E |  | 3-811 |  |
| 8- |  | Payload container type |  | Payload container type 9.11.3.40 |  | O |  | TV |  | 1 |  |
| 7B |  | Payload container |  | Payload container 9.11.3.39 |  | O |  | TLV-E |  | 4-65538 |  |
| 9- |  | Network slicing indication |  | Network slicing indication 9.11.3.36 |  | O |  | TV |  | 1 |  |
| 53 |  | 5GS update type |  | 5GS update type 9.11.3.9A |  | O |  | TLV |  | 3 |  |
| 41 |  | Mobile station classmark 2 |  | Mobile station classmark 2 9.11.3.31C |  | O |  | TLV |  | 5 |  |
| 42 |  | Supported codecs |  | Supported codec list 9.11.3.51A |  | O |  | TLV |  | 5-n |  |
| 71 |  | NAS message container |  | NAS message container 9.11.3.33 |  | O |  | TLV-E |  | 4-n |  |
| 60 |  | EPS bearer context status |  | EPS bearer context status 9.11.3.23A |  | O |  | TLV |  | 4 |  |
| 6E |  | Requested extended DRX parameters |  | Extended DRX parameters 9.11.3.26A |  | O |  | TLV |  | 3 |  |
| 6A |  | T3324 value |  | GPRS timer 3 9.11.2.5 |  | O |  | TLV |  | 3 |  |
| 67 |  | UE radio capability ID |  | UE radio capability ID 9.11.3.68 |  | O |  | TLV |  | 3-n |  |
| 35 |  | Requested mapped NSSAI |  | Mapped NSSAI 9.11.3.31B |  | O |  | TLV |  | 3-42 |  |
| 48 |  | Additional information requested |  | Additional information requested 9.11.3.12A |  | O |  | TLV |  | 3 |  |
| 1A |  | Requested WUS assistance information |  | WUS assistance information 9.11.3.71 |  | O |  | TLV |  | 3-n |  |
| A- |  | N5GC indication |  | N5GC indication 9.11.3.72 |  | O |  | T |  | 1 |  |
| 30 |  | Requested NB-N1 mode DRX parameters |  | NB-N1 mode DRX parameters 9.11.3.73 |  | O |  | TLV |  | 3 |  |

* * * Next Change * * * *

#### 9.11.3.72	N5GC indication

The purpose of the N5GC indication information element is to indicate to the network that the registration request by the W-AGF is on behalf of an N5GC device.

The N5GC indication information element is coded as shown in figure 9.11.3.72.1.

The N5GC indication is a type 1 information element.

| 8 | 7 | 6 | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N5GC indication IEI |  |  |  |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | N5GCREG |  | octet 1 |  |

Figure 9.11.3.72.1: N5GC indication

Table 9.11.3.72.1: N5GC indication

| N5GC device indication bit (N5GCREG) (octet 1, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | N5GC device registration is not requested |
| 1 | N5GC device registration is requested |
| Bits 2 to 4 are spare and shall be coded as zero, |  |

* * * End of Change * * * *