TSG-CT WG Meeting #89-e	CP-202206

E-Meeting, 14th – 16th September 2020											(Revision of C4-204433)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0381 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | H-SMF/SMF sending IPv6 Interface Identifier to V-SMF/I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT, ETSUN |  |  |  |  |  | Date: |  |  | 2020-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During a PDU session establishment if IPv6 ststless autoconfiguration is used, the H-SMF allocates an interface identifier to the UE, so that the UE can build its link-local address to ensure that the link-local address generated by the UE does not collide with the link-local address of the UPF and the SMF (see 3GPP TS 23.502, clause 4.3.2.2 steps 8 and 11). Clause 4.3.1 in 3GPP TS 23.502 reads: "information that both the V-SMF and H-SMF process: indication of the nature of the NAS SM signalling (e.g. PDU Session Establishment Request), PDU Session Type, Session-AMBR, UE addressing information (allocated IPv4 address, interface identifier)". Therefore, H-SMF needs to send the Interface Identifier to the SMF (see also clause 5.34.6.1 in 3GPP TS 23.501 fro I-SMF case). |  |  |  |  |  |  |  |  |
| Summary of change: |  | New attribute, 'ueIpv6InterfaceId' is added to Nsmf_PDUSession_Create Response, i.e. to PduSessionCreatedData data structure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDU session establishment procedure for HR roaming or PDU session with an I-SMF when H-SMF/SMF assigns IPv6 interface identifier to the UE will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.10, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR adds backward compatible new feature to OpenAPI. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Use case for PDU session establishment with an I-SMF is added to the new 'ueIpv6InterfaceId' attribute in Table 6.1.6.2.10-1. Consequently, the cover sheet was updated by replacing Category A with Category F, adding ETSUN to the WIC and otherwise. Rev2 (plenary): - In Table 6.1.6.2.10-1 and in the OpenAPI annex the description and the pattern of the new ueIpv6InterfaceId attribute has been corrected to encode 16 hexadecimal digits, instead of 8, which was incorrect; - In Table 6.1.6.2.10-1 the style of the font (red) has been corrected and “hex” has been spelled out as “hexadecimal”. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.10	Type: PduSessionCreatedData

Table 6.1.6.2.10-1: Definition of type PduSessionCreatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pduSessionType | PduSessionType | M | 1 | This IE shall indicate the selected PDU type. |  |
| sscMode | string | M | 1 | This IE shall indicate the SSC mode applicable to the PDU session. When present, it shall be encoded as one character in hexadecimal representation, taking a value of "0" to "7", representing the 3 bits of the SSC mode value of the SSC mode IE specified in clause 9.11.4.16 of 3GPP TS 24.501 [7]. Pattern: "^[0-7]$" Example: SSC mode 3 shall be encoded as "3". (NOTE 1). |  |
| hcnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a HR PDU session, except when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the N9 tunnel information of the home CN side, i.e. H-UPF. |  |
| cnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a PDU session involving an I-SMF, except when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the N9 tunnel information of the SMF. | DTSSA |
| additionalCnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present if a MA-PDU session is established for a UE registered over both 3GPP access and Non-3GPP access. When present, it shall contain additional N9 tunnel information of the UPF controlled by the H-SMF or SMF. | MAPDU |
| sessionAmbr | Ambr | C | 0..1 | This IE shall be present, except when Control Plane CIoT 5GS Optimisation is enabled for the PDU session. When present, this IE shall contain the Session AMBR granted to the PDU session. |  |
| qosFlowsSetupList | array(QosFlowSetupItem) | C | 1..N | This IE shall be present, except when Control Plane CIoT 5GS Optimisation is enabled for the PDU session. When present, this IE shall contain the full set of QoS flow(s) to establish for the PDU session. It shall contain at least the Qos flow associated to the default Qos rule. |  |
| hSmfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall contain the identifier of the home SMF. |  |
| smfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the identifier of the SMF. | DTSSA |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility or handover preparation using the N26 interface. When present, it shall be set to the PDU Session ID. |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain: - the S-NSSAI assigned to the PDU session in the Home PLMN, for a HR PDU session; - the S-NSSAI assigned to the PDU session in the serving PLMN, for a PDU session with an I-SMF. The Snssai shall overwrite the S-NSSAI earlier stored in I-SMF, if they are different. |  |
| enablePauseCharging | boolean | C | 0..1 | This IE shall be present, based on operator's policy, to enable the use of Pause of Charging for the PDU session (see clause 4.4.4 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: enable Pause of Charging; - false (default): disable Pause of Charging. |  |
| ueIpv4Address | Ipv4Addr | C | 0..1 | This IE shall be present if the SMF assigns a UE IPv4 address to the PDU session. |  |
| ueIpv6Prefix | Ipv6Prefix | C | 0..1 | This IE shall be present if the SMF assigns a UE IPv6 prefix to the PDU session. |  |
| n1SmInfoToUe | RefToBinaryData | C | 0..1 | This IE shall be present if the SMF needs to send N1 SM information to the UE that does not need to be interpreted by the V-SMF or I-SMF. When present, this IE shall reference the n1SmInfoToUe binary data (see clause 6.1.6.4.4). |  |
| epsPdnCnxInfo | EpsPdnCnxInfo | C | 0..1 | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |  |
| epsBearerInfo | array(EpsBearerInfo) | C | 1..N | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |
| maxIntegrityProtectedDataRate | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate for uplink. If the maxIntegrityProtectedDataRateDl IE is absent, this IE applies to both uplink and downlink. |  |
| maxIntegrityProtectedDataRateDl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE may be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate for downlink. |  |
| alwaysOnGranted | boolean | C | 0..1 | This IE shall be present if the alwaysOnRequested IE was received in the request or if the SMF determines, based on local policy, that the PDU session needs to be established as an always-on PDU session. When present, it shall be set as follows: - true: always-on PDU session granted. - false (default): always-on PDU session not granted. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if no GPSI IE is provided in the request, e.g. for a PDU session moved from another access or another system, and the SMF knows that a GPSI is already associated with the PDU session. When present, it shall contain the user's GPSI associated with the PDU session. |  |
| upSecurity | UpSecurity | O | 0..1 | When present, this IE shall indicate the security policy for integrity protection and encryption for the user plane of the PDU session. |  |
| roamingChargingProfile | RoamingChargingProfile | O | 0..1 | Roaming Charging Profile selected by the HPLMN (see clauses 5.1.9.1, 5.2.1.7 and 5.2.2.12.2 of 3GPP TS 32.255 [25]). |  |
| hSmfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the H-SMF service instance serving the PDU session, for a HR PDU session. This IE may be used by the V-SMF to identify PDU sessions affected by a failure or restart of the H-SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). |  |
| smfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the SMF service instance serving the PDU session, for a PDU session with an I-SMF. This IE may be used by the I-SMF to identify PDU sessions affected by a failure or restart of the SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). | DTSSA |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the SMF service instance serving the PDU session was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |  |
| dnaiList | array(Dnai) | C | 1..N | This IE shall be present over N16a, if available and an I-SMF has been inserted into a PDU session, during the following procedures: PDU session establishment, Registration, Service Request, Xn based handover, Inter NG-RAN node N2 based handover (see clause 4.23 of 3GPP TS 23.502 [3]). When present, it shall include the list of DNAIs of interest for the PDU session for local traffic steering at the I-SMF. | DTSSA |
| ipv6MultiHomingInd | boolean | C | 0..1 | This IE shall be present over N16a, if available and an I-SMF has been inserted into the PDU session during the following procedures: PDU session establishment, Registration, Service Request, Xn based handover, Inter NG-RAN node N2 based handover (see clause 4.23 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: IPv6 multi-homing is permitted. - false (default): IPv6 multi-homing is not allowed. | DTSSA |
| maAcceptedInd | boolean | C | 0..1 | This IE shall be present if a request to establish a MA PDU session was accepted or if a single access PDU session was upgraded into a MA PDU session (see clauses 4.22.2 and 4.22.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: MA PDU session - false (default): single access PDU session | MAPDU |
| homeProvidedChargingId | string | O | 0..1 | When present, this IE shall contain the Home provided Charging ID (see 3GPP TS 32.255 [25]). |  |
| nefExtBufSupportInd | boolean | C | 0..1 | This IE shall be present with value "true", if NEF has indicated Extended Buffering Support for mobile terminated data in SMF-NEF connection establishment response. When present, this IE shall be set as following: - true: Extended Buffering supported by NEF - false (default): Extended Buffering not supported by NEF | CIOT |
| smallDataRateControlEnabled | boolean | C | 0..1 | This IE shall be present and set to "true" if small data rate control is applicable on the PDU session. When present, it shall be set as follows: - true: small data rate control is applicable. - false (default): small data rate control is not applicable. | CIOT |
| ueIpv6InterfaceId | string | C | 0..1 | This IE shall be present if the H-SMF/SMF has assigned IPv6 interface identifier to the UE during the PDU session establishment for the Home-routed Roaming scenario or for a PDU session with an I-SMF. When present, it shall encode the UE IPv6 Interface Identifier to be used by the UE for its link-local address configuration with 16 hexadecimal digits. Pattern: "^[A-Fa-f0-9]{16}$" |  |
| NOTE 1: This IE contains information that the V-SMF or I-SMF only needs to transfer to the UE (without interpretation). It is sent as a separate IE rather than within the n1SmInfoToUE binary data because the Selected SSC mode IE is defined as a "V" IE (i.e. without a Type field) in the NAS PDU Session Establishment Accept message. NOTE 2: In scenarios with a V-SMF/I-SMF insertion, the V-SMF/I-SMF may receive in the Create Response some IEs it has already received during the earlier SM context retrieval from the SMF (e.g. due to the condition of presence of IEs in the Create Response). In such a case, the V-SMF/I-SMF shall overwrite the IEs earlier received with the new IEs received in the Create Response. |  |  |  |  |  |

* * * 2nd Change * * * *

## A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.1.0'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

*** Skipped for clarity ****

PduSessionCreatedData:

type: object

properties:

pduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

type: string

pattern: '^[0-7]$'

hcnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

cnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

qosFlowsSetupList:

type: array

items:

$ref: '#/components/schemas/QosFlowSetupItem'

minItems: 1

hSmfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

enablePauseCharging:

type: boolean

default: false

ueIpv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

maxIntegrityProtectedDataRate:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

alwaysOnGranted:

type: boolean

default: false

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

hSmfServiceInstanceId:

type: string

smfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

ipv6MultiHomingInd:

type: boolean

default: false

maAcceptedInd:

type: boolean

default: false

homeProvidedChargingId:

type: string

nefExtBufSupportInd:

type: boolean

default: false

smallDataRateControlEnabled:

type: boolean

default: false

ueIpv6InterfaceId:

type: string

pattern: '^[A-Fa-f0-9]{16}$'

required:

- pduSessionType

- sscMode

oneOf:

- required: [ hSmfInstanceId ]

- required: [ smfInstanceId ]

*** Skipped for clarity ****

* * * End of Changes * * * *