3GPP TSG-CT Meeting #89e	CP-202214

# E-Meeting, 14th – 16th September 2020

3GPP TSG-CT WG4 Meeting #99e	C4-204337

# E-Meeting, 18th – 28th August 2020

# Title:	LS on AUSF/UDM discovery based on SUCI information

# Release:	Rel-17

# Work Item:	SBIProtoc17

Source:	CT4

To:	SA2

Cc:	CT, SA, SA3

Contact Person:

Name:	Yue Song

Tel. Number:

E-mail Address:	songyue@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C4-204078 (DISC paper), C4-204339 (Rel-17 CR 29.510)

1. Overall Description:

CT4 has analyzed the issue of insufficient length of Routing Indicator within SUCI, see attached discussion paper in C4-204078 for details. The current 4 digits of the Routing Indicator enables only 10 K ranges of subscriptions, which may not provide enough granularity and flexibility for moving subscriptions across UDRs not controlled by the same AUSF/UDM, for operators with a large number of subscriptions (e.g. 100 K users per range for a PLMN with 1 billion subscriptions).

CT4 agreed that the above limitation should be addressed. CT4 has also discussed the solution proposed in C4-204339 (attached) for solving this problem, enabling to use the Home Public Key ID as an additional parameter (that allows to encode e.g. a 5th digit, in addition to the key ID on 4 bits) for the discovery of the AUSF/UDM. CT4 would be fine in principle to specify this solution in Rel-17, provided corresponding stage-2 requirements are agreed first.

2. Actions:

To SA WG2 group

ACTION: 	CT4 kindly asks SA2 to consider specifying stage 2 enhancements for selection of AUSF/UDM based on SUCI in Rel-17 to address the reported issue.

3. Date of Next CT4 Meetings:

3GPP TSG CT4#101e	11/2020	E-Meeting