3GPP TSG-CT Meeting #89-e	CP-202231

# Online, , 14th Sep 2020 - 16th Sep 2020												Was CP-202020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 0463 | rev | 3 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NodeType in HssAuthenticationInfoRequest |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Hewlett Packard Enterprise |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Hewlett Packard Enterprise, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | UDICOM |  |  |  |  |  | Date: |  |  | 2020-08-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - CR C4-203313 "HSS-AV-Generation" to TS29.505 introduced new requesting node type values to the lastIndexes attribute of the SequenceNumber type. However, it is currently not possible for the HSS to inform the UDM what the serving node type is. The HssAuthType provides information about the auth type but the operation doesn’t include any details about the requesting node type. - The HSSAuthType and HssAvType do not support a value for UMTS AKA |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Add the request node type in the HssAuthenticationInforequest type. -Add a value of UMTS_AKA to the HSSAuthType and HssAvType. -Add a description to the HssAuthenticationVectors data type to indicat that AvImsGbaEapAka is also applicable to UMTS AKA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UDM can’t retrieve appropriate ‘lastIndexes’ value from UDR and hence can not generate the proper AV for the requesting node type. The HSS cannot request UMTS AKA vectors from the UDM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6.2.10, 6.3.6.2.12, 6.3.6.3.5, 6.3.6.3.6, 6.3.6.3.x , A4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections, with impacts on the following APIs: - TS29503_Nudm_UEAU.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | rev2: corrected yaml (missing colon): requestingNodeType: $ref: '#/components/schemas/NodeType' rev3: Corrected the cardinality of the requestingNodeType in Table 6.3.6.2.10-1 and added Ericsson as a source to the TSG. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.3.6.2.10	Type: HssAuthenticationInfoRequest

Table 6.3.6.2.10-1: Definition of type HssAuthenticationInfoRequest

| Attribute name | Data type |  | P | Cardinality |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| hssAuthType | HssAuthType |  | M | 1 |  | Indicates the authentication method. |  |
| numOfRequestedVectors | NumOfRequestedVectors |  | M | 1 |  | Maximum 5 vectors are allowed per service request. |  |
| requestingNodeType | NodeType |  | C | 0..1 |  | Indicates the requesting node type. Should be included when known by the HSS. |  |
| servingNetworkId | PlmnId |  | C | 0..1 |  | Shall be present if the authentication method is EPS_AKA. |  |
| resynchronizationInfo | ResynchronizationInfo |  | O | 0..1 |  | Contains RAND and AUTS. |  |
| anId |  | AccessNetworkId | O |  | 0..1 |  | Contains the Access Network ID used in the derivation of authentication vectors in EAP-AKA'. |  |
| supportedFeatures | SupportedFeatures |  | O | 0..1 |  | See clause 6.3.8 |  |
| NOTE: For GBA authentication type, the number of requested vectors shall be set to 1; for other authentication types, the number of generated vectors by UDM, may be less than the number of requested vectors. |  |  |  |  |  |  |  |

* * * Next Change * * * *

##### 6.3.6.2.12	Type: HssAuthenticationVectors

Table 6.3.6.2.12-1: Definition of type HssAuthenticationVectors as a list of alternatives

| Data type | Cardinality | Description |
| --- | --- | --- |
| array(AvEpsAka) | 1 |  |
| array(AvImsGbaEapAka) | 1 | This data type is also used for UMTS AKA. |
| array(AvEapAkaPrime) | 1 |  |

* * * Next Change * * * *

##### 6.3.6.3.5	Enumeration: HssAuthType

Table 6.3.6.3.5-1: Enumeration HssAuthType

| Enumeration value | Description |
| --- | --- |
| "EPS_AKA" |  |
| "EAP_AKA" |  |
| "EAP_AKA_PRIME" | EAP-AKA' |
| "IMS_AKA" |  |
| "GBA_AKA" |  |
| "UMTS_AKA" |  |

* * * Next Change * * * *

##### 6.3.6.3.6	Enumeration: HssAvType

Table 6.3.6.3.6-1: Enumeration HssAvType

| Enumeration value | Description |
| --- | --- |
| "EPS_AKA" |  |
| "EAP_AKA" |  |
| "IMS_AKA" |  |
| "GBA_AKA" |  |
| "UMTS_AKA" |  |

* * * Next Change * * * *

##### 6.3.6.3.x	Enumeration: NodeType

Table 6.3.6.3.x-1: Enumeration NodeType

| Enumeration value | Description |
| --- | --- |
| "AUSF" | This value is not applicable to the HSS. |
| "VLR" |  |
| "SGSN" |  |
| "S_CSCF" |  |
| "BSF" |  |
| "GAN_AAA_SERVER" |  |
| “WLAN_AAA_SERVER” |  |
| “MME” |  |

* * * Next Change * * * *

## A.4	Nudm_UEAU API

openapi: 3.0.0

info:

version: '1.1.0'

title: 'NudmUEAU'

description: |

UDM UE Authentication Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.503 Unified Data Management Services, version 16.4.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.503/'

servers:

- url: '{apiRoot}/nudm-ueau/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause clause 4.4 of 3GPP TS 29.501.

security:

- oAuth2ClientCredentials:

- nudm-ueau

- {}

<contect removed for clarity>

HssAuthenticationInfoRequest:

type: object

required:

- hssAuthType

- numOfRequestedVectors

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

hssAuthType:

$ref: '#/components/schemas/HssAuthType'

numOfRequestedVectors:

$ref: '#/components/schemas/NumOfRequestedVectors'

requestingNodeType:

$ref: '#/components/schemas/NodeType'

servingNetworkId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

resynchronizationInfo:

$ref: '#/components/schemas/ResynchronizationInfo'

anId:

$ref: '#/components/schemas/AccessNetworkId'

<contect removed for clarity>

# ENUMS:

AuthType:

anyOf:

- type: string

enum:

- 5G_AKA

- EAP_AKA_PRIME

- EAP_TLS

- type: string

AvType:

anyOf:

- type: string

enum:

- 5G_HE_AKA

- EAP_AKA_PRIME

- type: string

HssAuthType:

anyOf:

- type: string

enum:

- EPS_AKA

- EAP_AKA

- EAP_AKA_PRIME

- IMS_AKA

- GBA_AKA

- UMTS_AKA

- type: string

HssAvType:

anyOf:

- type: string

enum:

- EPS_AKA

- EAP_AKA

- IMS_AKA

- GBA_AKA

- UMTS_AKA

- type: string

HssAuthTypeInUri:

anyOf:

- type: string

enum:

- eps-aka

- eap-aka

- eap-aka-prime

- ims-aka

- gba-aka

- type: string

AccessNetworkId:

anyOf:

- type: string

enum:

- HRPD

- WIMAX

- WLAN

- ETHERNET

- type: string

NodeType:

anyOf:

- type: string

enum:

- AUSF

- VLR

- SGSN

- S_CSCF

- BSF

- GAN_AAA_SERVER

- WLAN_AAA_SERVER

- MME

- type: string

* * * End of Changes * * * *